3GPP TSG-RAN WG3 Meeting #127-bis	R3-251911

Wuhan, China, 7-11 April 2025

Title: 	AIML enabled CCO - Single predicted CCO issue resolution

Source: 	Ericsson, InterDigital, Jio Platforms, Deutsche Telekom

Agenda item:	11.3

Document Type:	Discussion and agreement

# 1. Introduction

In this contribution we discuss aspects related to resolution of a single predicted CCO issue across multiple nodes.

# 2. Discussion

## Adapt to a future CCO state at neighbour gNB

According to the latest agreed TP for F1AP, the CU can send to the DU a Predicted CCO Assistance Information IE to indicate “predicted CCO Assistance Information for cells and beams served by the gNB-DU of the same NG-RAN node or for cells and beams not served by the gNB-DU.”.

| Predicted CCO Assistance Information | O |  | 9.3.1.A | Indicates predicted CCO Assistance Information for cells and beams served by the gNB-DU of the same NG-RAN node or for cells and beams not served by the gNB-DU. |
| --- | --- | --- | --- | --- |

As of now, the Predicted CCO Assistance Information IE includes: the Predicted CCO issue, indicating the type of predicted CCO issue, the Predicted Affected Cells and Beams, indicating a list of NR CGIs and SSB Indexes, pointing at the cells and beams predicted to be affected, and the Time for Predicted CCO Issue, indicating the time when the predicted CCO issue will happen.

Except for the timing information, the remaining piece of information is the same type of information that CU sends to DU for legacy CCO. The same information conveyed in Predicted CCO issue and Predicted Affected Cells and Beams is also sent via Xn from CU1 (gNB1) to CU2 (neighbour gNB2) in the Coverage Modification Cause and the Future Coverage Modification List, together with the Time for Future Coverage State. The CU2 forwards this information to DU2, but this is not enough for DU2 to generate the matching future CCO state.

In case of a detected CCO issue (legacy CCO), CU2 does not send to DU2 the CCO state generated by gNB1 (DU1), but the solution works, because DU2 can deduce the CCO state adopted for cells and beams of gNB1, thanks, for example, to collected UE measurements and reports that reveal how a cell/beam shape changed. With this information and the configured alternative configurations, DU2 is able to derive a matching CCO state for cells and beams under its control.

For a predicted CCO issue, however, DU1 has not applied yet the future CCO state, and DU2 cannot deduce it from measurements. An easy way to enable DU2 to generate a matching future CCO state, is that CU2 sends to DU2 the information it already has, i.e., the future CCO state generated by DU1, which CU2 received from CU1.

Observation 1: Sending the future CCO state generated at the gNB where the CCO issue is predicted to a neighbour gNB allows the neighbour gNB to adjust the future CCO state of its cells and beams.

Proposal 1: In a split gNB that receives via Xn a future CCO state for a CCO issue predicted at another gNB, the CU forwards the future CCO state to the DU, which generates a matching future CCO state for its own cells and beams.

Let’s now recap the steps to handle one detected CCO issue and one predicted CCO issue if Proposal 1 is agreed.

The logical steps to handle one detected CCO issue are:

- CU1 detects the CCO issue.

- To enable DU1 to generate a CCO state, CU1 sends to DU1 the CCO issue and the affected cells and beams (F1: CCO Assistance Information).

- DU1 generates the CCO state, it applies it immediately and sends the CCO state to CU1 (F1: Coverage Modification Notification).

- To enable gNB2 (DU2) to generate the matching CCO state, gNB1 (CU1) sends to gNB2 (CU2) the detected CCO issue (Xn: Coverage Modification Cause) and the CCO state at gNB1 (Xn: Cell Coverage State, SSB Coverage State).

- CU2 sends to DU2 the detected CCO issue and the affected cells and beams, not served by DU2 (F1: CCO Assistance Information).

- DU2 generates the matching CCO state to adjust the coverage of cells and beams of gNB2 to the CCO state generated by DU1, it applies it immediately and sends the matching CCO state to CU2.

The logical steps to handle one predicted CCO issue are:

- CU1 predicts the CCO issue.

- To enable DU1 to generate a future CCO state, CU1 sends to DU1 the predicted CCO issue and the predicted affected cells and beams (F1: Predicted CCO Assistance Information).

- DU1 generates the future CCO state and sends it to CU1 with the time when the future CCO state will be applied (F1: Future Coverage Modification Notification).

- To enable gNB2 (DU2) to generate the matching future CCO state and to determine when to apply the matching future CCO state, gNB1 (CU1) sends to gNB2 (CU2) the predicted CCO issue (Xn: Predicted Coverage Modification Cause), the future CCO state at gNB1 (Xn: Future Cell Coverage State, Future SSB Coverage State), and timing information (so far, only Xn: Time for Future Coverage State).

- CU2 sends to DU2 the predicted CCO issue, the predicted affected cells and beams (not served by DU2), the future CCO state and the timing information (so far, only F1: Time for Future Coverage State).

- DU2 generates the matching future CCO state that will adjust the coverage of cells and beams of gNB2 to the future CCO state generated by DU1 and sends the matching future CCO state to CU2.

From the steps above, it can be observed that, if CU2 receives a predicted CCO issue from a neighbour gNB1, there is no need for CU2 to predict a new CCO issue. The only relevant prediction is the predicted CCO issue received from gNB1 which CU2 can send to DU2, together with the Future CCO State at gNB1, to derive the matching future CCO state.

Observation 2: When CU2 receives a predicted CCO issue from a neighbouring gNB, CU2 does not need to predict a new CCO issue.

Proposal 2: If a CCO issue is predicted at gNB1 (CU1), CU2 sends to DU2 the same predicted CCO issue received from gNB1 (CU1).

# 3. Conclusions

Based on the discussion in this paper, the following observations and proposals are derived:

Observation 1: Sending the future CCO state generated at the gNB where the CCO issue is predicted to a neighbour gNB allows the neighbour gNB to adjust the future CCO state of its cells and beams.

Proposal 1: In a split gNB that receives via Xn a future CCO state for a CCO issue predicted at another gNB, the CU forwards the future CCO state to the DU, which generates a matching future CCO state for its own cells and beams.

Observation 2: When CU2 receives a predicted CCO issue from a neighbouring gNB, CU2 does not need to predict a new CCO issue.

Proposal 2: If a CCO issue is predicted at gNB1 (CU1), CU2 sends to DU2 the same predicted CCO issue received from gNB1 (CU1).