3GPP TSG-RAN WG3 Meeting #127-bis	R3-251926

Wuhan, CN, 7-11 Apr, 2025

Title:	(TP for SON BLCR for 38.300, 38.401, 38.473, 38.413) MRO for LTM

Agenda Item:	10.2

Source:	Huawei

Document for:	discussion

# 1	Introduction

In this paper, we discuss potential MRO enhancements for R18 mobility mechanisms.

# 2	Discussion

## 2.1	MRO for LTM

### 2.1.1	Failure scenarios

In last meeting we agreed that the CU identifies which node to send the information to. There was an agreement that the CU determine the failure type and send to the DU.

For failures shortly after successful handover, the source cell needs to identify the UE context based on the C-RNTI used in the source to perform the MRO analysis. It is assumed that the gNB-CU that supports this MRO feature will be able to store the association between the C-RNTI used in the target cell (last serving cell) and the corresponding C-RNTI used in the source cell. gNB-CU translates the C-RNTI in the target cell to the C-RNTI used in the source cell and convey it outside the RLF report when forwarding the information to the source gNB-DU.

In legacy MRO a similar solution was used for the same use case. The target gNB can store the C-RNTI coming in the HO preparation message and include it when sending the Handover report message. In order to facilitate this, the following stage2 text was added to 38.300:

If the cause for the failure is identified as a "Too Early HO" or a "HO to Wrong Cell", the NG-RAN node controlling the last serving cell shall, include in the HANDOVER REPORT message the C-RNTI used in the source cell of the last completed handover before the failure.

Without this stage2 text, there is nothing forcing/explaining when the C-RTNI should be included and therefore the intended functionality will not work, since implementations may choose to not include this information. Therefore, it is proposed to include a similar text in the stage2 for LTM. But since the behaviour is for F1AP, it makes more sense to capture this in 38.401 as previously proposed but not agreed in last meeting:

The gNB-CU receives a RLF report associated to an LTM mobility event from the UE or via the Failure indication message over Xn. The gNB-CU performs initial analysis and in case of failure due to inappropriate cell switch triggering may forward the RLF report to the last serving gNB-DU in case of too late LTM, or to the source gNB-DU in case of too early LTM or LTM to wrong cell. In case the gNB-DU responsible for the failure is not the gNB-DU serving the UE at the time of failure, the gNB-CU shall include the C-RNTI used in the source cell, if available.

Add stage2 text to specify when CRNTI should be included (see also the TP in the annex).

Early UL synchronization is the key point to shorten the interruption time. To achieve the intended goal, it is highly dependent on how well the early UL synchronization performs to get the possibly correct TA for accessing the target cell. Whether and how badly the TA becomes invalid depends on the relative mobility and relative position between the UE and the target cell. Thus, it is difficult for the source DU to estimate the validity time of TA. The obtained TA with some margin could enable the UE to access the target cell successfully, only causing some transmission drifting at the target cell side. The worst case of scenario is that, the UE encounters the HOF with the outdated TA, reselects to the same target cell and does the RA-based access successfully. It means that the target cell is the right cell, but the parameters to access the target cell is wrong. The reason could be the invalid TA or wrong TCI state. MRO mechanism should differentiate these two sub-cases.

MRO mechanism differentiates these two sub-cases: invalid TA and wrong TCI state when UE reselects to the same target cell and does the RA-based access successfully after HOF.

If we assume that the context retrieval can be done in the gNB-DU, this means that if we send the RLF report to the gNB-DU, the gNB-DU would be able to use the reported measurements and the access type to e.g. conclude that the failure was due to a wrong TA or beam failure. Therefore, the detection would be in the DU and we would only need to send the RLF report to the gNB-DU.

If context storage and retrieval is difficult to achieve in the DU, another option is to store some information in the CU and send to the DU when analysis is to be performed. But the gNB-CU will not be able to know whether the UE is configured with a TA for a cell switch command. One example of this is not only for intra-DU cell switch, but also for inter-DU scenarios. Despite of the fact that the CU has forwarded information regarding acquired TA(s) from the candidate DU one or more times, the CU does not know whether and which TA has been sent to the UE via CSC. One option to resolve this is to include the configured TA in the Cell Switch Notification.

Another option could be that the analysis is made in the gNB-CU. Similar to the above, the CU can be informed if TA was assigned to the UE in in the Cell Switch Notification and then use the L1 measurement to decide whether it is a beam failure or not (which may be difficult).

Yet another option is to let the UE include an indicator whether TA was configured for the cell switch.

From the above solutions, we think the solution of analysis in the DU with assistance from the CU is beneficial. This means we should include the configured TA in the Cell Switch Notification and send it back in Access and Mobility indication.

Include the configured TA in DU-CU Cell Switch Notification and Access and Mobility Indication message (see also the TP in the annex)

In order to enhance the analysis, it would be beneficial to be able to compare the TA assigned for the failed RA-less access attempt with the TA used in the RA-based LTM recovery. This would require that the target DU sends the TA that was used in the LTM recovery to the CU which could forward this to the source DU. For the CU to identify the UE which uses the TA value in the LTM recovery, the UE-associated signalling is used to convey the UE-specific TA value, e.g., the Access Success message from recovery DU or a new signalling.

Consider the possibility to compare the two TA in the source DU.

For the above cases, this may result in an LTM-recovery in the target cell. But the current framework does not allow the CU to forward the information to the DU in case LTM recovery is performed in the target cell. It is neither of the defined failure cases. Therefore, a new failure case is needed:

- LTM –recovery in target cell: Handover failure occurs during the handover procedure; then performs LTM cell recovery in the target cell.

Capture the new failure type in stage2 (see also the TP in the annex)

### 2.1.2	Near failure scenarios

RAN2 has excluded the RACH-based access and HO interruption time as new trigger conditions for SHR. There is no doubt to reuse the timer related trigger, i.e. T310/T312/T304, similar to legacy L3 handover. Once the timer related trigger is satisfied, the network would better know whether the handover is a LTM cell switch or L3 handover, in order to adjust the parameter precisely and efficiently, e.g. whether it is the CU or DU to optimize accordingly. Therefore, we propose, an indication of LTM cell switch is introduced to SHR report.

An indicator of LTM cell switch to distinguish from L3 handover is introduced to SHR report.

In last meeting, there is a discussion on the number of RLC re-transmission as a new trigger for SHR report. However, there is no RLC re-transmission for the first UL data transmission on the target cell when the UE is attempting to access the target cell, since RLC re-transmission at the UE side is triggered by the feedback from the network, i.e., RLC status report from the target cell. It is up to the UE implementation to retransmit in case the DCI received within T304. Therefore, it is not valuable to report the number of retransmissions, since this is not controlled by network.

Do not introduce trigger related to RLC re-transmission for SHR since there is no RLC re-transmission during LTM cell switch execution.

Regarding the analysis of the SHR associated to an LTM mobility, there is also a role split between the source DU and target DU. Similar to the successful L3 handover analysis mechanism, the CU may forward to the DU which configures the timers causing the SHR to be generated. If the trigger of SHR is T310/T312, which is source cell configured timers, the CU may forward the SHR to the source DU. If the trigger of SHR is T304, which is target cell configured timer, the CU may forward the SHR to the target DU.

If the trigger of SHR is T310/T312, which is source cell configured timers, the CU may forward the SHR to the source DU. If the trigger of SHR is T304, which is target cell configured timer, the CU may forward the SHR to the target DU (see also the TP in the annex).

If we know the TA is almost out of bounds, we can adjust the update frequency of acquired TA even before failure occurs. This could be achieved by measuring the needed TA correction in the target DU after the first transmitted uplink packet.

One option could be to report this from the UE. But it is also possible to solve this by network signalling, since the target DU calculates this difference of the TAs used by the first transmitted uplink packet and subsequent transmitted uplink packet. The target DU could then send it to the source DU, in a manner of either always, event triggered, or by an aggregated statistical value (max, mean, etc). The source DU could take actions to improve the TA management, e.g. enlarge/shorten the interval of two early TA attempts, enlarge/shorten the TA validity duration.

RAN3 agrees to report the adjusted TA after the first UL transmission to the source DU

### 2.1.3	Successful scenarios

Also, a potential risk has been raised during the Rel-18 discussion of supporting subsequent LTM feature, which is about the ping-pong issue. The L1 measurement may be unstable and leading the handover decision directing UE to access to the same cell back and forth in a limited time. This could also be solved by the DU if the DU keeps information stored when the UE has moved out of the cell. If the UE returns quickly, whe could choose to delay the handover a bit (increase hysteresis) or choose to send the UE to the target again hoping that the other side will increase the hysteresis.

Another scenario which may be more difficult to identify would be the case of three or more cell involved in ping pong. If for example the UE moves between cells A->B->C->A the hysteresis applied to A/B does not appear to be a problem. Resolving this requires that one of the cells in the chain delays the handover a bit.

But MHI is also used to help the target node to understand the behavior of the UE. If the UE stayed a long time in the source cell the target can assume that this is a slowly moving UE, whereas if the UE has made frequent mobility before, the UE can be considered as having high mobility and therefore likely to be handed over again in a short time. For comparison, the full MHI is included in PSCell addition procedures.

Therefore, we suggest to consider the UHI enhancement for LTM. The CU should be responsible to collect the MHI and further provide the latest MHI to the new DU timely. Specifically, the CU collect the UHI in a new cell upon reception of the LTM notification or access success message from the DU.

The CU collect the UHI in a new cell upon reception of the LTM notification or access success message from the DU and further provide the latest UHI to the new DU timely.

The MHI for DU to perform ping-pong analysis caused by LTM mobility events could be a simplified version of MHI. Because there is no need for DU to know the MHI caused by the legacy L3 handover. For example, the CU could collect the MHI, filter the MHI associated to LTM cell switch, and passed the filtered LTM MHI to the DU in the UE Context Setup procedure.

The CU could filter the LTM related UHI and pass to DU via the new IE in the UE Context Setup procedure.

For subsequent LTM, we would need to be able to update the information, since there is no UE context setup procedure initiated. Hence, we should add this to the UE context Modification procedure. In this case it may be beneficial to allow the possibility that the gNB-CU does not always send the MHI to the gNB-DU.

For subsequent LTM the gNB-CU should be able to send an updated UHI in the UE Context Modification or CU-DU CELL SWITCH NOTIFICATION message (see also the TP in the annex).

If the CU propagates the UHI to the target CU during connected UE’s mobility, an LTM indicator to distinguish LTM from the L3 handover should be added to legacy UHI. This is needed since the decision for LTM cell switch (by DU) may be different from legacy handover decisions (by CU) and if this UHI used e.g. for ping pong detection, it is useful for the CU to be able to distinguish between the two.

an LTM indicator to distinguish LTM from the L3 handover should be added to legacy UHI.

### 2.1.4	Beam management

RAN3 has received an LS from RAN2 regarding Beam management.

RAN2 have discussed the issues of:

- BFR shortly after successful LTM cell switch to the wrong beam (“near failure”), and

- LTM cell switch failure due to wrong beam

On the former issue, RAN2 have acknowledged the issue but agreed not to define a UE-based solution. RAN2 respectfully asks RAN3 to consider defining a network-based solution for this.

Furthermore, RAN2 asks RAN3 to see if/how the solution (if specified) could also be used for the latter issue

It seems that RAN2 discussed an issue but did not see the value to introduce a UE based solution.

Since the UE based solution is ruled out, the remaining question would be to discuss a network-based solution. For the near failure case, the event can be monitored by the target DU and notified of to the source DU.

One possible solution is to convey the BFR information via non UE-associated message (e.g., the Access and Mobility Indication message). For this solution, we could select between an aggregated report, e.g. including the percentage of BFR shortly after successful LTM switch, and the event triggered report including specific beams involved before and after.

Another possible solution is that the BFR information could be conveyed via UE-associated message in legacy messages (UL RRC MESSAGE TRANSFER and UE CONTEXT RELEASE COMMAND). The UE-associated signalling mitigates the risk of delaying the SON information transfer. If we prefer an immediate indication, the UE ID is added along with the BFR information for source DU to identify the UE context. The UE F1AP ID assigned by the CU and target/source DU, the C-RNTI assigned by the target Cell or the source Cell could be used as the UE ID.

RAN3 to agree on the need for a solution for observing BFR shortly after successful LTM cell switch to the wrong beam (“near failure”)

RAN3 to agree to adopt the aggregated report or an immediate solution for transferring the BFR information to source DU.

For the failure case, this relates to the discussion about TA acquisition where we discussed how to distinguish between TA or beam failures.

# 3	Conclusion

This document proposes:

Proposal 1:	Add stage2 text to specify when CRNTI should be included (see also the TP in the annex).

Proposal 2:	MRO mechanism differentiates these two sub-cases: invalid TA and wrong TCI state when UE reselects to the same target cell and does the RA-based access successfully after HOF.

Proposal 3:	Include the configured TA in DU-CU Cell Switch Notification and Access and Mobility Indication message (see also the TP in the annex)

Proposal 4:	Consider the possibility to compare the two TA in the source DU. (see also the TP in the annex)

Proposal 5:	Capture the new failure type in stage2 (see also the TP in the annex)

Proposal 6:	An indicator of LTM cell switch to distinguish from L3 handover is introduced to SHR report.

Proposal 7:	Do not introduce trigger related to RLC re-transmission for SHR since there is no RLC re-transmission during LTM cell switch execution.

Proposal 8:	If the trigger of SHR is T310/T312, which is source cell configured timers, the CU may forward the SHR to the source DU. If the trigger of SHR is T304, which is target cell configured timer, the CU may forward the SHR to the target DU (see also the TP in the annex).

Proposal 9:	RAN3 agrees to report the adjusted TA after the first UL transmission to the source DU

Proposal 10:	The CU collect the UHI in a new cell upon reception of the LTM notification or access success message from the DU and further provide the latest UHI to the new DU timely.

Proposal 11:	The CU could filter the LTM related UHI and pass to DU via the new IE in the UE Context Setup procedure.

Proposal 12:	For subsequent LTM the gNB-CU should be able to send an updated UHI in the UE Context Modification or CU-DU CELL SWITCH NOTIFICATION message (see also the TP in the annex).

Proposal 13:	an LTM indicator to distinguish LTM from the L3 handover should be added to legacy UHI.

Proposal 14:	RAN3 to agree on the need for a solution for observing BFR shortly after successful LTM cell switch to the wrong beam (“near failure”)

Proposal 15:	RAN3 to agree to adopt the aggregated report or an immediate solution for transferring the BFR information to source DU.

# Annex	TP for LTM for 38.300

##### 15.5.2.2.2	Connection failure due to intra-system mobility

One of the functions of Mobility Robustness Optimization is to detect connection failures that occur due to Too Early or Too Late Handovers, or Handover to Wrong Cell. These problems are defined as follows:

-	Intra-system Too Late Handover: an RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a different cell.

-	Intra-system Too Early Handover: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell.

-	Intra-system Handover to Wrong Cell: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell.

In the definition above, the "successful handover" refers to the UE state, namely the successful completion of the RA procedure.

In case of LTM cell switch, the "successful handover" in the definitions above refers to a successful completion of the RA procedure in case of RACH-based LTM or the successful reception of RRCReconfigurationComplete in case of RACH-less LTM.

In case of CHO, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell.

In case of LTM Cell switch, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late LTM cell switch, Too Early LTM cell switch and LTM cell switch to Wrong Cell. In addition to the problems defined above, the following applies to LTM Cell switch:

- LTM –recovery in target cell: Handover failure occurs during the handover procedure; then LTM cell recovery is performed in the target cell.

# Annex	TP for LTM for 38.401

## 7.X	MRO support for LTM

The gNB-CU receives a RLF report associated to an LTM mobility event from the UE or via the Failure indication message over Xn. The gNB-CU performs initial analysis and in case of failure due to inappropriate cell switch triggering may forward the RLF report to the last serving gNB-DU in case of too late LTM, or to the source gNB-DU in case of too early LTM or LTM to wrong cell. In case the gNB-DU responsible for the failure is not the gNB-DU serving the UE at the time of failure, the gNB-CU shall include the C-RNTI used in the source cell, if available.

The gNB-CU receives a SHR report associated to an LTM mobility event from the UE or via the Access and Mobility message over Xn or via Downlink RAN configuration transfer procedure over NG. The gNB-CU may forward the SHR report to the source gNB-DU in case the SHR was triggered by source cell timer values (e.g. T310/T312) or the target gNB-DU in case the SHR was triggered by target cell timer values (e.g. T304).

# Annex	TP for LTM for 38.473

### 8.3.9	DU-CU Cell Switch Notification

#### 8.3.9.1	General

The purpose of the DU-CU Cell Switch Notification procedure is to enable the gNB-DU to inform the gNB-CU about the initiation of the cell switch command to the UE. This procedure is also used to transfer the selected TCI state from the gNB-DU to the gNB-CU. The procedure uses UE-associated signalling.

#### 8.3.9.2	Successful Operation

Figure 8.3.9.2-1: DU-CU Cell Switch Notification procedure. Successful operation.

The gNB-DU initiates the procedure by sending a DU-CU CELL SWITCH NOTIFICATION message.

Upon reception of the DU-CU CELL SWITCH NOTIFICATION message, the gNB-CU shall, if supported, consider that a cell switch command was sent to the UE where the target cell is indicated by the included Cell ID IE.

Interaction with CU-DU Cell Switch Notification procedure:

If the LTM Cell Switch Information IE is included in the DU-CU CELL SWITCH NOTIFICATION message, the gNB-CU shall, if supported, forward it to the target gNB-DU in the CU-DU CELL SWITCH NOTIFICATION message.

If the TA Information List IE is included in the DU-CU CELL SWITCH NOTIFICATION message, the gNB-CU shall, if supported, forward it to the target gNB-DU in the CU-DU CELL SWITCH NOTIFICATION message to transfer the TA value(s).

#### 8.3.9.3	Unsuccessful Operation

Not applicable.

#### 8.3.9.4	Abnormal Conditions

Not applicable.

### 8.3.10	CU-DU Cell Switch Notification

#### 8.3.10.1	General

The purpose of the CU-DU Cell Switch Notification procedure is to enable the gNB-CU to inform the gNB-DU about the initiation of the cell switch command to the UE. This procedure is also used to transfer the selected TCI state from the gNB-CU to the gNB-DU. The procedure uses UE-associated signalling.

#### 8.3.10.2	Successful Operation

Figure 8.3.10.2-1: CU-DU Cell Switch Notification procedure. Successful operation.

The gNB-CU initiates the procedure by sending a CU-DU CELL SWITCH NOTIFICATION message.

Upon reception of the CU-DU CELL SWITCH NOTIFICATION message, the gNB-DU shall, if supported, consider that a cell switch command was sent to the UE where the target cell is indicated by the included Cell ID IE.

If the LTM Cell Switch Information IE is included in the CU-DU CELL SWITCH NOTIFICATION message, the gNB-DU shall, if supported, use it as specified in TS 38.401 [4].

If the TA Information List IE is included in the CU-DU CELL SWITCH NOTIFICATION message, the gNB-DU shall, if supported, use it as specified in TS 38.401 [4]. If the Tag ID Pointer IE is also included, the gNB-DU shall, if supported, consider it to determine the TAG corresponding to the received TA value.

#### 8.3.10.3	Unsuccessful Operation

Not applicable.

#### 8.3.10.4	Abnormal Conditions

Not applicable.

### 9.2.2	UE Context Management messages

#### 9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| ... |  |  |  |  |  |  |
| LTM UE History | O |  | 9.3.1.x |  | YES | Ignore |

#### 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| ... |  |  |  |  |  |  |
| LTM UE History | O |  | 9.3.1.x |  | YES | Ignore |

#### 9.2.2.15	DU-CU CELL SWITCH NOTIFICATION

This message is sent by the gNB-DU to inform the gNB-CU about the initiation of the cell switch  command to the UE.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cell ID | M |  | NR CGI 9.3.1.12 |  | YES | reject |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Includes the TCI-StateId IE, as defined in TS 38.331 [8]. | - |  |
| >UL TCI State ID | O |  | OCTET STRING | Includes the TCI-UL-StateId IE, as defined in TS 38.331 [8]. | - |  |
| TA Information List |  | 0..1 |  |  | YES | ignore |
| >TA Information Item IEs |  | 1 .. <maxnoofTAList> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. | - |  |
| >>Tag ID Pointer | O |  | OCTET STRING | Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [8]. | YES | ignore |
| Configured TA Value | O |  | INTEGER (0..4095) | Indicates the TA of the LTM target cell in cell switch command value as defined in TS 38.213 [31]. The absence of the IE indicates that there is no TA value provided to UE. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAList | Maximum no. of TA values to be sent, the maximum value is 8. |

#### 9.2.2.14	ACCESS SUCCESS

This message is sent by the gNB-DU to inform the gNB-CU of which cell the UE has successfully accessed during conditional handover, conditional PSCell addition, conditional PSCell change, LTM, or subsequent CPAC.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| NR CGI | M |  | 9.3.1.12 |  | YES | reject |
| RA based TA Value | O |  | INTEGER (0..4095) | Indicates the TA used in RA-based LTM execution. | YES | ignore |

### 9.2.10	Self Optimisation Support Messages

#### 9.2.10.1	ACCESS AND MOBILITY INDICATION

This message is sent by gNB-CU to gNB-DU to provide access and mobility information to the gNB-DU.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| RA Report List |  | 0..1 |  |  | YES | ignore |
| >RA Report Item |  | 1 .. <maxnoofRAReports> |  |  | - |  |
| >>RA Report Container | M |  | OCTET STRING | Includes the RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| RLF Report Information List |  | 0..1 |  |  | YES | ignore |
| >RLF Report Information Item |  | 1 .. <maxnoofRLFReports> |  |  | - |  |
| >>NR UE RLF Report Container | M |  | OCTET STRING | Includes the nr-RLF-Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| >>Configured TA Value | O |  | INTEGER (0..4095) | Indicates the TA of the LTM target cell in cell switch command value as defined in TS 38.213 [31]. The absence of the IE indicates that there is no TA value provided to UE. | YES | ignore |
| >>RA based TA Value | O |  | INTEGER (0..4095) | Indicates the TA used in RA-based LTM execution. | YES | ignore |
| Successful HO Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful HO Report Information Item |  | 1 .. <maxnoofSuccessfulHOReports> |  |  | - |  |
| >>Successful HO Report Container | M |  | OCTET STRING | Includes the SuccessHO-Report IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| Successful PSCell Change Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful PSCell Change Report information Item |  | 1..<maxnoofSuccessfulPSCellChangeReports> |  |  | - |  |
| >>Successful PSCell Change Report Container | M |  | OCTET STRING | Includes the SuccessPSCell-Report IE as defined in TS 38.331 [8]. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofRAReports | Maximum no. of RA Reports, the maximum value is 64. |
| maxnoofRLFReports | Maximum no. of RLF Reports, the maximum value is 64. |
| maxnoofSuccessfulHOReports | Maximum no. of Successful HO Reports, the maximum value is 64. |
| maxnoofSuccessfulPSCellChangeReports | Maximum no. of Successful PSCell Change Reports. Value is 64. |

#### 9.3.1.x	LTM UE History

This IE includes information on previously visited cells associated with LTM mobility.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| LTM UE History List |  | 1 |  |  |
| >LTM UE History Item |  | 1..<maxnoofCellsinLTMUEHistoryInfo> |  |  |
| >>NR CGI |  |  | 9.3.1.12 |  |
| >>Time UE Stayed in Cell |  |  | INTEGER (0..40950) | The duration of time the UE stayed in the cell. |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinLTMUEHistoryInfo | Maximum no. of cells in the LTM UE history information. Value is 16. |

#### 9.2.2.16	CU-DU CELL SWITCH NOTIFICATION

This message is sent by the gNB-CU to inform the gNB-DU about the initiation of the cell switch command to the UE.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cell ID | M |  | NR CGI 9.3.1.12 |  | YES | reject |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Includes the TCI-StateId IE, as defined in TS 38.331 [8]. | - |  |
| >UL TCI State ID | O |  | OCTET STRING | Includes the TCI-UL-StateId IE, as defined in TS 38.331 [8]. | - |  |
| TA Information List |  | 0..1 |  |  | YES | ignore |
| >TA Information Item IEs |  | 1 .. <maxnoofTAList> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. | - |  |
| >>Tag ID Pointer | O |  | OCTET STRING | Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [8]. | YES | ignore |
| Configured TA Value | O |  | INTEGER (0..4095) | Indicates the TA of the LTM target cell in cell switch command value as defined in TS 38.213 [31]. The absence of the IE indicates that there is no TA value provided to UE. | YES | ignore |
| LTM UE History | O |  | 9.3.1.x |  | YES | Ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAList | Maximum no. of TA values to be sent, the maximum value is 8. |

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

[snip]

RequestedSRSPreconfigurationCharacteristics-List,

SRSPreconfiguration-List,

Broadcast-MRBs-Transport-Request-Item,

TAInformation-List,

NonIntegerDRXCycle,

AggregatedPosSRSResourceSetList,

F1U-PathFailure,

LTMUEHistory,

ConfiguredTAValue

[snip]

id-AggregatedPosSRSResourceSetList,

id-RANSharingAssistanceInformation,

id-F1U-PathFailure,

id-LTMUEHistory,

id-ConfiguredTAValue

maxCellingNBDU,

maxnoofCandidateSpCells,

maxnoofDRBs,

maxnoofIndividualF1ConnectionsToReset,

[snip]

maxnoofTRPs,

maxnoofMRBs,

maxnoofUEIDforPaging,

maxnoofMRBsforUE,

maxnoofServingCellMOs,

maxnoofCellsinLTMUEHistoryInfo

[snip]

-- **************************************************************

--

-- CU-DU CELL SWITCH NOTIFICATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- CU-DU Cell Switch Notification

--

-- **************************************************************

CUDUCellSwitchNotification ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ CUDUCellSwitchNotificationIEs}},

...

}

CUDUCellSwitchNotificationIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID		CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID		CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-NRCGI					CRITICALITY reject	TYPE NRCGI							PRESENCE mandatory	}|

{ ID id-LTMCellSwitchInformation	 CRITICALITY ignore	TYPE LTMCellSwitchInformation	PRESENCE optional	}|

{ ID id-TAInformation-List			CRITICALITY ignore	TYPE TAInformation-List			PRESENCE optional		}|

{ ID id-ConfiguredTAValue	 		CRITICALITY ignore	TYPE TAValue					PRESENCE optional		},

...

}

[snip]

-- **************************************************************

--

##### -- UE CONTEXT SETUP REQUEST

--

-- **************************************************************

UEContextSetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextSetupRequestIEs} },

...

}

UEContextSetupRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID									PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID						CRITICALITY ignore	TYPE GNB-DU-UE-F1AP-ID									PRESENCE optional 	}|

{ ID id-SpCell-ID								CRITICALITY reject	TYPE NRCGI												PRESENCE mandatory	}|

{ ID id-ServCellIndex							CRITICALITY reject	TYPE ServCellIndex										PRESENCE mandatory	}|

[snip]

{ ID id-LTMUEHistory							CRITICALITY ignore	TYPE LTMUEHistory										PRESENCE optional	}|

}

[snip]

-- **************************************************************

--

##### -- UE CONTEXT MODIFICATION REQUEST

--

-- **************************************************************

UEContextModificationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextModificationRequestIEs} },

...

}

UEContextModificationRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID									PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID									PRESENCE mandatory	}|

{ ID id-SpCell-ID								CRITICALITY ignore	TYPE NRCGI												PRESENCE optional	}|

{ ID id-ServCellIndex							CRITICALITY reject	TYPE ServCellIndex										PRESENCE optional	}|

[snip]

{ ID id-LTMUEHistory							CRITICALITY ignore	TYPE LTMUEHistory										PRESENCE optional	}|

}

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

[snip]

LTMUEHistory = 	::= SEQUENCE {

lTMUEHistoryList 	LTMUEHistoryList,

iE-Extensions				ProtocolExtensionContainer { { LTMUEHistory-ExtIEs} }	OPTIONAL,

...

}

LTMUEHistory-ExtIEs		F1AP-PROTOCOL-EXTENSION ::= {

...

}

LTMUEHistoryList ::= SEQUENCE (SIZE(1..maxnoofCellsinLTMUEHistoryInfo)) OF LTMUEHistoryItem

LTMUEHistoryItem ::= SEQUENCE {

nRCGI					NRCGI,

timeUEStayedinCell		INTEGER (0..40950),

iE-Extensions			ProtocolExtensionContainer { { LTMUEHistoryItem-ExtIEs } }		OPTIONAL,

...

}

LTMUEHistoryItem-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

[snip]

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

F1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules

[snip]

id-LocalOrigin										ProtocolIE-ID ::= 854

id-LTMResetInformation								ProtocolIE-ID ::= 855

id-SRSPosPeriodicConfigHyperSFNIndex				ProtocolIE-ID ::= 856

id-PreconfiguredSRSInformation						ProtocolIE-ID ::= 857

id-rLFReportFailureType								ProtocolIE-ID ::= 999 --to be assigned by MCC

id-LTMUEHistory										ProtocolIE-ID ::= 998 --to be assigned by MCC

id-ConfiguredTAValue								ProtocolIE-ID ::= 997 --to be assigned by MCC

# Annex	TP for LTM for 38.413

#### 9.3.1.97	Last Visited NG-RAN Cell Information

This IE contains information about a cell. In case of NR cell, this IE contains information about a set of NR cells with the same NR ARFCN for reference point A, and the Global Cell ID IE identifies one of the NR cells in the set. The information is to be used for RRM purposes.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Global Cell ID | M |  | NG-RAN CGI 9.3.1.73 |  | - |  |
| Cell Type | M |  | 9.3.1.98 |  | - |  |
| Time UE Stayed in Cell | M |  | INTEGER (0..4095) | The duration of time the UE stayed in the cell, or set of NR cells with the same NR ARFCN for reference point A, in seconds. If the duration is more than 4095s, this IE is set to 4095. | - |  |
| Time UE Stayed in Cell Enhanced Granularity | O |  | INTEGER (0..40950) | The duration of time the UE stayed in the cell, or set of NR cells with the same NR ARFCN for reference point A, in 1/10 seconds. If the duration is more than 4095s, this IE is set to 40950. | - |  |
| HO Cause Value | O |  | Cause 9.3.1.2 | The cause for the handover. | - |  |
| Last Visited PSCell List |  | 0..<maxnoofPSCellsPerPrimaryCellinUEHistoryInfo> |  | List of cells configured as PSCells. Most recent PSCell related information is added to the top of the list. | YES | ignore |
| >Last Visited PSCell Information | M |  | 9.3.1.235 | The PSCell related information. | - |  |
| LTM cell switch | O |  | ENUMERATED (“true”,...) |  | YES | Ignore |

## 9.4	Message and Information Element Abstract Syntax (with ASN.1)

[snip]

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

[snip]

id-MaximumDataBurstVolume,

id-MBS-NGUFailureIndication,

id-UserPlaneFailureIndication,

id-UserPlaneFailureIndicationReport,

id-QoERVQoEReportingPaths,

id-LTMCellSwitch,

maxnoofAllowedAreas,

maxnoofAllowedCAGsperPLMN,

[snip]

LastVisitedNGRANCellInformation::= SEQUENCE {

globalCellID								NGRAN-CGI,

cellType									CellType,

timeUEStayedInCell							TimeUEStayedInCell,

timeUEStayedInCellEnhancedGranularity		TimeUEStayedInCellEnhancedGranularity 			OPTIONAL,

hOCauseValue								Cause		 									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {LastVisitedNGRANCellInformation-ExtIEs} }	OPTIONAL,

...

}

LastVisitedNGRANCellInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-LastVisitedPSCellList	CRITICALITY ignore	EXTENSION LastVisitedPSCellList	PRESENCE optional },

{ ID id-LTMCellSwitch			CRITICALITY ignore	EXTENSION ENUMERATED{true,...}	PRESENCE optional },

...

}

[snip]

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

[snip]

id-MBS-NGUFailureIndication								ProtocolIE-ID ::= 434

id-UserPlaneFailureIndication							ProtocolIE-ID ::= 435

id-UserPlaneFailureIndicationReport						ProtocolIE-ID ::= 436

id-SourceSN-to-TargetSN-QMCInfo							ProtocolIE-ID ::= 437

id-QoERVQoEReportingPaths								ProtocolIE-ID ::= 438

id-LTMCellSwitch										ProtocolIE-ID ::= 998 --to be assigned by MCC