3GPP TSG-RAN WG3 Meeting #127-bis	R3-251931

Wuhan, CN, 7-11 Apr, 2025

Title:	(TP for SON BLCR for 38.401) SLrelay service continuity

Agenda Item:	22.2

Source:	Huawei

Document for:	discussion

# 1	Introduction

In RAN3#127 meeting, we had the first time to discuss the RAN3 ascepts to support multi-hop relay, including the  control plan procedure, service continuty and the authorization. Based on the LS from SA2 [S2-2501296], we agreed the CRs for the authorization for multi-hop relay [1][2][3]. For the control plan procedures, the BL CR to 38.401[4] provided the overall procedure for Remote UE’s initial access in multi-hop relay.

RAN3 didn’t discuss the service continuity for multi-hop relay before. In the WID for NR sidelink multi-hop relay in [1] from RAN#105 meeting, we have the following description for service continuity scenarios for multi-hop U2N relay.

Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE): First Priority: - Intra-gNB multi-hop indirect to direct path switching using existing framework - Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: - Intra-gNB direct to multi-hop indirect path switching - Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell. NOTE: The current existing measurement framework and existing data forwarding mechanism should be reused.

In this paper, we will discuss the effect of service continuity for multi-hop relay on RAN3.

# 2	Discussion

## 2.1 Service continuity procedures

There is no need to discuss the path switch procedure for Scenario A and B, as the same as Rel-17. Because for intra-gNB multi-hop indirect path, the remote UE doesn’t need perform access procedure whether it's switched to direct path or single-hop indirect path, and the gNB-CU doesn’t need to inform the target gNB-DU of the multi-hop relay related information.

- Observation 1: There is no need to discuss the path switch procedure for Scenario A and B, as the same as Rel-17.

### 2.1.1 Intra-gNB direct to multi-hop indirect path switching

Figure 1 shows the procedure for the intra-gNB direct to multi-hop indirect path switching case which is extended from single hop intra-gNB direct to indirect path switching procedure as defined in clause 8.19.4.1 in TS 38.401.

- Figure 1: U2N Remote UE Direct-to-multihop-indirect Path Switch with gNB-DU change procedure

- The Uu measurement configuration and measurement report signalling is performed between U2N Remote UE and gNB-CU to evaluate both relay link measurement and Uu link measurement. The U2N Remote UE may report one or multiple candidate U2N Relay UE(s) and Uu measurement results after it measures/discovers the candidate U2N Relay UE(s).

- The gNB-CU decides to switch the U2N Remote UE to target U2N Relay UEs in the multihop link under a different gNB-DU (i.e., target gNB-DU).

- The reconfiguration to every L2 U2N multi-hop Relay UE is performed among L2 U2N multi-hop relay (the last relay, intermediate relay and first relay UE), the target gNB-DU and gNB-CU, if the U2N Relay UE in multihop link is in RRC_CONNECTED state. The gNB-CU sends the RRCReconfiguration message to every U2N Relay UE to reconfigure the connection for Remote UE. If a U2N Relay UE in multihop link is in RRC_IDLE/INACTIVE state, this step is skipped and the configuration to this U2N Relay UE is performed in Step 10.

- gNB-CU sends the UE CONTEXT SETUP REQUEST message for the U2N Remote UE to the target gNB-DU, which contains the path switch configuration at least.

- gNB-DU responds with the UE CONTEXT SETUP RESPONSE message to gNB-CU.

- gNB-CU sends the UE CONTEXT MODIFICATION REQUEST message by including the RRCReconfiguration message to the source gNB-DU. The contents in the RRCReconfiguration message may include at least path switch configuration, PC5 Relay RLC channel configuration for relaying traffic, bearer mapping configuration and the associated radio bearer(s).

- The source gNB-DU sends the RRCReconfiguration message to the U2N Remote UE. The U2N Remote UE stops UP and CP transmission over Uu after reception of RRCReconfiguration message from the gNB.

- The source gNB-DU sends the UE CONTEXT MODIFICATION RESPONSE message to the gNB-CU.

- The U2N Remote UE establishes PC5 connection with the First U2N Relay UE.

- The L2 U2N Remote UE completes the path switch procedure by sending the RRCReconfigurationComplete message to the gNB via the L2 U2N Relay UEs (the last relay, intermediate relay and first relay UE). In case a U2N relay UE in multihop link is in RRC_IDLE/ INACTIVE state, when the U2N relay UE receives the RRCReconfigurationComplete message, the reception of the RRCReconfigurationComplete message will first trigger RRC setup/resume procedure for the U2N relay UE to enter RRC_CONNECTED state.

- The target gNB-DU sends the UL RRC MESSAGE TRANSFER message to gNB-CU by including the RRCReconfigurationComplete message.

- The gNB-CU sends an UE CONTEXT RELEASE COMMAND message to the source gNB-DU.

- The source gNB-DU releases the UE context and responds the gNB-CU with an UE CONTEXT RELEASE COMPLETE message.

Agree the above procedure in Figure 1 as the baseline for intra-gNB direct to multi-hop indirect path switching with gNB-DU change procedure to be captured in TS 38.401.

### 2.1.2 Intra-gNB single-hop indirect to multi-hop indirect path switching

Figure 2 shows the procedure for the intra-gNB single-hop indirect to multi-hop indirect path switching case which is extended from single hop intra-gNB direct to indirect path switching procedure as defined in clause 8.19.4.1 in TS 38.401.

Actually, the single-hop indirect to multi-hop indirect path switching is similar to the direct to multi-hop indirect path switching, except for some steps.

- Figure 2: U2N Remote UE Singlehop-indirect -to-multihop-indirect Path Switch with gNB-DU change procedure

- The Uu measurement configuration and measurement report signalling is performed between U2N Remote UE and gNB-CU via the source L2 U2N Relay UE to evaluate both relay link measurement and Uu link measurement. The U2N Remote UE may report one or multiple candidate U2N Relay UE(s) and Uu measurement results after it measures/discovers the candidate U2N Relay UE(s).

- The gNB-CU decides to switch the U2N Remote UE to target U2N Relay UEs in the multihop link under a different gNB-DU (i.e., target gNB-DU).

- The reconfiguration to every L2 U2N multi-hop Relay UE is performed among L2 U2N multi-hop relay (the last relay, intermediate relay and first relay UE), the target gNB-DU and gNB-CU, if the U2N Relay UE in multihop link is in RRC_CONNECTED state. The gNB-CU sends the RRCReconfiguration message to every U2N Relay UE to reconfigure the connection for Remote UE. If a U2N Relay UE in multihop link is in RRC_IDLE/INACTIVE state, this step is skipped and the configuration to this U2N Relay UE is performed in Step 10.

- gNB-CU sends the UE CONTEXT SETUP REQUEST message for the U2N Remote UE to the target gNB-DU, which contains the path switch configuration at least.

- gNB-DU responds with the UE CONTEXT SETUP RESPONSE message to gNB-CU.

- gNB-CU sends the UE CONTEXT MODIFICATION REQUEST message by including the RRCReconfiguration message to the source gNB-DU. The contents in the RRCReconfiguration message may include at least path switch configuration, PC5 Relay RLC channel configuration for relaying traffic, bearer mapping configuration and the associated radio bearer(s).

- The source gNB-DU sends the RRCReconfiguration message to the U2N Remote UE via the source L2 U2N Relay UE. The U2N Remote UE stops UP and CP transmission over Uu after reception of RRCReconfiguration message from the gNB.

- The source gNB-DU sends the UE CONTEXT MODIFICATION RESPONSE message to the gNB-CU.

- The U2N Remote UE establishes PC5 connection with the target First U2N Relay UE.

- The L2 U2N Remote UE completes the path switch procedure by sending the RRCReconfigurationComplete message to the gNB via the L2 U2N Relay UEs (the last relay, intermediate relay and first relay UE). In case a U2N relay UE in multihop link is in RRC_IDLE/ INACTIVE state, when the U2N relay UE receives the RRCReconfigurationComplete message, the reception of the RRCReconfigurationComplete message will first trigger RRC setup/resume procedure for the U2N relay UE to enter RRC_CONNECTED state.

- The target gNB-DU sends the UL RRC MESSAGE TRANSFER message to gNB-CU by including the RRCReconfigurationComplete message.

- The gNB-CU sends an UE CONTEXT RELEASE COMMAND message to the source gNB-DU.

- The source gNB-DU releases the UE context and responds the gNB-CU with an UE CONTEXT RELEASE COMPLETE message.

- The gNB-CU sends the RRCReconfiguration message to the source L2 U2N relay to reconfigure the connection among the source L2 U2N relay UE, the source gNB-DU and gNB-CU. The RRCReconfiguration message to the source L2 U2N relay UE can be sent any time after step 7 based on gNB implementation (e.g., to release Uu or PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration related to the L2 U2N Remote UE).

- Either source L2 U2N Relay UE's AS layer or L2 U2N Remote UE's AS layer indicates upper layers to release PC5 unicast link after receiving the RRCReconfiguration messag. The timing to execute link release is up to UE implementation after step 7 or step14.

Agree the above procedure in Figure 2 as the baseline for the intra-gNB single-hop indirect to multi-hop indirect path switching with gNB-DU change procedure to be captured in TS 38.401.

# 3	Conclusion

This document proposes:

- Observation 1: There is no need to discuss the path switch procedure for Scenario A and B, as the same as Rel-17.

- Agree the above procedure in Figure 1 as the baseline for intra-gNB direct to multi-hop indirect path switching with gNB-DU change procedure to be captured in TS 38.401.

- Agree the above procedure in Figure 2 as the baseline for the intra-gNB single-hop indirect to multi-hop indirect path switching with gNB-DU change procedure to be captured in TS 38.401.

# 4	Reference

[1] R3-250940, (BL CR to 38.413) Support of Multi-hop relay, Nokia, Nokia Shanghai Bell, Ericsson, Huawei, LG Electronics, NEC, CATT, InterDigital, Samsung, ZTE, FirstNet, NIST, 3GPP TSG-RAN WG3 Meeting #127

[2] R3-250941, (BL CR to 38.423) Ericsson, LG Electronics, Nokia, Nokia Shanghai Bell, Huawei, NEC, CATT, InterDigital, Samsung, ZTE, FirstNet, NIST, 3GPP TSG-RAN WG3 Meeting #127

[3] R3-250942, (BL CR to 38.473) Support of Multi-hop relay, Huawei, LG Electronics, Nokia, Nokia Shanghai Bell, NEC, CATT, Ericsson, Samsung, ZTE, Interdigital, FirstNet, 3GPP TSG-RAN WG3 Meeting #127

[4] R3-250939, (BL CR to 38.401) Support of Multi-hop relay, LG Electronics Inc., ZTE, Nokia, Nokia Shanghai Bell, Ericsson, FirstNet, Huawei, 3GPP TSG-RAN WG3 Meeting #127

[5] RP-242349 Revised WID SL_relay_multihop,  3GPP TSG RAN Meeting #105

# Annex1	Text Proposal

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Start Changes>>>>>>>>>>>>>>>>>>>>>>>>>>

### 8.19.X	Service Continuity for multi-hop U2N relay

#### 8.19.X.1	Inter-gNB-DU switch from direct to multi-hop indirect path

The signalling flow for U2N Remote UE switch from direct to multi-hop indirect path with gNB-DU change is shown in Figure 8.19.X.1-1.

Figure 8.19.X.1-1: U2N Remote UE Direct-to-multihop-indirect Path Switch with gNB-DU change procedure

1.	The Uu measurement configuration and measurement report signalling is performed between U2N Remote UE and gNB-CU to evaluate both relay link measurement and Uu link measurement. The U2N Remote UE may report one or multiple candidate U2N Relay UE(s) and Uu measurement results after it measures/discovers the candidate U2N Relay UE(s).

2.	The gNB-CU decides to switch the U2N Remote UE to target U2N Relay UEs in the multihop link under a different gNB-DU (i.e., target gNB-DU).

3.	The reconfiguration to every L2 U2N multi-hop Relay UE is performed among L2 U2N multi-hop relay (the last relay, intermediate relay and first relay UE), the target gNB-DU and gNB-CU, if the U2N Relay UE in multihop link is in RRC_CONNECTED state. The gNB-CU sends the RRCReconfiguration message to every U2N Relay UE to reconfigure the connection for Remote UE. If a U2N Relay UE in multihop link is in RRC_IDLE/INACTIVE state, this step is skipped and the configuration to this U2N Relay UE is performed in Step 10.

4.	gNB-CU sends the UE CONTEXT SETUP REQUEST message for the U2N Remote UE to the target gNB-DU, which contains the path switch configuration at least.

5.	gNB-DU responds with the UE CONTEXT SETUP RESPONSE message to gNB-CU.

6.	gNB-CU sends the UE CONTEXT MODIFICATION REQUEST message by including the RRCReconfiguration message to the source gNB-DU. The contents in the RRCReconfiguration message may include at least path switch configuration, PC5 Relay RLC channel configuration for relaying traffic, bearer mapping configuration and the associated radio bearer(s).

7.	The source gNB-DU sends the RRCReconfiguration message to the U2N Remote UE. The U2N Remote UE stops UP and CP transmission over Uu after reception of RRCReconfiguration message from the gNB.

8.	The source gNB-DU sends the UE CONTEXT MODIFICATION RESPONSE message to the gNB-CU.

9.	The U2N Remote UE establishes PC5 connection with the First U2N Relay UE.

10.	The L2 U2N Remote UE completes the path switch procedure by sending the RRCReconfigurationComplete message to the gNB via the L2 U2N Relay UEs (the last relay, intermediate relay and first relay UE). In case a U2N relay UE in multihop link is in RRC_IDLE/ INACTIVE state, when the U2N relay UE receives the RRCReconfigurationComplete message, the reception of the RRCReconfigurationComplete message will first trigger RRC setup/resume procedure for the U2N relay UE to enter RRC_CONNECTED state.

11.	The target gNB-DU sends the UL RRC MESSAGE TRANSFER message to gNB-CU by including the RRCReconfigurationComplete message.

12.	The gNB-CU sends an UE CONTEXT RELEASE COMMAND message to the source gNB-DU.

13.	The source gNB-DU releases the UE context and responds the gNB-CU with an UE CONTEXT RELEASE COMPLETE message.

#### 8.19.X.2	Inter-gNB-DU switch from single-hop indirect to multi-hop indirect path

The signalling flow for U2N Remote UE switch from single-hop direct to multi-hop indirect path with gNB-DU change is shown in Figure 8.19.X.2-1.

Figure 8.19.X.2-1: U2N Remote UE Singlehop-indirect -to-multihop-indirect Path Switch with gNB-DU change procedure

1.	The Uu measurement configuration and measurement report signalling is performed between U2N Remote UE and gNB-CU via the source L2 U2N Relay UE to evaluate both relay link measurement and Uu link measurement. The U2N Remote UE may report one or multiple candidate U2N Relay UE(s) and Uu measurement results after it measures/discovers the candidate U2N Relay UE(s).

2.	The gNB-CU decides to switch the U2N Remote UE to target U2N Relay UEs in the multihop link under a different gNB-DU (i.e., target gNB-DU).

3.	The reconfiguration to every L2 U2N multi-hop Relay UE is performed among L2 U2N multi-hop relay (the last relay, intermediate relay and first relay UE), the target gNB-DU and gNB-CU, if the U2N Relay UE in multihop link is in RRC_CONNECTED state. The gNB-CU sends the RRCReconfiguration message to every U2N Relay UE to reconfigure the connection for Remote UE. If a U2N Relay UE in multihop link is in RRC_IDLE/INACTIVE state, this step is skipped and the configuration to this U2N Relay UE is performed in Step 10.

4.	gNB-CU sends the UE CONTEXT SETUP REQUEST message for the U2N Remote UE to the target gNB-DU, which contains the path switch configuration at least.

5.	gNB-DU responds with the UE CONTEXT SETUP RESPONSE message to gNB-CU.

6.	gNB-CU sends the UE CONTEXT MODIFICATION REQUEST message by including the RRCReconfiguration message to the source gNB-DU. The contents in the RRCReconfiguration message may include at least path switch configuration, PC5 Relay RLC channel configuration for relaying traffic, bearer mapping configuration and the associated radio bearer(s).

7.	The source gNB-DU sends the RRCReconfiguration message to the U2N Remote UE via the source L2 U2N Relay UE. The U2N Remote UE stops UP and CP transmission over Uu after reception of RRCReconfiguration message from the gNB.

8.	The source gNB-DU sends the UE CONTEXT MODIFICATION RESPONSE message to the gNB-CU.

9.	The U2N Remote UE establishes PC5 connection with the target First U2N Relay UE.

10.	The L2 U2N Remote UE completes the path switch procedure by sending the RRCReconfigurationComplete message to the gNB via the L2 U2N Relay UEs (the last relay, intermediate relay and first relay UE). In case a U2N relay UE in multihop link is in RRC_IDLE/ INACTIVE state, when the U2N relay UE receives the RRCReconfigurationComplete message, the reception of the RRCReconfigurationComplete message will first trigger RRC setup/resume procedure for the U2N relay UE to enter RRC_CONNECTED state.

11.	The target gNB-DU sends the UL RRC MESSAGE TRANSFER message to gNB-CU by including the RRCReconfigurationComplete message.

12.	The gNB-CU sends an UE CONTEXT RELEASE COMMAND message to the source gNB-DU.

13.	The source gNB-DU releases the UE context and responds the gNB-CU with an UE CONTEXT RELEASE COMPLETE message.

14.	The gNB-CU sends the RRCReconfiguration message to the source L2 U2N relay to reconfigure the connection among the source L2 U2N relay UE, the source gNB-DU and gNB-CU. The RRCReconfiguration message to the source L2 U2N relay UE can be sent any time after step 7 based on gNB implementation (e.g., to release Uu or PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration related to the L2 U2N Remote UE).

15.	Either source L2 U2N Relay UE's AS layer or L2 U2N Remote UE's AS layer indicates upper layers to release PC5 unicast link after receiving the RRCReconfiguration messag. The timing to execute link release is up to UE implementation after step 7 or step14.switching and the path switch type.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>