3GPP TSG-RAN WG3 Meeting #127bis	R3-251932

Wuhan, CN, April 7–11, 2025

Agenda item:	 10.2

Source:		Lenovo

Title:		(TP for SON BLCR for 38.473) Discussion on MRO for LTM

Document for:		Discussion and Decision

1	Introduction

RAN3#124 meeting agreed that:

For failures due to wrong selection of candidate LTM cell, CU is in charge of root cause analysis and performs optimization.

For failures due to inappropriate cell switch triggering (e.g. wrong cell selection at cell switch, wrong cell switch timing, …) source DU performs optimization.

RAN3 to discuss enhancements related to the information of RACH-less access and RACH-based access from UE related to MRO for LTM.

RAN3#125 meeting agreed that:

Reuse the existing connection failure definition of too late handover, too early handover, handover to wrong cell for LTM failure case. Add a sentence for LTM handover.

Take the existing detection mechanism descriptions as baseline.

For failures due to inappropriate cell switch triggering, CU initiates analysis and may forwards the RLF report to the DU responsible for the failure.

CU needs to forward the RLF related information to DU using existing Access and Mobility Indication procedure.

RAN3#125bis meeting agreed that:

For TP to BL CR to TS 38.300:

- Remove editor’s note 1 and 2. No need to capture LTM recovery in the failure definitions.

- For detection mechanism, use same approach as for CHO i.e. reuse existing sub-bullets to add LTM.

- Change “successful handover" definition, as proposed in R3-245230.

For TP to BL CR to TS 38.401:

- Capture wrong LTM prepared cell selection.

Add failure type to F1AP ACCESS AND MOBILITY INDICATION message.

CU does not forward the RLF Report to DU if failure is due to wrong selection of prepared LTM cell.

For TP to BL CR to TS 38.473:

- In ACCESS AND MOBILITY INDICATION message, the presence of the C-RNTI IE is kept optional. FFS on stage2 or stage3 to define the condition to include this optional C-RNTI.

For TP to BL CR to TS 38.401:

- For failure due to inappropriate cell switch triggering, CU forwards RLF Report to the failure DU in case of too late LTM.

- For failure due to inappropriate cell switch triggering, CU forwards RLF Report to the source DU in case of too early LTM, or LTM to wrong cell.

RAN3#126 meeting further agreed that:

MRO will cover the scenario that RACH-less LTM fails including outdate TA and UE performs RACH based LTM?

CU can adjust the candidate Cell list for LTM based on UHI.

DU should be able to resolve LTM ping-pong based on information over F1.

Network based solution is used for LTM candidate cell list.

Network based or UE based solution for TA acquisition type?

Will TA acquisition type always be available in the CU or is there a need for the DU to store and retrieve (partial) UE Context?

In this paper, we would further discuss the left issues of MRO for LTM.

2	Discussion

Stage2 or stage3 to define the condition of the optional C-RNTI IE in ACCESS AND MOBILITY INDICATION message?

Based on agreements achieved so far, we can sumarize the failure cause analysis and optimisation as well as RLF report forwarding in F1 as below:

- For failures due to wrong selection of candidate LTM cell, CU is in charge of root cause analysis and performs optimization, CU does not forward the RLF Report to DU;

- For failures due to inappropriate cell switch triggering (e.g. wrong cell selection at cell switch, wrong cell switch timing, …), CU initiates analysis and may forward the RLF report to the DU responsible for the failure (e.g. CU forwards RLF Report to the failure DU in case of too late LTM, or, CU forwards RLF Report to the source DU in case of too early LTM or LTM to wrong cell) , the DU responsible for the failure performs optimization.

For the RLF shortly after successful LTM cell switch, C-RNTI of target cell is included in RLF report, however, in case of too early LTM or LTM to wrong cell, the C-RNTI of source PSCell needs to be transferred in F1 interface together with the RLF report to enable source DU to identify UE context. Compared with adding description in TS38.401, it is more straightforward to clarify the presence of the optional C-RNTI IE in ACCESS AND MOBILITY INDICATION message in TS38.473 to improve text readability. The TP for TS38.473 is attached in the Annex.

Proposal 1: Clarify the presence of the optional C-RNTI IE in ACCESS AND MOBILITY INDICATION message in TS38.473.

Network based or UE based solution for TA acquisition type?

RAN2#127bis meeting agreed that unless RAN3 defines a NW-based solution, the UE logs and reports whether and how the UE got the TA value used for a failed LTM switch (gNB indicated or UE determined). Since it is CU that configures the UE to perform UE based TA measurement or PDCCH ordered early TA acquisition, CU can identify how the UE got the TA value when receiving the RLF report, based on the stored or retrieved UE context (e.g. configuration related to early TA acquisition, configuration related to UE based TA measurement). One concern was raised during online discussion e.g. source DU may not send the TA value received from CU to the UE even the TA value is valid, but we think it is a network implementation issue rather than a MRO issue, we should not consider this case. Therefore, it is not needed for the UE to log and report whether and how the UE got the TA value used for a failed LTM cell switch.

Proposal 2: It is not needed for the UE to report whether and how the UE got the TA value used for a failed LTM cell switch in RLF report.

UHI and ping-pong avoidance

Last RAN3#126 meeting agreed that CU can adjust the candidate Cell list for LTM based on UHI, and DU should be able to resolve LTM ping-pong based on information over F1. For intra-DU LTM, the source DU itself can store the UHI, but for subsequent LTM in inter-DU intra-CU LTM procedure, source DU may not always have the whole UHI information. Upon LTM cell switch is successful, CU can collect or update the UHI upon receiving the ACCESS SUCCESS message. For LTM ping-pong resolution at DU, CU can detect whether ping-pong event exists, and CU can forward the UHI to DU e.g. via the ACCESS AND MOBILITY INDICATION message when ping-pong is detected. Based on the indicated UHI from CU, DU can perform MRO optimisation.

Proposal 3: CU can collect or update the UHI upon receiving the ACCESS SUCCESS message.

Proposal 4: CU can forward the UHI to DU e.g. via the ACCESS AND MOBILITY INDICATION message when ping-pong is detected.

LTM failure due to wrong beam

RAN2 have discussed the issues as below and sent an LS to RAN3 [1]:

- BFR shortly after successful LTM cell switch to the wrong beam (“near failure”), and

- LTM cell switch failure due to wrong beam

On the former issue, RAN2 have acknowledged the issue but agreed not to define a UE-based solution. RAN2 respectfully asks RAN3 to consider defining a network-based solution for this. Furthermore, RAN2 asks RAN3 to see if/how the solution (if specified) could also be used for the latter issue.

From RAN3 point of view, both the two issues can be acknowledged since either BFR or HOF may be caused due to wrong beam selection at network side, and a unified solution can be considered to solve both issues.

Observation 1: The following two issues are acknowledged:

- Issue 1: BFR shortly after successful LTM cell switch due to wrong beam;

- Issue 2: LTM cell switch failure due to wrong beam.

The two issues may happen in the case that:

- Case 1 (BFR shortly after successful LTM cell switch due to wrong beam): shortly after a successful LTM cell switch, a beam failure occurs and the UE performs BFR, e.g. the beam where the beam failure happened and the beam where the UE succeeded to perform BFR are different but of the same cell.

- Case 2 (LTM cell switch failure due to wrong beam): a failure occurs during the LTM cell switch execution; the UE recovers or re-establishes the radio link connection in a cell where the connection failure occurs, and the beam where LTM cell switch execution failed and the beam where the UE succeeded to recover or re-establish the radio link connection are different but of the same cell.

To detect the failure is caused due to wrong beam:

- source DU may send TCI state ID which is activated or deactivated via the Candidate Cell TCI States Activation/Deactivation MAC CE to CU;

- candidate DU may send TCI state ID where beam failure or LTM cell switch failure happened, and, TCI state ID where the UE succeeded to perform BFR or LTM failure recovery or RRC re-establishment to CU;

- then, CU may identify whether the failure is caused due to wrong beam selection in LTM cell switch command MAC CE or in Candidate Cell TCI States Activation/Deactivation MAC CE; then, CU may indicate a new failure type (e.g. wrong TCI state in LTM cell switch command MAC CE, or wrong TCI state in Candidate Cell TCI States Activation/Deactivation MAC CE) to source DU via the Access and Mobility Indication message;

- then, source DU may perform corresponding MRO optimisation, e.g. update TCI state ID in the LTM cell switch command MAC CE, or update the TCI state ID in the Candidate Cell TCI States Activation/Deactivation MAC CE.

Proposal 5: To detect the failure is caused due to wrong beam:

- source DU may send TCI state ID which is activated or deactivated via the Candidate Cell TCI States Activation/Deactivation MAC CE to CU;

- candidate DU may send TCI state ID where beam failure or LTM cell switch failure happened, and, TCI state ID where the UE succeeded to perform BFR or LTM failure recovery or RRC re-establishment to CU;

- CU may identify whether the failure is caused due to wrong beam selection in LTM cell switch command MAC CE or in Candidate Cell TCI States Activation/Deactivation MAC CE; then, CU may indicate a new failure type (e.g. wrong TCI state in LTM cell switch command MAC CE, or wrong TCI state in Candidate Cell TCI States Activation/Deactivation MAC CE) to source DU via the Access and Mobility Indication message.

LTM failure due to outdated TA

In LTM, source DU is in charge of checking the TA validity (e.g., based on a TAT related timer in source DU), for example, the source DU starts a TAT related timer upon receiving the TA value from the CU, if the TAT related timer is running when source DU decides to send LTM Cell Switch Command MAC CE to the UE, source DU understands the TA value is valid and sends the TA value to the UE via the LTM Cell Switch Command MAC CE. It is not easy for the source DU to check the TA validity, e.g. if TAT related timer is near to be expired when sending the TA value to the UE, RACH-less LTM may fail due to the outdated TA, the UE may recover or re-establish the radio link connection to same target cell/beam via RA-based access, the TA used in RA-based access is different from the TA indicated in the LTM Cell Switch Command MAC CE. This failure case caused due to outdated TA should be considered.

Observation 2: Consider the LTM failure case due to outdated TA: RACH-less LTM cell switch to the target cell/beam fails, the UE recovers or re-establishes the radio link connection via RA-based access to the same target cell/beam.

Since RAN2 has less TU, similar as MRO for LTM failure due to wrong beam, we should prioritize to consider network based solution.

To detect LTM failure due to outdated TA:

- source DU may send following information to CU:

  - TA value which is indicated in the LTM Cell Switch Command MAC CE

  - timer duration for checking the TA validity, e.g. timer duration of the TAT related timer

  - elapsed or remaining duration of the timer when sending the LTM Cell Switch Command MAC CE to the UE

- candidate DU may send TA value which is used/applied for LTM failure recovery/RRC re-establishment to CU;

- then, CU may identify whether the failure is caused due to outdated TA, if yes, CU may indicate a new failure type (e.g. wrong early TA acquisition, or outdated TA) to source DU via the Access and Mobility Indication message;

- then, source DU may perform corresponding MRO optimisation, e.g. update the timer duration for checking the TA validity, or exclude early TA value in the LTM cell switch command MAC CE.

Proposal 6: To detect LTM failure due to outdated TA:

- source DU may send following information to CU:

  - TA value which is indicated in the LTM Cell Switch Command MAC CE

  - timer duration for checking the TA validity, e.g. timer duration of the TAT related timer

  - elapsed or remaining duration of the timer when sending the LTM Cell Switch Command MAC CE to the UE

- candidate DU may send TA value which is used/applied for LTM failure recovery/RRC re-establishment to CU;

- CU may identify whether the failure is caused due to outdated TA, if yes, CU may indicate a new failure type (e.g. wrong early TA acquisition, or outdated TA) to source DU via the Access and Mobility Indication message.

3	Conclusion

In this paper, MRO for LTM are further discussed. We have the following observations and proposals:

Observation 1: The following two issues are acknowledged:

- Issue 1: BFR shortly after successful LTM cell switch due to wrong beam;

- Issue 2: LTM cell switch failure due to wrong beam.

Observation 2: Consider the LTM failure case due to outdated TA: RACH-less LTM cell switch to the target cell/beam fails, the UE recovers or re-establishes the radio link connection via RA-based access to the same target cell/beam.

Proposal 1: Clarify the presence of the optional C-RNTI IE in ACCESS AND MOBILITY INDICATION message in TS38.473.

Proposal 2: It is not needed for the UE to report whether and how the UE got the TA value used for a failed LTM cell switch in RLF report.

Proposal 3: CU can collect or update the UHI upon receiving the ACCESS SUCCESS message.

Proposal 4: CU can forward the UHI to DU e.g. via the ACCESS AND MOBILITY INDICATION message when ping-pong is detected.

Proposal 5: To detect the failure is caused due to wrong beam:

- source DU may send TCI state ID which is activated or deactivated via the Candidate Cell TCI States Activation/Deactivation MAC CE to CU;

- candidate DU may send TCI state ID where beam failure or LTM cell switch failure happened, and, TCI state ID where the UE succeeded to perform BFR or LTM failure recovery or RRC re-establishment to CU;

- CU may identify whether the failure is caused due to wrong beam selection in LTM cell switch command MAC CE or in Candidate Cell TCI States Activation/Deactivation MAC CE; then, CU may indicate a new failure type (e.g. wrong TCI state in LTM cell switch command MAC CE, or wrong TCI state in Candidate Cell TCI States Activation/Deactivation MAC CE) to source DU via the Access and Mobility Indication message.

Proposal 6: To detect LTM failure due to outdated TA:

- source DU may send following information to CU:

  - TA value which is indicated in the LTM Cell Switch Command MAC CE

  - timer duration for checking the TA validity, e.g. timer duration of the TAT related timer

  - elapsed or remaining duration of the timer when sending the LTM Cell Switch Command MAC CE to the UE

- candidate DU may send TA value which is used/applied for LTM failure recovery/RRC re-establishment to CU;

- CU may identify whether the failure is caused due to outdated TA, if yes, CU may indicate a new failure type (e.g. wrong early TA acquisition, or outdated TA) to source DU via the Access and Mobility Indication message.

Reference

[1] R2-2501378, LS on SON for LTM, Apple

Annex: TP for TS38.473

### 9.2.10	Self Optimisation Support Messages

#### 9.2.10.1	ACCESS AND MOBILITY INDICATION

This message is sent by gNB-CU to gNB-DU to provide access and mobility information to the gNB-DU.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| RA Report List |  | 0..1 |  |  | YES | ignore |
| >RA Report Item |  | 1 .. <maxnoofRAReports> |  |  | - |  |
| >>RA Report Container | M |  | OCTET STRING | Includes the RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| RLF Report Information List |  | 0..1 |  |  | YES | ignore |
| >RLF Report Information Item |  | 1 .. <maxnoofRLFReports> |  |  | - |  |
| >>NR UE RLF Report Container | M |  | OCTET STRING | Includes the nr-RLF-Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| >>C-RNTI | O |  | 9.3.1.32 | C-RNTI allocated at the source gNB-DU in case that the connection Failure type IE in the nr-RLF-Report-r16 IE is set to “RLF” and RLF Report Failure Type IE is set to “too early LTM” or “LTM to wrong cell” | YES | ignore |
| >>RLF Report Failure Type | O |  | ENUMERATED (too late LTM, too early LTM, LTM to wrong cell,...) |  | YES | ignore |
| Successful HO Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful HO Report Information Item |  | 1 .. <maxnoofSuccessfulHOReports> |  |  | - |  |
| >>Successful HO Report Container | M |  | OCTET STRING | Includes the SuccessHO-Report IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| Successful PSCell Change Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful PSCell Change Report information Item |  | 1..<maxnoofSuccessfulPSCellChangeReports> |  |  | - |  |
| >>Successful PSCell Change Report Container | M |  | OCTET STRING | Includes the SuccessPSCell-Report IE as defined in TS 38.331 [8]. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofRAReports | Maximum no. of RA Reports, the maximum value is 64. |
| maxnoofRLFReports | Maximum no. of RLF Reports, the maximum value is 64. |
| maxnoofSuccessfulHOReports | Maximum no. of Successful HO Reports, the maximum value is 64. |
| maxnoofSuccessfulPSCellChangeReports | Maximum no. of Successful PSCell Change Reports. Value is 64. |