3GPP TSG-RAN WG3 Meeting #127bis	R3-251935

Wuhan, CN, April 7–11, 2025

Agenda item:	 10.4

Source:		Lenovo

Title:		Discussion on MRO for R18 leftovers

Document for:		Discussion and Decision

1	Introduction

The following agreements have been achieved until now:

MRO for MR-DC SCG failure:

Keep the following R18 agreement in R19:

Support MRO for SCG failure in EN-DC, NGEN-DC and NE-DC scenarios.

Take Stage 2 descriptions of PSCell change failure in TS37.340 as baseline for EN-DC and NGEN-DC SCG failure, and necessary updates can be added on top of it if needed.

Introduce new SCG FAILURE INFORMATION REPORT message and SCG FAILURE TRANSFER message over X2 interface.

RACH optimization for SDT:

Work on scenario of RACH optimization for SDT first.

There is no need for UE to report SDT configurations, e.g., RSRP threshold, data volume threshold, T319a, and other configurations.

The information that stored and reported in the RA report for SDT:

c. the time from the start of RA-SDT to the reporting of RA report

We will send an LS to RAN2 to inform about the previous agreement to report the time from the start of RA-SDT to the reporting of RA report.

MHI Enhancement for SCG Deactivation/Activation:

Wait for RAN2 progress on whether to use absolute time or relative time to express SCG activated time.

In this paper, we would further discuss the details of MRO for R18 leftovers.

2	Discussion

  - 2.1 MRO for MR-DC SCG failure

FFS after RAN3#126 meeting:

- Do we see any need to send additional info from SN to MN: suitable cell?

- Do we see any need to send additional info from MN to SN: e.g., failure type, perRA-InfoList?

RAN2#127bis meeting agreed to add reporting of the following parameters for SCG failure report in EN-DC scenario:

⁻	For failedPSCellId and previousPSCellId: frequency and the PCI of the PSCell;

⁻	For timeSCGFailure: value range 0-1023;

⁻	For failureType: Reuse the legacy field;

-	perRA-InfoList.

RAN3#125 meeting agreed to introduce new SCG FAILURE INFORMATION REPORT message and SCG FAILURE TRANSFER message over X2 interface, and explicit IEs of SCG failure information (e.g. Source PSCell CGI, Failed PSCell CGI and Time SCG Failure) were included in SCG FAILURE INFORMATION REPORT message for SCG failure reason detection and optimisation. Besides source PSCell CGI, failed PSCell CGI and Time SCG Failure, failureType and perRA-InfoList are also beneficial for optimising SCG related parameters (e.g. SCG RACH configuration(s) during SCG change). So, it is better to enhance SCG FAILURE INFORMATION REPORT message in X2 interface to include failureType and perRA-InfoList.

Proposal 1: Enhance SCG FAILURE INFORMATION REPORT message in X2 interface for EN-DC SCG failure to include failureType and perRA-InfoList.

Similarly, previousPSCellId, failedPSCellId and Time SCG Failure are included in SCG FAILURE INFORMATION REPORT message in Xn interface for NGEN-DC, similar as enhancements for EN-DC SCG failure in X2 interface, the failureType and perRA-InfoList IEs can also be included in SCG FAILURE INFORMATION REPORT message in Xn interface.

Proposal 2: Enhance SCG FAILURE INFORMATION REPORT message in Xn interface for NGEN-DC SCG failure to include failureType and perRA-InfoList.

Regarding the FFS on whether there is any need to send suitable cell info from SN to MN, we think MN can select the suitable PSCell based on SSB measurement results instead of CSI-RS measurement results from the UE which can’t be understood by MN, no critical issue exists if we follow legacy principle (e.g. for SCG failure in NR-NR DC, it is MN selects the suitable PSCell). Therefore, SN is not essential to select the suitable PSCell after decoding the CSI-RS measurement results. and there is no need to transfer the suitable PSCell id from SN to MN in X2 or Xn interface.

Proposal 3: Not transfer the suitable PSCell id from SN to MN in X2 or Xn interface.

  - 2.2 RACH optimization for SDT

FFS after RAN3#126 meeting:

- Remaining Data volume at the end of SDT transmission and/or SDT failure

Until now RAN2 have agreed that:

For the purpose of SON enhancements for SDT, include RSRP/data volume related information

Downlink RSRP value and buffered uplink data volume at the time when the UE evaluates if it should perform SDT.

When SDT failure happens, the UE can indicate the failure cause of SDT to the network, e.g. T319a expiration. Details are TBD, e.g. if RSRP and data volume can also be included in such report.

Do not add logging of sdt-RSRP-Threshold, since already agreed by RAN3 to not support it.

UE logs and reports the failure cause for SDT to the network.

For failed SDT case, UE includes the DL RSRP and UL data volume at the time of SDT evaluation in SON report. For successful SDT procedure, the UE does not log.

RAN2 understands for SON/MDT R19 the SDT enhancements only relate to the RA-SDT procedure. This does not rule out the case when the UE falls back from RA-SDT.

The UE logs the DL RSRP and UL data volume for the failed to initiate of RA-SDT in RA report.

The following failure causes can be logged for failed SDT in RA report:

⁻	upon receiving indication from the MCG RLC that the maximum number of retransmissions has been reached while SDT procedure is ongoing;

⁻	upon random access problem indication is received from MCG MAC while SDT procedure is ongoing;

⁻	upon T319a expires;

⁻	upon the lower layers indicate that cg-SDT-TimeAlignmentTimer or the configuredGrantTimer expired before receiving network response for the UL CG-SDT transmission with CCCH message while SDT procedure is ongoing.

Include cell re-selection as a failure cause for failed RA-SDT in RA-Report

Introduce a new field in RA-Report to indicate the elapsed time since the execution of RA-SDT. Value in seconds. The maximum value is 172800 seconds.

Previous RAN2 meeting agreed UE includes the DL RSRP and UL data volume at the time of SDT evaluation in SON report for failed SDT case. However, even condition(s) of SDT evaluation is successful and a RA-SDT is triggered, it may also fail due to a low threshold of radio link quality or due to a high threshold of data volume, for example, the UE triggers RA-SDT but data transmission fails because of bad radio link quality or heavy data volume. When the triggered RA-SDT fails, the UE may re-trigger SDT transmission or enter RRC connected for data transmission.

To improve RA-SDT performance, it is beneficial to store and report the RSRP when the triggered RA-SDT fails, then network can analyse the failure issue e.g. whether the RSRP threshold used to decide whether to trigger RA-SDT is set properly, for example, in the case that the RSRP threshold and RSRP when RA-SDT fails are close, it means that the RSRP threshold is set too low, network may modify with a higher RSRP threshold. Similarly, it is beneficial to store and report the data volume when the triggered RA-SDT fails, network can analyse whether the data volume threshold used to decide whether to trigger RA-SDT is set properly, e.g., in the case that the data volume threshold and data volume when RA-SDT fails are close, it means that the data volume threshold is set too high, network may modify with a lower data volume threshold. The detailed informtion to be stored or reported by the UE in the RA report can be further checked with RAN2, and an LS to RAN2 may be needed if RAN3 achieves some conclusions.

Proposal 4: RSRP and data volume when triggered RA-SDT fails can be stored and reported in the RA report.

3	Conclusion

In this paper, MRO for R18 leftovers are discussed. We have the following proposals:

Proposal 1: Enhance SCG FAILURE INFORMATION REPORT message in X2 interface for EN-DC SCG failure to include failureType and perRA-InfoList.

Proposal 2: Enhance SCG FAILURE INFORMATION REPORT message in Xn interface for NGEN-DC SCG failure to include failureType and perRA-InfoList.

Proposal 3: Not transfer the suitable PSCell id from SN to MN in X2 or Xn interface.

Proposal 4: RSRP and data volume when triggered SDT fails can be stored and reported in the RA report.