3GPP TSG-RAN WG3 #127-bis	R3-251937

Wuhan, China, 7th – 11th Mar 2025

Agenda Item:	11.2

Source:	Lenovo

Title: 		Left issues with AI enabled slicing

Document for:	Discussion and Approval

# Introduction

In this paper, we try to discuss the left issues with per slice UE performance measurement.

# Discussion

Whether predicted composite available capacity should be introduced and configured to be reported together with the predicted slice available capacity? To be checked in next meeting…

It was raised by some company that in the legacy that the value of Slice Available Capacity can be only correctly understood if the corresponding total NG-RAN resources of that cell is also given (since slice available capacity is relative to the total NG-RAN resources per cell).

#### 9.2.2.51 Composite Available Capacity Group The Composite Available Capacity Group IE indicates the overall available resource level per cell and per SSB area in the cell in Downlink, Uplink and Supplementary Uplink. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Composite Available Capacity Downlink; M; Composite Available Capacity 9.2.2.52; For the Downlink; –. Composite Available Capacity Uplink; M; Composite Available Capacity 9.2.2.52; For the Uplink, including both NUL and SUL (if available); –. Composite Available Capacity Supplementary Uplink; O; Composite Available Capacity 9.2.2.52; For the SUL; YES; ignore. #### 9.2.2.52 Composite Available Capacity The Composite Available Capacity IE indicates the overall available resource level in the cell in either Downlink or Uplink. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Cell Capacity Class Value; O; 9.2.2.53. Capacity Value; M; 9.2.2.54; ‘0’ indicates no resource is available, Measured on a linear scale.. #### 9.2.2.53 Cell Capacity Class Value The Cell Capacity Class Value IE indicates the value that classifies the cell capacity with regards to the other cells. The Cell Capacity Class Value IE only indicates resources that are configured for traffic purposes. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Capacity Class Value; M; INTEGER (1..100,...); Value 1 indicates the minimum cell capacity, and 100 indicates the maximum cell capacity. There should be a linear relation between cell capacity and Cell Capacity Class Value.. #### 9.2.2.54 Capacity Value The Capacity Value IE indicates the amount of resources per cell and per SSB area that are available relative to the total NG-RAN resources. The capacity value should be measured and reported so that the minimum NG-RAN resource usage of existing services is reserved according to implementation. The Capacity Value IE can be weighted according to the ratio of cell capacity class values, if available. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Capacity Value; M; INTEGER (0..100); Value 0 indicates no available capacity, and 100 indicates maximum available capacity with respect to the whole cell. Capacity Value should be measured on a linear scale.. SSB Area Capacity Value List; 0..1. >SSB Area Capacity Value Item; 1..<maxnoofSSBAreas>. >>SSB Index; M; INTEGER (0..63). >>SSB Area Capacity Value; M; INTEGER (0..100); Value 0 indicates no available capacity, and 100 indicates maximum available capacity . SSB Area Capacity Value should be measured on a linear scale.. #### 9.2.2.55 Slice Available Capacity The Slice Available Capacity IE indicates the amount of resources per network slice that are available per cell relative to the total NG-RAN resources per cell. The Slice Available Capacity Value Downlink IE and the Slice Available Capacity Value Uplink IE can be weighted according to the ratio of the corresponding cell capacity class values contained in the Composite Available Capacity Group IE, if available. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Slice Available Capacity; 1..< maxnoofBPLMNs >. >PLMN Identity; M; 9.2.2.4; Broadcast PLMN. >S-NSSAI Available Capacity List; 1. >>S-NSSAI Available Capacity Item; 1 .. < maxnoofSliceItems>. >>>S-NSSAI; M; 9.2.3.21. >>>Slice Available Capacity Value Downlink; M; INTEGER (0..100); Value 0 indicates no available capacity, and 100 indicates maximum available capacity . Slice Available Capacity Value Downlink should be measured on a linear scale.. >>>Slice Available Capacity Value Uplink; M; INTEGER (0..100); Value 0 indicates no available capacity, and 100 indicates maximum available capacity. Slice Available Capacity Value Uplink should be measured on a linear scale..

Firstly, we believe the total NG-RAN resources per cell is a rather static configuration. And in the legacy, the total NG-RAN resources per cell in the legacy is indicated by the Cell Capacity Class Value conveyed in the Composite Available Capacity Group. In addition, Cell Capacity Class Value is an optional IE which may or may not be provided by the neighbour RAN.

In legacy, the total NG-RAN resources per cell is indicated by the Cell Capacity Class Value IE OPTIONALLY conveyed in the Composite Available Capacity Group IE in Resource Status Report procedure, which is optionally provided by the neighbor NG-RAN.

When it comes to the predicted Slice Available Capacity, even if the corresponding predicted Cell Capacity Class Value is not given, there is no critical issue as the local NG-RAN node that receives the predicted Slice Available Capacity can either:

- Handle as in legacy when the Cell Capacity Class Value is not provided (which is an optional IE), or

- Rely on the Cell Capacity Class Value received via the legacy Resource Status Report procedure (if provided). Since the cell capacity class is not dynamically configured.

For predicted Slice Available Capacity exchange, the predicted Cell Capacity Class Value is not necessary and thus not supported in Rel19.

UE performance per S-NSSAI defined in SA5 can be referred in RAN3 which can be used as the feedback. Other granularity of UE performance are not excluded and to be continued based on cases ...

In the last meeting, it was agreed to use the UE performance per S-NSSAI defined in SA5. Considering the issue has been discussed since the beginning of this release, RAN3 is suggested to not further discuss the finer granularity of UE performance in the agenda of AI enabled slicing. It can be discussed e.g. in 11.4 in case of split architecture or DC.

RAN3 is suggested to not further discuss the finer granularity of UE performance in the agenda of AI enabled slicing.

Whether to introduce a new IE or extending existing bitmap to request slice UE performance in the Data Collection Request message? Taking the R18 signalling design as the starting point? How to include the failed report characteristic in the Data Collection Response message?

R3-250786 SOD of AI for slicing It seems there is two option to reflect the request of slice UE performance in the DATA COLLECTION REQUEST message. Option 1: To introduce a new IE to request slice UE performance in the DATA COLLECTION REQUEST message. Details of implementation about Option1 : 9.2.3.186 UE Performance Collection Configuration This IE indicates the configuration for UE performance measurement collection. If the Slice Level UE Performance Reporting IE is set to “true” then the UE Performance measurements are collected on a per slice granularity. IE/Group Name; Presence; Range; IE Type and Reference; Semantics Description; Criticality; Assigned Criticality. Collection Time Duration for UE Performance; M; INTEGER(1..5000, ...); Time duration starting at successful handover within which the UE performance measurements are collected. Unit: millisecond; -. Slice Level UE Performance Reporting; O; ENUMERATED (true, …); Indicates that UE Performance measurements are collected on a per slice granularity.; YES; ignore. Option 2a: Extending existing bitmap to request slice UE performance in the DATA COLLECTION REQUEST message: Report Characteristics for Data Collection; C-ifRegistration Request ForDataCollectionStart; BITSTRING (SIZE(32)); Each position in the bitmap indicates the object the NG-RAN node2 is requested to report. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC Connections Fourth Bit = Average UE Throughput DL, Fifth Bit = Average UE Throughput UL, Sixth Bit = Average Packet Delay, Seventh Bit = Average Packet Loss DL Eighth Bit = Energy Cost Ninth Bit = Measured UE Trajectory Tenth Bit = Predicted Slice Available Capacity Eleventh Bit = Slice Average UE Throughput DL Twelfth Bit = Slice Average UE Throughput UL Thirteenth Bit = Slice Average Packet Delay, Fourteenth Bit = Slice Average Packet Loss DL Other bits are ignored by the NG-RAN node2.; YES; reject.

With respect to indicate the request of slice UE performance in the Data Collection Request message, there are two options identified from last meeting discussion

- Option 1: A new “slice level performance” flag IE together with the existing bit (e.g., Fourth Bit = Average UE Throughput DL) in Report Characteristics for Data Collection IE

- Option 2: Additional bits in existing Report Characteristics for Data Collection IE (e.g, Eleventh Bit = Slice Average UE Throughput DL)

In our understanding, one fundamental difference is that if there is a need to support a NG-RAN node to request both i) Per UE Performance and ii) Per UE per S-NSSAI performance at the same time in the same Data Collection Initialization procedure.

In Option 1, either i) Per UE Performance or ii) Per UE per S-NSSAI performance can be requested.

In Option 2, both i) Per UE Performance and ii) Per UE per S-NSSAI performance can be requested at the same time.

RAN3 clarifies if there is a need of requesting both i) Per UE Performance and ii) Per UE per S-NSSAI performance at the same time before deciding the way to request per UE per S-NSSAI performance.

  - Option 1: A new “slice level performance” flag IE together with the existing bit (e.g., Fourth Bit = Average UE Throughput DL) in Report Characteristics for Data Collection IE

  - Option 2: Additional bits in existing Report Characteristics for Data Collection IE (e.g, Eleventh Bit = Slice Average UE Throughput DL)

# Conclusion

Based on the discussion above, we observe:

Observation 1	In legacy, the total NG-RAN resources per cell is indicated by the Cell Capacity Class Value IE OPTIONALLY conveyed in the Composite Available Capacity Group IE in Resource Status Report procedure, which is optionally provided by the neighbor NG-RAN.

Based on the discussion above, we propose:

Proposal 1	For predicted Slice Available Capacity exchange, the predicted Cell Capacity Class Value is not necessary and thus not supported in Rel19.

Proposal 2	RAN3 is suggested to not further discuss the finer granularity of UE performance in the agenda of AI enabled slicing.

Proposal 3	RAN3 clarifies if there is a need of requesting both i) Per UE Performance and ii) Per UE per S-NSSAI performance at the same time before deciding the way to request per UE per S-NSSAI performance.

a.	Option 1: A new “slice level performance” flag IE together with the existing bit (e.g., Fourth Bit = Average UE Throughput DL) in Report Characteristics for Data Collection IE

b.	Option 2: Additional bits in existing Report Characteristics for Data Collection IE (e.g, Eleventh Bit = Slice Average UE Throughput DL)