3GPP TSG-RAN WG3 #127-bis	R3-251939

Wuhan, China, 7th – 11th Mar 2025

Agenda Item:	11.4

Source:	Lenovo

Title: 		Discussion on UE performance collection in split architecture

Document for:	Discussion and Approval

# Introduction

In the previous meeting, it was agreed that the packet delay is calculated by CU-UP considering the DDDS received from DU.

Packet delay measured for UE Performance feedback is sent from CU-UP (the delay in DU side is aggregated in the CU-UP) to the CU-CP in the case of CP-UP split architecture.

Packet delay measured for UE Performance feedback is sent from DU to CU via DDDS in the case of CU-DU split architecture.

In this paper, we further discuss the left issues.

# Discussion

## 2.1	UE performance measurement

The existing UE performance IE consists of average UE throughput DL, average UE throughput UL, and average packet loss DL as below:

#### 9.2.3.179 UE Performance This IE indicates the UE performance measurements metrics. IE/Group Name; Presence; Range; IE Type and Reference; Semantics Description. Average UE Throughput DL; O; Bit Rate 9.2.3.4. Average UE Throughput UL; O; Bit Rate 9.2.3.4. Average Packet Delay; O; 9.2.3.187. Average Packet Loss DL; O; Packet Loss Rate 9.2.3.11.

TS 38.314 4.2.1.5 Packet Loss Rate 4.2.1.5.1 Packet Uu Loss Rate in the DL per DRB per UE The objective of this measurement is to measure packets that are lost at Uu transmission, for OAM performance observability or for QoS verification of MDT. Protocol Layer: RLC Table 4.2.1.5.1-1: Definition for Packet Uu Loss Rate in the DL per DRB per UE Definition; Uu Packet Loss Rate in the DL per DRB per UE. One packet corresponds to one RLC SDU. The measurement is done separately per DRB. Detailed Definition: M(T,drbid)=Dloss(T,drbid)*1000000N(T,drbid)+Dloss(T,drbid), where explanations can be found in the table 4.2.1.5.1-2 below.. 4.2.1.5.2 Packet Uu Loss Rate with delay threshold in the DL per DRB per UE The objective of this measurement is to measure the DL packets loss including any packets not successfully transmitted or packets successfully received but delayed more than a delay threshold at Uu transmission, for OAM performance observability or for QoS verification of MDT. Protocol Layer: RLC Table 4.2.1.5.2-1: Definition for Packet Uu Loss Rate with delay threshold in the DL per DRB per UE Definition; Uu Packet Loss Rate with delay threshold in the DL per DRB per UE: One packet corresponds to one RLC SDU. The measurement is done separately per DRB. Detailed definition: M_dt(T,drbid)=[DlossT,drbid+DexdT, drbid]*1000000N_dtT,drbid+DlossT,drbid+DexdT, drbid Where explanations can be found in the table 4.2.1.5.2-2 below..

TS 28.558 6.3.1.4 UE throughput 6.3.1.4.1 Average DL UE throughput in gNB a) This measurement provides the average UE throughput in downlink. This measurement is intended for data bursts that are large enough to require transmissions to be split across multiple slots. The UE data volume refers to the total volume scheduled for each UE regardless of if using only primary- or also supplemental aggregated carriers. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in EN-DC architecture [16]) and subcounters per supported S-NSSAI. This measurement is also referred to as DL M5 in TS 37.320 [9]. b) DER(N=1) c) This measurement is obtained according to the following formula based on the "ThpVolDl" and "ThpTimeDl" defined below. Separate counters are maintained for each mapped 5QI (or QCI for EN-DC architecture [16]) and for each supported S-NSSAI. If ThpTimeDl>0, ThpVolDlThpTimeDl×1000 [kbit/s] If ThpTimeDl=0, 0 [kbit/s] For small data bursts, where all buffered data is included in one initial HARQ transmission,, otherwise ThpTimeDl; The time to transmit a data burst excluding the data transmitted in the slot when the buffer is emptied. A sample of "ThpTimeDl" for each time the DL buffer for one Data Radio Bearer (DRB) is emptied.. The point in time after T2 when data up until the second last piece of data in the transmitted data burst which emptied the RLC SDU available for transmission for the particular DRB was successfully transmitted, as acknowledged by the UE.. The point in time when the first transmission begins after a RLC SDU becomes available for transmission, where previously no RLC SDUs were available for transmission for the particular DRB.. The RLC level volume of a data burst, excluding the data transmitted in the slot when the buffer is emptied. A sample for ThpVolDl is the data volume, counted on RLC SDU level, in kbit successfully transmitted (acknowledged by UE) in DL for one DRB during a sample of ThpTimeDl. (It shall exclude the volume of the last piece of data emptying the buffer).. 6.3.1.4.2 Average UL UE throughput in gNB a) This measurement provides the average UE throughput in uplink. This measurement is intended for data bursts that are large enough to require transmissions to be split across multiple slots. The UE data volume refers to the total volume scheduled for each UE regardless of using only primary- or also supplemental aggregated carriers. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in EN-DC architecture [16]) and subcounters per supported S-NSSAI. This measurement is also referred to as UL M5 in TS 37.320[9]. B) DER(N=1) c) This measurement is obtained according to the following formula based on the "ThpVolUl" and "ThpTimeUl" defined below. Separate counters are maintained for each mapped 5QI (or QCI for EN-DC architecture [16]) and for each supported S-NSSAI. If ThpTimeUl>0, ThpVolUlThpTimeUl×1000 [kbit/s] If ThpTimeUl=0, 0 [kbit/s] For small data bursts, where all buffered data is included in one initial HARQ transmission otherwise ThpTimeUl; The time to transmit a data burst excluding the data transmitted in the slot when the buffer is emptied. A sample of "ThpTimeUl" for each time the UL buffer for one Data Radio Bearer (DRB) is emptied.. The point in time when the data up until the second last piece of data in data burst has been successfully received for a particular DRB. The point in time when transmission is started for the first data in data burst for a particular DRB.. The RLC level volume of a data burst, excluding the data transmitted in the slot when the buffer is emptied. A sample for ThpVolUl is the data volume counted on RLC SDU level in kbit received in UL for one DRB during a sample of ThpTimeUl, (It shall exclude the volume of the last piece of data emptying the buffer)..

In CU and DU split architecture, DU can measure DL packet loss (38.314), UL/DL throughput (28.558).

Besides, the UE performance feedback measurement and report is triggered by the handover event. For split architecture discussion, both intra-gNB (with or without DU/CU-UP change) or inter-gNB handover should be considered.

For UE performance collection in split architecture, both intra-gNB (with or without DU/CU-UP change) and inter-gNB handover should be considered.

Over Xn interface, the UE performance feedback collection is done in two steps:

1. Source gNB configures the target gNB to measure and report required UE performance if the concerned handover happens

2. In the HANDOVER REQUEST message, source gNB includes an indicator that triggers target gNB to measure and report accordingly.

The advantage of above solution in Rel18 discussion is that source gNB does not need to configure the UE performance collection every time there is a concerned UE handed over to the target gNB, which could cause heavy signalling overhead if many UEs are handed over to the target gNB.

UE performance collection over Xn interface is designed in a way to avoid heavy signaling overhead.

In our view, the procedures over E1 and F1 interface can follow the similar design. For example,

- For E1 interface, the CU-CP can configure the CU-UP to measure and report required UE performance via a E1 Data Collection Reporting Initiation procedure if concerned handover happens. Then in the BEARER CONTEXT SETUP REQUEST message (e.g., for inter gNB handover with CU-UP change) or in the BEARER CONTEXT MODIFICATOIN REQUEST message (e.g., for intra gNB handover without CU-UP change), an indicator (e.g., Data Collection ID IE) can be added to trigger the UE performance measurement. The CU-UP will then measure and report the UE performance to CU-CP via a E1 Data Collection Reporting procedure.

- For F1 interface, the CU can configure the DU to measure and report required UE performance via a F1 Data Collection Reporting Initiation procedure if concerned handover happens. Then in the UE CONTEXT SETUP REQUEST message (e.g., for inter gNB handover with DU change) or in the UE CONTEXT MODIFICATOIN REQUEST message (e.g., for intra gNB handover without DU change), an indicator (e.g., Data Collection ID IE) can be added to trigger the UE performance measurement. The DU will then measure and report the UE performance to CU via a F1 Data Collection Reporting procedure.

For E1 interface, RAN3 uses the new DATA COLLECTION REQUEST/RESPONSE/UPDATE messages for CU-CP to configure the CU-UP to measure and report required UE performance per DRB, e.g., UL/DL delay.

For E1 interface, BEARER CONTEXT SETUP/MODIFICATION REQUEST messages are enhanced by including an indicator (e.g., Data Collection ID IE) to trigger the UE performance measurement and report.

For F1 interface, RAN3 uses new DATA COLLECTION REQUEST/RESPONSE/UPDATE messages for CU to configure the DU to measure and report required UE performance per DRB, e.g., DL packet loss, UL/DL throughput.

For F1 interface, UE CONTEXT SETUP/MODIFICATION REQUEST messages are enhanced by including an indicator (e.g., Data Collection ID IE) to trigger the UE performance measurement and report.

# 3	Conclusion

Based on the discussion above, we observe:

Observation 1	In CU and DU split architecture, DU can measure DL packet loss (38.314), UL/DL throughput (28.558).

Observation 2	UE performance collection over Xn interface is designed in a way to avoid heavy signaling overhead.

Based on the discussion above, we propose:

Proposal 1	For UE performance collection in split architecture, both intra-gNB (with or without DU/CU-UP change) and inter-gNB handover should be considered.

Proposal 2	For E1 interface, RAN3 uses the new DATA COLLECTION REQUEST/RESPONSE/UPDATE messages for CU-CP to configure the CU-UP to measure and report required UE performance per DRB, e.g., UL/DL delay.

Proposal 3	For E1 interface, BEARER CONTEXT SETUP/MODIFICATION REQUEST messages are enhanced by including an indicator (e.g., Data Collection ID IE) to trigger the UE performance measurement and report.

Proposal 4	For F1 interface, RAN3 uses new DATA COLLECTION REQUEST/RESPONSE/UPDATE messages for CU to configure the DU to measure and report required UE performance per DRB, e.g., DL packet loss, UL/DL throughput.

Proposal 5	For F1 interface, UE CONTEXT SETUP/MODIFICATION REQUEST messages are enhanced by including an indicator (e.g., Data Collection ID IE) to trigger the UE performance measurement and report.