3GPP TSG-RAN WG3 Meeting #127bis	R3-251942

Wuhan, China, 7th – 11th April, 2025

Agenda Item:	12.2

Source: 	Lenovo

Title: 	(TP to BL CR 38.401) Architecture and configuration for WAB-node

Document for:	Discussion and decision

- Introduction

A new WID for additional topological enhancements [1] has been approved to be discussed in Rel-19, and following detail objectives are identified for Wireless Access Backhaul (WAB) focus on RAN3.

The objectives of the Wireless Access Backhaul (WAB) work are as follows: - Specifications for the support of WAB including [RAN3]: - Support of a WAB-node including a WAB-gNB and a WAB-MT. - Support of backhauling of the WAB-gNB’s NG, Xn and OAM traffic over the WAB-MT’s PDU session(s). - Support of Xn interface(s) by the WAB-gNB with the WAB-MTs serving BH RAN node and with other surrounding gNBs, including how to avoid setting up Xn between WAB-gNBs. - Defining the behaviour of WAB-node in case the authorization status of WAB-MT and/or WAB-gNB changes. - Network integration procedures for WAB nodes. - Handling of WAB-gNB’s traffic (including Xn, NG and OAM traffic) during WAB-node mobility, including the case where the WAB-MT’s BH PDU session changes. - Support the UE’s AMF change for UEs connected to, or camped on, a WAB-gNB. - UE’s ULI that reflect the WAB node’s location. - The handling of: - PCI collision avoidance. - Reconfiguration of TAC and RANAC on WAB-gNBs. - Mechanisms to avoid multi-hop WAB topology. - Radio-resource coordination between access and backhaul links. - NG connection management.

In this contribution, the architecture and configuration for WAB are discussed.

- Discussion

Xn interface in WAB architecture

- Support of Xn interface(s) by the WAB-gNB with the WAB-MTs serving BH RAN node and with other surrounding gNBs, including how to avoid setting up Xn between WAB-gNBs.

Different to IAB-node, WAB-node has the gNB functionality and the WAB-gNB may establish Xn connection for the following scenarios.

Scenario 1: Xn connection between WAB-gNBs of two WAB-nodes

Scenario 2: Xn connection between a WAB-gNB and its BH RAN node

Scenario 3: Xn connection between a WAB-gNB and a surrounding RAN node

Currently, in the WAB-node integration procedure, the WAB-gNB may establish Xn connection(s) towards the BH RAN node and/or other surrounding NG RAN node(s), if needed.

For the Xn connection between the WAB-gNB and its BH RAN node, the WAB-gNB can trigger the Xn connection setup to the BH RAN node after WAB-gNB setup procedure. And it’s not optimal for the BH RAN node to trigger the Xn setup procedure because the BH RAN node is transparent for the gNB setup of the WAB-node. As for the TNL address for the BH RAN node, the WAB-gNB can obtain it via legacy Xn-C TNL address discovery procedure or it can be indicated from the BH RAN node directly.

Proposal 1: For Xn connection between WAB-gNB and its BH RAN node, it's the WAB-gNB to trigger the Xn setup procedure.

For the Xn connection between the WAB-gNB and the surrounding RAN node, both the WAB-gNB and the surrounding RAN node need to discovery each other before Xn connection setup. As legacy, a gNB can find another gNB for Xn connection via measurement report or OAM configuration. However, legacy procedure may be not timely enough due to the mobility of the WAB-node. Alternatively, the WAB-gNB and the surrounding RAN node can discovery each other with the help of the BH RAN node, e.g., the WAB-gNB and the surrounding RAN node can discovery each other upon receiving the neighbour cell information from the BH RAN node. Then each of the two nodes can trigger Xn connection setup procedure.

Proposal 2: For Xn connection between WAB-gNB and surrounding RAN node, both the WAB-gNB and the surrounding RAN node can trigger the Xn setup procedure.

The main motivation to setup Xn connection for a WAB-node is to accelerate the Xn based handover of the served UEs of the WAB-node. However, due to mobility of the WAB-node, it’s unexpected to perform UE handover between two mobile WAB-nodes. Therefore, Xn connection between two WAB-nodes needs to be avoided.

The WAB-gNB can be aware of the type of another WAB-gNB (whether the gNB is WAB-gNB or not) via 1) served UE’s measurement report, 2) WAB-MT’s measurement report or 3) neighbour information from BH RAN node. And then the WAB-gNB avoid to setup Xn interface to another WAB-gNB. Alternatively, without type information of another WAB-gNB, the type of node is included in Xn setup request message, based on the type information in the Xn setup request message, the target RAN node can determine whether to accept Xn interface setup or not.

Proposal 3: WAB-gNB can obtain the type of another WAB-gNB and then avoid to setup Xn interface to another WAB-gNB. Or the WAB-gNB can include the type information in the Xn setup request message.

Multi-hop WAB topology

Agree on Solution 2: For HO, the target WAB-gNB should reject HO preparation including the S-NSSAI used for Backhauling. To be further discussed whether Solution 1 or Solution 3 (or both) need to be selected 1. For initial access, WAB-gNB may use dedicated frequencies and/or PCIs and potential other legacy OTA parameters (e.g. NR CGI), to ensure that the WAB-MTs of other WAB-nodes avoid (re)selecting the WAB-gNB cells. 3. For initial access, WAB-gNB-cells broadcast a new indicator in SIB to bar WAB-MT, and the WAB-MT avoids (re)selection of cells broadcasting this indicator.

As discussed in last meeting, we had agreed that target WAB-gNB can identify the handover of WAB-MT via the S-NSSAI I the handover request message. While in case of WAB-node initial access, companies still show different views on Solution 1 and Solution 3.

As for Solution 1, the legacy OTA parameter may be used to avoid WAB-MT selecting to other WAB-gNB cells. However, this solution may be not always workable due to the network planning and has strong limitation on the deployment of the operator. While for Solution 3, this is always workable, and only simple impact is expected to be introduced in SIB1. Then we prefer to use Solution 3 in case of WAB-node initial access.

Proposal 4: In case of WAB-node initial access, WAB-gNB-cells broadcast a new indicator in SIB to bar WAB-MT, and the WAB-MT avoids (re)selection of cells broadcasting this indicator.

Radio resource coordination

WAB network in R19 only supports one-hop backhauling, where the WAB-node is directly connected to the BH RAN node. In this WAB topology, the downlink transmission from WAB-gNB to the served UEs is scheduled by the WAB-gNB itself, and the uplink transmission from WAB-MT to the BH RAN node is scheduled by the BH RAN node.

In a centralized resource allocation to avoid inter-cell or inter-node resource interference, the BH RAN node should provide some semi-static reserved DL/UL resources for each served WAB-nodes, and how to schedule on these semi-static resources is left to the implementation of WAB-node’s scheduling algorithm. However, there is no F1 interface between the BH RAN node and the WAB-node, the legacy F1AP message used for resource configuration on the IAB-node cannot be reused. Then the resource configuration may need to be delivered via the XnAP sigalling between the BH RAN node and the WAB-gNB.

According to the IAB specifications, interpreting the resources available to the IAB-DU depends on the D/U/F configuration at the IAB-DU because the L1 availability indication indicates availability of soft DL symbols, soft UL symbols, and soft flexible symbols in a slot instead of indicating availability of each symbol separately. That, in part, reduces the overhead of the L1 availability indication signaling. A similar principle can be adopted for the L1/L2 resource usage indication for WAB. As a result, the BH RAN node may require knowledge of the D/U/F configuration at the WAB-gNB. For this purpose, the WAB-gNB can use Xn signaling to inform the BH RAN node of its own D/U/F resource configurations.

Proposal 5: The WAB-gNB can signal its D/U/F configuration to the BH RAN node over Xn.

As for the resource configuration on DU of the BH RAN node, legacy F1AP GNB-DU RESOURCE CONFIGURATION can be reused, which includes the resource configuration (D/U/F/NA/H/S) on the gNB-DU of the BH RAN node, and it further includes the resource configuration and the multiplexing info of the WAB-gNB, which is helpful for the gNB-DU of the BH RAN node to avoid scheduling the WAB-MT in the slot/symbol with hard resources in the collocated WAB-gNB.

Proposal 6: Legacy F1AP GNB-DU RESOURCE CONFIGURATION can be reused for resource configuration on gNB-DU of the BH RAN node.

WAB-node co-location discovery

For resource coordination between the access link and backhaul link, the BH-gNB may need to discover co-location of the WAB-MT and the WAB-gNB. There are maybe three different cases considering WAB-node integration and WAB-node migration as follow.

- Case 1: WAB-node integration, where the WAB-MT connects to the network in the same way as a UE by performing RRC connection setup procedure with the BH gNB, and the WAB-gNB setup Xn connection with the same BH-gNB.

- Case 2: The WAB-MT performs handover to another neighbour gNB which doesn’t have setup Xn with WAB-gNB.

- Case 3: The WAB-MT performs handover to another neighbour gNB which has setup Xn with WAB-gNB.

In Case 1, during Xn setup between the WAB-gNB and the BH gNB, the WAB-gNB can trigger Xn setup to the BH gNB and send the Xn setup request message to the BH gNB, and the Xn setup request message may include an ID of WAB-MT (e.g., CRNTI of WAB-MT). In this case, the BH gNB can discover the WAB-MT and the WAB-gNB are co-location based on the CRNTI allocated by itself.

For Case 2, the solution in Case 1 can be reused. Alternatively, during RRC connection establishment between the WAB-MT and the target gNB, the WAB-MT can include the gNB ID of the WAB-gNB in the RRCReconfigurationComplete message, and then during Xn setup procedure between the WAB-gNB and the target gNB, the target gNB can discover the WAB-MT and the WAB-gNB are co-location based on the gNB ID received from the WAB-gNB.

As for Case 3, because the WAB node has already set up Xn interface with the target gNB (neighbour RAN node), the legacy Xn setup procedure cannot be used for WAB-node discovery in the target gNB. In an easy way, other XnAP procedure between the WAB-gNB and the target gNB can be used. Or the WAB-MT can include the gNB ID of the WAB-gNB in the RRCReconfigurationComplete message, then the target gNB can identify the WAB-MT is co-located with the WAB-gNB which has set up Xn connection. Furthermore, during the handover procedure of the WAB-MT, including the gNB ID in the handover request message is more efficient than in the RRCReconfigurationComplete message, in this way, the target gNB can discover the WAB-node earlier.

Proposal 7: To enable BH gNB discovering co-location of WAB-MT and WAB-gNB:

- Solution 1: Xn setup request message includes an ID of WAB-MT (e.g., CRNTI of WAB-MT)

- Solution 2: RRCReconfigurationComplete message includes a gNB ID of the WAB-gNB

- Solution 3: Handover request message includes a gNB ID of the WAB-gNB

Backhaul traffic mapping

After the WAB-MT setup with 5GC, at least one PDU session needs to be established for transporting the signalling related to NG setup. While for the NG-U traffic for the served UEs, all the NG-U traffic may be mapped to the same PDU session for the NG-C signaling. Alternatively, additional PDU sessions can be established to transport the NG-U and different PDU sessions are used to the NG-U traffic with different QoS requirements. In addition, different types of control plane signalling (e.g., non-UE associated signalling and UE associated signalling) may be mapped to different PDU sessions.

Proposal 8: NG-C signalling and NG-U traffic can be mapped to the same PDU session between WAB-MT and MT’s UPF, alternatively, different types of NG-C signalling and NG-U traffic with different QoS requirements can be mapped to different PDU sessions.

WAB dual connectivity

It is FFS whether a WAB gNB can act as an MN or SN in Dual Connectivity.

As defined in the objectives of the WID, we need to avoid setting up Xn connection between WAB-gNBs. That is, due to the mobility of the WAB-node, the Xn relation between WAB-gNBs need to be frequently updated. Similarity, as for the dual connectivity of WAB, once a WAB-gNB becomes a MN or a SN of served UE, both UE and another RAN node need to frequently maintain the relation with the mobile WAB-node.

In addition for R18 mobile IAB, dual-connectivity is also down-prioritized when considering mobility of the mobile IAB node. Then we also prefer to deprioritize the combination of WAB and dual-connectivity.

Proposal 9: The combination of WAB and dual-connectivity is deprioritized in this release.

WAB-gNB indication

FFS whether to Introduce a “WAB-gNB” indication in the NG SETUP REQUEST message.

In last meeting, some companies proposed to introduce a WAB-gNB indication in the NG SETUP REQUEST message, in order for the CN to be aware of the RAN node being a WAB-gNB. However, the motivation and benefit for the CN is ambiguous on the awareness of WAB-gNB. And anyway, additional ULI is supported by the WAB, the CN can always recognize the RAN node as a WAB-gNB based on the received additional ULI.

Proposal 10: No need to introduce a WAB-gNB indication in the NG SETUP REQUEST message.

WAB-gNB migration

It is FFS whether the single-gNB solution with two logical cells can be included in the description in TS38.401

As for the single logical WAB-gNB solutions, an LS was sent to SA2 (and CC RAN2) in last meeting to check whether these single-gNB solutions can be achieved without CN impact. Based on the Reply LS, there is no consensus on the support of a single-gNB solution in SA2. From RAN3 point of view, the discussion on the single-gNB solution is suspended in RAN3 and we only need to revisit the solution based on further inputs from other WG.

Proposal 11: Single-gNB solution is deprioritized in this release unless further inputs from other WGs.

# Conclusion

This contribution aims to discuss WAB architecture and configuration. And following observations and proposals are concluded.

Proposal 1: For Xn connection between WAB-gNB and its BH RAN node, it's the WAB-gNB to trigger the Xn setup procedure.

Proposal 2: For Xn connection between WAB-gNB and surrounding RAN node, both the WAB-gNB and the surrounding RAN node can trigger the Xn setup procedure.

Proposal 3: WAB-gNB can obtain the type of another WAB-gNB and then avoid to setup Xn interface to another WAB-gNB. Or the WAB-gNB can include the type information in the Xn setup request message.

Proposal 4: In case of WAB-node initial access, WAB-gNB-cells broadcast a new indicator in SIB to bar WAB-MT, and the WAB-MT avoids (re)selection of cells broadcasting this indicator.

Proposal 5: The WAB-gNB can signal its D/U/F configuration to the BH RAN node over Xn.

Proposal 6: Legacy F1AP GNB-DU RESOURCE CONFIGURATION can be reused for resource configuration on gNB-DU of the BH RAN node.

Proposal 7: To enable BH gNB discovering co-location of WAB-MT and WAB-gNB:

- Solution 1: Xn setup request message includes an ID of WAB-MT (e.g., CRNTI of WAB-MT)

- Solution 2: RRCReconfigurationComplete message includes a gNB ID of the WAB-gNB

- Solution 3: Handover request message includes a gNB ID of the WAB-gNB

Proposal 8: NG-C signalling and NG-U traffic can be mapped to the same PDU session between WAB-MT and MT’s UPF, alternatively, different types of NG-C signalling and NG-U traffic with different QoS requirements can be mapped to different PDU sessions.

Proposal 9: The combination of WAB and dual-connectivity is deprioritized in this release.

Proposal 10: No need to introduce a WAB-gNB indication in the NG SETUP REQUEST message.

Proposal 11: Single-gNB solution is deprioritized in this release unless further inputs from other WGs.

And a TP is drafted to reflect Solution 3 of Proposal 7 in another paper [3].

# Reference

- RP-242395. New WID on additional topological enhancements for NR.

- R3-244019. LS on questions regarding FS_VMR_Ph2. SA2.

- R3-251943. (TP to BL CR 38.423) WAB-node co-location discovery. Lenovo.

# TP to BL CR 38.401

# X	Wireless Access Backhaul

NEXT CHANGE

## X.a 	Xn connection management

For Xn connection between the WAB-gNB and its BH-gNB, the WAB-gNB can trigger the Xn connection setup to the BH-gNB after WAB-gNB setup procedure.

For Xn connection between the WAB-gNB and the surrounding RAN node, both the WAB-gNB and the surrounding RAN node can trigger the Xn setup procedure. And the WAB-gNB and the surrounding RAN node can discovery each other with the assist of the BH-gNB.

Due to mobility of the WAB-node, Xn connection between two WAB-nodes needs to be avoided. To achieve the avoidance, WAB-gNB can obtain the type of another WAB-gNB and then avoid to setup Xn interface to another WAB-gNB. Or the WAB-gNB can include the type information in the Xn setup request message.

In addition, dual-connectivity for WAB also needs to be avoided.

NEXT CHANGE

## X.b 	Radio resource management

In case of out-band backhaul, there is no issue on radio resource collision between WAB-node’s access links and backhaul link. While in the in-band case, resource coordination may be needed to facilitate the resource multiplexing of the WAB-node’s access links and backhaul link.

For resource coordination between the access link and backhaul link, the BH-gNB may need to discover co-location of the WAB-MT and the WAB-gNB as defined in clause X.c. And the BH-gNB may require knowledge of the resource configuration at the WAB-gNB. For this purpose, the WAB-gNB can use Xn signaling to inform the BH-gNB of its own resource configurations. As for the resource configuration on DU of the BH-gNB, legacy F1AP GNB-DU RESOURCE CONFIGURATION can be reused.

NEXT CHANGE

## X.c 	WAB-node co-location discovery

For resource coordination between the access link and backhaul link, the BH-gNB may need to discover co-location of the WAB-MT and the WAB-gNB. There are maybe three different solutions considering WAB-node integration and WAB-node migration as follow.

- Solution 1: Xn setup request message includes an ID of WAB-MT (e.g., CRNTI of WAB-MT)

- Solution 2: RRCReconfigurationComplete message includes a gNB ID of the WAB-gNB

- Solution 3: Handover request message includes a gNB ID of the WAB-gNB