3GPP TSG-RAN WG3 Meeting #127bis	R3-251954

Wuhan, China, 7th – 11th April, 2025

Agenda item:	21.3

Source: 	Lenovo

Title: 	Discussion on XR Enhancements

Document for:	Discussion and Approval

- Introduction

This paper discusses the rel-19 XR related enhancements on the following aspects:

-	Multi-modal service ID;

-	PDU set information marking support indication;

-	Alternative PDU set QoS;

-	TTNB and data burst size;

-	Available date rate reporting.

- Discussion

  - Multi-modal Service ID

In the incoming LS [1], SA2 has already agreed to provide the Multi-modal Service ID (MMSID) of QoS flows to NG-RAN. The MMSID should be provided per QoS flow basis, which means that the MMSID should be included in QoS Flow Level QoS Parameters IE. Introduction of MMSID could have specification impact on NG-AP, Xn-AP, F1-AP and E1-AP.

As an example for NG-AP, the MMSID can be defined as:

--------------------------------------------------------------------------------------

| MMSID | O |  | OCTET STRING (SIZE(FFS)) | Multi-modal service ID from the application, used to indicates QoS flows are related to a multi-modal service, as specified in TS 23.501 [9] and TS 38.300[8]. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

--------------------------------------------------------------------------------------

Proposal 1: Introduce MMSID in the QoS Flow Level QoS Parameters IE of NG-AP, Xn-AP, F1-AP and E1-AP.

Regarding how to use the MMSID by NG-RAN, it could rely on gNB’s implementation. However, there are two use cases in which the MMSID can be clearly applied, that are joint admission control and QoS flows to DRB mapping. In the stage-2 description, it is recommended to use joint admission control and QoS flow-to-DRB mapping as examples to illustrate how the MMSID can be applied within an NG-RAN node.

Proposal 2: In the stage-2 description, it is recommended to use joint admission control and QoS flow-to-DRB mapping as examples to illustrate how the MMSID can be applied by an NG-RAN node.

  - PDU Set Information Marking Support Indication

SA2 concluded that in the case of no DL PDU Set QoS parameters provided by the AF, according to the Protocol Description and PSA UPF supports of PDU Set Information marking, the SMF sends NG-RAN a new indicator indicating the DL PDU Set Information Marking is supported. The SMF activates the PDU Set Information marking based on information from NG-RAN. If the DL PDU Set Information Marking Support Indication has been sent to the NG-RAN node and the NG-RAN node has provided a PDU Set based handling support Indication indicating that PDU Set handling is supported, the SMF instructs PSA UPF to perform PDU Set marking and provides the PSA UPF the DL Protocol Description used by the service data flow.

As an example for NG-AP, the PDU Set based handling support Indication can be defined as:

--------------------------------------------------------------------------------------

9.3.1.XXX	PDU Set Information Marking Support Indication

This IE indicates whether PDU set information marking is supported for the core network as specified in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PDU Information Marking Support Indication | M |  | ENUMERATED (supported, …) |  |

--------------------------------------------------------------------------------------

For NG-AP, the PDU Set Information Marking Support Indication per PDU session should be included in the following IEs:

-	PDU Session Resource Setup Request Transfer;

-	PDU Session Resource Modify Request Transfer;

-	Handover Command Transfer.

For Xn-AP, the PDU Set Information Marking Support Indication should be included in the following messages for handover and UE context retrieval cases:

-	PDU Session Resources To Be Setup List.

For DC case, it is FFS whether and how  the PDU Set Information Marking Support Indication should be introduced.

For F1-AP, the PDU Set Information Marking Support Indication per DRB should be included in the following messages:

-	UE CONTEXT SETUP REQUEST message;

-	UE CONTEXT MODIFICATION REQUEST message;

-	UE CONTEXT MODIFICATION CONFIRM message.

For E1-AP, the PDU Set Information Marking Support Indication per PDU Session should be included in the following IEs:

-	PDU Session Resource To Setup List;

-	PDU Session Resource To Modify List.

Proposal 3: Introduce PDU Set Information Marking Support Indication in NG-AP, Xn-AP, F1-AP and E1-AP.

  - Alternative PDU Set QoS

As per SA2’s agreement, an Alternative QoS Profile represents a combination of QoS parameters PDB, PER, Averaging Window and GFBR to which the application traffic is able to adapt. The Alternative QoS Profile(s) shall also contain the PDU Set QoS parameters PSDB and PSER in the respective direction(s) when the QoS Profile contains PDU Set QoS parameters PSDB and PSER; otherwise, the corresponding Alternative QoS Profile(s) shall not contain the PDU Set QoS parameters PSDB and PSER.

In this sense, DL/UL alternative PSDB and PSER should be included in the Alternative QoS Parameters Set List IE.

--------------------------------------------------------------------------------------

9.3.1.151	Alternative QoS Parameters Set List

This IE contains alternative sets of QoS parameters which the NG-RAN node can indicate to be fulfilled when notification control is enabled and it cannot fulfil the requested list of QoS parameters.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Alternative QoS Parameters Set Item |  | 1..<maxnoofQoSparaSets> |  |  |  |  |
| >Alternative QoS Parameters Set Index | M |  | 9.3.1.152 |  | - |  |
| >Guaranteed Flow Bit Rate Downlink | O |  | Bit Rate 9.3.1.4 |  | - |  |
| >Guaranteed Flow Bit Rate Uplink | O |  | Bit Rate 9.3.1.4 |  | - |  |
| >Packet Delay Budget | O |  | 9.3.1.80 |  | - |  |
| >Packet Error Rate | O |  | 9.3.1.81 |  | - |  |
| >Maximum Data Burst Volume | O |  | 9.3.1.83 | Maximum Data Burst Volume is specified in TS 23.501 [9]. This IE is included if the Delay Critical IE is set to "delay critical" and is ignored otherwise. | YES | ignore |
| >UL PDU Set Packet Delay Budget | O |  | Extended Packet Delay Budget 9.3.1.135 | Alternative PDU Set Delay Budget for UL as specified in TS 23.501 [9] | - | - |
| >DL PDU Set Packet Delay Budget | O |  | Extended Packet Delay Budget 9.3.1.135 | Alternative PDU Set Delay Budget for DL as specified in TS 23.501 [9] | - | - |
| >UL PDU Set Error Rate | O |  | Packet Error Rate 9.3.1.81 | Alternative PDU Set Error Rate for UL as specified in TS 23.501 [9]. | - | - |
| >DL PDU Set Error Rate | O |  | Packet Error Rate 9.3.1.81 | Alternative PDU Set Error Rate for DL as specified in TS 23.501 [9]. | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSparaSets | Maximum no. of alternative sets of QoS Parameters allowed for the QoS profile. Value is 8. |

--------------------------------------------------------------------------------------

If the PDU Set QoS handling is applied in UL and/or DL, the NG-RAN uses the PDU Set QoS parameters PSDB and PSER for UL and/or DL instead of PDB and PER in the respective direction(s) to determine whether "GFBR can no longer be guaranteed" or "GFBR can again be guaranteed".

If the NG-RAN determines that only in one direction the GFBR, the PDB or the PER cannot be fulfilled, the NG-RAN shall indicate that "GFBR can no longer be guaranteed" together with the corresponding direction.

Proposal 4: Introduce Alternative PDU Set QoS in the Alternative QoS Parameters Set List IE of  NG-AP, Xn-AP, F1-AP and E1-AP.

  - TTNB and Burst Size

In Rel-18, data burst concept was introduced in XR. Data Burst is a set of multiple PDUs generated and sent by the application in a short period of time. An End of Data Burst (EoDB) indication can be added to the last PDU of each Data Burst in the GTP-U header. This End of Data Burst can be used by the gNB to push the UE back to sleep when possible for the purpose of UE power saving, e.g., entering DRX sleep state.

The time to next burst (TTNB) may be useful for the network scheduling for downlink, if it is provided in advance and is reliable and accurate at RAN. TTNB is the time interval between the transmission of the last packet in the current burst and first packet of the next burst, i.e., inter-burst time. SA4 thinks that if the N6 link is covered by transport level agreements and N6 jitter and bandwidth variation is known or can be predicted, senders can estimate how much time it will take to send all packets in a data burst and determine the start time of the next burst within an implementation-dependent error margin. Therefore, application servers may be able to provide an estimate of TTNB in the first packet of the burst for such cases and conditions.

The Data Burst Size may be provided to the NG-RAN in the downlink GTP-U header by the UPF in order to assist radio resource management.

| Bits |  |  |  |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 |  | 6 | 5 |  | 4 |  | 3 | 2 | 1 | 0 |  |
| PDU Type (=0) |  |  |  |  |  |  | EDB | EPDU | PSSI | New IE flag | 1 |
| QoS Flow Identifier (QFI) |  |  |  |  |  |  |  |  | PSSN |  | 2 |
| PSSN |  |  |  |  |  |  |  |  |  |  |  |
| Spare |  |  |  |  |  |  | PSI |  |  |  | 1 |
| PSN |  |  |  |  |  |  |  |  |  |  | 1 |
| PSSize |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| New IE flag 7(E) | New IE Flag 6 |  |  | New IE Flag 5 |  | New IE Flag 4 | New IE Flag 3 | New IE Flag 2 | New IE Flag 1 | New IE Flag 0 | 0 or 1 |
| TTNB |  |  |  |  |  |  |  |  |  |  | 0 or 1 (FFS) |
| DBSize |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| Padding |  |  |  |  |  |  |  |  |  |  | 0-3 |

Proposal 5: Introduce TTNB and Data Burst Size in the DL PDU Set Information frame of the PDU Set Information user plane protocol.

During handover case, the source gNB may receive a data burst with providing the TTNB. During data transmission of the data burst, handover and data forwarding may happen. When forwards the TTNB to the target gNB, the source gNB may already buffer the data burst for a while. The TTNB is not accurate any more due to the buffering time in source gNB. In order to target gNB gets the accurate TTNB, the buffering time in source time should be excluded. In other words, the remaining TTNB should be forwarded to the target gNB.RAN3 needs to discuss whether such kind of enhancement is needed or not.

Proposal 6: During handover, the source gNB forwards the remaining TTNB to the target gNB in the DL PDU Set Information frame of the PDU Set Information user plane protocol.

  - Available Data Rate

As per SA2 agreement, the QoS Monitoring for available bitrate allows the measurement and report of the UL and/or DL available bitrate for a GBR QoS Flow without Alternative QoS Profiles at the NG-RAN. The available bitrate refers to the bitrate that NG-RAN is able to provide for a QoS Flow within a duration. The duration is based on RAN implementation. The available bitrate can be between the GFBR and the MFBR. The available bitrate can also be below the GFBR for a QoS Flow with QoS Notification Control enabled without Alternative QoS Profiles.

For the reporting of the available bitrate, periodic reporting is not applied and only threshold-based reporting applies. Based on the PCC rule containing available bitrate as QoS monitoring parameter and Reporting threshold(s) per direction in the QoS Monitoring policy (as described in clause 6.1.3.27.1 of TS 23.503 [45]), the SMF configures the NG-RAN to measure and report the available bitrate by sending QoS monitoring request including the threshold(s). Based on the QoS monitoring request, if the bitrate that NG-RAN is able to provide matches or exceeds indicated Reporting threshold(s), the NG-RAN reports the UL and/or DL available bitrate information of the highest fulfilled threshold to the PSA UPF via the GTP-U header of UL packets.

9.3.1.12	QoS Flow Level QoS Parameters

This IE defines the QoS parameters to be applied to a QoS flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE QoS Characteristics | M |  |  |  | - |  |
| >Non-dynamic 5QI |  |  |  |  |  |  |
| >>Non Dynamic 5QI Descriptor | M |  | 9.3.1.28 |  | - |  |
| >Dynamic 5QI |  |  |  |  |  |  |
| >>Dynamic 5QI Descriptor | M |  | 9.3.1.18 |  | - |  |
| ……. |  |  |  |  |  |  |
| >UL Available Date Rate Threshold | O |  | Bit Rate 9.3.1.4 | The threshold triggering available date rate reporting for UL as specified in TS 23.501 [9]. | - | - |
| >DL Available Date Rate Threshold | O |  | Bit Rate 9.3.1.4 | The threshold triggering available date rate reporting for DL as specified in TS 23.501 [9]. | - | - |

Proposal 7: Introduce UL/DL Available Date Rate Threshold for a QoS flow to request NG-RAN reports the UL and/or DL available bitrate information.

Proposal 8: Introduce UL/DL Available Date Rate in the UL PDU SESSION INFORMATION frame in the PDU session user plane protocol.

For F1-AP, since the gNB-DU is in charge of scheduling, it would be gNB-DU to perform available data rate monitoring. It relies on one-to-one mapping between QoS flow and DRB. In case of one-to-one QoS flow to DRB mapping, the gNB-CU provides the UL/DL Available Date Rate Threshold for a QoS flow to the gNB-DU. And gNB-DU feedbacks the UL/DL Available Date Rate in the ASSISTANCE INFORMATION DATA frame of "NR RAN Container" GTP-U extension header.

Proposal 9: Introduce UL/DL Available Date Rate Threshold for a QoS flow to request gNB-DU the UL and/or DL available bitrate information over F1-AP.

Proposal 10: Introduce UL/DL Available Date Rate in the ASSISTANCE INFORMATION DATA frame of "NR RAN Container" GTP-U extension header.

3. Conclusion

This paper discusses the rel-19 XR related enhancements. The following observations and proposals are made.

Proposal 1: Introduce MMSID in the QoS Flow Level QoS Parameters IE of NG-AP, Xn-AP, F1-AP and E1-AP.

Proposal 2: In the stage-2 description, it is recommended to use joint admission control and QoS flow-to-DRB mapping as examples to illustrate how the MMSID can be applied by an NG-RAN node.

Proposal 3: Introduce PDU Set Information Marking Support Indication in NG-AP, Xn-AP, F1-AP and E1-AP.

Proposal 4: Introduce Alternative PDU Set QoS in the Alternative QoS Parameters Set List IE of  NG-AP, Xn-AP, F1-AP and E1-AP.

Proposal 5: Introduce TTNB and Data Burst Size in the DL PDU Set Information frame of the PDU Set Information user plane protocol.

Proposal 6: During handover, the source gNB forwards the remaining TTNB to the target gNB in the DL PDU Set Information frame of the PDU Set Information user plane protocol.

Proposal 7: Introduce UL/DL Available Date Rate Threshold for a QoS flow to request NG-RAN reports the UL and/or DL available bitrate information.

Proposal 8: Introduce UL/DL Available Date Rate in the UL PDU SESSION INFORMATION frame in the PDU session user plane protocol.

Proposal 9: Introduce UL/DL Available Date Rate Threshold for a QoS flow to request gNB-DU the UL and/or DL available bitrate information over F1-AP.

Proposal 10: Introduce UL/DL Available Date Rate in the ASSISTANCE INFORMATION DATA frame of "NR RAN Container" GTP-U extension header.