3GPP TSG-RAN WG3 #127bis								R3-251958

Wuhan, China, 7th – 11st April 2025

Agenda Item:	9.2

Source:	China Telecom, Huawei, CATT, ZTE

Title: 	Discussion on Security Issue for State Transition from RRC_INACTIVE to RRC_CONNECTED

Document for:	Discussion

Introduction

RRC_INACTIVE is a state in which a UE remains in the CM-CONNECTED state, allowing it to move within an area configured by NG-RAN (the RNA) without notifying NG-RAN. The UE-triggered transition procedure from RRC_INACTIVE to RRC_CONNECTED, in the case of UE context retrieval success, has been supported since Release 15. The security behavior of both the UE and the network is specified in Clause 6.8.2.1.3 of TS33.501[1]. We have examined the security aspects of the transition procedure from RRC_INACTIVE to RRC_CONNECTED in both SA3 and RAN3 and understand that some security information may be missing in RAN3-related specifications. Therefore, this paper aims to address the missing information and offer insights into potential CRs.

Discussion

As shown in Figure 1, the following describes the UE triggered transition from RRC_INACTIVE to RRC_CONNECTED in case of UE context retrieval success.

Figure 1 UE triggered transition from RRC_INACTIVE to RRC_CONNECTED

In step 3, the last serving gNB or source gNB needs to response retrieve UE Context Response, including security information from source gNB. The following texts on security information on UE 5G AS security context sent to the target gNB/ng-eNB are quoted from the clause 6.8.2.1.3 of TS33.501.

The source gNB/ng-eNB retrieves the stored UE context including the UE 5G AS security context from its database using the I-RNTI. The source gNB/ng-eNB verifies the ResumeMAC-I/shortResumeMAC-I using the current KRRCint key stored in the retrieved UE 5G AS security context (calculating the ResumeMAC-I/shortResumeMAC-I in the same way as described above). If the verification of the ResumeMAC-I/shortResumeMAC-I is successful, then the source gNB/ng-eNB calculates KNG-RAN* using the target cell PCI, target ARFCN-DL/EARFCN-DL and the KgNB/NH in the current UE 5G AS security context based on either a horizontal key derivation or a vertical key derivation according to whether the source gNB/ng-eNB has an unused pair of {NCC, NH} as described in Annex A.11/Annex A.12. The source gNB/ng-eNB can obtain the target PCI and target ARFCN-DL/EARFCN-DL from a cell configuration database by means of the target Cell-ID which was received from the target gNB/ng-eNB. Then the source gNB/ng-eNB shall respond with an Xn-AP Retrieve UE Context Response message to the target gNB/ng-eNB including the UE context that contains the UE 5G AS security context. The UE 5G AS security context sent to the target gNB/ng-eNB shall include the newly derived KNG-RAN*, the NCC associated to the KNG-RAN*, the UE 5G security capabilities, UP security policy, the UP security activation status with the corresponding PDU session ID(s), and the ciphering and integrity algorithms used by the UE with the source cell.

The target gNB/ng-eNB shall check if it supports the ciphering and integrity algorithms the UE used with the last source cell. If the target gNB/ng-eNB does not support the ciphering and integrity algorithms used in the last source cell or if the target gNB/ng-eNB prefers to use different algorithms than the source gNB/ng-eNB, then the target gNB/ng-eNB shall send an RRC Setup/RRCSetup message on SRB0 to the UE in order to proceed with RRC connection establishment as if the UE was in RRC_IDLE (i.e., a fallback procedure).

If the target gNB/ng-eNB supports the ciphering and integrity algorithms used with the last source cell and these algorithms are the chosen algorithms by the target gNB/ng-eNB, the target gNB/ng-eNB shall derive new AS keys (RRC integrity key, RRC encryption key and UP keys) using the algorithms the UE used with the source cell and the received KNG-RAN*. The target gNB/ng-eNB shall reset all PDCP COUNTs to 0 and activate the new keys in PDCP layer. The target gNB/ng-eNB shall respond to the UE with an RRC Resume message on SRB1 which is integrity protected and ciphered in PDCP layer using the new RRC keys.

If the UP security activation status can be supported in the target gNB/ng-eNB, the target gNB/ng-eNB shall use the UP security activations that the UE used at the last source cell. Otherwise, the target gNB/ng-eNB shall respond with an RRC Setup message to establish a new RRC connection with the UE.

Per TS33.501, the following mandatory security information on UE 5G AS security context shall be included in Retrieve UE Context Response message. In addition, the corresponding IEs specified in XnAP message are also listed as below:

- the newly derived KNG-RAN* and the NCC associated to the KNG-RAN*: AS Security Information IE specified in clause 9.2.3.50 of TS38.423[2], which contains Key NG-RAN Star(KNG-RAN*) and Next Hop Chaining Count(NCC)

- UE 5G security capabilities: UE Security Capabilities IE specified in clause 9.2.3.49 of TS38.423, which contains NR Encryption Algorithms, NR Integrity Protection Algorithms, E-UTRA Encryption Algorithms, and E-UTRA Integrity Protection Algorithms

- UP security policy: The UP security policy can be seen in RETRIEVE UE CONTEXT RESPONSE->UE Context Information – Retrieve UE Context Response (clause 9.2.1.13)->PDU Session Resources To Be Setup List(clause 9.2.1.1)->Security Indication(clause 9.2.3.52). The security indication IE specified in TS38.423 contains the user plane integrity protection indication and confidentiality protection indication, contain three codepoints “required”, “preferred”, and “not needed”, which indicates the requirements on UP integrity protection and ciphering for the corresponding PDU session, respectively.

- UP security activation status with the corresponding PDU session ID(s)：this information is contained in RRC Container

- the ciphering and integrity algorithms used by the UE with the source cell: this information is contained in RRC Container

As highlighted in the referenced texts, the target node must verify whether it can support the UP security activations previously used by the UE in the last source cell. If the target node does not support these UP security activations, it will prompt the UE to establish an RRC connection via SRB0 as if the UE were in the IDLE state.

For E1 interface, the Bearer Context Setup Request message from CU-CP to CU-UP contains only the UP security policy. When the UP security policy is set to "preferred," the CU-UP can determine whether to activate UP security based on its local strategy. As shown in Figure 1, if the CU-UP responds with "not perform" in the security result IE, then the CU-CP has to send RRC connection message to establish a new RRC connection with the UE as specified in TS 33.501 above. Therefore, it is necessary to incorporate an optional IE indicating the UP security activation status in the Bearer Context Setup Request message when the UP security policy is set to "preferred." Based on the indication, the gNB-CU-UP can follow to perform or not perform the UP security (including the Integrity Protection and Confidentiality Protection)

Fig.1 the procedure of the new serving gNB after receiving the retrieve UE context response message.

Proposal 1: to introduce a new optional UP security activation status IE in the Bearer Context Setup Request message when UP security policy is "preferred".

Based on the above analysis, we kindly ask RAN3 to agree the corresponding CRs for E1 interface [5-8].

Proposal 2: to agree the corresponding CRs [5-8] for E1 interface.

Proposal

Based on the above analysis, we have the following proposals:

Proposal 1: to introduce a new optional UP security activation status IE in the Bearer Context Setup Request message when UP security policy is "preferred".

Proposal 7: to agree the corresponding CRs [5-12] for Xn and E1 interface.

Reference

- 3GPP TS 33.501 V19.1.0, Security architecture and procedures for 5G system (Release 19)

- 3GPP TS 38.423 V18.4.0,Xn application protocol (XnAP) (Release 18)

- R3-184541,(TP for NR BL CR for TS 38.423) Corrections for UP Security Handling in DC,ZTE

- S3-250946, SA3 Rel-20 Planning, SA3 chair

- R3-25xxxx, Correction on Security Configuration in Bearer Context Setup message (TS38.463 Rel-15)

- R3-25xxxx, Correction on Security Configuration in Bearer Context Setup message (TS38.463 Rel-16)

- R3-25xxxx, Correction on Security Configuration in Bearer Context Setup message (TS37.483 Rel-17)

- R3-25xxxx, Correction on Security Configuration in Bearer Context Setup message (TS37.483 Rel-18)