3GPP TSG-RAN WG3 Meeting #127-bis	R3-251963

Wuhan, China, 7-11 April 2025

Agenda Item:	13.2

Source:	Ericsson

Title:	Further discussion on inter-CU LTM

Document for:	Discussion, Agreement

# Introduction

At the last RAN3 meeting, good progress was achieved for inter-CU LTM. In this contribution, we discuss further details to support inter-CU LTM for standalone and NR-DC. The corresponding TPs can be found in R3-251964 for XnAP, F1AP, and stage-2 for standalone and NR-DC.

# Discussion

## Reference configuration

The following point was left to be continued from last meeting: FFS on whether the source gNB can request a candidate gNB to provide a reference configuration.

We observe that if there are multiple candidate gNBs from different vendors (e.g., the source gNB is gNB A and there are two candidate gNBs, gNB B from vendor 1 and gNB C from vendor 2), a reference configuration sent by gNB A may not be suitable for gNB B. It is then simpler if only the source gNB A prepares the reference configuration. In any case, if a candidate gNB receives the reference configuration from the source gNB and does not find it useful, the solution still works, because the candidate gNB can respond with a complete configuration. If we extend this to the NR-DC scenarios it becomes complex to handle the different reference configuration provided by different nodes, where the MN or the SN can provide reference configuration associated with MCG or SCG. We propose to resolve the FFS and only allow the source gNB to provide the reference configuration.

- Only the source gNB can provide a reference configuration.

## Support for Early UL sync

In relation to the support for Early UL sync some agreements and a WA were reached at last meeting:

The source gNB-CU sends the pair of (gNB ID, new ID of early RACH configuration resource (to be further discussed)) to the candidate gNB-CU to request early RACH configuration.

WA: Introduce a new non-UE associated class 2 procedure on Xn, namely TA information Transfer message, to transfer the TA information from the candidate gNB-CU to the source gNB-CU.

The format of the new introduced IE is same with the gNB-DU ID.

In current XnAP baseline, the source CU sends to the candidate CU a pair of identifiers (Global gNB ID, Early RACH Resources Requester ID (naming FFS)) to request the early RACH configuration, and receives from the candidate a list of Preamble Index. Effectively the pair (Global gNB ID, Early RACH Resources Requester ID (naming FFS)), from the source CU perspective, represents the entity providing the list of Preamble Index. Because of this, we think that using the wording “Early RACH Resources Provider ID” reflects more precisely the purpose of the entity, and we propose to resolve the FFS and name the new IE Early RACH Resources Provider ID.

- For inter-CU LTM, use a new identifier to support Early UL sync (e.g., an “Early RACH Resources Provider ID”).

A RACH-less LTM is only possible if PRACH resources are available, and the amount of such resources is quite limited in general (even if same Preamble Index is reused with multiple PRACH occasions). The availability of PRACH resources for LTM is further limited if the candidate (DU or gNB) decides to use them for other purposes (i.e., for non-LTM UEs). If we consider an inter-CU LTM scenario with 3 gNBs, where gNB1 is the source gNB, gNB2 and gNB3 are candidate gNBs, it is possible that for example that gNB2 decides to release some (or all) the PRACH resources initially assigned to gNB1. If this happens, we think gNB1 should be informed, or otherwise it may try to acquire TA values for the UEs towards gNB2 using PDCCH orders that point at the PRACH resources provided by gNB2, and the Early UL sync will fail. A simple way is that gNB2 informs gNB1 which preamble indexes have been released.

- For inter-CU LTM, a candidate gNB informs the source gNB of the release of Preamble Indexes assigned for Early UL Sync.

- For intra-CU inter-DU LTM, a candidate DU informs the source DU (via the CU) of the release of Preamble Indexes assigned for Early UL Sync.

We further discuss now the signalling to use for transferring the TA values. First of all, a certain PDCCH order sent by the source DU for Early UL sync is always for one UE, and the source DU expects to receive one TA value associated with that UE. Therefore, even if multiple TA values can be transferred in the DU-CU/CU-DU TA INFORMATION TRANSFER, the source DU still needs to associate one TA value with only one UE. From previous meetings, we know that there is a preference in RAN3 to adopt in XnAP a non-UE associated procedure to transfer TA values, reusing the same approach as for F1AP. We think that reapplying this approach is not optimal, because while, in one hand it can reduce the number of messages exchanged by grouping the TA values before transferring them in Xn, on the other hand it can make the whole Early UL sync procedure useless, due to an excessive delay in transferring the TA values. The delay to transfer all the TA values for a certain UE from all the candidate cells to the source DU includes: 1) in the candidate gNB, the delay due to the CU grouping the TA values of potentially multiple UEs from all the candidate DUs; 2) the delay over Xn; 3) the delay at the source gNB, where the CU groups the TA values of potentially multiple UEs coming from different candidate gNBs; 4) the delay to transfer the group of TA values from source CU to source DU. Even if the TA value is still valid after incurring the delay in the transfer, such delay may be excessive in terms of mobility. Before even receiving the TA value, the source DU may realize that it is time to send the LTM Cell Switch command before losing the UE and the TA acquisition process becomes useless. Because of the reasons just explained, we definitively prefer to differentiate the Xn solution compared to the F1 Rel-18 solution, and transfer TA information in XnAP via a UE-associated procedure. This can be a class 2 procedure, as in F1AP.

- The Xn procedure to transfer TA information is a class 2 procedure.

- The Xn procedure to transfer a TA information is UE-associated and is used to transfer a list of TA values, each TA value for a different LTM candidate cell.

If the previous proposal is not agreeable, the bare minimum is that in F1 and Xn signaling there is a mean to identify the UE for which a certain TA value is being transferred. This can help to avoid unnecessary delays in transferring TA values that can be outdated before being sent.

- Include the UE XnAP IDs of the source and target gNB in the XnAP messages for TA information transfer.

- Include the UE F1AP IDs in the DU-CU/CU-DU TA Information Transfer messages.

## Single UE association for multiple LTM handover requests

At the last meeting the group discussed how to address/avoid the complex error handling if multiple UE association are used in the LTM initial preparation. The following WA was reached:

WA: Use a single UE association for multiple LTM handover request to the same candidate gNB.

It remains to be clarified how to identify the same UE from multiple Handover preparation procedure. We example with an example how this can be achieved, when two candidate cells used for one UE belong to the same candidate gNB:

- the source gNB sends the first HANDOVER REQUEST to a candidate gNB to prepare the first candidate cell of that candidate gNB, and includes in the message its assigned source XnAP ID

- the candidate gNB sends to the source gNB the HANDOVER REQUEST ACKNOWLEDGE and includes the source XnAP ID and the target XnAP ID that it has assigned for the UE. At this point, the UE association is established at both source and target gNBs

- the source gNB sends the second HANDOVER REQUEST to the candidate gNB to prepare the second candidate cell of that candidate gNB. The source gNB includes in the message the source XnAP ID used in step 1 and the target XnAP ID used in step 2.

- The target gNB understands that the UE association for which the second candidate cell is being prepared is the same UE association for which the first candidate cell was prepared, i.e., the UE is the same.

The source XnAP ID (Source NG-RAN node UE XnAP ID reference) is already included of the HANDOVER REQUEST, and the HANDOVER REQUEST ACKNOWLEDGE already contain a Source XnAP ID and a Target XnAP ID, so the only piece of information missing is the Target XnAP ID in the HANDOVER REQUEST (e.g., Target NG-RAN node UE XnAP ID). If the source gNB sends this information in the second, third, … HANDOVER REQUEST, the candidate gNB can understand that a UE association already exists for the UE and identify the UE. The source gNB needs to send the Target XnAP ID only if needed, i.e., the IE needs to be present only two conditions are satisfied: 1) there is an ongoing LTM preparation, and 2) a UE association was already established for the UE. One way to indicate the conditions above can be to introduce in the HANDOVER REQUEST two IEs:

- an IE similar to the LTM Indicator IE included in F1AP, with two codepoints, e.g., “first-candidate”, “other-candidate”,

- a Target NG-RAN node UE XnAP ID IE, present if the value of the LTM Indicator is “other-candidate”.

| >LTM Indicator | M |  | ENUMERATED (first-candidate, other-candidate, …) |  | - |  |
| --- | --- | --- | --- | --- | --- | --- |
| >Target NG-RAN node UE XnAP ID | C-ifOtherCand |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node. | - |  |

| Condition | Explanation |
| --- | --- |
| ifOtherCand | This IE shall be present if the LTM Indicator IE is present and set to "other-candidate". |

In our view the WA can be turned into agreement. However, we should avoid normative text that restricts the implementations by describing the sender behaviour. The wanted behaviour (one UE association for multiple LTM candidate cells prepared for the same UE in the same candidate gNB) can instead be achieved describing that the receiver (candidate gNB) can use an indication of the source gNB (as in the example, a specific value of the new LTM Indicator IE) to understand whether there is (or there is not) an existing UE association, pointing to a certain UE, for the LTM candidate cells being prepared.

- It is possible to describe that only one UE association is used for one UE when multiple LTM candidate cells are prepared in the same gNB without imposing restrictions on implementation at the sender.

- Add an LTM Indicator IE and a Target NG-RAN node UE XnAP ID IE to the HANDOVER REQUEST.

## Subsequent inter-CU LTM

In this section we provide some further discussion on support for subsequent inter-CU LTM.

At RAN3#126 meeting it was agreed that: “To support subsequent LTM, the LTM Configuration Update procedure is reused to establish UE association between the new source gNB and the other candidate gNB(s) after each inter-CU LTM Cell Switch.”.

At RAN3#127 meeting the following was further agreed:

- Reuse the LTM Configuration Update procedure to sync up configurations among candidate gNBs for subsequent LTM, including early sync configuration, configuration ID, and data forwarding addresses, etc.

- It is up to the network implementation when the sync up is performed: during the preparation step, or during the cell switch execution step, or after successful cell switch.

To capture the above, we propose to introduce in TS 38.300 a signaling sequence for subsequent LTM, as captured in our companion contribution in R3-250366.

- Introduce a signaling sequence in TS 38.300 for subsequent LTM operations.

Regarding the sync up among candidate, in the current XnAP baseline, it is FFS whether the encoding of the Data Forwarding Info IE in the LTM CONFIGURATION UPDATE is a single item or a list is needed:

| Data Forwarding Info | FFS if List is needed |  | 9.2.1.16 |  | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

During the first round of LTM preparation, in the HANDOVER REQUEST ACKNOWLEDGE message, the source gNB receives from each candidate gNB the respective data forwarding information (Data Forwarding Info from target NG-RAN node IE within the PDU Session Resources Admitted List IE). As agreed, one possibility is that the source gNB, to sync up the configurations among candidate gNBs, during the preparation phase sends an LTM CONFIGURATION UPDATE message to each candidate gNB. If there are multiple gNBs, the source gNB needs to send in each LTM CONFIGURATION UPDATE message the data forwarding information for more than one candidate gNBs. Because of that, we think a natural choice to resolve the FFS is that the source gNB sends a list of Data Forwarding Info. In particular, the source gNB sends in the LTM CONFIGURATION UPDATE message to a certain candidate gNB the list of the list of Data Forwarding Info received from all other candidate gNBs.

Another piece of information that one candidate gNB need to know to support subsequent LTM is the setting of Rel-19 Set ID of other candidate gNBs. We think that a simple solution is that the source gNB assigns the Rel-19 Set ID of the candidate gNBs (in addition to its own Rel-19 Set ID). To make sure that all the candidate gNBs are in sync, the source gNB sends the Rel-19 Set ID assigned to a certain candidate gNB in the HANDOVER REQUEST and in the LTM CONFIGURATION UPDATE sent to a certain candidate gNB it includes the list of the Rel-19 Set ID assigned to all other candidate gNBs. With that all the candidate gNBs are in sync.

- The source gNB sends the Rel-19 Set ID assigned to the candidate gNB in the HANDOVER REQUEST message.

- A list of Rel-19 Set IDs should be introduced to the LTM CONFIGURATION UPDATE message as well.

- Add in the LTM CONFIGURATION UPDATE message a list of Data Forwarding Info.

We want now to raise an issue that can occur, with impact on how to achieve a proper sync up of the configurations among the candidate gNBs for subsequent LTM. We think that a deployment scenario as in the figure below is possible, where there is NOT a full mesh Xn connectivity among all the gNBs involved in the LTM. The source gNB is gNB A and gNBs B, C, D, E are candidate gNBs for the first LTM Cell Switch. There is no Xn connectivity between gNB B and gNB E, nor between gNB D and gNB C. An obvious reason for the absence of the Xn connectivity can be that there is no coverage continuity between gNB B and gNB E (nor between gNB D and gNB C).

Figure 4. Subsequent LTM when no full mesh Xn connectivity is present among the gNBs.

If the UE moves from gNB A to gNB B, the gNB E will not be used for the very next LTM Cell Switch, i.e., the gNB B will never send an LTM Cell Switch command to move the UE from gNB B to gNB E, so, for example, what would be the purpose of sending the Data Forwarding Info of gNB E from gNB A to gNB B? Also, if gNB A sends to gNB B the Data Forwarding Info of gNB E, how can gNB B understand it?

As per RAN3#126 agreement, after an inter-CU LTM Cell Switch, the new source gNB can use the LTM Configuration Update to establish the UE association between itself and the other candidate gNBs. In the example, when the UE moves from gNB A to gNB B, the gNB B can establish a new UE association between itself and gNB C and gNB D (and keep the UE association towards gNB A). Even if the UE continues to keep the same LTM configuration provided by gNB A, at the network side, there is no need to keep the previously established UE associations between gNB A and gNB E, nor between gNB E and gNB C. In fact, keeping this UE association is a waste of resources in gNB E. We would like to discuss this issue, and hopefully find a solution for that.

- RAN3 to discuss how to sync up configuration among candidate gNBs in case of non-full Xn connectivity among the candidate gNBs.

## Inter-CU LTM with NR-DC

For inter-CU LTM with NR-DC, the following agreements and open points were captured at last meeting:

RAN3 agree the following scenarios to support LTM with NR-DC:

1.SN initiated inter-CU SCG LTM without MCG changes (high priority)

2.Inter-CU MCG LTM without SN release

3.Inter-CU MCG LTM with SN release

4.Inter-CU MCG LTM with SN addition

For SN initiated inter-CU SCG LTM, the source SN initiates the inter-CU SCG LTM preparation procedure by sending an SN Change Required message to the MN.

The MN requests each candidate SN to allocate resources for the UE via SN Addition request message.

Within the list of cells suggested by the source SN, the candidate SN provides the SCG part configuration of each candidate PSCell and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL sync configuration or TCI-state configuration, to the MN, via SN addition request ACK message.

In order to support subsequent inter-CU SCG LTM, the MN needs to transfer the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s), via SN modification request message. Accordingly, the candidate SN(s) responds with the updated candidate SCG configuration to the MN via SN modification request ACK message.

The Cell Switch Notification message can be reused from the source SN to the target SN via the MN. The detailed IE can be further discussed.

One of the aspects that remains to be discussed for Inter-CU LTM in NR-DC, relates to how to avoid that an inter-CU LTM configuration is present at the same time at MN and SN. In our understanding, the MN already indicates to the SN via Inter-Node RRC signaling, the amount of candidate cells the SN can prepare (in ConfigRestrictInfoSCG the maxNumberLTM-CandidatesSCG-r18 IE), and the running RRC CR already includes a configureInterSN-LTM-r19 IE. It is our understanding that if this information is sent from MN to SN, the SN is not allowed to configure inter-CU LTM, and in the running RRC CR it is FFS whether this indication is provided in Xn.

ConfigRestrictInfoSCG ::= SEQUENCE { (skip) [[ maxNumberLTM-CandidatesSCG-r18 INTEGER(0..maxNrofLTM-Configs-r18) OPTIONAL ]], (skip) [[ configureInterSN-LTM-r19 ENUMERATED{allowed} OPTIONAL ]] Editor’s Note: FFS whether the indication whether the SN is allowed to configure an inter-CU LTM configuration is provided in the Xn signalling.

In our view, given that some information related to the coordination between MN and SN for intra-CU SN LTM and inter-CU SN LTM is already included in Inter-Node RRC signaling (e.g., the number of candidate cells an SN can prepare), it is preferrable to use the same type of signaling also to convey other piece of information related the same coordination (such as: whether the SN can configure inter-CU LTM or not). So, unless the group identifies an obvious benefit to pass this information explicitly via Xn, we propose to wait for RAN2 discussion. If RAN2 decides that the indication is not to be passed via RRC, RAN3 can adopt a different approach.

- Leave to RAN2 the decision on an indication to the SN on whether SN is allowed to configure inter-CU LTM.

At RAN3#127, it was agreed to reuse the Cell Switch Notification message from source SN to target SN via MN. Here we further discuss the details left FFS.

The Cell Switch Notification message can be reused from the source SN to the target SN via the MN. The detailed IE can be further discussed.

Depending on the NR-DC scenario, the sending of security key material from MN to SN needs to be analysed. The agreed use cases:

For Inter-CU MCG LTM without SN change, it is not expected that the UE will apply a new sk-counter for the SN, and the SN is the same, so there is no need for the MN to send security key material to the SN.

For Inter-CU MCG LTM with SN release, no security key material needs to be sent to SN.

For Inter-CU MCG LTM with SN addition and for SN initiated inter-CU SCG LTM (without MCG change), the MN needs to send to the new SN the security configuration that SN can use (SN security key and the list of possible sk-counters the UE can apply).

- For SN initiated inter-CU SCG LTM, the MN sends to the target SN the SN security key and a list of sk-counters in the CELL SWITCH NOTIFICATION message.

- For Inter-CU MCG LTM with SN addition, the MN sends to the target SN the SN security key in the CELL SWITCH NOTIFICATION message.

At completion of the SN initiated inter-CU SCG LTM, the target SN (i.e. the new serving SN) informs the source SN (i.e., the old serving SN), via the MN, that UE has successfully accessed the target cell. We propose to reuse the HANDOVER SUCCESS for this purpose.

- At completion of SN initiated inter-CU SCG LTM, the target SN sends a HANDOVER SUCCESS to the MN, which forwards it to the source SN, to inform the source SN the UE successfully accessed the target cell.

- Agree on the TPs for 38.423, 38.473, 38.300, and 37.340 in R3-251964 that capture the above proposals.

# Conclusions and Proposals

In this contribution we made observations and proposals for different LTM related topics, as listed below.

Observation 1	It is possible to describe that only one UE association is used for one UE when multiple LTM candidate cells are prepared in the same gNB without imposing restrictions on implementation at the sender.

Proposal 1	Only the source gNB can provide a reference configuration.

Proposal 2	For inter-CU LTM, use a new identifier to support Early UL sync (e.g., an “Early RACH Resources Provider ID”).

Proposal 3	For inter-CU LTM, a candidate gNB informs the source gNB of the release of Preamble Indexes assigned for Early UL Sync.

Proposal 4	For intra-CU inter-DU LTM, a candidate DU informs the source DU (via the CU) of the release of Preamble Indexes assigned for Early UL Sync.

Proposal 5	The Xn procedure to transfer TA information is a class 2 procedure.

Proposal 6	The Xn procedure to transfer a TA information is UE-associated and is used to transfer a list of TA values, each TA value for a different LTM candidate cell.

Proposal 7	Include the UE XnAP IDs of the source and target gNB in the XnAP messages for TA information transfer.

Proposal 8	Include the UE F1AP IDs in the DU-CU/CU-DU TA Information Transfer messages.

Proposal 9	Add an LTM Indicator IE and a Target NG-RAN node UE XnAP ID IE to the HANDOVER REQUEST.

Proposal 10	Introduce a signaling sequence in TS 38.300 for subsequent LTM operations.

Proposal 11	The source gNB sends the Rel-19 Set ID assigned to the candidate gNB in the HANDOVER REQUEST message.

Proposal 12	A list of Rel-19 Set IDs should be introduced to the LTM CONFIGURATION UPDATE message as well.

Proposal 13	Add in the LTM CONFIGURATION UPDATE message a list of Data Forwarding Info.

Proposal 14	RAN3 to discuss how to sync up configuration among candidate gNBs in case of non-full Xn connectivity among the candidate gNBs.

Proposal 15	Leave to RAN2 the decision on an indication to the SN on whether SN is allowed to configure inter-CU LTM.

Proposal 16	For SN initiated inter-CU SCG LTM, the MN sends to the target SN the SN security key and a list of sk-counters in the CELL SWITCH NOTIFICATION message.

Proposal 17	For Inter-CU MCG LTM with SN addition, the MN sends to the target SN the SN security key in the CELL SWITCH NOTIFICATION message.

Proposal 18	At completion of SN initiated inter-CU SCG LTM, the target SN sends a HANDOVER SUCCESS to the MN, which forwards it to the source SN, to inform the source SN the UE successfully accessed the target cell.

Proposal 19	Agree on the TPs for 38.423, 38.473, 38.300, and 37.340 in R3-251964 that capture the above proposals.