3GPP TSG-RAN WG3 Meeting #127bis	R3-252012

Wuhan, China, 07 – 11 April 2025

Agenda item:	12.3

Source:	Nokia, BT

Title:	Completion of Functional Aspects of NR Femto

WID/SID:	FS_WAB_5GFemto_NR - Release 19

Document for:	Discussion and Decision

# 1	Introduction

At RAN3#125 the study item of NR Femto was successfully completed and the outcome is visible in TR 38.799 which was agreed in R3-244849.

At the following RAN Plenary#105, the corresponding work item was agreed in [1] including the following objective:

The objectives of the 5G Femto work are as follows:

-	Specification to support NR Femto architecture with optional NR Femto GW for NG interface [RAN3].

-	Specification to support access control for NR Femtos operating in open, hybrid and closed modes reusing existing CAG functionality [RAN3].

NOTE 10: For NR Femto access control, only stage 2 impact is expected on this objective.

NOTE 11: Coordination with other WGs (e.g. SA2, SA3) when needed.

At the last RAN3#126, baseline CRs was endorsed for TS 38.413 which is almost complete.

At the last RAN3#127, two text proposals for functional aspects of stage 2 were agreed and the baseline CR was endorsed for TS 38.300 which is almost complete as well.

This paper investigates remaining open points.

# 2	Discussion

- Completion of stage 2

The following conclusion was reached in the TR 38.799 for the NG interface:

### 6.2.1	Architecture for NG interface

Option 1, namely direct connection of an NR Femto node to the 5GC via the NG interface, is already possible.

In order to maintain the existing infrastructure for an operator who has deployed LTE HeNBs, Option 2 with an optional NR Femto GW is recommended for a normative phase.

At the last RAN3#126 meeting, the specific functional aspects related to NR Femtos were captured in the baseline CR for TS 38.300 along these lines.

At the last RAN3#127 meeting, the specific functional aspects related to NR Femto Gateway and to AMF were captured in the baseline CR for TS 38.300 along these lines.

The two TPs agreed at RAN3#127 do not consider the security aspects.

Therefore, at RAN3#127 an LS was also sent to SA3 in tdoc [2] to check the security aspects as follows:

RAN3 would like SA3 feedback on whether the following verifications need to be specified for the NR Femto GW (and AMF, respectively), as outlined below:

For the NR Femto Gateway:

1:	In case of NGAP INITIAL UE MESSAGE message, whether to verify that reported CAG ID is valid for that NR Femto.

2:	In case of NGAP SETUP REQUEST message, whether to verify that the identity used by the NR Femto is valid.

3:	In case of NGAP PWS RESTART INDICATION message and NGAP PWS FAILURE INDICATION message, whether to verify that the indicated cell identity is valid.

For the AMF:

1:	In case of an NR Femto directly connected, whether to verify that the CAG ID reported in the NGAP INITIAL UE MESSAGE message is valid for the indicated cell.

2:	In case of an NR Femto directly connected, whether to verify that the identity used by the NR Femto is valid when receiving the NGAP SETUP REQUEST message.

3:	In case of an NR Femto directly connected, whether to verify that the indicated gNB identity of the NR Femto is valid when receiving the NGAP PWS RESTART INDICATION message and the NGAP PWS FAILURE INDICATION message.

SA3 has well progressed their normative work which is captured in TS 33.545. It is likely that verification actions similar to HeNB GW functions specified in TS 36.300 would be needed, but RAN3 did not yet get the reply from SA3.

Proposal 1: wait for SA3 reply to complete the stage 2 baseline TS 38.300 CR.

- Remaining open points

At RAN3#127 RAN3 started to discuss the access control in AMF in case of shared cell between PNI NPN and PLMN and whether current specification in SA2 TS 23.401 was good enough.

Simultaneously, SA2 also sent RAN3 an LS on this topic in tdoc [3] to verify that AMF access control checks specified in TS 23.501 are aligned with RAN3.

Network sharing concepts are defined in TS 23.501 in clause 5.18.1

5G MOCN also supports the following sharing scenarios involving non-public networks, i.e.NG-RAN can be shared by any combination of PLMNs, PNI-NPNs (with CAG) and SNPNs (each identified by PLMN ID and NID).

From the above statement it is clear that RAN sharing is possible with one logical cell of PLMN and one logical cell of PNI NPN. Further text says:

In all non-public network sharing scenarios, each Cell Identity as specified in TS 38.331 [28] is associated with one of the following configuration options:

-	one or multiple SNPNs;

-	one or multiple PNI-NPNs (with CAG); or

-	one or multiple PLMNs only.

From the above statement, it is clear that one logical cell associated with a cell identity belongs to only one type i.e. either PLMN or PNI NPN.

However, this doesn’t prevent from a physical cell to be shared with one logical cell of PLMN and one logical cell of PNI NPN as explained below:

NOTE 2:	This allows the assignment of multiple cell identities to a cell and also allows the cell identities to be independently assigned, i.e. without need for coordination, by the network sharing partners, between PLMNs and/or non-public networks.

Given that there is one cell identity per logical cell, the second “cell” in above sentence can only refer to physical cell.

The same is also well specified in TS 38.300 clause 4.6:

If NR access is shared, system information broadcast in a shared cell indicates a TAC and a Cell Identity for each subset of PLMNs, PNI-NPNs and SNPNs

This means that the shared physical cell can support a logical cell for PLMN and a logical cell for PNI NPN and these two logical cells can have different cell identities.

Observation 1: TS 23.501 and TS 38.300 are well aligned which foresee the sharing of one physical cell by multiple logical cells, one can be PLMN one can be PNI NPN.

Any UE connecting to gNB will indicate in RRC Setup Complete message the logical cell identity with which it connects. From this the gNB can determine if UE accesses via the PLMN logical cell or via the CAG logical cell. Only in the second case, the gNB will include the list of CAG IDs supported by this logical cell towards the AMF in the NGAP Initial UE Message message.

From the above the AMF can apply the relevant access control check:

- if the logical cell accessed is CAG, then AMF checks if the allowed CAG list of the UE matches,

- if the logical cell accessed is PLMN, then AMF just needs to check if the UE is CAG-only or not.

This is as specified in TS 23.501 clause 5.30.3.4:

-	If the UE is accessing the 5GS via a CAG cell and if at least one of the CAG Identifier(s) received from the NG-RAN is part of the UE's Allowed CAG list (for entries with time validity information if any, the evaluation of the condition is true), then the AMF accepts the NAS request;

-	If the UE is accessing the 5GS via a CAG cell and if none of the CAG Identifier(s) received from the NG-RAN are part of the UE's Allowed CAG list (for entries with time validity information if any, the evaluation of the condition is true), then the AMF rejects the NAS request and the AMF should include CAG information in the NAS reject message. The AMF shall then release the NAS signalling connection for the UE by triggering the AN release procedure; and

-	If the UE is accessing the 5GS via a non-CAG cell and the UE's subscription contains an indication that the UE is only allowed to access CAG cells, then the AMF rejects the NAS request and the AMF should include CAG information in the NAS reject message. The AMF shall then release the NAS signalling connection for the UE by triggering the AN release procedure.

Observation 2: AMF has all the information to perform the correct access control when a UE connects via a cell shared between PLMN and PNI NPN.

We conclude that both TS 38.300 and TS 23.501 are well aligned for the case of cells shared PLMN and PNI NPN. We also observe that this function was discussed and is supported by RAN3 since the beginning of release 16.

Proposal 2: agree the reply LS in [4] to explain to SA2 the sharing feature specified by RAN3 since release 16 and that both TS 38.300 and TS 23.501 are well aligned.

# 3	Conclusion and Proposals

This paper has investigated remaining open points and incoming LSs and proposes:

To progress the stage 2 and stage 3:

Proposal 1: wait for SA3 reply to complete the stage 2 baseline CR.

To address remaining open points:

Observation 1: TS 23.501 and 38.300 are well aligned which foresee the sharing of one physical cell by multiple logical cells, one logical cell can be PLMN cell and one can be PNI NPN cell.

Observation 2: AMF has all the information to perform the correct access control when a UE connects via a cell shared between PLMN and PNI NPN.

Proposal 2: agree the reply LS in [4] to explain to SA2 the sharing feature specified by RAN3 since release 16 and that both TS 38.300 and TS 23.501 are well aligned.

# 4	References

- RP-242395, Additional topological enhancements for NR, NTT Docomo, AT&T

- R3-250822, LS on Security Verification related to NR Femtos

- R3-251524, LS on NR Femto Node shared by PLMN and PNI NPN (S2-2502787)

- R3-252013, Reply LS on NR Femto Noe shared by PLMN and PNI NPN