3GPP TSG-RAN WG3 Meeting #127bis	R3-252014

Wuhan, China, 07 – 11 April 2025

Agenda item:	9.2

Source:	Nokia, CATT, Qualcomm Incorporated, Huawei, Ericsson, Orange, ZTE

Title:	Correction of MBS Distribution Setup

WID/SID:	NR_MBS-Core

Document for:	Discussion and Decision

# Discussion

TS 23.247 specifies in clause 7.2.4.2.1 that during shared delivery setup the NG-RAN node may provide the list of tracking areas supported by the gNB:

If the RAN determines the shared delivery is not established for the MBS Session ID and Area Session ID, the NG-RAN initiates the shared delivery establishment as specified in clause 7.2.1.4 and provides the Area Session ID, tracking area ID(s) concerning the RAN node in addition to MBS Session ID in the request for shared delivery establishment. The MB-SMF provides MBS service area information (Area Session ID(s), MBS service area(s)) associated with the same MBS session to NG-RAN in the shared delivery establishment response. MB-SMF may either provide only the service areas concerning the target RAN node or all service areas associated with the MBS session. If the MB-SMF only provides the service areas concerning the target RAN node, which includes the cell ID(s), the cell ID is included if the tracking area IDs associated with the indicated cell ID is received by the MB-SMF.

As specified above, SA2 agreed this to enable MB-SMF to avoid sending the full service area but instead may limit to the service area related to the gNB which triggers the distribution setup.

However, looking at MBS Distribution Setup procedure in TS 38.413, NGAP is not aligned with this. CRs are provided in [1] and [2] to align TS 38.413 with TS 23.247.

# Conclusion and Proposals

A discrepancy is found between TS 23.247 and TS 38.413 for the sending of MBS TAIs from gNB to MB-SMF during the MBS Distribution Setup procedure.

Proposal 1: agree the CRs in [1] and [2] to add the TAI MBS Support List IE in the MBS Distribution Setup Request Transfer IE container to align with TS 23.247.

# References

- R3-252016, Correction of MBS Distribution Setup, CR TS 38.413 R17, Nokia, CATT, Qualcomm Incorporated, Huawei, Ericsson, Orange, ZTE

- R3-252017, Correction of MBS Distribution Setup, CR TS 38.413 R17, Nokia, CATT, Qualcomm Incorporated, Huawei, Ericsson, Orange, ZTE