3GPP TSG-RAN WG3 Meeting #127 Bis	             R3-252037

Wuhan, China, 7 – 11 April 2025

Agenda Item:	21.3

Source:	CMCC

Title:	Support of Uplink Congestion Signalling and PDU Set Based QoS Handling Enhancement

Document for:	Discussions & Approval

# Introduction

The following revised WID on XR (eXtended Reality) for NR Phase 3 [1] was approved at RAN Plenary meeting #107:

The Rel-19 XR Phase 3 objectives are as follows: - Support and specify multi-modality awareness for QoS flows in both DL and UL RAN [RAN3]: - Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4] - Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4] - Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]: - Specify additional Logical Channel priority handling using delay/deadline information of packets; - Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG. - Specify the following user plane enhancements [RAN2]: - RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget. - Extend Release 18 standalone mechanism to support NR-NR dual connectivity as follows [RAN3]: - PDU set based handling; - ECN marking; - Burst Arrival Time reporting, if needed; - PSI Discard coordination, if needed; NOTE: No RAN2 impact from above items. - Specify uplink congestion signalling [RAN2, RAN3]: - Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB and the associated F1 signalling, if any, to enable faster source rate adaption to uplink congestion. - Support of PDU set based QoS handling enhancement [RAN3]: - Support of DL PDU Set marking without PDU Set QoS - Support of Alternative PDU Set QoS, which may contain UL and/or DL PDU Set QoS Parameters (i.e. UL PSDB, DL PSDB, UL PSER and/or DL PSER). - QoS Handling when Traffic Characteristics Change Dynamically [RAN3] - TTNB and burst size to be provided over GTP-U - Support of exposure of available data rate [RAN3] Note: Coordination with SA2, as needed - Specify Core requirements related to the above objectives as necessary [RAN4].

In this document, we focus on the following topics:

Support and specify multi-modality awareness for QoS flows in both DL and UL RAN [RAN3]:

-	Specify uplink congestion signalling [RAN2, RAN3]:

-	Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB and the associated F1 signalling, if any, to enable faster source rate adaption to uplink congestion.

-	Support of PDU set based QoS handling enhancement [RAN3]:

-	Support of DL PDU Set marking without PDU Set QoS

-	Support of Alternative PDU Set QoS, which may contain UL and/or DL PDU Set QoS Parameters (i.e. UL PSDB, DL PSDB, UL PSER and/or DL PSER).

# Discussions

## MMSID

The QoS flows that are associated with a multi-modal service are identified by a MMSID. An MMSID IE needs to be included in the QoS Flow Level QoS Parameters.

From RAN3’s perspective, the addition of MMSID IE to QoS Flow Level QoS Parameters has impact on XnAP, F1AP, E1AP, and NGAP.

## Support of Uplink Congestion Signalling

In the current 3GPP architecture, the gNB-DU operates at the DRB level and the gNB-CU is responsible for mapping QoS flows to DRBs and providing the gNB-DU with DRB-level QoS parameters. However, there are scenarios where it may be beneficial for the gNB-DU to have more granular control over QoS flows, such as deriving recommended bit rates for individual QoS flows based on available resources and QoS requirements.

The F1AP protocol already supports the transfer of QoS flow-level QoS parameters (e.g., 5QI, GFBR, MFBR) from the gNB-CU to the gNB-DU during the UE Context Setup or UE Context Modification procedures. This information could potentially be leveraged to enable the gNB-DU to make more informed decisions about resource allocation and scheduling.

The gNB-CU can configure the gNB-DU to enforce uplink bit rate limits at the DRB level. If a DRB carries only one QoS flow, this effectively controls the uplink bit rate at the QoS flow level.

In CU-DU split architecture, the gNB-CU can configure the gNB-DU to enforce uplink bit rate control at the DRB level.

The gNB-DU sends ﻿Recommended bit rate MAC CE to UE for the DRB.

According to TS 38.323 clause 5.8 [2], it states that the SDAP header will not be ciphered. Thus, the gNB-DU is able to utilize the QFI carried in SDAP header to differentiate different QoS flows mapped to the same DRB.

The ciphering function includes both ciphering and deciphering and is performed in PDCP, if configured. The data unit that is ciphered is the MAC-I (see clause 6.3.4) and the data part of the PDCP Data PDU (see clause 6.3.3) except the SDAP header and the SDAP Control PDU if included in the PDCP SDU. The ciphering is not applicable to PDCP Control PDUs.

The gNB-DU has the information of the QoS flows that are mapped to a specific DRB, thus the gNB-DU is capable of reconfiguring the uplink data rate for each QoS flow mapped to the DRB based on the available uplink resource, congestion status of the DRB, as well as the QoS requirements contained in the ﻿QoS Flow Level QoS Parameters. If the gNB-DU perceives/estimates the current data rate of each QoS flow by leveraging the QFI contained in the SDAP header, the uplink data rate reconfiguration by the gNB-DU would be feasible at the QoS flow level.

The QFI carried in the SDAP header can be considered visible to the gNB-DU, which can be utilized by the gNB-DU for uplink data rate reconfiguration at the QoS flow level.

With the above proposal being made, if we stick to the functional and protocol stack split of gNB-CU and gNB-DU, QFI is likely not be leveraged by the gNB-DU. In the following, we discuss the solutions to provide QoS flow level in CU-DU split architecture if we assume the gNB-DU would not utilize the QFI contained in the SDAP header.

Solution I:

Since the PDCP layer is handled by the gNB-CU in the 3GPP functional split, the gNB-CU can enforce QoS flow-level bit rate control directly. The gNB-CU can request the gNB-DU to provide updates on:

- The available uplink resources for a specific DRB.

- The congestion status of the DRB (e.g., whether the DRB is experiencing high load or latency).

This feedback can be sent from the gNB-DU to the gNB-CU over the F1. The gNB-CU can use to reconfigure the uplink data rate for each QoS flow mapped to the DRB. The gNB-CU could use RRC to notify the UE regarding the uplink rate control for the QoS flows.

The gNB-DU sends ﻿available uplink resource, congestion status of a specific DRB to the gNB-CU, the gNB-CU reconfigures the uplink data rate for each QoS flow based on the information.  ﻿

The gNB-CU could send the recommended uplink data rate for a QoS Flow by RRC to the UE.

Solution II:

The gNB-DU sends the Recommended bit rate MAC CE to UE for the DRB. The gNB-CU notifies the UE which QoS flows are subject to uplink rate control by RRC, how to update uplink rate for those QoS flows is up to the UE.

The gNB-CU informs the UE via RRC which QoS flows are subject to uplink rate control. The UE decides how to adjust the uplink rate for the affected QoS flows.

## Support of PDU Set Based QoS Handling Enhancement

SA2 has agreed the following updates regarding PDU Set based QoS handling enhancement [3][4] [5].

- In the case no DL PDU Set QoS parameters provided, when DL PDU Set based handling without PDU Set QoS parameters is allowed(i.e. SMF is configured to allow), if PSA UPF supports of PDU Set Information marking, the SMF sends NG-RAN a indication that the DL PDU Set Information Marking is supported. - If NG-RAN got the DL PDU Set Information Marking support indication from SMF, NG-RAN includes the PDU Set Based Handling Support Indication in N2 SM information.

In R18, NG-RAN nodes that support PDU Set-based handling are capable of sending the PDU Set Based Handling Indicator to the SMF when PDU Set QoS parameters are received. This indicator enables the SMF to instruct the UPF to apply PDU Set Information Marking in the GTP-U header. According to SA2, in cases where no PDU Set QoS parameters are available, the SMF will send a specific indicator named DL PDU Set Information Marking Support Indication to NG-RAN to signal that the CN supports DL PDU Set Information Marking. The PDU Set Based Handling Indicator can be reused by the NG-RAN to respond that the PDU Set Information Marking is needed at UPF.  This does not require changes to RAN-to-CN messages from the ASN.1 perspective, and only minor updates to the procedural text are needed.

The existing PDU Set Based Handling Indicator can be reused to respond DL PDU Set Information Marking Support Indication in Rel-19. There is impact on NGAP and XnAP to support DL PDU Set Information Marking Support Indication.

## Support of Alternative PDU Set QoS

SA2 has agreed the following updates regarding support of alternative PDU Set QoS [6]:

#### 5.7.1.2a Alternative QoS Profile The Alternative QoS Profile(s) can be optionally provided for a GBR QoS Flow with Notification control enabled. If the corresponding PCC rule contains the related information (as described in TS 23.503 [45]), the SMF shall provide, in addition to the QoS profile, a prioritized list of Alternative QoS Profile(s) to the NG-RAN. If the SMF provides a new prioritized list of Alternative QoS Profile(s) to the NG-RAN (if the corresponding PCC rule information changes), the NG-RAN shall replace any previously stored list with it. An Alternative QoS Profile represents a combination of QoS parameters PDB, PER, Averaging Window and GFBR to which the application traffic is able to adapt. The Alternative QoS Profile(s) shall also contain the PDU Set QoS parameters PSDB and PSER in the respective direction(s) when the QoS Profile contains PDU Set QoS parameters PSDB and PSER in UL and/or DL as described in clause 5.7.7; otherwise, the corresponding Alternative QoS Profile(s) shall not contain the PDU Set QoS parameters PSDB and PSER. NOTE 1: There is no requirement that the GFBR monotonically decreases, nor that the PDB or PER (or PSDB or PSER) monotonically increase as the Alternative QoS Profiles become less preferred.

If a QoS Profile includes PDU Set QoS parameters (PSDB - PDU Set Delay Budget and PSER - PDU Set Error Rate) for either uplink (UL) or downlink (DL), then the corresponding Alternative QoS Profile(s) must also include PSDB and PSER in the respective directions.

Alternative QoS Profiles shall include PDU Set QoS parameters (PSDB, PSER) on the same direction(s), if QoS Profile includes PDU Set QoS parameters (PSDB, PSER) for either uplink or downlink or both, which has impact on NGAP, XnAP, F1AP, E1AP.

# Conclusions

In this submission, we discussed the support of multi-modality awareness for QoS flows in both DL and UL RAN, support of uplink rate control at DRB and QoS flow granularities, support of support of DL PDU Set marking without PDU Set QoS, and support of Alternative PDU Set QoS, which may contain UL and/or DL PDU Set QoS Parameters (i.e. UL PSDB, DL PSDB, UL PSER and/or DL PSER).. The following proposals are presented in the paper:

- From RAN3’s perspective, the addition of MMSID IE to QoS Flow Level QoS Parameters has impact on XnAP, F1AP, E1AP, and NGAP.

- In CU-DU split architecture, the gNB-CU can configure the gNB-DU to enforce uplink bit rate control at the DRB level.

- The gNB-DU sends ﻿Recommended bit rate MAC CE to UE for the DRB.

- The QFI carried in the SDAP header can be considered visible to the gNB-DU, which can be utilized by the gNB-DU for uplink data rate reconfiguration at the QoS flow level.

- The gNB-DU sends ﻿available uplink resource, congestion status of a specific DRB to the gNB-CU, the gNB-CU reconfigures the uplink data rate for each QoS flow based on the information.  ﻿

- The gNB-CU could send the recommended uplink data rate for a QoS Flow by RRC to the UE.

- The gNB-CU informs the UE via RRC which QoS flows are subject to uplink rate control. The UE decides how to adjust the uplink rate for the affected QoS flows.

- The existing PDU Set Based Handling Indicator can be reused to respond DL PDU Set Information Marking Support Indication in Rel-19. There is impact on NGAP and XnAP to support DL PDU Set Information Marking Support Indication.

- Alternative QoS Profiles shall include PDU Set QoS parameters (PSDB, PSER) on the same direction(s), if QoS Profile includes PDU Set QoS parameters (PSDB, PSER) for either uplink or downlink or both, which has impact on NGAP, XnAP, F1AP, E1AP.

# References

- RP-240107, Revised WID on XR (eXtended Reality) for NR Phase 3

- Packet Data Convergence Protocol (PDCP) specification, (3GPP TS 38.323 version 18.4.0 Release 18)

- S2-2502672(pre2502467_02282_02208_01423_2401423_11439_09898)_XRM r19_KI#1 501 PS Info Marking

- S2-2502468(pre2502281_2501425_S2-2411441_09875)_XRM r19_KI#1 503 PS Info Marking

- S2-2502673(was2466)(was2280)(was2209)(was1794)_23.502_XRMph2-PDUSethandlingwithoutQoSparameter

- S2-2502678 was2476 was2220 was1911 was1102 was 00706r10 TS23.501 PDU Set AQP and NC﻿