3GPP TSG-RAN WG3 Meeting #127 Bis	             R3-252042

Wuhan, China, 7 – 11 April 2025

Agenda Item:	17.3

Source:	CMCC

Title:	(TPs to BL CRs) Support On-Demand SIB1 for UEs

Document for:	Discussions & Approval

# Introduction

In the RAN3 # 125 Bis meeting, the followings were agreed [1]:

- The UL WUS configuration will be transferred over Xn via a new class1 defined procedure. - Cell A gNB can decide and signal to NES Cell gNB that it stops the UL WUS Configuration Provision in its SIB.

In the RAN3 # 126 meeting, the followings were agreed [2]:

The purpose of the new XnAP procedure is to enable an NG-RAN node1 to provide UL WUS configuration information to NG-RAN node2. The procedure uses non UE-associated signaling. Cell A stores the UL WUS configuration information after it has received it. A Class 2 procedure is to be introduced for Cell A gNB to signal to NES Cell gNB that it stops the UL WUS Configuration Provision in its SIB. The procedure uses non UE-associated signalling. Reuse the existing IE “Served Cell Information” IE to include UL WUS configuration from gNB-DU to gNB -CU in F1 Setup Request and gNB-DU Configuration Update procedures.

In the RAN3 # 127 meeting, the followings were agreed [3]:

After Cell A gNB has received and accepted the UL WUS Configuration request from the NES Cell gNB, it broadcasts in SIBx at the first opportunity. From the signalling point of view, in UL WUS Configuration Provision Request (naming to be further discussed) from NES Cell gNB to Cell A gNB, one option is “start with UL WUS Configuration”. This codepoint/choice is used to trigger Cell A to broadcast UL WUS configuration in SIBx. NES Cell gNB can “stop” (naming to be further discussed) Cell A to broadcast UL WUS configuration.

# Discussion on OD-SIB1 Mode

## UL WUS Configuration Provision Request

UL WUS Configuration Provision Request from the NES Cell indicates the needs from the NES Cell to request the Cell A to broadcast the UL WUS configuration on the NES Cell’s behalf. The UL WUS Configuration Provision Request is sent from the NES Cell gNB to the Cell A gNB, which could optionally include the number of NES Cell, NES Cell List, corresponding UL WUS configuration for each NES Cell, the action associated with the NES cell, i.e., Start (UL WUS Configuration Broadcast), Stop (UL WUS Configuration Broadcast), Cancel (UL WUS Configuration Broadcast).

- Start: it means the NES Cell gNB wants the Cell A gNB to broadcast the UL WUS configuration for a list of NES Cells. Each NES Cell might be associated with a UL WUS configuration, time of period that the UL WUS Configuration Provision is requested by the NES Cell.

- Stop: it means that the NES Cell gNB wants the Cell A gNB to temporally stop the UL WUS Configuration Provision for a list of NES Cells.

- Cancel: it means that the NES Cell gNB wants the Cell A gNB to completely cancel the UL WUS Configuration Provision for a list of NES Cells. The cancellation will remove the configuration and state that was maintained by the Cell A gNB for the list of NES Cells.

Remove FFS regarding the naming of UL WUS Configuration Provision Request.

The UL WUS Configuration Provision Request includes three codepoints: start, stop, cancel.

The UL WUS Configuration Provision Request includes the requested NES Cell List, each item contains the NR CGI of the cell, corresponding UL WUS configuration.

For the request that contains the action of “Start UL WUS Configuration Provision”, the Cell A gNB might return a “Success” or “Failure” response:

- The “Success” response could simply indicate that the Cell A accepts the request, and will broadcast the UL WUS Configuration for the NES Cell at its first opportunity as agreed in the last RAN3 127 meeting.

- The “Failure” response tells the NES Cell the requested Cell A refuses to broadcast the UL WUS configuration for the NES Cell at this moment. It might be associated with a suggested time that the NES Cell could request again.

The response to UL WUS Configuration Provision Request could be a “Confirm”, which include a list of cells whose UL WUS Configuration Provision Request is accepted.

The Cell A gNB responds NES gNB with a failure message if it cannot accept UL WUS Configuration Provision Request for any NES cell in the requested list.

“Stop UL WUS Configuration Provision” message from the NES Cell requests the Cell A to pause the UL WUS Configuration Provision for the list of requested NES Cells.The Cell A needs to maintain the configuration and status for the NES Cells.

“Cancel UL WUS Configuration Provision” message from the NES Cell requests the Cell A to completely cancel the UL WUS Configuration Provision for the list of requested NES Cells. The Cell A removes the configuration and status for the NES Cells.

As agreed: A Class 2 procedure is to be introduced for Cell A gNB to signal to NES Cell gNB that it stops the UL WUS Configuration Provision in its SIB. We could name the class 2 procedure as UL WUS Configuration Provision Status Update. Such update could include two types of lists of cells:

- Stop: which indicates the UL WUS Configuration Provision is temporally stopped.

- Ready: which indicates the UL WUS Configuration Provision is ready to be resumed for the NES Cell.

Naming the class 2 procedure as UL WUS Configuration Provision Status Update.

UL WUS Configuration Provision Status Update could contain two types of lists of cells associated with a previously accepted UL WUS Configuration Provision Request: stop, ready.

“List of Cells to Stop UL WUS Configuration Provision” indicates that the Cell A needs to stop UL WUS Configuration Provision for a list of NES Cells.

“List of Cells Ready to Resume UL WUS Configuration Provision” indicates that the Cell A is ready to resume the UL WUS Configuration Provision for a list of NES Cells.

## Split Architecture

If the CU/DU split architecture is adopted in gNB, a NES Cell and Cell A could be in different DU(s). In order to transit the UL WUS Configuration, both F1 interface and Xn interface could be impacted.

- The NES Cell and Cell A are on the same gNB, but on different DU(s). gNB-CU is in the middle to forward the UL WUS configuration between two gNB-DUs on the same gNB.

- The NES Cell and Cell A are on different gNB(s). Two gNB-CU(s) are in the middle to forward the UL WUS Configuration between two gNB-DUs on different gNB(s).

The procedures and messages that are discussed above are also applicable to F1 interface.

### F1AP Signalling in NES Cell gNB

The CU manages signalling between the network and the UE, including connection setup, release, mobility management, and security functions. And it has more global picture of the neighbouring cell deployment, inference level, signal strength and quality, energy consumption, power status, etc.  The CU can be helpful by providing information before the DU initiates the on-demand SIB1 operation for some NES cells. For instance, the CU could supply a list of cells where the on-demand SIB1 operation should be enabled. However, the NES Cell DU itself should be the one to decide whether and when to start or stop the SIB1 broadcast.

The NES Cell gNB-CU could provide some assistance information (a list of cells where the on-demand SIB1 operation should be enabled, and a list of corresponding cells could be requested as Cell A) before a NES Cell gNB-DU initiates the OD SIB1 operation, which can reuse the gNB-CU Configuration Update procedure over F1.

The NES Cell gNB-DU itself decides the OD-SIB1 status switch.

After NES Cell gNB-DU decides to enter OD-SIB1 mode for a list of NES cells, it generates the UL WUS configuration and sends the OD-SIB1 mode request to its gNB-CU. The gNB-DU Configuration Update procedure over F1 can be reused.

Remove the FFS on the UL WUS configuration in the F1AP BLCR.

The gNB-DU Configuration Update procedure over F1 can be reused for the gNB-DU to send the on-demand SIB1 status switch request to the gNB-DU.

The NES Cell gNB-CU selects an appropriate Cell A gNB-CU to send the UL WUS Configuration Provision Request over Xn interface as discussed in the previous section. After the NES Cell gNB-CU receives a response from the Cell A gNB-CU, which could be a “Confirm” as acceptance, or a “Reject” as failure.

The gNB-DU Configuration Update Acknowledge procedure over F1 can be reused for the gNB-CU to send the on-demand SIB1 status switch response for each requested cell to the gNB-DU.

If the Cell A gNB-CU notifies the NES Cell gNB-CU that it stops the UL WUS Configuration Provision for a list of NES Cells, the NES Cell gNB-CU needs to inform the NES Cell gNB-DU .

If the Cell A gNB-CU notifies the NES Cell gNB-CU that it is ready to resume the UL WUS Configuration Provision for a list of NES Cells, the NES Cell gNB-CU can take this into consideration, and inform the gNB-DU by List of Cells to be OD-SIB1.

The gNB-CU Configuration Update procedure over F1 can be reused for a gNB-CU to send the Stopped OD-SIB1 List to the gNB-DU.

### F1AP Signalling in Cell A gNB

The Cell A gNB-CU receives the UL WUS Configuration Provision Request from NES gNB-CU. The UL WUS configuration for the requesting NES cells is encoded by Cell A gNB-CU into SIB-x and is sent to the Cell A DU over the F1 interface. In case of an update or termination of the UL WUS broadcast, the Cell A CU modifies or removes the relevant SIB information and sends the updated system information to the Cell A DU, which then handles the broadcasting to UEs.

At Cell A, the UL WUS configuration can be encoded into a new SIB by the gNB-CU, which is included in gNB-CU System Information and sent to the gNB-DU.

# Conclusions

In this submission, we discussed the procedures of the OD-SIB1. The following proposals are presented in the paper:

- Remove FFS regarding the naming of UL WUS Configuration Provision Request.

- The UL WUS Configuration Provision Request includes three codepoints: start, stop, cancel.

The UL WUS Configuration Provision Request includes the requested NES Cell List, each item contains the NR CGI of the cell, corresponding UL WUS configuration.

The response to UL WUS Configuration Provision Request could be a “Confirm”, which include a list of cells whose UL WUS Configuration Provision Request is accepted.

The Cell A gNB responds NES gNB with a failure message if it cannot accept UL WUS Configuration Provision Request for any NES cell in the requested list.

“Stop UL WUS Configuration Provision” message from the NES Cell requests the Cell A to pause the UL WUS Configuration Provision for the list of requested NES Cells.The Cell A needs to maintain the configuration and status for the NES Cells.

“Cancel UL WUS Configuration Provision” message from the NES Cell requests the Cell A to completely cancel the UL WUS Configuration Provision for the list of requested NES Cells. The Cell A removes the configuration and status for the NES Cells.

Naming the class 2 procedure as UL WUS Configuration Provision Status Update.

UL WUS Configuration Provision Status Update could contain two types of lists of cells associated with a previously accepted UL WUS Configuration Provision Request: stop, ready.

“List of Cells to Stop UL WUS Configuration Provision” indicates that the Cell A needs to stop UL WUS Configuration Provision for a list of NES Cells.

“List of Cells Ready to Resume UL WUS Configuration Provision” indicates that the Cell A is ready to resume the UL WUS Configuration Provision for a list of NES Cells.

The NES Cell gNB-CU could provide some assistance information (a list of cells where the on-demand SIB1 operation should be enabled, and a list of corresponding cells could be requested as Cell A) before a NES Cell gNB-DU initiates the OD SIB1 operation, which can reuse the gNB-CU Configuration Update procedure over F1.

The NES Cell gNB-DU itself decides the OD-SIB1 status switch.

Remove the FFS on the UL WUS configuration in the F1AP BLCR.

The gNB-DU Configuration Update procedure over F1 can be reused for the gNB-DU to send the on-demand SIB1 status switch request to the gNB-DU.

The gNB-DU Configuration Update Acknowledge procedure over F1 can be reused for the gNB-CU to send the on-demand SIB1 status switch response for each requested cell to the gNB-DU.

The gNB-CU Configuration Update procedure over F1 can be reused for a gNB-CU to send the Stopped OD-SIB1 List to the gNB-DU.

At Cell A, the UL WUS configuration can be encoded into a new SIB by the gNB-CU, which is included in gNB-CU System Information and sent to the gNB-DU.

# Reference

- draft_MeetingReport_RAN3#125-bis_vOct25

- draft_MeetingReport_RAN3#126_vDec05

- General_information_on_3GPP_RAN3#127-bisMeetings

# TP for TS 38.423

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<start of change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

#### 9.1.3.x1	 UL WUS CONFIGURATION PROVISION REQUEST

This message is sent by the NG-RAN node1 to the peer NG-RAN node2 to request the provision of the UL WUS configuration.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| List of Cells in UL WUS Config Provision Request |  | 1 |  | Cell list to which the request applies. | YES | ignore |
| >List of Cells in UL WUS Config Provision Request Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>CHOICE UL WUS Config Provision Action | M |  |  |  |  |  |
| >>>UL WUS Config Broadcast Start |  |  |  |  |  |  |
| >>>>UL WUS Config | O |  | OCTET STRING | If UL WUS Configuration is included, it means the cell requests the broadcast of this specified UL WUS Configuration. | – |  |
| >>>UL WUS Config Broadcast Stop |  |  |  |  |  |  |
| >>>UL WUS Config Broadcast Cancel |  |  |  |  |  |  |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by an NG-RAN node. Value is 16384. |

#### 9.1.3.x2 	UL WUS CONFIGURATION PROVISION RESPONSE

This message is sent by an NG-RAN node2 to a peer NG-RAN node1 to indicate that one or more cell(s) has (have) been successfully provisioned with the UL WUS configuration.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| List of Cells in UL WUS Config Provision Request Accepted |  | 0..1 |  | List of cells that is being accepted for UL WUS Config Provision Request. | YES | ignore |
| > List of Cells in UL WUS Config Provision Request Accepted Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR Cell Identity. | – |  |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by an NG-RAN node. Value is 16384. |

#### 9.1.3.x3 UL WUS CONFIGURATION PROVISION FAILURE

This message is sent by an NG-RAN node2 to a peer NG-RAN node1 to indicate UL WUS configuration provision failure.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Provision ID | M |  | INTEGER (0..255) | Allocated by the NG-RAN node1 | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

//omitted text unchanged//

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<next change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

#### 9.1.3.y1	 UL WUS CONFIGURATION PROVISION Status Update

This message is sent by the NG-RAN node1 to the peer NG-RAN node2 to update the status of the UL WUS configuration provision.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Provision ID | M |  | INTEGER (0..255) |  | YES | reject |
| List of Cells To Stop UL WUS Provision | O | 1 |  | Cell ID list to which the indication applies. | YES | ignore |
| > List of Cells To Stop UL WUS Provision Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID |  |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| List of Cells Ready to Resume UL WUS Config provision | O | 1 |  | Cell ID list to which the indication applies. | YES | ignore |
| > List of Cells Ready to Resume UL WUS Provision Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID |  |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by an NG-RAN node. Value is 16384. |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<end of change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

# TP for TS 38.473

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<start of change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 8.2.3	F1 Setup

#### 8.2.3.1	General

The purpose of the F1 Setup procedure is to exchange application level data needed for the gNB-DU and the gNB-CU to correctly interoperate on the F1 interface. This procedure shall be the first F1AP procedure triggered for the F1-C interface instance after a TNL association has become operational.

NOTE:	If F1-C signalling transport is shared among multiple F1-C interface instances, one F1 Setup procedure is issued per F1-C interface instance to be setup, i.e. several F1 Setup procedures may be issued via the same TNL association after that TNL association has become operational.

NOTE:	Exchange of application level configuration data also applies between the gNB-DU and the gNB-CU in case the DU does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [7]. How to use this information when this option is used is not explicitly specified.

The procedure uses non-UE associated signalling.

This procedure erases any existing application level configuration data in the two nodes and replaces it by the one received. This procedure also re-initialises the F1AP UE-related contexts (if any) and erases all related signalling connections in the two nodes like a Reset procedure would do.

#### 8.2.3.2	Successful Operation

Figure 8.2.3.2-1: F1 Setup procedure: Successful Operation

//omitted text unchanged//

If the F1 SETUP REQUEST message contains the Mobile IAB-MT User Location Information IE, the gNB-CU shall, if supported, take it into account when reporting UE location information to the AMF for a UE served by the mobile IAB-node.

If the XR Broadcast Information IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU shall, if supported, consider the indicated cell does not allow 2Rx XR UEs in case of subsequent outgoing mobility involving XR UEs.

If the NCGI to be Updated List IE is included in the F1 SETUP RESPONSE message, the gNB-DU shall, if supported, change the NCGI of the cell indicated by the Old NCGI IE to the NCGI indicated by the New NCGI IE.

If the Barring Exemption for Emergency Call Information IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU may store the information and consider the indicated cell allows emergency bearer services for UEs who would otherwise consider the cell as barred as specified in TS 38.304 [24].

If the UL WUS Configuration Information IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU shall, if supported, use this information for coordination of on-demand SIB1 transmission for network energy saving as specified in TS 38.300 [6].

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<next change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 8.2.4	gNB-DU Configuration Update

#### 8.2.4.1	General

The purpose of the gNB-DU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and the gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

NOTE:	Update of application level configuration data also applies between the gNB-DU and the gNB-CU in case the DU does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [7]. How to use this information when this option is used is not explicitly specified.

#### 8.2.4.2	Successful Operation

Figure 8.2.4.2-1: gNB-DU Configuration Update procedure: Successful Operation

//omitted text unchanged//

If the GNB-DU CONFIGURATION UPDATE message contains the Mobile IAB-MT User Location Information IE, the gNB-CU shall, if supported, take it into account when reporting UE location information to the AMF for a UE served by the mobile IAB-node.

If the XR Broadcast Information IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, consider the indicated cell does not allow 2Rx XR UEs in case of subsequent outgoing mobility involving XR UEs.

If the Barring Exemption for Emergency Call Information IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store the information and consider the indicated cell allows emergency bearer services for UEs who would otherwise consider the cell as barred as specified in TS 38.304 [24].

If the UL WUS Configuration Information IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, use this information for coordination of on-demand SIB1 transmission for network energy saving as specified in TS 38.300 [6].

If the On-Demand SIB1 Status Switch Request IE is included in the Served Cell To Modify IE in the gNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, use this information to request UL WUS Configuration Provision as specified in TS 38.423, and return the response in the On-Demand SIB1 Status Switch Response List IE in the gNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message.

If the On-Demand SIB1 Status Switch Response List IE in the gNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB-DU shall, if supported, take into account to decide On-Demand SIB1 status switch for related cells.

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<next change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 8.2.5	gNB-CU Configuration Update

#### 8.2.5.1	General

The purpose of the gNB-CU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

#### 8.2.5.2	Successful Operation

Figure 8.2.5.2-1: gNB-CU Configuration Update procedure: Successful Operation

The gNB-CU initiates the procedure by sending a GNB-CU CONFIGURATION UPDATE message including the appropriate updated configuration data to the gNB-DU. The gNB-DU responds with a GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall interpret that the corresponding configuration data is not changed and shall continue to operate the F1-C interface with the existing related configuration data.

The updated configuration data shall be stored in the respective node and used as long as there is an operational TNL association or until any further update is performed.

If Cells to be Activated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall activate the cell indicated by NR CGI IE and reconfigure the physical cell identity for which the NR PCI IE is included.

If the SSBs within the cell to be Activated List IE is included in the Cells to be Activated List Item IE within the gNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, only activate those SSB beams indicated by the SSB Index IE.

If at least one requested SSB beam in the SSBs within the cell to be Activated List IE is activated, the gNB-DU includes the Cells with SSBs Activated List IE in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message. The gNB-CU shall consider that the SSB beams indicated by the SSBs activated List IE as activated.

If Cells to be Deactivated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall deactivate the cell indicated by NR CGI IE.

If Cells to be Activated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message and the indicated cells are already activated, the gNB-DU shall update the cell information received in Cells to be Activated List Item IE.

If Cells to be Activated List Item IE is included in the GNB-CU CONFIGURATION UPDATE message, and the information for the cell indicated by the NR CGI IE includes the IAB Info IAB-donor-CU IE, the gNB-DU shall, if supported, apply the IAB STC Info IE therein to the indicated cell.

If the Cells Allowed to be Deactivated List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that it is allowed to deactivate the SSB beams within the indicated cells for network energy saving purpose.

If the Cells Allowed to be On-Demand SIB1 List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that it is allowed to be operated on on-demand SIB1 status within the indicated cells for network energy saving purpose.

If the Cells to Stop UL WUS Provision List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that the indicated cells shall stop on-demand SIB1 status.

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<next change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

#### 9.2.1.7	GNB-DU CONFIGURATION UPDATE

This message is sent by the gNB-DU to transfer updated information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Served Cells To Add List |  | 0..1 |  | Complete list of added cells served by the gNB-DU | YES | reject |
| >Served Cells To Add Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | reject |
| >>Served Cell Information | M |  | 9.3.1.10 | Information about the cells configured in the gNB-DU | - |  |
| >>gNB-DU System Information | O |  | 9.3.1.18 | RRC container with system information owned by gNB-DU | - |  |
| Served Cells To Modify List |  | 0..1 |  | Complete list of modified cells served by the gNB-DU | YES | reject |
| >Served Cells To Modify Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | reject |
| >>Old NR CGI | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>Served Cell Information | M |  | 9.3.1.10 | Information about the cells configured in the gNB-DU | - |  |
| >>On-Demand SIB1 Status Switch Request | O |  | ENUMERATED (start, stop, cancel, ...) |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<next change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

#### 9.2.1.8	GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE

This message is sent by a gNB-CU to a gNB-DU to acknowledge update of information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| //omitted text unchanged// |  |  |  |  |  |  |
| Transport Layer Address Info | O |  | 9.3.2.5 |  | YES | ignore |
| Uplink BH Non-UP Traffic Mapping | O |  | 9.3.1.103 |  | YES | reject |
| BAP Address | O |  | 9.3.1.111 | Indicates a BAP address assigned to the IAB-donor-DU. | YES | ignore |
| Cells for SON List | O |  | 9.3.1.214 |  | YES | Ignore |
| On-Demand SIB1 Status Switch Response List |  | 0..1 |  | Only used between the NES Cell gNB-DU and NES gNB-CU | YES | reject |
| > On-Demand SIB1 Status Switch Response List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >> On-Demand SIB1 Status Switch Response | O |  | ENUMERATED (accept, reject, ...) |  | - |  |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<next change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

#### 9.2.1.10	GNB-CU CONFIGURATION UPDATE

This message is sent by the gNB-CU to transfer updated information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| //omitted text unchanged// |  |  |  |  |  |  |
| Extended gNB-CU Name | O |  | 9.3.1.206 |  | YES | ignore |
| Cells Allowed to be Deactivated List |  | 0..1 |  |  | YES | ignore |
| >Cells Allowed to be Deactivated List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| Cells Allowed to be On-Demand SIB1 List |  | 0..1 |  |  | YES | ignore |
| > Cells Allowed to be On-Demand SIB1 List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| Cells To Stop UL WUS Provision List |  | 0..1 |  |  | YES | ignore |
| > Cells To Stop UL WUS Provision Item |  | 1.. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum numbers of cells that can be served by a gNB-DU. Value is 512. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU and the gNB-DU. Value is 32. |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a cell. Value is 64. |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<next change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

#### 9.3.1.10	Served Cell Information

This IE contains cell configuration information of a cell in the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| NR CGI | M |  | 9.3.1.12 |  | - |  |
| NR PCI | M |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| 5GS TAC | O |  | 9.3.1.29 | 5GS Tracking Area Code | - |  |
| Configured EPS TAC | O |  | 9.3.1.29a |  | - |  |
| Served PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs in SIB 1 associated to the NR Cell Identity in the NR CGI IE | - |  |
| >PLMN Identity | M |  | 9.3.1.14 |  | - |  |
| >TAI Slice Support List | O |  | Slice Support List 9.3.1.37 | Supported S-NSSAIs per PLMN or per SNPN. | YES | ignore |
| >NPN Support Information | O |  | 9.3.1.156 | Supported NPNs per PLMN. | YES | reject |
| >Extended TAI Slice Support List | O |  | Extended Slice Support List 9.3.1.165 | Additional Supported S-NSSAIs per PLMN or per SNPN. | YES | reject |
| >TAI NSAG Support List | O |  | 9.3.1.273 | NSAG information associated with the slices per TAC, per PLMN or per SNPN. | YES | ignore |
| //omitted text unchanged// |  |  |  |  |  |  |
| XR Broadcast Information | O |  | ENUMERATED (true, …) | Corresponds to information provided in the cellBarred2RxXR contained in the SIB1 message as defined in TS 38.331 [8]. | YES | ignore |
| Barring Exemption for Emergency Call Information | O |  | ENUMERATED (true, …) | Corresponds to information provided in the barringExemptEmergencyCall contained in the SIB1 message as defined in 38.331 [10]. | YES | ignore |
| UL WUS Configuration Information | O |  | Octet String | Includes the [FFS] IE, as defined in TS 38.331 [8]. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMN Ids. Value is 6. |
| maxnoofExtendedBPLMNs | Maximum no. of Extended Broadcast PLMN Ids. Value is 6. |
| maxnoofBPLMNsNR | Maximum no. of PLMN Ids.broadcast in an NR cell. Value is 12. |
| maxnoofNR-UChannelIDs | Maximum no. NR-U Channel IDs in a cell. Value is 16. |
| maxnoofMBSFSAs | Maximum no. of MBS FSAs by a cell. Value is 256. |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<end of change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>