3GPP TSG-RAN WG3 #127bis	R3-252048

Wuhan, PRC, Apr 7-11, 2025

Agenda Item:	10.2

Source:	Ericsson

Title:	(TP for BL CR to 38.473 for SON) MRO enhancements for LTM

Document for:	Discussion and Decision

# 1	Introduction

One of the main objectives of the SONMDT Rel-19 WID [RP-234038] is to enhance the MRO features to optimize the LTM cell switch procedure.

MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs, subsequent CPAC [RAN3, RAN2]:

- Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3]

- Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2]

This contribution discusses considerations regarding MRO feature enhancements for LTM.

# 2	Discussion

## 2.1	Source C-RNTI over F1 for RLF Report

During RAN3#126, there was a discussion on addition of the source C-RNTI to the ACCESS AND MOBILITY INDICATION message, for the case where that message transfers a RLF Report from the CU to the DU. The source C-RNTI has already been added to the message in the BL CR to 38.473, however the documentation of the attribute containing the C-RNTI is still under discussion.

The C-RNTI attribute is specified to be optional. The reason is that the attribute is only relevant to send if the source DU is different from the target DU, i.e. for a failure during an Inter-DU cell switch.

There are several possible places for specifying the details on when to include the source C-RNTI in the ACCESS AND MOBILITY INDICATION message. RAN3#126 contribution R3-247159 cl. 2.3.1 proposes that the documentation for this attribute is located in 38.401, NG-RAN; Architecture description. That proposal suggests specifying details of the attribute in a Stage 2 document. We believe that this is not a proper place for such details.

Instead, we propose to place the specification of the details of the Source C-RNTI in the Stage 3 document where this attribute is actually detailed, i.e. in TS 38.473, NG-RAN; F1 Application Protocol (F1), see TP for BL CR in Annex.

Avoid specifying the conditions of the sending of the source C-RNTI in ACCESS AND MOBILITY message in TS 38.401

Specify the conditions of the sending of the source C-RNTI in ACCESS AND MOBILITY message in TS 38.473, clause 9.2.10.1. See TP for BLCR in Annex 1.

## 2.2	Optimization of early UL synchronisation

Early UL synchronization is an important part of LTM, since it is one of the mechanisms that allows the LTM cell switch execution to take place with really short interruption. To achieve this benefit, the source DU has to ensure that the early UL synchronization takes place with some margin before the LTM cell switch execution is actually triggered. On the other hand, if the source DU lets the TA obtained through early UL synchronization become too old, there is a risk that it is outdated at the time of the LTM cell switch execution, potentially leading to an LTM failure. How fast a TA obtained through early UL synchronization becomes outdated is highly dependent on a number of circumstances, e.g. if UEs generally moves fast or slow in the source cell or in certain beams in the source cell. This implies that it would be useful for the source DU to receive feedback on how well the early UL synchronization it triggers perform, e.g. how large the UEs' transmission errors are when they access the LTM target cell. Was it close to being so large that successful reception/decoding would have been precluded in the target DU? It is therefore proposed to acknowledge this scenario and to propose solutions to optimize early UL synchronization.

RAN3 to acknowledge that the source DU needs to receive information about uplink synchronisation performance, to optimise uplink synchronisation.

- As discussed in RAN3#127, the RACH-less performance depends on the TA used by the UE at LTM cell switch. And as explained above, the TA can be outdated if not maintained properly by the network. Another possibility is that the network configures the UE to calculate its own TA, which may in some situations (high speed, high reflections, …) may be difficult and lead to errors. In that case it would be difficult for the UE to correct these mistakes, but with proper feedback, the network can identify these situations and learn how calculate itself an optimized TA, and not configure the UE with autonomous TA in these cases. Therefore, the discussion of RACH-less optimization is now equivalent to a TA optimization problem, including outdated TA and miscalculated TA from the UE.

We can split the discussion about TA value optimisation for RACH-less into two cases, one for the failure case and one for the near-failure case. The reason is that the discussion and proposed solution are dissimilar.

### 2.2.1	Optimising for the failure case

One way to optimize the TA value would be to calculate the difference between the TA that was used at LTM cell switch, and the TA that was used at successful LTM recovery or RRC re-establishment. This case is limited to LTM recovery or re-establishment in the failed cell. For transferring information about incorrect TA value to the source DU, one could imagine a UE based solution or a network based solution. These two solutions are discussed below:

Network based solution:

In a network based solution the target DU sends LTM recovery TA to the source DU (via the CU). The source DU compares it with the TA value (and the source of this value) that was used at the failed attempt.

One drawback for a network-based solution is that even if the CU provides a TA to the source DU, it is the DU that decides to use a network provided TA or a UE calculated TA for uplink synchronization. The knowledge of this decision resides in the DU.

Yet another aspect is that a network based solution can only be used when the network provides the TA value. When the TA value is determined by the UE, there is no way the source DU can know this value.

UE based solution:

In a UE based solution the UE logs, in the RLF report, the TA value used in the failed attempt. If the failed cell and LTM recovery cell are the same, it can also log the TA value received from the LTM recovery cell at RACH-based access.

Conclusion:

The UE based solution works, whereas the network based solution works only for a few cases, but does not work in some cases (for UE-calculated TA). For these reasons, we propose a UE based solution.

RAN3 to agree on a UE based solution for TA value optimisation in the case of a failed LTM cell switch. RAN3 to send an LS to RAN2 about this agreement.

### 2.2.2	Optimising for the near-failure case

The near-failure case occurs when the first transmission from the UE was received almost outside the cyclic prefix. In other words, the access succeeded, but the timing was off so much that a better TA value could have been used. In that case the network will need to correct the TA with the UE.

A near-failure solution can actually be more useful than the failure case solution, because the failure case solution only works when a cell switch fails and at the same time LTM recovery cell is the same as the target cell. The analysis of whether there is a near-failure in a cell switch works for all cell switches. And if near-failure are avoided, failure will be less likely

As there was no failure, no RLF Report was generated, and no SHR report either. New conditions for a new SHR trigger would be complicated to obtain, because it would be hard for the UE to detect that a near-failure has occurred. Therefore, a UE based solution would be complicated.

However, a network based solution is feasible: The target DU, upon determining that the TA value used in LTM cell switch almost lead to a failure, can signal this to the source DU via the CU.

RAN3 to agree on a network-based solution for TA value optimisation in the case of a near-failed LTM cell switch.

## 2.3	Optimization of LTM cell switch ping-pong

Ping-pong for LTM cell switches is a condition that needs to be avoided. Optimizing configuration to avoid ping-pong from an LTM perspective is done differently for the Intra-DU ping pong case and the Inter-DU ping pong case. These cases will be detailed in the following subsections. It is worth noting that during RAN3#127, this problem was broken down into two questions, namely:

- Which node is responsible for identifying LTM ping-pong (CU or DU)?

- Which node is responsible for resolving LTM ping-pong (CU or DU)?

And the following agreements were taken:

CU can adjust the candidate Cell list for LTM based on UHI.

DU should be able to resolve LTM ping-pong based on information over F1.

### 2.3.1	Intra-DU ping-pong

For intra-DU ping-pong, both CU and DU can detect ping-pong (UE Context is available in both nodes), and therefore take actions to stop it and avoid it in the future for similar situations. Therefore, no communication between nodes needs to be specified.

There is no need for RAN3 to specify a solution for intra-DU ping-pong.

### 2.3.2	Inter-DU ping-pong

In Rel-18 LTM, all mobility occurs within the oversight of one gNB. In other words, the CU of the gNB has information of all LTM mobility concerning its own cells. For this reason, the CU has the ability to detect inter-DU ping-pong. And this feature already exist in the CU for L3 inter-gNB HO. Furthermore, the parameters controlling the LTM mobility behavior is controlled by the DU. Therefore, the DU has the ability to resolve ping pong, as long as it can be made aware if the existence of a ping-pong.

It is the task of the CU to detect inter-DU ping-pong.

As the CU detects ping-pong and the DU should be able to resolve LTM ping-pong based on information over F1, different solutions can be imagined:

- CU sends the UHI to DU after every mobility event

- CU sends the UHI to DU only if ping-pong is detected

- CU sends the information that ping-pong occurred to the DU

Solution 1 is not optimized regarding the signaling load, especially in case of subsequent LTM, where the UHI needs to be updated in all candidate DUs, by sending messages that would not be needed in normal operation. Solution 2 allows to reuse the detection mechanism in the CU but implies that the DU will “decode” the full UHI to find the source of the ping-pong. Solution 3 needs new signaling (in existing messages), but relies on the existing CU detection mechanism and limit the impact on the DU. Therefore, it is proposed to agree solution 3.

CU sends the information that ping-pong occurred to the DU.

If we consider ping-pong to be mobility from one cell to a second cell, and then back to the first cell, all within a small amount of time (similar to the definition of Inter-system ping-pong in TS 38.300), then the CU can detect the ping-pong when the “pong” has been successful.

Looking at TS 38.401, figure 8.2.1.5-1 (copied below), consider the case where firstly a cell switch occurs from the source gNB-DU to the candidate gNB-DU, and shortly after that a cell switch in the opposite direction. Here we see that during the second cell switch, the CU get notified about the successful cell switch in message 19, DU-CU CELL SWITCH NOTIFICATION. At this point in time, the CU can discover that a ping-pong has occurred, according to the definition above, by realizing that a cell switch was made in the opposite direction a short time ago.

The CU can discover a ping-pong after receiving message 19 in TS 38.401 figure 8.2.1.5-1.

When this ping-pong has been detected, the CU might want to tell this to the first DU and/or the second DU. The first occasion to tell the first DU is in message 20, CU-DU CELL SWITCH NOTIFICATION. A new IE added to this message could tell the DU that a ping-pong has occurred for this UE.

Add a new IE to CU-DU CELL SWITCH NOTIFICATION message telling the first DU that a ping-pong has occurred.

If the ping-pong has not stopped (yet) and a cell switch occurs one again (thus yielding a “ping-pong-ping”), then the second DU will be the “candidate gNB-DU” in the figure. By the same procedure as above, the CU can then tell the second DU that a ping-pong has occurred.

The IE in the previous proposal can be used for notifying the second DU that a ping-pong has occurred for a particular UE as well.

A text proposal to TS 38.473 for this new IE is proposed in Annex.

## 2.4	Beam failure recovery after successful LTM cell switch

In LTM, the LTM Cell Switch command from the network to the UE includes a beam indication. That “beam indication” is specified as a Transmission Configuration Identifier (TCI) State Identity (ID) associated with the LTM candidate cell.

In a typical case, the network determines the beam to be included based on lower layer measurement reports from the UE (also called L1 measurement reports for LTM), which includes measurement per beam (SSB measurements, such as L1 RSRP for one or more SSBs). Since these measurements may not be layer 3 (L3) filtered, the radio conditions may change quite quickly, so it might not always be simple for the network to determine the optimal beam to indicate to the UE in an LTM cell switch. TCI states are sent from target DU to source DU, via the CU, at LTM preparation. There can be up to 128 TCI states signaled to the source DU. For DL pre-sync, the source DU can pick at most 16 of them. In the LTM cell switch command, only one TCI state from the 16 can be chosen by the source DU.

One problem that may occur is that the network may include a beam indication (e.g. TCI State ID=X) and, shortly after a successful LTM cell switch to an LTM candidate cell, the UE declares a Beam Failure Detection (BFD) for the beam indicated in the LTM cell switch for the indicated LTM candidate cell (let’s call it the target cell once the UE has performed the LTM cell switch there). Upon BFD in the target cell, the UE triggers a Beam Failure Recovery (BFR), selects another beam (e.g. corresponding to TCI State ID=Y) and continues the communication with the target cell without a Radio Link Failure (RLF).

This failure case can have a detrimental effect on the performance of LTM cell switch and has also been identified and discussed by RAN2 during RAN2#129. As a result, RAN2 sent an LS to RAN3 in R3-251514, in which they ask RAN3 to:

consider defining a network-based solution for the issue of BFR shortly after successful LTM cell switch (“near failure”).

Therefore, in order to avoid these beam failures, it would be beneficial for the source DU to understand that it selected the wrong TCI state, and to know the TCI state in which the UE successfully recovered. It is therefore proposed to signal this information from target DU to CU and to CU to source DU. Also, a new “near-failure” definition will be needed in stage-2 to captured this use-case

Target DU to signal BFD and BFR information (e.g. recovery TCI state) to the CU, which will forward this information to source DU and capture the use-case as new near-failure definition in stage-2.

The signaling from target DU to CU, and from CU to source DU can either be done using UE-associated signaling or non-UE-associated signaling. As there are benefits of UE-associated signaling only if the UE-associated logical connection between CU and source DU exists, and that this connection may be released very quickly by CU by sending the UE Context Release message, we propose to use non-UE-associated signaling for notifying the target DU of beam failure after LTM cell switch. It should also be noted that even if the UE-associated logical connection still exists between target DU and CU, there is no existing class-2 UE-associated procedure that could be reused. In that case UE-associated solution would need the definition of a new message.

RAN3 to agree to use non-UE-associated signaling for signaling BFD and BFR information (e.g. recovery TCI state) from the target DU to the CU, and from the CU to the source DU.

### 2.4.1	From target DU to CU

Notifying the CU that there was a beam failure just after LTM cell switch to the target DU will occur within a short time after a cell switch, in the order of a few seconds. As the cell switch failed, this notification will be sent outside the LTM cell switch procedure. One suitable message is the DU-CU ACCESS AND MOBILITY INDICATION. We propose adding IEs to this message for conveying beam failure after a cell switch.

RAN3 to agree to use the DU-CU ACCESS AND MOBILITY INDICATION message for conveying beam failure information after an LTM cell switch from target DU to CU.

The beam failure information sent from the target DU to the CU should include the fact that there was an LTM cell switch directly followed by a beam failure. This information could be implicit. Also, the gNB-CU UE F1AP ID should be included, for identification of the UE Context in the CU. As this procedure is for intra-CU LTM cell switch, the CU should still keep the UE context. Furthermore, the TCI state (or beam ID) of the recovery beam should be included, as this can hint to the source DU that this beam should have been used in the cell switch.

RAN3 to agree to include in the DU-CU ACCESS AND MOBILITY INDICATION message the following information: An (implicit) beam-failure-after-cell-switch indication; the gNB-CU UE F1AP ID of the failed UE; and the TCI state of the beam after beam failure recovery.

This proposal is reflected in the TP in Annex.

### 2.4.2	From CU to source DU

The CU, after receiving the DU-CU ACCESS AND MOBILITY INDICATION from the target DU should relay the information in that message to the source DU. It should also add more information to help the source DU. One suitable message for this is the ACCESS AND MOBILITY INDICATION message, which is already used for conveying similar information from the CU to a DU.

RAN3 to agree to use the ACCESS AND MOBILITY INDICATION message for conveying beam failure information from the CU to the source DU after an LTM cell switch.

The beam failure information sent from the CU to the source DU should include the fact that there was an LTM cell switch directly followed by a beam failure. This information could be implicit.

From the UE context in the CU, the CU should extract the following information to send to the source DU:

- The source C-RNTI (optional, and if available, for UE Context identification)

- the source cell ID

- the target cell ID

- TCI state (or beam ID) used during LTM cell switch

- TCI state (or beam ID) used at BFR (received from target DU)

- gNB-DU UE F1AP ID (if existing e.g. subsequent LTM).

RAN3 to agree to include in the ACCESS AND MOBILITY INDICATION message from the CU to the source DU the following information: An (implicit) beam-failure-after-cell-switch indication; the source C-RNTI; the source cell ID; the target cell ID; the TCI state (or beam ID) used during LTM cell switch; the TCI state (or beam ID) used at BFR.

This proposal is reflected in the TP in Annex.

# 4	Conclusion

- Avoid specifying the conditions of the sending of the source C-RNTI in ACCESS AND MOBILITY message in TS 38.401

Specify the conditions of the sending of the source C-RNTI in ACCESS AND MOBILITY message in TS 38.473, clause 9.2.10.1. See TP for BLCR in Annex 1.

RAN3 to acknowledge that the source DU needs to receive information about uplink synchronisation performance, to optimise uplink synchronisation.

RAN3 to agree on a UE based solution for TA value optimisation in the case of a failed LTM cell switch. RAN3 to send an LS to RAN2 about this agreement.

RAN3 to agree on a network-based solution for TA value optimisation in the case of a near-failed LTM cell switch.

There is no need for RAN3 to specify a solution for intra-DU ping-pong.

It is the task of the CU to detect inter-DU ping-pong.

CU sends the information that ping-pong occurred to the DU.

- The CU can discover a ping-pong after receiving message 19 in TS 38.401 figure 8.2.1.5-1.

Add a new IE to CU-DU CELL SWITCH NOTIFICATION message telling the first DU that a ping-pong has occurred.

The IE in the previous proposal can be used for notifying the second DU that a ping-pong has occurred for a particular UE as well.

Target DU to signal BFD and BFR information (e.g. recovery TCI state) to the CU, which will forward this information to source DU and capture the use-case as new near-failure definition in stage-2.

RAN3 to agree to use non-UE-associated signaling for signaling BFD and BFR information (e.g. recovery TCI state) from the target DU to the CU, and from the CU to the source DU.

RAN3 to agree to use the DU-CU ACCESS AND MOBILITY INDICATION message for conveying beam failure information after an LTM cell switch from target DU to CU.

RAN3 to agree to include in the DU-CU ACCESS AND MOBILITY INDICATION message the following information: An (implicit) beam-failure-after-cell-switch indication; the gNB-CU UE F1AP ID of the failed UE; and the TCI state of the beam after beam failure recovery.

RAN3 to agree to use the ACCESS AND MOBILITY INDICATION message for conveying beam failure information from the CU to the source DU after an LTM cell switch.

RAN3 to agree to include in the ACCESS AND MOBILITY INDICATION message from the CU to the source DU the following information: An (implicit) beam-failure-after-cell-switch indication; the source C-RNTI; the source cell ID; the target cell ID; the TCI state (or beam ID) used during LTM cell switch; the TCI state (or beam ID) used at BFR.

# 5	References

R2-125bis_Maint_eRedCap_SON-MDT_(Mattias)_20240419_Final.docx

3GPP TS 38.300, "NR and NG-RAN Overall Description; Stage 2"

R3-243846, Summary of Offline Discussion on MRO for LTM

# Annex:	 TP to SON BL CR for 38.473

First change

#### 9.2.2.16	CU-DU CELL SWITCH NOTIFICATION

This message is sent by the gNB-CU to inform the gNB-DU about the initiation of the cell switch command to the UE.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cell ID | M |  | NR CGI 9.3.1.12 |  | YES | reject |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Includes the TCI-StateId IE, as defined in TS 38.331 [8]. | - |  |
| >UL TCI State ID | O |  | OCTET STRING | Includes the TCI-UL-StateId IE, as defined in TS 38.331 [8]. | - |  |
| TA Information List |  | 0..1 |  |  | YES | ignore |
| >TA Information Item IEs |  | 1 .. <maxnoofTAList> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. | - |  |
| >>Tag ID Pointer | O |  | OCTET STRING | Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [8]. | YES | ignore |
| LTM ping-pong detection | O |  |  |  | YES | ignore |
| >Previous Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >Time UE Stayed in Previous Cell | M |  | INTEGER (0..4095) | The duration of time the UE stayed in the previous cell, or set of previous NR cells with the same NR ARFCN for reference point A, in seconds. If the duration is more than 4095s, this IE is set to 4095. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAList | Maximum no. of TA values to be sent, the maximum value is 8. |

Next change

#### 9.2.10.1	ACCESS AND MOBILITY INDICATION

This message is sent by gNB-CU to gNB-DU to provide access and mobility information to the gNB-DU.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| RA Report List |  | 0..1 |  |  | YES | ignore |
| >RA Report Item |  | 1 .. <maxnoofRAReports> |  |  | - |  |
| >>RA Report Container | M |  | OCTET STRING | Includes the RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| RLF Report Information List |  | 0..1 |  |  | YES | ignore |
| >RLF Report Information Item |  | 1 .. <maxnoofRLFReports> |  |  | - |  |
| >>NR UE RLF Report Container | M |  | OCTET STRING | Includes the nr-RLF-Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| >>C-RNTI | O |  | 9.3.1.32 | C-RNTI allocated at the source gNB-DU in case the gNB-DU responsible for the failure is not the gNB-DU serving the UE at the time of failure | YES | ignore |
| >>RLF Report Failure Type | O |  | ENUMERATED (too late LTM, too early LTM, LTM to wrong cell,...) |  | YES | ignore |
| Successful HO Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful HO Report Information Item |  | 1 .. <maxnoofSuccessfulHOReports> |  |  | - |  |
| >>Successful HO Report Container | M |  | OCTET STRING | Includes the SuccessHO-Report IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| Successful PSCell Change Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful PSCell Change Report information Item |  | 1..<maxnoofSuccessfulPSCellChangeReports> |  |  | - |  |
| >>Successful PSCell Change Report Container | M |  | OCTET STRING | Includes the SuccessPSCell-Report IE as defined in TS 38.331 [8]. | - |  |
| Beam Failure Recovery Information | O |  |  |  | YES | ignore |
| >gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | - |  |
| >Recovery TCI State | M |  | OCTET STRING | Includes the TCI-StateId IE used at Beam Failure Recovery, as defined in TS 38.331 [8]. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofRAReports | Maximum no. of RA Reports, the maximum value is 64. |
| maxnoofRLFReports | Maximum no. of RLF Reports, the maximum value is 64. |
| maxnoofSuccessfulHOReports | Maximum no. of Successful HO Reports, the maximum value is 64. |
| maxnoofSuccessfulPSCellChangeReports | Maximum no. of Successful PSCell Change Reports. Value is 64. |

Next change

#### 9.2.10.2	DU-CU ACCESS AND MOBILITY INDICATION

This message is sent by the gNB-DU to provide access and mobility information to the gNB-CU.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| DL LBT Failure Information List |  | 0..1 |  |  | YES | ignore |
| > DL LBT Failure Information Item |  | 1 .. <maxnoofLBTFailureInformation> |  |  | - |  |
| >>DL LBT Failure Information | M |  | 9.3.1.327 |  | - |  |
| Beam Failure Recovery Information | O |  |  |  | YES | ignore |
| >Source Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >Target Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >Failed TCI State | M |  | OCTET STRING | Includes the TCI-StateId IE used by the UE at LTM cell switch, as defined in TS 38.331 [8]. | - |  |
| >Recovery TCI State | M |  | OCTET STRING | Includes the TCI-StateId IE used at Beam Failure Recovery, as defined in TS 38.331 [8]. | - |  |
| >gNB-DU UE F1AP ID | O |  | 9.3.1.5 | Included if available in gNB-CU. | - |  |
| >Source C-RNTI | O |  | C-RNTI 9.3.1.32 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofLBTFailureInformation | Maximum no. of UEs for which LBT Failure Information is provided, the maximum value is 64. |

End of changes