3GPP TSG-RAN WG3 Meeting #127bis	R3-252053

# Wuhan, China, 7th – 11th April, 2025

Title: 	Data collection for slice coverage observability enhancement

Source: 	Ericsson, Jio Platforms, BT, InterDigital, Deutsche Telekom

Agenda item:	10.3.2

Document Type:	Discussion and decision

# 1	Introduction

The release 19 work item for Data Collection for SON/MDT in NR standalone and MR-DC focuses on enhancements for Rel-17/Rel-18 SON/MDT features such as MRO enhancement for LTM, CHO with candidate SCGs and subsequent CPAC, as well as on SON/MDT enhancements for Intra NTN mobility and Network Slicing [1].

In meetings leading up to TSG-RAN WG3 #127 discussion on proposals regarding enhancements for logging denied slice service request at NAS level due to slice not being supported in the cell from which the UE service request has been discussed in the past meetings without much consensus. In this contribution, we share our views and proposals on these topics.

# 2	Discussion

In NR slices are uniformly available across a Tracking Area within a Registration Area, however the cells in the same tracking area, which support a certain slice, may have no resources allocated to it. This mechanism, where a slice is supported in a cell but has no resources allocated to it, allows the operator to deploy slices in the network which are specific to a certain location, e.g., shopping mall, stadium, while keeping the overall Registration Area with the corresponding Tracking Areas and its planning unaffected.

Due to the above, an area where it is beneficial to have better observability is in the overlap between UE originated Service Request to establish sessions on certain slices and its relation to the existing slice coverage across the network. The above problem i.e., configuring a perfect match between the coverage of the cells in which a slice is supported and available and the geographical areas where the users should have access to the services provided by a slice, may be challenging in some cases and mismatches may occur. Another dimension of the same problem is to anticipate from which geographical areas/locations UEs may request to access services over a certain slice, and if the slice deployment must be changed to satisfy such requests.

For example, let us consider the case of an operator providing services over a certain slice for a customer such that the employees of the customer can access the slice services, e.g., the enterprise slice, for work-related purposes.  The users may request access to slice services while working from the office premises of their employer but also while working remotely from home. While configuring a good overlap between the slice coverage and the customer’s office premises is rather easy, it may be comparatively more difficult to configure a good slice coverage overlapping with the remote working location of each employee.

Observation 1: Ensuring a good overlap between the slice coverage and the areas in which the authorized UEs should have access to the services provided by the slice requires a good observability of the UEs’ slice service requests at the RAN/OAM.

Below we describe solutions to the above problem and motivate our proposals based on the observed pros and cons. In general, the configuration problem is two-fold. The first is to configure the cells/tracking areas that overlap with the enterprise location. This part of the configuration task is relatively simple and can be achieved with careful network planning and correlating the network deployment with measurements using existing MDT measurements. The second is to configure the cells/tracking areas in remote locations, e.g., employee households, is a much harder and a dynamic task. Below we discuss solutions to this second configuration problem.

As one potential solution to support the configuration task described above, the RAN may attempt to configure all cells in a geographic area to support the enterprise slice, however, these slices may have zero resources allocated to them. As a consequence of the above, the RAN will become aware of each UE request trying to access the slice. The RAN after receiving one or more request to access the slice in a cell may decide to allocate resources to the enterprise slice in certain cells in the registration area. The drawback of the above solution is that one or more registration areas may need to advertise support for the enterprise slice, which has both a configuration overhead and an overhead during execution, i.e., UEs may handover to the cell with an expectation that the slice is supported/has resources allocated to it, but the lack of allocated resources to the slice is only known after the UE tries to setup a session on the target cell after the handover. Due to the above drawbacks, logging UE’s service request in the network by advertising slice support with no allocated resources to the enterprise slice is not a viable solution.

A potential solution to the problem is to log the service requests from the UE and the corresponding service response/rejections at the core network at the time of a service request. The UE as part of the NAS Registration procedure requests for a list of slices to which the UE may eventually connect to. The core network, depending on UE subscription, current registration area, and other existing policies, may either allow or reject the use of certain slices by the UE. On one hand, the AMF and core network entities are unaware, by design, of cells in the RAN. However, on the other hand, the core network entities are aware of per-UE-level request to access services over a slice. This information is not available at the gNB at a per UE-level as NAS messages are transparent to the gNB.

Observation 1: Part of the information required to correlate the service request to a cell/location is available at the gNB, while another is available at the AMF. The information available at the gNB and the AMF may be correlated in order to obtain per-cell per-UE-level slice access requests.

Proposal 1: RAN3 to discuss mechanisms to enable the gNB to obtain per-UE-level information from the AMF on allowed and rejected slice requests.

In the appendix below, a example of text proposal is provided that details a mechanism by which the gNB could request NAS level UE slice service related information, e.g., rejected S-NSSAI, from the AMF. In this case, the request is performed via an Information Element in the INITIAL UE MESSAGE to request from the AMF the reporting of NAS rejected slice service requests for this specific UE. Having received the request, and after the UE performs the Service Request, the NAS may then communicate the list of accepted/reject slices to the gNB using INITIAL CONTEXT SETUP REQUEST message.

Proposal 2: RAN3 to discuss normative solutions to enable gNB to obtain per-UE-level information from the AMF on allowed and rejected slice requests.

Once collected, the gNB may then signal information from the AMF, together with other information available at the gNB, e.g., cell-level information to the OAM for logging and as input to future slice re-planning procedures.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1: RAN3 to discuss mechanisms to enable the gNB to obtain per-UE-level information from the AMF on allowed and rejected slice requests.

Proposal 2: RAN3 to discuss normative solutions to enable gNB to obtain per-UE-level information from

the AMF on allowed and rejected slice requests.

# References

RP-234038: WID: Data collection for SON/MDT in NR standalone and MR-DC

# TP to TS 38.413

### --First change—

### 9.2.2	UE Context Management Messages

#### 9.2.2.1	INITIAL CONTEXT SETUP REQUEST

This message is sent by the AMF to request the setup of a UE context.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| Old AMF | O |  | AMF Name 9.3.3.21 |  | YES | reject |
| UE Aggregate Maximum Bit Rate | C-ifPDUsessionResourceSetup |  | 9.3.1.58 |  | YES | reject |
| Core Network Assistance Information for RRC INACTIVE | O |  | 9.3.1.15 |  | YES | ignore |
| GUAMI | M |  | 9.3.3.3 |  | YES | reject |
| PDU Session Resource Setup Request List |  | 0..1 |  |  | YES | reject |
| >PDU Session Resource Setup Request Item |  | 1..<maxnoofPDUSessions> |  |  | - |  |
| >>PDU Session ID | M |  | 9.3.1.50 |  | - |  |
| >>PDU Session NAS-PDU | O |  | NAS-PDU 9.3.3.4 |  | - |  |
| >>S-NSSAI | M |  | 9.3.1.24 |  | - |  |
| >>PDU Session Resource Setup Request Transfer | M |  | OCTET STRING | Containing the PDU Session Resource Setup Request Transfer IE specified in subclause 9.3.4.1. | - |  |
| >>PDU Session Expected UE Activity Behaviour | O |  | Expected UE Activity Behaviour 9.3.1.94 | Expected UE Activity Behaviour for the PDU Session. | YES | ignore |
| Allowed NSSAI | M |  | 9.3.1.31 | Indicates the S-NSSAIs permitted by the network | YES | reject |
| Rejected NSSAI | O |  |  | Indicates the S-NSSAIs rejected by the network | YES | reject |
| UE Security Capabilities | M |  | 9.3.1.86 |  | YES | reject |
| Security Key | M |  | 9.3.1.87 |  | YES | reject |
| Trace Activation | O |  | 9.3.1.14 |  | YES | ignore |
| Mobility Restriction List | O |  | 9.3.1.85 |  | YES | ignore |
| UE Radio Capability | O |  | 9.3.1.74 |  | YES | ignore |
| Index to RAT/Frequency Selection Priority | O |  | 9.3.1.61 |  | YES | ignore |
| Masked IMEISV | O |  | 9.3.1.54 |  | YES | ignore |
| NAS-PDU | O |  | 9.3.3.4 |  | YES | ignore |
| Emergency Fallback Indicator | O |  | 9.3.1.26 |  | YES | reject |
| RRC Inactive Transition Report Request | O |  | 9.3.1.91 |  | YES | ignore |
| UE Radio Capability for Paging | O |  | 9.3.1.68 |  | YES | ignore |
| Redirection for Voice EPS Fallback | O |  | 9.3.1.116 |  | YES | ignore |
| Location Reporting Request Type | O |  | 9.3.1.65 |  | YES | ignore |
| CN Assisted RAN Parameters Tuning | O |  | 9.3.1.119 |  | YES | ignore |
| SRVCC Operation Possible | O |  | 9.3.1.128 |  | YES | ignore |
| IAB Authorized | O |  | 9.3.1.129 |  | YES | ignore |
| Enhanced Coverage Restriction | O |  | 9.3.1.140 |  | YES | ignore |
| Extended Connected Time | O |  | 9.3.3.31 |  | YES | ignore |
| UE Differentiation Information | O |  | 9.3.1.144 |  | YES | ignore |
| NR V2X Services Authorized | O |  | 9.3.1.146 |  | YES | ignore |
| LTE V2X Services Authorized | O |  | 9.3.1.147 |  | YES | ignore |
| NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.3.1.148 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| LTE UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.3.1.149 | This IE applies only if the UE is authorized for LTE V2X services. | YES | ignore |
| PC5 QoS Parameters | O |  | 9.3.1.150 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| CE-mode-B Restricted | O |  | 9.3.1.155 |  | YES | ignore |
| UE User Plane CIoT Support Indicator | O |  | 9.3.1.160 |  | YES | ignore |
| RG Level Wireline Access Characteristics | O |  | OCTET STRING | Specified in TS 23.316 [34]. Indicates the wireline access technology specific QoS information corresponding to a specific wireline access subscription. | YES | ignore |
| Management Based MDT PLMN List | O |  | MDT PLMN List 9.3.1.168 |  | YES | ignore |
| UE Radio Capability ID | O |  | 9.3.1.142 |  | YES | reject |
| Time Synchronisation Assistance Information | O |  | 9.3.1.220 |  | YES | ignore |
| QMC Configuration Information | O |  | 9.3.1.223 |  | YES | ignore |
| Target NSSAI Information | O |  | 9.3.1.229 |  | YES | ignore |
| UE Slice Maximum Bit Rate List | O |  | 9.3.1.231 |  | YES | ignore |
| 5G ProSe Authorized | O |  | 9.3.1.233 |  | YES | ignore |
| 5G ProSe UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.148 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| 5G ProSe PC5 QoS Parameters | O |  | 9.3.1.234 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| Network Controlled Repeater Authorized | O |  | 9.3.1.245 |  | YES | ignore |
| Aerial UE Subscription Information | O |  | 9.3.1.246 |  | YES | ignore |
| NR A2X Services Authorized | O |  | 9.3.1.247 |  | YES | ignore |
| LTE A2X Services Authorized | O |  | 9.3.1.248 |  | YES | ignore |
| NR A2X UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.148 | This IE applies only if the UE is authorized for NR A2X services. | YES | ignore |
| LTE A2X UE PC5 Aggregate Maximum Bit Rate | O |  | LTE UE Sidelink Aggregate Maximum Bit Rate 9.3.1.149 | This IE applies only if the UE is authorized for LTE A2X services. | YES | ignore |
| A2X PC5 QoS Parameters | O |  | 9.3.1.249 | This IE applies only if the UE is authorized for A2X services. | YES | ignore |
| Mobile IAB Authorized | O |  | 9.3.1.259 |  | YES | ignore |
| Partially Allowed NSSAI | O |  | 9.3.1.261 | Indicates the S-NSSAIs partially permitted by the network. | YES | ignore |
| Ranging and Sidelink Positioning Service Information | O |  | 9.3.1.269 | This IE applies only if the UE is authorized for NR V2X services and/or 5G ProSe services. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions allowed towards one UE. Value is 256. |

| Condition | Explanation |
| --- | --- |
| ifPDUsessionResourceSetup | This IE shall be present if the PDU Session Resource Setup List IE is present. |

--Unmodified text ignored--

### 9.2.5	NAS Transport Messages

#### 9.2.5.1	INITIAL UE MESSAGE

This message is sent by the NG-RAN node to transfer the initial layer 3 message to the AMF over the NG interface.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range |  | IE type and reference |  | Semantics description |  | Criticality | Assigned Criticality |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  |  | 9.3.1.1 |  |  |  | YES | ignore |  |
| RAN UE NGAP ID | M |  |  | 9.3.3.2 |  |  |  | YES | reject |  |
| NAS-PDU | M |  |  | 9.3.3.4 |  |  |  | YES | reject |  |
| User Location Information | M |  |  | 9.3.1.16 |  |  |  | YES | reject |  |
| RRC Establishment Cause | M |  |  | 9.3.1.111 |  |  |  | YES | ignore |  |
| 5G-S-TMSI | O |  |  | 9.3.3.20 |  |  |  | YES | reject |  |
| AMF Set ID | O |  |  | 9.3.3.12 |  |  |  | YES | ignore |  |
| UE Context Request | O |  |  | ENUMERATED (requested, ...) |  |  |  | YES | ignore |  |
| Allowed NSSAI | O |  |  | 9.3.1.31 |  |  |  | YES | reject |  |
| Source to Target AMF Information Reroute | O |  |  | 9.3.3.27 |  |  |  | YES | ignore |  |
| Selected PLMN Identity | O |  |  | PLMN Identity 9.3.3.5 |  | Indicates the selected PLMN id for the non-3GPP access. |  | YES | ignore |  |
| IAB Node Indication | O |  |  | ENUMERATED (true, ...) |  | Indication of an IAB node |  | YES | reject |  |
| CE-mode-B Support Indicator | O |  |  | 9.3.1.156 |  |  |  | YES | reject |  |
| LTE-M Indication | O |  |  | 9.3.1.157 |  |  |  | YES | ignore |  |
| EDT Session | O |  |  | ENUMERATED (true, …) |  |  |  | YES | ignore |  |
| Authenticated Indication | O |  |  | ENUMERATED (true, …) |  | Indicates the FN-RG has been authenticated by the access network. |  | YES | ignore |  |
| NPN Access Information | O |  |  | 9.3.3.46 |  |  |  | YES | reject |  |
| RedCap Indication | O |  |  | 9.3.1.228 |  |  |  | YES | ignore |  |
| Selected NID | O |  |  | NID 9.3.3.42 |  | Indicates together with the Selected PLMN Identity IE, the selected SNPN Identity for the non-3GPP access. |  | YES | ignore |  |
| Mobile IAB Node Indication | O |  |  | ENUMERATED (true, ...) |  | Indicates that the UE is a mobile IAB-node |  | YES | reject |  |
| Partially Allowed NSSAI | O |  |  | 9.3.1.261 |  | Indicates the S-NSSAIs partially permitted by the network. |  | YES | ignore |  |
| NAS Rejected NSSAI Information Request |  |  | O |  |  |  | ENUMERATED (requested, ...) |  |  | Indicates a request to receive the NAS rejected S-NSSAIs requested by the UE |  | YES | ignore |
| eRedCap Indication | O |  |  | ENUMERATED (true, ...) |  |  |  | YES | ignore |  |
| AUN3 Device Access Information | O |  |  | 9.3.3.66 |  |  |  | YES | ignore |  |

--end of changes--