3GPP TSG-RAN WG3 #127bis	             		                           R3-252109

Wuhan, China, 7th – 11th April 2025

Agenda Item:	15.2

Source:	Jio Platforms Ltd (JPL)

Title:		Seamless Feeder Link Switch-Over Support for Regenerative Payload Operation

Document for:	Discussion

# Introduction

The evolution of non-terrestrial networks (NTNs), particularly for IoT services over LEO satellite constellations, has led to increased interest in regenerative payload architectures. In such configurations, the satellite itself hosts a full LTE eNodeB (eNB) and communicates with the core network through feeder links to ground-based NTN Gateways. As these satellites move, the feeder link often needs to switch from one gateway to another.

Without enhancements to current 3GPP specifications, this feeder link switch-over process can cause temporary loss of connectivity between the satellite eNB and the EPC, resulting in disruption of signaling, dropped bearer contexts, and degraded user experience. This is especially problematic for IoT devices that depend on persistent low-data-rate sessions with minimal overhead.

To address this challenge, this proposal introduces a new subclause in TS 36.410 v18.0.0 to explicitly define the architecture and operational principles for seamless feeder link switch-over, leveraging dual S1 connectivity from the regenerative eNB to the EPC via two NTN gateways.

Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore:

- Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2, RAN4]

- Specify necessary enhancements for full eNB as regenerative payload e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3]

Note: Strive to minimise UE impact.

Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required.

RAN3 decides to work on Split MME architecture and Full CN on board architecture. Whether eNB only on board architecture is feasible or not can be discussed later.

RAN3#127:

Introduce stage 2 description of Store and Forward operation.

Introduce stage 2 description of full eNB as regenerative payload.

Introduce the S1 Removal procedure.

Check progress in NR NTN and any further impact on IoT NTN…

# Discussion

## Background on Regenerative Payloads and Feeder Links

In a regenerative NTN system, the eNB operates onboard the satellite. It interfaces with the EPC via ground gateways using S1-MME and S1-U links. These gateways serve as S1 interface endpoints and are typically connected via feeder links (e.g., Ka-band). As satellites move, maintaining continuous visibility with a single gateway is not always feasible, necessitating gateway switch-over.

## Problem Statement

During the gateway switch, if the S1 interface is momentarily lost, all active UE contexts at the MME may be released. This results in session drop, delayed re-attachment, and increased signaling overhead. While SCTP multi-homing at the transport layer offers limited resilience, it cannot fully manage the handover between logically distinct gateways.

## Architectural Requirements for Seamless Operation

To ensure service continuity:

- The eNB should be able to maintain two simultaneous S1 connections, one via each gateway.

- The MME should recognize these as paths from the same eNB (same Global eNB ID).

- One connection serves as the active S1 signaling and bearer path, while the other is prepared for immediate switch-over.

- Upon successful switch, bearer traffic and signaling can be re-routed, and the old S1 path can be gracefully released.

This make-before-break approach ensures at least one path is always available during the handover process

## Observations

### Observation 1: Existing S1 Architecture Permits Multiple Associations

The current TS 36.410 (Clause 4.1 and Clause 5) already supports the concept of multiple S1 associations between an eNB and multiple MMEs. However, it does not explicitly document the use of concurrent S1 paths between the same eNB and MME across distinct transport layers, as required for NTN feeder link resilience.

### Observation 2: Feeder Link Transitions Are Not Yet Standardized

While feeder link switch-over is functionally similar to handovers, there is currently no standard procedure to maintain EPC connectivity across distinct gateway nodes. Without clear guidance, implementations may rely on hard switch-over, leading to service interruption.

### Observation 3: SCTP Multi-Homing Is Not Sufficient

While SCTP multi-homing provides transport-layer redundancy, it is not sufficient when the eNB must transition to an entirely new gateway with a different IP address and physical interface. A dual S1 connection model at the control plane is needed to manage this transition at the application level.

### Limitation

In the current specification:

- Feeder link change may result in S1 connection teardown.

- MME releases UE contexts upon S1 interface loss.

- UE is forced to reattach, reestablishing bearers from scratch.

- This adds latency, signaling overhead, and risks data loss, especially for delay-sensitive IoT services.

## Proposed Seamless Feeder Link Switch-Over Support for Regenerative Payload Operation

In an NTN deployment with a regenerative payload, where an E-UTRAN eNodeB is located on a non-terrestrial platform (e.g. a satellite), the S1 interface architecture supports mechanisms to maintain connectivity during a feeder link switch-over.

A feeder link switch-over is the procedure by which the eNB's connection to the core network is transferred from a source satellite gateway (NTN Gateway) to a target satellite gateway.

To achieve seamless transition of the feeder link, the eNB may establish and maintain dual S1 connectivity via both the source and target gateways in parallel.

With dual S1 connectivity, the eNB simultaneously uses two S1-MME/S1-U connections: one through the current (source) gateway and one through the new (target) gateway.

This allows the eNB to continue core network signaling and user-plane traffic over one S1 path while the other path is being added or removed, thereby enabling a soft feeder link switch-over with no interruption in service.

In practice, prior to switching over, the eNB initiates an additional S1 setup via the target NTN gateway, establishing S1 interfaces to the relevant core network nodes (MME/S-GW) over the new feeder link.

After the new link is confirmed active, the eNB can seamlessly redirect traffic to use the new S1-U path (as instructed by the core network via existing S1 procedures), and finally release the S1 connection over the old feeder link.

This architecture enhancement ensures that at any given time during the switch-over, at least one S1 connection remains active between the eNB and the core network.

As a result, UEs served by the satellite eNB experience uninterrupted connectivity, with ongoing sessions maintained across the feeder link change.

If only a single S1 link were used (a hard feeder link switch-over), a brief loss of the S1 interface would occur during gateway transition; by contrast, the dual connectivity approach avoids this outage by overlapping the connections.

No changes to S1 Application Protocol or messages are required to support this feature, since the eNB's ability to have multiple concurrent S1 connections is already permitted (see subclause 4.1 and the NAS node selection function in clause 5 for multiple S1-MME support). The addition introduced in Release-19 simply applies this capability to the NTN regenerative payload scenario to fulfill feeder link switch-over requirements.

Figure 1 Seamless Feeder Link Switch-Over Support for Regenerative Payload Operation

# Conclusion and Proposals

### Proposal 1: Introduce Dual S1 Connectivity Capability for Regenerative Payloads

Define architecture enhancements that allow regenerative payload eNBs to maintain two concurrent S1 interfaces to the EPC via distinct feeder gateways. One link serves as the primary and the other as a standby or switch-over candidate. This enables the eNB to perform a soft switch-over by transitioning active signaling and bearer sessions without disconnecting from the core network.

### Proposal 2: Define Logical Behavior for MME During Dual Connection Handling

Ensure the MME logically maps both S1 connections to the same eNB context by matching the Global eNB ID. This requires no change to S1AP messages but defines how the MME should treat concurrent S1 associations. For example, the MME should avoid releasing UE contexts if one of the transport links is lost, provided the other link is still active.

### Proposal 3: Promote Make-Before-Break Strategy as Best Practice

Encourage use of make-before-break procedures, where the eNB establishes a new feeder link before releasing the old one. This allows traffic redirection to be coordinated, minimizing risk of data loss or service outage. The strategy aligns with 3GPP architectural principles and is achievable using existing signaling flows.