3GPP TSG-RAN WG3 Meeting #127bis	R3-252116

Wuhan, China, 7th – 11th April 2025

Title:	Ambient IoT support

Agenda Item:	16.2

Source:	Jio Platforms Ltd (JPL)

Document for:	Discussion

# 1	Introduction

This Change Request proposes the introduction of Ambient IoT (A-IoT) support in the NG-RAN architecture, aligned with the objectives and agreements under the approved Release-19 Work Item Ambient_IoT_solutions-Core (WID: RP-250796). Ambient IoT refers to a new class of ultra-low complexity, low-power devices such as passive or semi-passive tags and sensors, which operate without continuous battery-powered radios. Instead, these devices are energized by environmental RF sources (e.g., a Carrier-Wave (CW) signal from a base station or reader), and communicate with the network using lightweight protocols, typically via backscatter or pulse-modulated signaling. Such devices are expected to enable a wide range of 5G use cases across industrial automation, logistics, retail, supply chain tracking, and smart infrastructure.

Unlike conventional NR UEs, Ambient IoT devices do not engage in NAS registration, RRC connection establishment, or standard mobility procedures. As a result, existing 5G RAN and core architecture do not provide mechanisms for the network to detect, inventory, or command these devices. The introduction of Ambient IoT support in NG-RAN therefore requires new architectural constructs and procedures, distinct from those used for UE-based communication, but fully integrated into the existing 5G system.

This CR introduces the necessary functional architecture, procedures, and interface considerations at the NG-RAN level to support Ambient IoT devices. Specifically, it defines:

A new Ambient IoT RAN Reader function, which may be co-located within the gNB or implemented as a separate but gNB-controlled radio unit. This function is responsible for querying, energizing, and receiving data from A-IoT devices.

Support for both direct topology (gNB-integrated reader) and indirect topology (a UE acting as a reader under gNB control), offering flexibility in deployment models.

A new Inventory procedure, by which the NG-RAN scans for nearby Ambient IoT devices, collects their unique identifiers (and optional small payloads), and reports the results to the 5G Core network.

A new Command procedure, enabling the 5GC to deliver one-time downlink instructions to specific A-IoT devices (e.g. read sensor value, activate actuator, etc.), with optional feedback from the tag.

The use of non-UE-associated signaling over the NG interface (NG-C), via new NGAP procedures such as AmbientIoTInventoryRequest, AmbientIoTInventoryReport, and AmbientIoTCommandRequest/Response. These are integrated into the existing 5G architecture without requiring any new physical interfaces.

The introduction of identifiers such as Ambient IoT Device ID, Reader ID, and Ambient IoT Service Area ID, to enable granular control and device localization.

Informative guidance and placeholders (FFS) on future capabilities, such as advanced device positioning, authentication, multicast commands, or extended device telemetry.

The architecture ensures that A-IoT support is scalable, low-overhead, and flexible, catering to high-density tag deployments (hundreds to thousands of tags per cell) without overloading NG-RAN resources. It also ensures backward compatibility with existing UE procedures and avoids introducing new network node types or layers. The proposal reuses and minimally extends the NGAP protocol for A-IoT procedures, based on conclusions reached in RAN3#127 and aligned with SA2 and RAN1/RAN2 developments in Release-19.

By introducing this A-IoT support in TS 38.300 (or TS 38.401), 3GPP enables 5G to natively support battery-free and passive IoT tags, fulfilling the vision of massive, ambient sensing and tracking in future smart environments. This CR is a foundational step to ensure standardized integration of Ambient IoT in NG-RAN, supporting upcoming enhancements in PHY/MAC (TS 38.291/38.391), RF (TS 38.191), and signaling (TS 38.413) layers.

RAN3 scope:

Specify necessary architectural aspects, and signaling and procedures between A-IoT RAN and A-IoT CN to support the A-IoT functions, assuming an architecture of aggregated gNB, including:

- Inventory and command operations

Note: The above A-IoT functions are supported over the existing NG interface, based on architecture(s) defined by RAN3/SA2.

RAN3#127:

RAN3 agrees to support both the direct and indirect options for the A-IoT RAN node to communicate with AIoTF in Topology 1.

WA: Take Option1 (Including AIoTF information in NGAP).

WA: The AIoT RAN node may only provide the AIoT radio.

AIoTF obtain the AIoT RAN information via OAM configuration. The AIoT RAN information includes supported AIoT area, served reader ID list.

AIoT RAN node receives the requested service area information (encoded as area and/or reader ID list) from AIoT CN.

Architecture aspects:

AIoT RAN node is an aggregated gNB. And this gNB may or may not only provide the A-IoT radio.

AIOTF may be aware of the location of reader via OAM configuration.

FFS: The details of the location of reader (e.g., room number, GNSS info) are FFS, and it is FFS on signaling support.

FFS: To support selection of RAN node by AIOTF, define a new A-IoT Area, or use Tracking Area dedicated for A-IoT?

Upon receiving only the area in Inventory Request, the gNB selects readers within the indicated area.

Upon receiving neither the area nor the reader list in Inventory Request, the gNB selects all the served readers.

WA: Upon receiving only the reader list in Inventory Request, the gNB selects the readers indicated by the reader list.

FFS: Upon receiving both the area and the reader list in Inventory Request, the gNB selects the readers within the indicated area and the readers within the reader list.

About Inventory Procedures:

WA: Introduce a NGAP Class 1 Inventory Request procedure.

Introduce INVENTORY REQUEST message, which includes Inventory Request Transfer IE.

Introduce INVENTORY RESPONSE message, which includes Inventory Response Transfer IE.

Introduce INVENTORY FAILURE messages, which includes Inventory Failure Transfer IE.

Introduce a NGAP Class 2 Inventory Report procedure.

Introduce INVENTORY REPORT message, which includes Inventory Report Transfer IE.

The above Transfer IEs are transparent to AMF in case of indirect communication.

FFS: For Inventory related messages, the inclusion of Correlation Identifier and AIOTF Identifier.

About Command Procedure:

WA: Introduce a NGAP Class 1 Command Request procedure

Introduce COMMAND REQUEST message, which includes Command Request Transfer IE.

Introduce COMMAND RESPONSE message, which includes Command Response Transfer IE.

Introduce COMMAND FAILURE messages, which includes Command Failure Transfer IE.

The above Transfer IEs are transparent to AMF in case of indirect communication.

FFS: For Command related messages, the inclusion of Correlation Identifier and AIOTF Identifier.

FFS: For command related messages, whether the Correlation Identifier IE if included is the same one as in the related Inventory procedure?

FFS: Command request procedure is a per device procedure or to be used for multiple devices?

FFS: The need for a Command Report procedure?

FFS: Device association between gNB and AIOTF is used in Command procedure.

About Interface management procedures:

FFS: In indirect path communication, existing NG interface management procedures are used between the gNB and the AMF.

FFS: In direct path communication, existing NG Setup, RAN Configuration Update, NG Reset, Error Indication procedures are used between the gNB and the AIOTF.

FFS: For interface management procedures, if reuse existing ones, how to handle the not applicable mandatory IEs, and how to handle A-IoT only support case.

Work on the stage2/3 details and open issues....

# Discussions

To incorporate Ambient IoT supportv.

## X.0 Ambient IoT Support in NG-RAN – Overview:

A brief introduction of Ambient IoT in Release-19, its purpose and scope. For example: “Ambient IoT refers to ultra-low complexity tags that harvest energy from a CW signal and communicate via the NR air interface​. This subclause introduces how NG-RAN supports discovery (inventory) and command of Ambient IoT devices as part of the 5G system.” (A short paragraph, giving context and referencing the overall architecture.)

## X.1 Architecture and Network Elements:

Describe any new or modified network elements for Ambient IoT. For instance, clarify that Ambient IoT tags are not full UEs but are managed by the gNB. Mention the role of a Carrier-Wave (CW) node if applicable (e.g. a CW node provides continuous wave power and may be co-located with a gNB or controlled via O&M). Also state that the gNB handles Ambient IoT inventory and command functions, interfacing with the 5G Core via NGAP. This section should be kept high-level (stage-2 view), possibly with a simple figure (if allowed) or text describing the relationships. It can reference TS 38.401 for further architectural details. For example: “The NG-RAN architecture is enhanced to support Ambient IoT devices. The gNB shall implement an Ambient IoT Controller function responsible for scanning and communicating with Ambient IoT tags. In some deployments, a dedicated CW node may be used to illuminate tags with RF energy; this node is managed by the NG-RAN (or via OAM) but does not have an NG interface.” (This could be split into X.1.1, X.1.2 if multiple topics like “CW Node” need separate treatment.)

## X.2 Ambient IoT Inventory Procedure:

This subclause would detail the procedure for discovering and inventorying Ambient IoT devices. It should be written in an ordered list form to reflect the step-by-step process, for example:

  - Initiation: The gNB initiates an Ambient IoT scan (either periodically or triggered by the core network). This may involve the gNB transmitting a wake-up signal or beacon to energize nearby Ambient IoT tags.

  - Tag Response: Ambient IoT tags in the cell respond with identification messages (using the A-IoT MAC/PHY protocols defined in TS 38.291/38.391) containing their unique ID and possibly status information.

  - Collection: The gNB collects the responses. (If numerous tags respond, the MAC layer may use ALOHA or polling – but those details are covered in the MAC spec. Here we assume the gNB successfully receives a set of tag IDs.)

  - NGAP Report: The gNB sends an Ambient IoT Inventory Report to the 5G Core over NGAP. Working assumption in RAN3 is that this is done via a new NGAP message (or IE) rather than a completely new interface​. The NGAP Inventory Report contains a list of discovered Tag IDs (and any associated data).

  - Core Handling: The AMF (or other core function as defined by the core WID) receives the inventory information. It may forward the list to an IoT application server or trigger other actions (core network aspects are defined in the SA2 spec, outside the scope of TS 38.300).

Each step above would be described in a sentence or two, written in the imperative “shall” style as appropriate for normative text. Numbered lists make the procedure clear and conform to 3GPP’s typical stage-2 presentation (similar to how, say, small data transmission or handover procedures are listed).

## X.3 Ambient IoT Command Procedure:

This subclause describes how the network can send a command or downlink data to an Ambient IoT tag. Again, an enumerated sequence is appropriate. For example:

  - Command Trigger: An external application or core network function determines a command should be sent to a specific Ambient IoT tag (identified by its ID). The core network triggers the gNB via NGAP. This could use a new NGAP message, e.g. A-IoT Command Request, carrying the Tag ID and the command payload (for example, an instruction to toggle a sensor or an acknowledgment request).

  - Paging/Notification: Upon receiving the command from the core, the gNB schedules a downlink Ambient IoT command to the target tag. Since Ambient IoT tags don’t maintain an RRC connection, the gNB may use a specialized paging or broadcast mechanism to address the tag (e.g. using the tag’s ID in a MAC layer address field).

  - Tag Reception: The Ambient IoT tag, upon being energized by the CW signal, listens for its command and executes it (for instance, activating an LED, or returning some data).

  - Optional Response: If the procedure requires a response (e.g. the tag sends back a small acknowledgment or sensor reading), the tag transmits a reply which the gNB receives. The gNB then forwards any reply to the core (likely via another NGAP message such as A-IoT Command Feedback).

Like the inventory procedure, these steps would be written out as normative behavior. The use of sub-bullets or sub-subclauses can clarify any alternate flows or failure cases (for instance, if a tag doesn’t respond, the gNB may retry or report a failure).

## X.4 NGAP Signalling and Information Elements:

In this section, we enumerate the new signaling additions to support Ambient IoT, aligning with 3GPP conventions for procedure specification. This can be done with a concise table or list of messages. For example, a table could summarize:

| New NGAP Message/IE | Purpose | Reference |
| --- | --- | --- |
| AmbientIoTInventoryReport (NGAP message) | GNB -> AMF: Conveys list of discovered Ambient IoT device IDs and attributes. | (Defined in TS 38.413) |
| AmbientIoTCommandRequest (NGAP message) | AMF -> GNB: Requests the gNB to send a downlink command to a specific Ambient IoT tag. | (TS 38.413) |
| AmbientIoTDeviceID (IE) | New Information Element used to identify an Ambient IoT device (could be, e.g., a 48-bit ID). | (TS 38.413 ASN.1) |

- The actual definitions would be in the stage-3 spec TS 38.413, but listing them here (in TS 38.300) provides context. We should also mention any needed alignment with core network procedures (e.g. the AMF may need to handle these messages – which is covered in 3GPP TS 23.502 stage-2 procedures). This subclause essentially bridges the stage-2/stage-3, ensuring the reader knows what new signals are introduced for A-IoT. It should follow typical 3GPP editorial style: succinct descriptions, possibly referencing an ASN.1 structure for the new IE (without reproducing it fully in stage-2).

## X.5 (Optional) Parameter Formats and Miscellaneous:

- If there are any special parameters (for instance, a timing or power parameter related to A-IoT operation), this could be captured in either a small table or an ASN.1 snippet. However, since most of the low-level parameters (like tag ID lengths, frequency used for CW, etc.) would be in the RRC/MAC/PHY specs, TS 38.300 might not need a dedicated parameters section. It could simply refer to the relevant specs. Any O&M or performance requirements can be briefly mentioned if needed (or left to 38.401/OAM specs).

Using the above structure, the long narrative text would be broken down into logical, labeled subclauses. Each subclause addresses a specific aspect of Ambient IoT support, which aligns with standard 3GPP drafting practice (e.g., compare to how NR RedCap or NTN support were organized in short subclauses​). The procedures are presented as ordered lists for clarity, and key information is summarized in tables. This not only follows 3GPP’s editorial conventions but will make the CR much easier to read and approve. The resulting CR would clearly show the insertion of the new clause X in TS 38.300 (or TS 38.401) with all the required sub-sections, rather than a wall of text, thereby aligning the documentation with the style used in existing features and the agreements from RAN3#127 and the Ambient IoT study conclusions​

# Conclusion

### Proposal 1: Introduce a new clause (Clause 22) in TS 38.300 [Rel-19] titled “Ambient IoT Support in NG-RAN”

This clause will define:

- The NG-RAN functional architecture for Ambient IoT support, including the role of the gNB-integrated A-IoT RAN Reader, optional CW node, and control of UE-based readers in indirect topology.

- The A-IoT Inventory Procedure, allowing the gNB to discover and report A-IoT Device IDs (and optional metadata) upon request from the 5GC.

- The A-IoT Command Procedure, allowing the gNB to deliver downlink command instructions to individual Ambient IoT devices with optional tag response.

- The use of Ambient IoT Device ID, Reader ID, and Ambient IoT Service Area ID for addressing, targeting, and scoping of inventory and command operations.

- Deployment support for both direct (gNB-integrated) and indirect (UE-assisted) topologies.

- References to NGAP procedures defined in TS 38.413 and relevant physical/MAC layer behavior in TS 38.291/38.391.

- Forward-looking FFS (For Further Study) notes for items such as UE-based reader control, A-IoT device authentication, group commands, device-initiated reports, and uplink data transfer enhancements.

This clause provides the high-level description and procedural framework for Ambient IoT support within the overall NG-RAN architecture, targeting implementation in commercial Rel-19 5G systems.

### Proposal 2: Optionally, introduce a corresponding architectural clause in TS 38.401 [Rel-19]

This clause would describe:

- The logical and physical architecture of the A-IoT RAN Reader, including management of external reader units (e.g. CW nodes or distributed antennas) under a gNB.

- Handling of OAM configuration for A-IoT reader parameters and service area definitions.

- Internal gNB interfaces (e.g., between DU/CU and A-IoT reader elements) if standardized in the future.

- Coordination between A-IoT reading and conventional NR scheduling to avoid resource conflict or interference.

This architectural clause would provide complementary detail to the stage-2 procedure description in TS 38.300, capturing any new or refined logical node behaviors or reference points related to A-IoT introduced in the RAN architecture.