3GPP TSG-RAN WG3 #127-bis                                   R3-252152

Wuhan, China, 07-11 April, 2025

Agenda item:	13.2

Source:	Samsung

Title:	Discussion on inter-gNB-CU LTM

Document for:	Discussion & Decision

# 1 Introduction

RAN3 had some discussions on inter-CU LTM in previous meetings, and below agreements are achieved:

RAN3#126: RAN3 move forward on Legacy framework with PDCP change/switch for inter-CU LTM in this release, not considering the PDCP anchor based solution. The source CU can request candidate CU to provide CSI-RS configuration in HANDOVER REQUEST message, and candidate CU signals the CSI-RS configuration in HANDOVER REQUEST ACKNOWLEDGEMENT message. The source CU generates common CSI-RS Resource Configuration and sends it to candidate CU in LTM CONFIGURATION UPDATE message, the candidate CU signals the CSI-RS Report Configuration in LTM CONFIGURATION UPDATE ACKNOWLEDGEMENT message. Turn the WA into agreement: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell. To support subsequent LTM, the LTM Configuration Update procedure is reused to establish UE association between the new source gNB and the other candidate gNB(s) after each inter-CU LTM Cell Switch. Confirm the message name as LTM Cancel for candidate gNB-initiated LTM cancellation. Turn the WA into agreement: Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exists in the source gNB. Late data forwarding may be initiated after the source gNB decides to trigger the LTM Cell Switch Command to the UE, when exactly it is initiated is left to implementation? Continue to work on the solutions comparison and details… RAN3#127: Inter-CU LTM The candidate gNB-CU responds the full SSB Time/Frequency Configuration (in SSB Information IE) of candidate cells to the source gNB-CU in the Handover Request ACK message. The source gNB-CU sends the pair of (gNB ID, new ID of early RACH configuration resource (to be further discussed)) to the candidate gNB-CU to request early RACH configuration. The format of the new introduced IE is same with the gNB-DU ID. WA: Introduce a new non-UE associated class 2 procedure on Xn, namely TA information Transfer message, to transfer the TA information from the candidate gNB-CU to the source gNB-CU. The source gNB can generate reference configuration and provide a reference configuration for LTM in a Handover Request and LTM configuration update message. The candidate gNB indicates whether a LTM candidate configuration is a complete candidate configuration in the Handover Request Acknowledge and LTM configuration update acknowledge message. FFS on whether the source gNB can request a candidate gNB to provide a reference configuration. WA: Use a single UE association for multiple LTM handover request to the same candidate gNB. Reuse the LTM Configuration Update procedure to sync up configurations among candidate gNBs for subsequent LTM, including early sync configuration, configuration ID, and data forwarding addresses, etc. It is up to the network implementation when the sync up is performed: during the preparation step, or during the cell switch execution step, or after successful cell switch. Normal data forwarding may be initiated after the source gNB decides to trigger the LTM cell switch for the UE, and when exactly it is initiated is left to implementation. Inter-CU LTM with DC RAN3 agree the following scenarios to support LTM with NR-DC: 1.SN initiated inter-CU SCG LTM without MCG changes (high priority) 2.Inter-CU MCG LTM without SN release 3.Inter-CU MCG LTM with SN release 4.Inter-CU MCG LTM with SN addition For SN initiated inter-CU SCG LTM, the source SN initiates the inter-CU SCG LTM preparation procedure by sending an SN Change Required message to the MN. The MN requests each candidate SN to allocate resources for the UE via SN Addition request message. Within the list of cells suggested by the source SN, the candidate SN provides the SCG part configuration of each candidate PSCell and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL sync configuration or TCI-state configuration, to the MN, via SN addition request ACK message. In order to support subsequent inter-CU SCG LTM, the MN needs to transfer the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s), via SN modification request message. Accordingly, the candidate SN(s) responds with the updated candidate SCG configuration to the MN via SN modification request ACK message. The Cell Switch Notification message can be reused from the source SN to the target SN via the MN. The detailed IE can be further discussed.

In this paper, we will further discuss the basic procedure for inter-gNB-CU LTM based on agreed BLCR in [1], [2], and discuss the potential issues on Xn/F1 interface.

# 2 Discussion

To achieve the analysis for network interface impact for the scenario of inter-gNB-CU LTM, we draw the potential message flows of the whole inter-gNB-CU LTM process as shown in the below Fig.1 to provide the basic description for each involved message flows and our view for the issues raised in stage 2 BLCR in [1] and [2]. After that, we finally summarize our proposals.

Fig.1 Message Flows for Inter-gNB-CU LTM

The procedure for inter-gNB-CU LTM is as follows:

1.	The UE sends a MeasurementReport message (L3 measurement result) to the source gNB-DU containing measurements of inter-gNB neighbouring cells. The source gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

2.	The source gNB-CU determines to initiate inter-gNB-CU LTM configuration.

3.	The source gNB requests inter-gNB LTM for one or more candidate cells belonging to one or more candidate gNB(s). The source gNB initiates a HANDOVER REQUEST message per candidate cell containing one candidate cell ID and the CSI resource configuration for subsequent LTM. The source gNB may request the candidate gNB(s) to provide the CSI-RS configuration.

4.  The candidate gNB-CU sends a UE CONTEXT SETUP REQUEST message to the candidate gNB-DU(s) for each candidate cell to request LTM configurations.

5.	If the candidate gNB-DU accepts the request of LTM configuration, it responds with a UE CONTEXT SETUP RESPONSE message including the LTM lower layer RRC configurations.

6.  The candidate gNB(s) prepares the LTM configuration(s) and sends inter-gNB response (HO REQUEST ACKNOWLEDGE) to the source gNB including the generated RRC configurations for the accepted candidate cell. The candidate gNB(s) may also include the CSI-RS configuration upon request.

7.	The source gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source gNB-DU including the information related to early sync and the LTM configuration IDs for the accepted target candidate cell(s) in other candidate gNB-CU(s).

8.	The source gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message which includes an updated lower layer configuration, e.g., containing the updated CSI report configuration of the source cell.

9.	The source gNB sends an LTM CONFIGURATION UPDATE to the candidate gNB(s) to update the LTM configurations of candidate cell(s). The source gNB may include the common CSI-RS resource configuration.

10.	The candidate gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the candidate gNB-DU(s) containing the information of candidate cells for subsequent LTM. The candidate gNB-CU may include the common CSI-RS resource configuration.

11.	The candidate gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message. The candidate gNB-DU may also provide the CSI-RS report configuration.

12.	The candidate gNB(s) sends the LTM CONFIGURATION UPDATE ACKNOWLEDGE message to the source gNB. The candidate gNB(s) may also provide the CSI-RS report configuration.

Editor’s Note: FFS on whether step 9 and 12 are mandatory or optional.

NOTE: Step 9 may also be triggered after step 24, or after step 32 by implementation for subsequent LTM.

13.	The source gNB-CU sends a DL RRC MESSAGE TRANSFER message to the source gNB-DU, which includes the generated RRCReconfiguration message with the LTM configuration.

14.	The source gNB-DU forwards the received RRCReconfiguration message to the UE.

15.	The UE stores the LTM candidate configurations and sends an RRCReconfigurationComplete message to the source gNB-DU.

16.	The source gNB-DU forwards the RRCReconfigurationComplete message to the source gNB-CU via an UL RRC MESSAGE TRANSFER message.

17. If early data forwarding is applied, the source gNB sends the EARLY STATUS TRANSFER message to other candidate gNB(s).

NOTE: Early data forwarding can be triggered before the source gNB triggers a MAC CE Cell Switch Command to the UE to change cell, timing is left to implementation.

18.	Early synchronization to the target candidate cell(s) may be performed.

19. The candidate gNB-DU sends a DU-CU TA INFORMATION TRANSFER message to the candidate gNB-CU, which includes the TA value, the associated PRACH resource information, and etc.

20.	The candidate gNB-CU forwards the TA value and the associated CFRA resource information to source gNB-CU via the TA INFORMATION TRANSFER message.

Editor’s Note: Details are FFS on step 20.

21. The source gNB-CU forwards the TA value, and the associated CFRA resource information to the source gNB-DU in the CU-DU TA INFORMATION TRANSFER message.

22.	The UE performs L1 measurements on the configured LTM candidate cell(s) and transmits L1 measurement reports to the source gNB-DU.

23.	The source gNB-DU determines to initiate inter-gNB-CU LTM.

24.	The source gNB-DU decides to execute cell switch to a target cell and transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID which indicates the index of the candidate configuration of the target cell, a beam indicated with a TCI state or beams indicated with DL and UL TCI states, and a timing advance command for the target cell, if available. The UE switches to the target cell and applies the candidate configuration indicated by the target configuration ID.

25.	The source gNB-DU sends the DU-CU CELL SWITCH NOTIFICATION message to source gNB-CU to indicate the initiation of the Cell Switch Command to the UE including the target cell ID, the TCI state ID(s), and the TA values for subsequent LTM.

26.	The source gNB-CU sends the CELL SWITCH NOTIFICATION message to the target gNB-CU to indicate the initiation of Cell Switch command to the UE.

27.	The target gNB-CU forwards the target cell ID, the TCI state ID, and the TA values for subsequent LTM to the target gNB-DU in the CU-DU CELL SWITCH NOTIFICATION message.

28.	The target gNB-DU detects the UE access.

29.	The target gNB-DU sends the ACCESS SUCCESS message to the target gNB-CU with the target cell ID.

30.	The UE sends an RRCReconfigurationComplete message to the target gNB-DU.

31.	The target gNB-DU forwards the RRCReconfigurationComplete message to the target gNB-CU via an UL RRC MESSAGE TRANSFER message.

32/33. The target gNB-CU sends the HANDOVER SUCCESS message to the source gNB-CU to inform that the UE has successfully accessed the target cell. In return, the source gNB-CU sends the SN STATUS TRANSFER message following the principles described in step 7 of Intra-AMF/UPF Handover in clause 9.2.3.2.1.

NOTE: Late data forwarding may be initiated as soon as the source gNB receives the HANDOVER SUCCESS message. Up to implementation, late data forwarding and SN Status Transfer may be initiated once the source gNB triggers the inter-gNB LTM cell switch for the UE in Step 24.

34. The target gNB-CU sends a PATH SWITCH REQUEST message to 5GC to trigger 5GC to switch the DL data path towards the target gNB-CU and to establish an NG-C interface instance towards the target gNB-CU.

35.	The 5GC confirms the PATH SWITCH REQUEST message with the PATH SWITCH REQUEST ACKNOWLEDGE message to target gNB-CU.

36.	The new source gNB (i.e., the target gNB) sends the LTM CONFIGURATION UPDATE message to the candidate gNBs.

Editor’s Note: The new source gNB may initiate to setup the UE associated Xn signalling connection with the candidate gNBs. Details are FFS.

37. The candidate gNB(s) responds the LTM CONFIGURATION UPDATE ACKNOWLEDGE message to the new source gNB.

38. The new source gNB may send the UE CONTEXT RELEASE message to inform the old source gNB to release radio and C-plane related resources associated to the UE context if no LTM candidate cell(s) exist in the old source gNB. Any ongoing data forwarding may continue.

39.	The source gNB-CU may send the UE CONTEXT RELEASE COMMAND message to the source gNB-DU to release the resources of source cell.

40.	The source gNB-DU responds with a UE CONTEXT RELEASE COMPLETE message.

NOTE: The steps 17-40 can be performed multiple times for subsequent LTM cell switch executions.

From the above overview of inter-gNB-CU LTM procedure, the impacts on Xn/F1 interface are mainly reflected in the following aspects:

## Issue 1: Early TA RACH resource allocation

In RAN3#127 meeting, the following agreement was reached on this issue:

- The source gNB-CU sends the pair of (gNB ID, new ID of early RACH configuration resource (to be further discussed)) to the candidate gNB-CU to request early RACH configuration. - The format of the new introduced IE is same with the gNB-DU ID.

From the RAN3’s agreements, we note that early RACH configuration of each candidate cell is allocated based on gNB-DU which is replaced with an implicit ID(new ID of early RACH configuration resource) in XnAP specification. And the dedicated different early RACH configuration is allocated by candidate cell for each candidate gNB-DU. RACH configuration could be shared only among the UEs in a single gNB-DU to avoid the RACH access conflict between UEs from different gNB-DU.

However, another case is existing and if one LTM gNB is not CU-DU split architecture, how does source gNB request early RACH configuration for this LTM gNB to each candidate cell? We think RACH configuration allocation based on gNB-DU cannot be achieved, should be based on gNB in case of aggregated gNB.

Therefore, we should announce below mechanism on early RACH configuration allocation.

- Case#1: If one LTM gNB is CU-DU split architecture, early RACH configuration of candidate cell is allocated based on gNB-DU.

- Case#2: If one LTM gNB is not CU-DU split architecture, i.e., aggregated gNB, early RACH configuration of candidate cell is allocated based on gNB.

Proposal 1-1: RAN3 support the early TA RACH resource allocation is based on gNB-DU in case of split gNB and/or based on gNB in case of aggregated gNB.

Additionally, in case#1, we should note before the step that the source gNB-CU sends the pair of (gNB ID, new ID of early RACH configuration resource) to the candidate gNB-CU to request early RACH configuration is initiated from the source gNB-CU, the source gNB-CU should be aware of the gNB-DU ID of each candidate cell belonging.

However, how does source gNB-CU acquire the gNB-DU ID information to which inter-CU candidate cell belongs? Review current specification as below highlight from TS 38473, it cannot be achieved via Cell ID information due to no connection between gNB-DU ID and cell identifiers.

9.3.1.9 gNB-DU ID [F1 IE] The gNB-DU ID uniquely identifies the gNB-DU at least within a gNB-CU. IE/Group Name; Presence; Range; IE type and reference; Semantics description. gNB-DU ID; M; INTEGER (0 .. 236-1); The gNB-DU ID is independently configured from cell identifiers, i.e. no connection between gNB-DU ID and cell identifiers..

Therefore, RAN3 should discuss this issue on how does source gNB-CU acquires the gNB-DU ID information to which inter-CU candidate cell belongs?

Proposal 1-2: RAN3 should discuss how source gNB-CU acquires the gNB-DU ID information to which inter-CU candidate cell belongs before initiation of LTM preparation.

Based on the mechanism of early TA RACH resource allocation proposed by Proposal 1-1, in the Case#2: If one LTM gNB is not CU-DU split architecture, i.e., aggregated gNB, early RACH configuration of candidate cell is allocated based on gNB. In case that source gNB is aggregated gNB, and one candidate gNB with one candidate cell is CU-DU split gNB. When source gNB initiates the HO REQ to candidate gNB with Early Sync Information Request IE for requesting early TA RACH resource from candidate cell, the IE of Early RACH Resources Requester ID seem to be reasonable as an optional IE since source gNB is an aggregated gNB.

Below IEs are from BLCR in [2] #### 9.2.1.xx3 Early Sync Information Request This IE contains a request for Early synchronization information. Editor’s note: Details on IEs need to be continued. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Early Sync Configuration Request; M; ENUMERATED (true, …). Early RACH Resources List; 0..1. >Early RACH Resources Item; 1..< maxnoofEarlyRACHResourcesID>. >>Global gNB ID; M; 9.2.2.1. >>Early RACH Resources Requester ID; M; 9.2.1.xx9; Identifies the entity requesting Early RACH resources.. Range bound; Explanation. maxnoofEarlyRACHResourcesID; Maximum no. of Early RACH resources identities allowed to be configured with LTM towards one UE, the maximum value is 8..

Furthermore, when candidate gNB-CU initiates UE Context Setup Request to candidate gNB-DU with Early Sync Information Request IE for requesting early TA RACH resource from candidate cell, it will introduce another issue if reusing current IE tabular on LTM gNB-DUs List IE. LTM gNB-DU ID IE is a mandatory IE for intra-CU LTM but it is possible to be an optional IE for inter-CU LTM in case source gNB is an aggregated gNB.

Regarding the DU-CU TA INFORMATION TRANSFER message, the same issue is existing in current IE tabular, and the mandatory IE on the Source gNB-DU ID IE is an optional information for inter-CU LTM in case source gNB is an aggregated gNB.

Below IEs are from BLCR in [3] Early Sync Information Request; 0..1; YES; ignore. >Request for RACH Configuration; O; ENUMERATED (true, …); -. >LTM gNB-DUs List; 0..1; This IE contains the IDs of the source gNB-DU and candidate gNB-DU(s).; YES; reject. >>LTM gNB-DUs Item IEs; 1..< maxnoofLTMg NBDUs>. >>>LTM gNB-DU ID; M; gNB-DU ID 9.3.1.9. >>>LTM gNB ID; O; Global gNB ID 9.3.1.305; -. #### 9.2.1.24 DU-CU TA INFORMATION TRANSFER This message is sent by the gNB-DU to inform the gNB-CU about TA information. Direction: gNB-DU gNB-CU IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.3.1.1; YES; ignore. Transaction ID; M; 9.3.1.23; YES; reject. DU to CU TA Information List; 0..1; YES; ignore. >DU to CU TA Information Item IEs; 1 .. <maxnoof TAList>; EACH; ignore. >>Candidate Cell ID; M; NR CGI 9.3.1.12; -. >>TA Value; M; INTEGER (0..4095); Indicates the TA value as defined in TS 38.213 [31].; -. >>Preamble Index; M; INTEGER (0..63); -. >>RA-RNTI; M; INTEGER (0..65535, ...); RA-RNTI as defined in TS 38.321 [16].; -. >>Source gNB-DU ID; M; gNB-DU ID 9.3.1.9; -. >>Tag ID Pointer; O; OCTET STRING; Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [8].; -. >>Source gNB ID; O; Global gNB ID 9.3.1.305; -.

Therefore, RAN3 should re-think the F1/Xn IEs design on early TA RACH resource request & allocation & TA information transfer. One simple way is that define the related gNB-DU ID IE to an optional IE in the Rel-19 specification.

Proposal 1-3: RAN3 should define F1 gNB-DU ID IE to be an optional IE in the Rel-19 specification on the procedure of early TA RACH resource request & allocation & TA information transfer.

## Issue 2: FFS the message type and detail in TA INFORMATION TRANSFER message

Regarding early TA information transfer from candidate gNB-DU to source gNB-DU via candidate gNB-CU and source gNB-CU in step 20,

We already agreed to introduce a new TA INFORMATION TRANSFER message to transfer the TA information. And the WA is achieved on message type last meeting as below:

− WA: Introduce a new non-UE associated class 2 procedure on Xn, namely TA information Transfer message, to transfer the TA information from the candidate gNB-CU to the source gNB-CU.

Generally, since R18 F1AP DU-CU/CU-DU TA INFORMATION TRANSFER message is a non-UE associated message, to inherit the legacy principle, we suggest to turn this WA into agreement and define XnAP TA INFORMATION TRANSFER message as a non-UE associated message.

Proposal 2-1: Propose to turn WA into agreement that introduce a new non-UE associated class 2 procedure on Xn, namely TA information Transfer message, to transfer the TA information from the candidate gNB-CU to the source gNB-CU.

Furthermore, following the current detail IEs in F1AP DU-CU/CU-DU TA INFORMATION TRANSFER, we still proposal to include a candidate cell list that include related TA information and rach resource information for each candidate cell. Additionally, if source gNB is a split gNB and early TA RACH resource is allocated based on gNB-DU as mentioned in the issue#1, the implicit gNB-DU ID should be indicated in the XnAP TA INFORMATION TRANSFER message to inform source gNB-CU to forward TA information to the indicated source gNB-DU.

Proposal 2-2: Propose to include below information in XnAP TA INFORMATION TRANSFER message.

- TA Information List

  - Candidate Cell ID

  - TA value

  - Preamble Index

  - RA-RNTI

  - Early RACH Resources Requester ID (optional IE, present in case of split gNB, implicit indicate source gNB-DU ID)

## Issue 3: Stage 3 IEs design on Data forwarding Info in LTM Configuration Update message

For supporting data forwarding in the subsequent inter-CU LTM scenarios, RAN3 have reached the following agreement in last meeting:

Reuse the LTM Configuration Update procedure to sync up configurations among candidate gNBs for subsequent LTM, including early sync configuration, configuration ID, and data forwarding addresses, etc. It is up to the network implementation when the sync up is performed: during the preparation step, or during the cell switch execution step, or after successful cell switch.

From the agreement, we know that the data forwarding addresses information is sent to other candidate gNBs or target gNB via the LTM Configuration Update message. And post last RAN3 meeting, the merged XnAP BLCR in [2] is archived, we can see the Data Forwarding Info IE is included in the LTM Configuration Update message, and a note is indicated that FFS if List is needed. Therefore, we will continue to discuss this issue on FFS the list is needed and FFS the details on data forwarding information IE.

#### 9.1.1.x3 LTM CONFIGURATION UPDATE This message is sent by the NG-RAN node1 to update LTM configuration data. Direction: NG-RAN node1  NG-RAN node2. Editor’s note: Details on IEs need to be continued. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.2.3.1; YES; reject. NG-RAN node 1 UE XnAP ID; M; NG-RAN node UE XnAP ID 9.2.3.16; Allocated at the NG-RAN node 1; YES; reject. NG-RAN node 2 UE XnAP ID; O; NG-RAN node UE XnAP ID 9.2.3.16; Allocated at the NG-RAN node 2; YES; reject. Target Cell Global ID (FFS); M; NR CGI 9.2.2.7; YES; reject. LTM Information Request; O; 9.2.1.xx1; YES; ignore. Early Sync Information Request; O; 9.2.1.xx3; YES; ignore. Early Sync Information; O; F9.2.1.xx6; YES; ignore. Early Sync Information for SUL; O; F9.2.1.xx6; YES; ignore. Data Forwarding Info; FFS if List is needed; 9.2.1.16; YES; ignore.

During the LTM preparation phase, source gNB collects data forwarding address information from each candidate gNB using Ho Req Ack based on each candidate cell. Therefore, it is obvious that data forwarding information is a list including multiple data forwarding addresses from each candidate gNB/cell.

Proposal 3-1: Data Forwarding Information IE is a list including multiple data forwarding addresses from each candidate gNB/cell.

And FFS the details on Data Forwarding Information IE, review the current the specification in XnAP TS 38.423, the basic principle is that one data forwarding address is allocated based on each SN in case of DC, but different data forwarding address can be allocated based on each candidate PCell in case of SA, therefore, we propose that data forwarding address per-candidate PCell is reasonable for inter-CU LTM.

Proposal 3-2: Regarding inter-CU LTM, Data forwarding address is based on candidate PCell not candidate gNB.

How to express the data forwarding address for each candidate PCell, generally using Data Forwarding Info from target NG-RAN node IE in the previous examples. Since this IE is per-PDU session, therefore, we can design the Data Forwarding Information IE as below:

9.2.1.xx Data Forwarding Info List This IE contains different data forwarding addresses information per-PDU session for different candidate LTM cells. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Data Forwarding Info List; 1; YES; ignore. >Data Forwarding Info Item; 1.. <maxnoof LTMCells>; –. >>LTM Cell ID; M; NR CGI 9.2.2.7; –. >>PDU Session Resources List; 1; –. >>>PDU Session Resources Item; 1. .<maxnoofPDU Sessions>; –. >>>>PDU Session ID; M; 9.2.3.18; –. >>>>Data Forwarding Info from target NG-RAN node; M; 9.2.1.16; –. Range bound; Explanation. maxnoofLTMCells; Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8.. Range bound; Explanation. maxnoofPDUSsessions; Maximum no. of PDU sessions. Value is 256.

Proposal 3-3: RAN3 to agree to use Data Forwarding Info from target NG-RAN node IE per-PDU session per-candidate Cell as the included details in the Data Forwarding Info List IE.

## Issue 4: FFS TA Information List IE in Cell Switch Notification message

Regarding Cell Switch Notification from source gNB-DU to candidate gNB-DU via source gNB-CU and candidate gNB-CU, we already agree to introduce a new XnAP UE-associated procedure, namely Cell Switch Notification message.

Following the current detail IEs in F1AP DU-CU CELL SWITCH NOTIFICATION in R18 intra-CU LTM, since TA values of candidate cells may be also forwarded from source gNB-DU to candidate gNB-DU for subsequent LTM use, we have a below proposal.

Proposal 4: TA Information List IE can be included in XnAP Cell Switch Notification message to forward the valid TA values of candidate cells for reuse in subsequent LTM.

## Issue 5: FFS how to identify the UE for the candidate gNB when the new source gNB initiates to setup the UE associated Xn signalling connection with the candidate gNBs

In RAN3#126 meeting, below agreement is reached to support subsequent LTM, and the LTM Configuration Update procedure is reused to establish UE association between the new source gNB and the other candidate gNB(s) after each inter-CU LTM Cell Switch.

To support subsequent LTM, the LTM Configuration Update procedure is reused to establish UE association between the new source gNB and the other candidate gNB(s) after each inter-CU LTM Cell Switch.

In plus, we need to consider how to identify the same UE when candidate gNB receives the LTM Configuration Update message from new source gNB. As below figure shown, when the C-gNB2 (new source gNB2) initiates to establish UE association with C-gNB3, sending LTM Configuration Update carrying one UE ID reference info for the candidate gNB to identify the same UE.

Through the discussion in last meeting, the below information can be included as the related UE ID reference info for the candidate gNB to identify the same UE.

Opt.1 UE XnAP ID allocated by the candidate gNB3 for the source gNB1

Opt.2 UE XnAP ID allocated by the last source gNB1(also including source gNB1 ID)

Regarding the Opt.1, S-gNB1 need transfer the UE XnAP ID allocated by other candidate gNBs(including C-gNB3) to the new source gNB2 via Cell Switch Notification or other message(LTM Configuration Update), then new source gNB2 can carry the corresponding UE XnAP ID information to C-gNB3. This solution has more impact.

Regarding the Opt.2, C-gNB2 (new source gNB2) knows the UE XnAP ID allocated by the last source gNB1 and source gNB1 ID information, therefore, this solution has less impact.

Proposal 5: Introduce last source gNB UE XnAP ID/source gNB ID in LTM Configuration Update message from new source gNB to other candidate gNBs, so that candidate gNB can identify the same UE and setup the UE associated Xn signalling connection.

## Issue 6: Enhancement for the LTM candidate cells cancel via the procedure of LTM Configuration Update/Acknowledge

Regarding the LTM candidate cells cancel or release, based on previous agreements, source gNB or candidate gNB can initiate LTM cells cancel/release by Handover Cancel message or LTM Cancel message.

The Handover Cancel message is reused by the source gNB to release the reserved resource for LTM candidate cells in the candidate gNBs. A candidate gNB can initiate cancellation of configured LTM candidate cell(s) of its own. Adopt Class-2 procedure for candidate gNB-initiated LTM cancellation. Confirm the message name as LTM Cancel for candidate gNB-initiated LTM cancellation.

However, after each inter-CU LTM Cell Switch, new source gNB may initiate to setup the UE associated Xn signalling connection with the candidate gNBs via LTM Configuration Update/Acknowledge. If some LTM candidate cells are released due to some reasons, this procedure cannot be initiated by source gNB or candidate gNB until the setup for the UE associated Xn signalling connection between new source gNB and candidate gNB. Therefore, if LTM candidate cells cancel/release information can be included in the LTM Configuration Update/Acknowledge message, the following Handover Cancel or LTM Cancel does not need be initiated which can save signalling procedure.

Therefore, it has a benefit to introduce LTM Candidate Cells To Be Cancelled List IE in the LTM Configuration Update/Acknowledge if there are LTM candidate cells to be released during the setup for UE associated Xn signalling connection between new source gNB and candidate gNB.

Proposal 6: Introduce LTM Candidate Cells To Be Cancelled List IE in the LTM Configuration Update/Acknowledge from new source gNB or other candidate gNB to trigger LTM candidate cells cancel.

## Issue 7: Co-existence between L3 SN change and inter-CU SCG LTM

Review the R19 discussion on the co-existence between L3 HO and inter-CU LTM, below agreement is reached in the RAN2#127

L3 mobility (including both the network triggered L3 HO and CHO) can be configured to UE, while the inter-CU LTM is configured (w/o DC), and the following items can be considered (follow Rel-18 intra-CU LTM): - When performing the L3 mobility (HO or CHO), the UE does not autonomously release inter-CU LTM configurations, unless these are explicitly released by the network. - The RRCReconfiguration message to execute an L3 mobility (HO or CHO) procedure may reconfigure inter-CU LTM configurations. - For the execution order between CHO and LTM, Rel-18 principle is applied.

Let us review the R18 discussion from RAN3, the below agreements on LTM and L3 handover collision is achieved in the RAN3#121(-bis)

In case that L3 handover is triggered earlier than LTM (the gNB-DU receives the L3 handover command before LTM is triggered), L3 handover has high priority. In case that LTM is triggered earlier than L3 (the gNB-CU receives the LTM notify message from gNB-DU before L3 handover is triggered), LTM has high priority, In case that LTM and L3 handover are triggered almost simultaneously (cross signaling on F1). The (source) gNB-DU fails the L3 handover by responding with UE Context Modification Failure message with proper cause meaning LTM has high priority.

Look through the above conclusions, one important enhancement is that introduce a cause indication of LTM command triggered in the UE Context Modification Failure message when LTM and L3 handover are triggered almost simultaneously (refer to below figure). And below procedure text is specified in TS 38.473.

“If the gNB-DU is not able to accept the UE CONTEXT MODIFICATION REQUEST message for mobility because an LTM command has been triggered to the UE, it shall reply with the UE CONTEXT MODIFICATION FAILURE message with an appropriate cause value.”

gNB-CU

gNB-DU

UE

L3 HO trigger

LTM trigger

UE Context Modification Request

Containing RRC Reconfiguration

LTM Cell Switch Command

UE Context Modification Failure

Cause: LTM command triggered

This enhancement is also applicable for the case of inter-CU LTM with SA and following scenarios on LTM with NR-DC

1. Inter-CU MCG LTM without SN release

2. Inter-CU MCG LTM with SN release

3. Inter-CU MCG LTM with SN addition

In above scenarios, when L3 HO is triggered, source gNB-CU also send UE Context Modification Request message containing RRC Reconfiguration to source gNB-DU, in case if LTM is triggered by source gNB-DU at this moment, source gNB-DU responses with the UE Context Modification Failure message including a cause indication of LTM command triggered. It can avoid to trigger simultaneously LTM and L3 HO from the view of UE.

However, regarding R19 SN initiated inter-CU SCG LTM, LTM is triggered in source SN-DU in case S-SN is a split gNB. When L3 SN change is triggered, MN sends the RRC Reconfiguration message to UE after the complete of target PSCell resource preparation regardless SN change is initiated by MN or S-SN. (refer to the message flows in below figures on SN change procedure-SN/MN initiated)

Due to different node finally sends L3 SN change command and LTM cell switch command message to UE, it is possible for the UE to almost simultaneously receive LTM Cell Switch Command from source SN-DU and RRC Reconfiguration message from MN. In order to avoid inter-CU SCG LTM and L3 SN change collision, we propose to introduce one coordination mechanism to solve this issue.

MN

UE

S-SN-CU

S-SN-DU

Target-SN

Target PSCell resource preparation between MN and target SN

LTM trigger

LTM Cell Switch Command

L3 HO trigger

Coordination mechanism to avoid L3 SN change and inter-CU SCG LTM triggered simultaneously

RRC Reconfiguration

Proposal 7: RAN3 to discuss one coordination mechanism to avoid inter-CU SCG LTM and L3 SN change triggered almost simultaneously.

## Issue 8: Reference Configuration

As RAN2 decided, single reference configuration is used when preparing inter-CU LTM. However, it was left FFS on whether the source gNB can request a candidate gNB to provide a reference configuration.

Recalling agreement on Rel-18 LTM, which the scenario is limited to intra-CU, inter-DU case, the source gNB-DU was allowed to request the candidate gNB-DU to generate reference configuration. Of course, it is up to the candidate gNB-DU whether to provide the reference configuration to the source gNB-DU even when the candidate gNB-DU receives the request.

Following same principle adopted in Rel-18, it is natural for the source gNB requests a candidate gNB to provide a reference configuration.

In last meeting, some companies raised the issue of interoperability. It is true there is interoperability issue when vendors of gNBs are not same. However, this is not the matter of who makes the reference configuration. When we limit the case that only source gNB can generate a reference configuration, IOT issue is still remain.

Therefore, regardless of IOT issue, reference configuration can be generated at both source and candidate side. Again, the same philosophy was used in inter-DU scenario where the vendors of gNB-DUs are different.

Proposal 8: RAN3 agree to allow the source gNB to request a candidate gNB to provide a reference configuration.

# 3 Conclusion

This contribution firstly analyses the whole message flows of inter-CU LTM and propose some enhancements for Xn interface on below aspects. Therefore, we invite RAN3 to agree below proposals for supporting the basic inter-CU LTM procedure.

## Issue 1: Early TA RACH resource allocation

Proposal 1-1: RAN3 support the early TA RACH resource allocation is based on gNB-DU in case of split gNB and/or based on gNB in case of aggregated gNB.

Proposal 1-2: RAN3 should discuss how source gNB-CU acquires the gNB-DU ID information to which inter-CU candidate cell belongs before initiation of LTM preparation.

Proposal 1-3: RAN3 should define F1 gNB-DU ID IE to be an optional IE in the Rel-19 specification on the procedure of early TA RACH resource request & allocation & TA information transfer.

## Issue 2: FFS the message type and detail in TA INFORMATION TRANSFER message

Proposal 2-1: Propose to turn WA into agreement that introduce a new non-UE associated class 2 procedure on Xn, namely TA information Transfer message, to transfer the TA information from the candidate gNB-CU to the source gNB-CU.

Proposal 2-2: Propose to include below information in XnAP TA INFORMATION TRANSFER message.

- TA Information List

  - Candidate Cell ID

  - TA value

  - Preamble Index

  - RA-RNTI

  - Early RACH Resources Requester ID (optional IE, present in case of split gNB, implicit indicate source gNB-DU ID)

## Issue 3: Stage 3 IEs design on Data forwarding Info in LTM Configuration Update message

Proposal 3-1: Data Forwarding Information IE is a list including multiple data forwarding addresses from each candidate gNB/cell.

Proposal 3-2: Regarding inter-CU LTM, Data forwarding address is based on candidate PCell not candidate gNB.

Proposal 3-3: RAN3 to agree to use Data Forwarding Info from target NG-RAN node IE per-PDU session per-candidate Cell as the included details in the Data Forwarding Info List IE.

## Issue 4: FFS TA Information List IE in Cell Switch Notification message

Proposal 4: TA Information List IE can be included in XnAP Cell Switch Notification message to forward the valid TA values of candidate cells for reuse in subsequent LTM.

## Issue 5: FFS how to identify the UE for the candidate gNB when the new source gNB initiates to setup the UE associated Xn signalling connection with the candidate gNBs

Proposal 5: Introduce last source gNB UE XnAP ID/source gNB ID in LTM Configuration Update message from new source gNB to other candidate gNBs, so that candidate gNB can identify the same UE and setup the UE associated Xn signalling connection.

## Issue 6: Enhancement for the LTM candidate cells cancel via the procedure of LTM Configuration Update/Acknowledge

Proposal 6: Introduce LTM Candidate Cells To Be Cancelled List IE in the LTM Configuration Update/Acknowledge from new source gNB or other candidate gNB to trigger LTM candidate cells cancel.

## Issue 7: Co-existence between L3 SN change and inter-CU SCG LTM

Proposal 7: RAN3 to discuss one coordination mechanism to avoid inter-CU SCG LTM and L3 SN change triggered almost simultaneously.

## Issue 8: Reference Configuration

Proposal 8: RAN3 agree to allow the source gNB to request a candidate gNB to provide a reference configuration.

# 4 Reference

[1] R3-250933 (BL CR to 38.300) Support for Inter-CU LTM

[2] R3-250935 (BL CR to 38.423) Xn support for inter-CU LTM

[3] R3-250936 (BL CR to 38.473) F1 support for inter-CU LTM