3GPP TSG-RAN WG3 Meeting #127bis	             R3-252186

Wuhan, CN, 7th – 11th Apr 2025

Agenda Item:	16.3

Source:	CMCC

Title:	(TP to TS 38.300) Discussion on A-IoT device location reporting

Document type:	Other

# 1	Introduction

During last meeting, we agreed the following agreements on the topic of A-IoT device location reporting.

The A-IoT RAN node always sends the AIoT device’s location at reader ID granularity to the AIoTF in Inventory Report. AIoT RAN node sends the Inventory Report aggregated per Reader. “Global gNB ID + Reader index” is used to uniquely identify the Reader globally. FFS whether only Reader index or “Global gNB ID + Reader index” is sent in Inventory Report. Continue to discuss the open issues....

In this contribution, we will continue to provide our views on A-IoT device location reporting.

# 2	Discussion

From the WID, the scope of RAN3 restricts the granularity for device location reporting at reader ID granularity. As the location of the A-IoT device, the AIOTF already be aware of the reader ID received through the existing A-IoT Inventory Report procedure over NGAP. Since the command is always initiated after the inventory and the reader within a gNB is stable, it is unnecessary for gNB to provide the A-IoT device’s location at reader ID granularity to AIoTF in Command message.

Proposal 1: The gNB does not need to provide the A-IoT device’s location at reader ID granularity to AIoTF in Command message.

Currently, the A-IoT service defined in SA2 only supports Inventory service and Command service, there is currently no demand to introduce a new service directly obtaining the A-IoT device location. Thus, we think it is unnecessary to introduce a dedicated locating procedure for A-IoT device to solely report device ID.

Proposal 2: It is unnecessary to introduce a dedicated locating procedure for A-IoT device to solely report device ID.

Regarding the encode of Reader ID, it is already agreed that “Global gNB ID + Reader index” is used to uniquely identify the reader. The left issue is whether only Reader index or “Global gNB ID + Reader index” is sent in Inventory Report. From our view, no matter the direct or indirect deployment, the Reader index included in Inventory Report message is enough to identify the reader. In direct deployment, the AIoTF can directly identify the originating gNB of the transmission. Likewise, in indirect deployment, the AMF can directly identify the originating gNB of the transmission. From a logical perspective, the AMF is required to relay the originating gNB to the AIOTF. The detail signalling exchange between AMF and AIoTF should be designed in SA. We should believe SA2 is professional enough to contain the Global gNB ID towards AIOTF. Considered the maximum reader number within a gNB, in my opinion, a sufficiently large number is required, similar to maximum cell number within an NG-RAN node. From the NR spec, the maximum number of cells served by a NG-RAN node is 16384. Thus, the maximum reader number served by an gNB also can set to 16384.

Proposal 3: The Reader index included in Inventory Report message is enough to identify the reader.

Proposal 4: The maximum reader number served by a gNB can set to 16384.

# 3	Conclusion

Based on the discussions mentioned above, in this contribution we provide some observations and proposals on A-IoT device location reporting:

Proposal 1: The gNB does not need to provide the A-IoT device’s location at reader ID granularity to AIoTF in Command message.

Proposal 2: It is unnecessary to introduce a dedicated locating procedure for A-IoT device to solely report device ID.

Proposal 3: The Reader index included in Inventory Report message is enough to identify the reader.

Proposal 4: The maximum reader number served by a gNB can set to 16384.

# 4	Reference

[1] RP-250796, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA;

# 5	TP to TS 38.300 BL CR

### 16.xx.x3	Inventory procedure

Figure 16.xx.x3-1 depicts the basic communication between the gNB and the A-IoT CN node for the Inventory procedure.

Editor’s Note 3:	Entity names, message names and question whether to depict direct and indirect connectivity somehow in a different way are subject to further discussions.

Figure 16.xx.x3-1: Inventory procedure

1.	The A-IoT CN node initiates the Inventory procedure over NG-C by sending the Inventory Request message to the gNB. The Inventory Request may include information concerning one or multiple A-IoT devices.

Editor’s Note 4:	Specification of further parameterisation of the Inventory Request message (e.g., assistance information from 5GC) needs further work.

2.	The gNB allocates and co-ordinates the usage of A-IoT radio resources.

3.	The gNB confirms the request from the A-IoT CN node by replying with the Inventory Response message.

NOTE 2:	If the gNB is not able to perform the Inventory procedure, it rejects the request and sends an Inventory Failure message to the A-IoT CN node.

4.	The gNB performs the Inventory procedure towards the A-IoT device(s) over the A-IoT radio interface.

5./6.	Upon receiving the inventory result(s) from the A-IoT device(s), the gNB sends the inventory result(s) and the A-IoT device(s)’ location in Inventory Report message to the A-IoT CN node. If the Inventory procedure concerns multiple A-IoT devices, multiple Inventory Reports may be sent to the A-IoT CN node.

NOTE 3:	If the Inventory procedure concerns multiple A-IoT devices, an Inventory Report message may include inventory reports from multiple A-IoT devices,