3GPP TSG-RAN WG3 #127-bis					 				                                  R3-252244

Wuhan, China, 7-11 April 2025

Title: 	TP to (BL CR to 38.300) Location AIoT device

Source: 	ZTE Corporation, China Telecom

Agenda item:	16.3

Document for:	Discussion and Decision

- Introduction

This paper is used to discuss locating AIoT devices.

- Discussion

In the SI phase, the TR38.769 v2.0.0 has captured the solution on locating Ambient IoT devices.

In the WI phase, in the last RAN3 #127 meeting, we have made the following agreement.

AIoT RAN node sends the Inventory Report aggregated per Reader.

“Global gNB ID + Reader index” is used to uniquely identify the Reader globally.

FFS whether only Reader index or “Global gNB ID + Reader index” is sent in Inventory Report.

Proposal 1: Agree to capture the content of section 6.9 Locating ambient IoT devices in TR38.769 as baseline.

Meanwhile, in this release, AIoT command only procedure is excluded, so that AIoT RAN node does not need to report the reader ID to the AIoT CN by the Command response message.

Proposal 2: Agree to locate AIoT device by Inventory report message.

Agree to capture the following TP into the BLCR to 38.300

- TP to TS38.300

<<<<<<<<<<<<<<<<<<<< Start of the Change >>>>>>>>>>>>>>>>>>>>

16.xx	Support of Ambient IoT

16.xx.x1	Introduction

<Skip unchanged part>

16.xx.xa Locating Ambient IoT devices

16.xx.xa.1	General

A-IoT device location information may be used for the following purposes:

(a) improving the A-IoT operation itself, e.g., by A-IoT CN sending a Command to one or more readers (e.g., the last reader(s)) associated to the device rather than sending it blindly.

(b) providing location information to the consumer of the A-IoT service.

Locating an Ambient IoT device at "reader ID granularity" is useful for both purposes and is to be supported.

NOTE: "reader ID granularity" is likely not sufficient for purpose (b) and may be refined in the future.

16.xx.xa.2	Topology 1

For support of A-IoT device location, the A-IoT RAN node may report the Global gNB ID to the A-IoT CN in the Inventory Report messages.

The A-IoT RAN node may additionally report a Reader index to the A-IoT CN in the Inventory Report messages, which corresponds to the reader to which the A-IoT device responded.

NOTE 1: Whether multiple Reader index (s) can be included by the A-IoT RAN node in the Inventory Report message needs further discussion.

NOTE 2: Definition and format of the Global gNB ID and Reader index needs further discussion.

NOTE 3: Whether and how to have awareness of Reader location in A-IoT CN needs further discussion.

<<<<<<<<<<<<<<<<<<<< End of the Change >>>>>>>>>>>>>>>>>>>>