3GPP TSG RAN WG3#127bis	R3-252321

7th April – 11th April - 2025

Wuhan, China

Agenda Item:	13.3

Source:	Nokia (moderator)

Title:	Summary of Offline Discussion for Conditional LTM

Document for:	Approval

# Introduction

CB: # Mobility_CLTM

- continue discuss and capture the agreements.

- update the TPs if needed.

- capture the open issues for next meeting.

(Moderator - Nokia)

Offline summary in R3-252321

# For the Chairman’s Notes

Propose to agree the following

For CLTM focus firstly on scenarios without CU-UP change.

The gNB-DU signals the generated L1 execution conditions over F1AP for CLTM via a new IE in the UE CONTEXT SETUP RESPONSE and UE CONTEXT MODIFICATION RESPONSE messages, details FFS

Reuse the ACCESS SUCCESS message from Candidate DU to CU to inform the cell change

For Inter-DU CLTM, reuse the UE CONTEXT MODIFICATION REQUEST message from CU to Source DU to inform the cell change and stop data transmission

For CLTM, the existing LTM network signaling for LTM Cell Switch Completion phase is reused.

TPs:

TS 38.473: R3-252120 is revised in R3-252435

TS 38.401: R3-251882 is revised in R3-252423

Propose to be continued

FFS whether a cell can be simultaneously configured for LTM and CLTM, to be continued

FFS whether its needed that for each candidate cell, the CU indicates to the source DU and the candidate DUs in the LTM Configuration ID Mapping List the need of generating L1 execution condition. To be continued.

FFS, whether L3 CLTM is configured needs to be explicitly indicated separately from L1 CLTM. To be continued.

# Discussion

In this offline, we plan to continue to discuss the remaining issues on support of Conditional LTM.

## 3.1	Conditional LTM configuration

With respect to C-LTM configuration phase, RAN3 agreed the following at previous meeting.

RAN3#127: The CU decides to configure C-LTM. The CU determines which type of C-LTM to initiate: L1 event-triggered C-LTM or L3 event-triggered C-LTM. Reuse the current LTM preparation procedure (i.e. UE Context Setup/Modification procedure) for DU to generate the C-LTM L1 execution condition. UE Context Setup procedure for the candidate DU to generate the subsequent C-LTM L1 execution condition. UE Context Modification procedure for the source DU to generate the C-LTM L1 execution condition. Indicator for C-LTM shall be added in the UE CONTEXT SETUP/MODIFICATION REQUEST message. FFS for the detail.

### CLTM Scenarios (First Round)

Proposal (A) Contributions propose to prioritize the intra-DU and Inter-DU CLTM scenarios without CU-UP change (Ericsson)

Moderator’s view is to agree to focus firstly on scenarios without CU-UP change.

Moderator’ summary:

Proposal: For CLTM, focus firstly on scenarios without CU-UP change.

### CLTM Configuration from CU to DU (First Round)

Contributions at this meeting propose several variations regarding how the CU requests the DU for the L1 execution conditions when configuring CLTM.

- Proposal (B) New IE to request CLTM L1 execution conditions (Nokia, ZTE, NEC, Huawei, Samsung)

- Proposal (C) New codepoint to request CLTM L1 execution conditions as part of existing LTM Indicator IE (Ericsson, Lenovo, CATT, LGE)

- Proposal (D) For each LTM candidate cell, CU indicates to Source/Candidate DU which LTM candidate cells are configured with L1 CLTM (Huawei, Qualcomm)

- Proposal (E) CU indicates to DU whether a cell is configured with L3 CLTM (Ericsson, Qualcomm, CATT).

With respect to Proposal (D) the reasoning in R3-252120 (Huawei) indicates that the LTM candidates can consist of a mixture of LTM and CLTM cells for the same UE (e.g., 2 x cells are LTM and other 2 x cells are CLTM). Thus, that for each candidate cell, the CU indicates both to the Source DU and Candidate DUs about the need of generating L1 execution conditions within the existing LTM Configuration Mapping List.

The moderator’s view is that the variants proposed show difference in understanding from companies as to: Firstly, whether an LTM cell can be configured simultaneously for both CLTM and LTM, since some proposals were made in a way that the L1-CLTM indication mutually excludes the LTM configuration for the same cell. Secondly, whether for each candidate cell, both Source DU and Candidate DUs need to be aware of which candidate LTM cells are configured with CLTM.  Hence, we need to clarify in the comeback companies understanding on simultaneous LTM and CLTM configuration prior to deciding Stage 3 detail.

- Question 1: Can a cell be simultaneously configured for LTM and CLTM (L1 or L3) for the same UE?

- Question 2: Does the (Source/Candidate) DUs need to be aware of which other candidate LTM cells are configured with L1 CLTM?

- Question 3: In case of L3-CLTM is configured, does the (Source/Candidate) DU needs to be informed?

Moderator’ summary:

Q1 discussion: Can a cell be simultaneously configured for LTM and CLTM (L1 or L3) for the same UE?

LGE: In LTM DU generates report configuration, in L1 CLTM generates L1 execution conditions, how to distinguish?

E///: CU should be able to configure both (e.g., first LTM and then L3 CLTM)

ZTE: For the same cell, CLTM and LTM cannot be configured for same UE

Huawei: Unclear why RAN2 has allowed such configuration

LGE: Simultaenous configuration is odd for the UE, DU receives measurements and at same time evaluates execution conditions

FFS whether a cell can be simultaneously configured for LTM and CLTM, to be continued

Q2 discussion: Does the (Source/Candidate) DUs need to be aware of which other candidate LTM cells are configured with L1 CLTM?

Moderator’s note: this relates to the following proposal in R3-252120 (“Proposal 2: For each candidate cell, the CU indicates to the source DU and the candidate DUs in the LTM Configuration ID Mapping List the need of generating L1 execution condition”)

CATT: Source needs to know by why candidates also need this information

QC. Huawei: Other candidates need to know because they may become source after LTM mobility and subsequent LTM

Huawei: Refer to the following agreement from RAN2: “RAN2#129 meeting Agreements on C-LTM:  1.	From a given source cell, network configures either L1 execution condition or L3 execution condition for a CLTM candidate cell.”

Lenovo: Support this proposal

FFS whether its needed that for each candidate cell, the CU indicates to the source DU and the candidate DUs in the LTM Configuration ID Mapping List the need of generating L1 execution condition. To be continued.

Q3 discussion: In case of L3-CLTM is configured, does the (Source/Candidate) DU needs to be informed?

Huawei, CATT, Ericsson: Behavior for legacy LTM and L3 CLTM is different and DU needs to be aware.

E///: We need to explicitly indicate whether it is L1 or L3 based CLTM. Compared to Rel-18 LTM, behavior will differ at the DU.

Lenovo: Relates to question 1.

FFS, whether L3 CLTM is configured needs to be explicitly indicated separately from L1 CLTM. To be continued.

### L1 CLTM execution conditions DU to CU (First Round)

Proposal (F) For L1 CLTM execution conditions generated by the gNB-DU, contributions propose a new F1AP IE to signal the L1 execution conditions generated by the DU taking into account RAN2 running RRC CR (Nokia, ZTE, Ericsson, NEC, China Telecom, LGE).

Moderator’s view is to propose the following. “Proposal: The gNB-DU signals the generated L1 execution conditions for CLTM via a new F1AP in the UE CONTEXT SETUP RESPONSE and UE CONTEXT MODIFICATION RESPONSE messages”

Moderator’ summary:

LGE: Regardless of new IE, OCTETSTRING will not suffice

Moderator: Stage 3 details let’s try to discuss further in the draft TP, at this moment structure is FFS.

Proposal: The gNB-DU signals the generated L1 execution conditions over F1AP for CLTM via a new IE in the UE CONTEXT SETUP RESPONSE and UE CONTEXT MODIFICATION RESPONSE messages, details FFS

## 3.2	Early Synchronization

### TCI States

Contributions propose to collect the TCI State Configuration from Candidates and provided it to the Source DU by reusing UE Context Setup and Modification procedures (Huawei) as follows:

- Proposal (G) Reuse the TCI States Configurations List field in the UE CONTEXT SETUP RESPONSE and UE CONTEXT MODIFICATION RESPONSE messages to collect candidate cell TCI state configuration.

- Proposal (H) Reuse the TCI States Configurations List field in the UE CONTEXT MODIFICATION REQUEST message to send the candidate cell TCI state configuration to the source DU, as well as to the candidate DUs for the subsequent C-LTM.

Additionally, with respect to the activated TCI States for a UE, there are opposing views as whether

- Proposal (I) The Source DU signals the activated TCI States to the Candidate DU (CATT, Nokia)

- Proposal (J) There is NO need to signal the Activated TCI States (Huawei, LGE)

Moderator’s view is to propose to reuse existing procedures for collecting/informing TCI State Configuration, and to note proposals for signaling activated TCI States from Source DU to Candidate DU. “Proposal: Reuse UE Context Setup and Modification procedures to collect the TCI state configuration from candidate cells and provide it to the Source DU.”

Moderator’ summary:

Skipped, discuss next meeting.

### UE-based TA

Contributions indicate that RAN2 agreed that UE-based TA is applicable for CLTM hence, propose the following:

- Proposal (K) Reuse the existing UE-based TA Measurement Configuration field in UE CONTEXT MODIFICATION REQUEST message to notify the DU of UE-based TA for CLTM (Huawei)

Moderator’s view is to discuss whether the proposal above is agreeable.

Moderator’ summary:

Skipped, discuss next meeting.

### Time Alignment Timer (TAT)

Contributions propose that for both L1 or L3 CLTM, that the TAT value from Candidate LTM cells is also provided to the Source DU. Following variants are proposed.

- Proposal (L) Include TAT in DU-CU/CU-DU TA INFORMATION TRANSFER messages (China Telecom, Lenovo)

- Proposal (M) Include TAT generated for C-LTM in UE CONTEXT MODIFICATION RESPONSE and UE CONTEXT MODIFICATION RESPONSE messages (Ericsson, Huawei)

- Proposal (N) The new TAT for CLTM is maintained at source DU via implementation (CATT)

Moderator’s view is to discuss further both proposals above (legacy TAT and the newly specified TA Timer for CLTM).

Moderator’ summary:

Skipped, discuss next meeting.

### Early Synch for L3 CLTM

For early synchronization for L3 CLTM, contributions propose the following

- Proposal (O) The CU triggers TA Acquisition to the DU via a new F1AP procedure (China Telecom, Huawei, LGE)

  - Note: It is noted that it is FFS whether DL synch can also be triggered (LGE)

- Proposal (P) L3 CLTM requires capability to trigger early synch procedures independently, that is, without having to also trigger an LTM cell switch (Nokia)

Moderator’s view is to agree that in for L3 based CLTM, a new F1AP procedure is introduced to trigger TA Acquisition at the Source DU. “Proposal: For L3 based CLTM, introduce a new F1AP procedure to trigger TA Acquisition at the Source DU.”

Moderator’ summary:

Skipped, discuss next meeting.

## 3.3	Conditional LTM Cell Switch Execution (First Round)

Contributions indicate that given that source DU is unaware of CLTM execution by the DU, that signaling is required to inform that the UE has accessed a candidate cell. The following are proposed:

- Proposal (Q) Candidate DU signals to the Source DU (via the CU) that CLTM has taken place and that the UE has accessed the candidate cell.

  - Proposal (R) Reuse the ACCESS SUCCESS message from Candidate DU to CU (Nokia, ZTE, China Telecom, Lenovo, Huawei, LGE)

  - Proposal (S) For Inter-DU CLTM, reuse the UE CONTEXT MODIFICATION REQUEST message from CU to Source DU to inform the cell change and stop data transmission (Nokia, China Telecom, LGE, Samsung(?), Huawei)

  - Proposal (T) Investigate possible solutions (Ericsson)

Moderator’s view is to agree to agree to reuse ACCESS SUCCESS, and UE CONTEXT MODIFICATION REQUEST messages to inform the Source DU that the UE has accessed the Candidate DU after CLTM and stop data transmission.

Moderator’ summary:

Proposal: Reuse the ACCESS SUCCESS message from Candidate DU to CU to inform the cell change

Proposal: For Inter-DU CLTM, reuse the UE CONTEXT MODIFICATION REQUEST message from CU to Source DU to inform the cell change and stop data transmission

## 3.4	Conditional LTM Cell Switch Completion (First Round)

Contributions propose to follow LTM network signaling for cell switch completion phase as follows:

as follow:

- Proposal (U) Existing LTM network signaling is reused for CLTM (Ericsson)

- Proposal (V) The CU sends the UE CONTEXT RELEASE COMMAND message to the source DU to release the UE context if the source DU does not have any candidate cells for subsequent C-LTM (Huawei).

Moderator’s view is to agree to that existing LTM network signaling is reused also for CLTM scenarios.

Moderator’ summary:

Proposal: For CLTM, the existing LTM network signaling for LTM Cell Switch Completion phase is reused.

## 3.5	Other Proposals

Contributions on other topics include the following proposals:

- Proposal (W) Early data forwarding is supported for CLTM (Lenovo)

- Proposal (Y) For CU triggered modification or release of CLTM configuration, the CU reuses the UE CONTEXT MODIFICATION REQUEST or UE CONTEXT RELEASE COMMAND messages (China Telecom).

- Proposal (X) For DU triggered release of CLTM configuration, the DU reuses the UE CONTEXT MODIFICATION REQUIRED or the UE CONTEXT RELEASE REQUEST messages (China Telecom).

Moderator’s view is to discuss further proposals above.

Moderator’ summary:

Skipped, discuss next meeting.

## TP/BLCRs

Check stage2/3 TPs if possible.

Moderator’s view is to revise TPs as follows:

- TP for TS 38.473: R3-252120 (Huawei) is revised in R3-25XXXX

- TP for TS 38.401: R3-251882 (ZTE) is revised in R3-25XXXX

Moderator’ summary:

- TP for TS 38.473: R3-252120 (Huawei) is revised in R3-25XXXX

- TP for TS 38.401: R3-251882 (ZTE) is revised in R3-25XXXX

# Conclusion, Recommendations [if needed]

If needed

# References

| R3-251534 | Discussion on Conditional LTM in disaggregated gNB architecture (Nokia) | discussion |
| --- | --- | --- |
| R3-251882 | (TP for 38.473, 38.401) Conditional intra-SN LTM (ZTE Corporation) | other |
| R3-251965 | (TP to CLTM BL CR for TS 38.473, TS 38.401) – Further discussion on Intra-CU Conditional LTM (Ericsson) | other |
| R3-251672 | (TP to BL CR for TS 38.401and 38.473 on conditional intra-CU LTM) Discussion of Rel-19 intra-CU Conditional LTM (NEC) | other |
| R3-251864 | (TP for TS38.401) On support of intra-CU Conditional LTM (China Telecom) | other |
| R3-252120 | (TP for LTM BLCR for TS38.473, TS38.401): Discussion on intra-CU conditional LTM (Huawei) | other |
| R3-251760 | Signalling enhancements for Conditional Intra-CU LTM (Qualcomm Incorporated) | discussion |
| R3-251743 | (TP for TS 38.401 and TS 38.473) Discussions on RAN3 impacts from Conditional Intra-CU LTM (LG Electronics Inc.) | other |
| R3-251948 | [TP to BLCR for TS 38.401] Conditional intra-CU LTM (Lenovo) | other |
| R3-252164 | Discussion on Intra-CU Conditional LTM (Samsung) | discussion |
| R3-251838 | (TP to 38.473)Discussion for C-LTM (CATT) | Discussion Move to 13.3 |

# Appendix

Previous meeting agreements:

RAN3#127:

The CU decides to configure C-LTM.

The CU determines which type of C-LTM to initiate: L1 event-triggered C-LTM or L3 event-triggered C-LTM.

Reuse the current LTM preparation procedure (i.e. UE Context Setup/Modification procedure) for DU to generate the C-LTM L1 execution condition.

UE Context Setup procedure for the candidate DU to generate the subsequent C-LTM L1 execution condition.

UE Context Modification procedure for the source DU to generate the C-LTM L1 execution condition.

Indicator for C-LTM shall be added in the UE CONTEXT SETUP/MODIFICATION REQUEST message. FFS for the detail.

For L1 measurement based C-LTM, to support Early Sync, the existing F1: DU-CU/ CU-DU TA Information Transfer is reused.

Continue to work on the solutions comparison and details…