3GPP TSG-RAN WG3 Meeting #127bis														R3-252343

Wuhan, China, 7th – 11th April 2025

Agenda item:	14.2

Source:	Nokia, Nokia Shanghai Bell, Qualcomm, Huawei, Ericsson, Samsung, CATT, ZTE, NEC

Title:	(TP to BL CR for TS 38.413) Update on the support of MBS Broadcast Service

Document for:	Discussion and Decision

# Annex B – TP for TS 38.413 BL CR

-----------------Start of Change-------------------

### 8.17.2	Broadcast Session Modification

#### 8.17.2.1	General

The purpose of the Broadcast Session Modification procedure is to request the NG-RAN node to update the MBS session resources or the area related to a previously established broadcast MBS session. The procedure uses non-UE associated signalling.

#### 8.17.2.2	Successful Operation

Figure 8.17.2.2-1: Broadcast Session Modification, successful operation.

The AMF initiates the procedure by sending a BROADCAST SESSION MODIFICATION REQUEST message to the NG-RAN node.

If the MBS Service Area IE is included in the BROADCAST SESSION MODIFICATION REQUEST message, the NG-RAN node shall update the MBS service area as needed, and send the BROADCAST SESSION MODIFICATION RESPONSE message. For NTN operation, the NG-RAN node shall, if supported, update the intended service area information as specified in TS 38.300 [8].

If the MBS Session Modification Request Transfer IE is included in the BROADCAST SESSION MODIFICATION REQUEST message, the NG-RAN node shall replace the previously provided information by the newly received one and update the MBS session resources and area as requested and send the BROADCAST SESSION MODIFICATION RESPONSE message.

-----------------End of the Change-------------------