3GPP TSG-RAN WG3 Meeting #127bis	R3-252393

# Wuhan, China, 7 – 11 Avril, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 1266 | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Trace Depth for Vendor Specific Trace Record |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Ericsson, Huawei, ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TraceQoE_OAM |  |  |  |  |  | Date: |  |  | 2025-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In S5-250077, SA5 agreed to add 3 new codepoints for the Trace Depth parameter for Trace Session level, better support vendor-defined content in a consistent manner. In XnAP, the existing Trace Depth IE contained in the Trace Activation IE needs to be updated to introduce these new codepoints, namely: - MinimumOnlyVendorSpecificTraceRecord - MediumOnlyVendorSpecificTraceRecord - MaximumOnlyVendorSpecificTraceRecord |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add 3 new codepoints to the Trace Depth IE: - MinimumOnlyVendorSpecificTraceRecord - MediumOnlyVendorSpecificTraceRecord - MaximumOnlyVendorSpecificTraceRecord |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The new vendor specific Trace Depth introduced by SA5 will not be supported in NG-RAN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.14, 9.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.423 CR1485, TS 37.483 CR0169, TS 38.473 CR1564 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 9.3.1.14		Trace Activation

This IE defines parameters related to a trace session activation.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| NG-RAN Trace ID | M |  | OCTET STRING (SIZE(8)) | This IE is composed of the following: Trace Reference defined in TS 32.422 [11] (leftmost 6 octets, with PLMN information encoded as in 9.3.3.5), and Trace Recording Session Reference defined in TS 32.422 [11] (last 2 octets). | - |  |
| Interfaces to Trace | M |  | BIT STRING (SIZE(8)) | Each position in the bitmap represents an NG-RAN node interface: first bit = NG-C, second bit = Xn-C, third bit = Uu, fourth bit = F1-C, fifth bit = E1: other bits reserved for future use. Value '1' indicates 'should be traced'. Value '0' indicates 'should not be traced'. | - |  |
| Trace Depth | M |  | ENUMERATED (minimum, medium, maximum, minimumWithoutVendorSpecificExtension, mediumWithoutVendorSpecificExtension, maximumWithoutVendorSpecificExtension, …, minimumOnlyVendorSpecificTraceRecord, mediumOnlyVendorSpecificTraceRecord, maximumOnlyVendorSpecificTraceRecord) | Defined in TS 32.422 [11]. | - |  |
| Trace Collection Entity IP Address | M |  | Transport Layer Address 9.3.2.4 | For File based Reporting. Defined in TS 32.422 [11]. This IE is ignored if the Trace Collection Entity URI IE is present. | - |  |
| MDT Configuration | O |  | 9.3.1.167 |  | YES | ignore |
| Trace Collection Entity URI | O |  | URI 9.3.2.14 | For Streaming based Reporting. Defined in TS 32.422 [11]. | YES | ignore |

### 9.4.5	Information Element Definitions

<<< skip unchanged ASN.1 >>>

TraceActivation ::= SEQUENCE {

nGRANTraceID						NGRANTraceID,

interfacesToTrace					InterfacesToTrace,

traceDepth							TraceDepth,

traceCollectionEntityIPAddress		TransportLayerAddress,

iE-Extensions		ProtocolExtensionContainer { {TraceActivation-ExtIEs} }	OPTIONAL,

...

}

TraceActivation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-MDTConfiguration			CRITICALITY ignore	EXTENSION MDT-Configuration		PRESENCE optional 	}|

{ ID id-TraceCollectionEntityURI	CRITICALITY ignore	EXTENSION URI-address			PRESENCE optional		},

...

}

TraceDepth ::= ENUMERATED {

minimum,

medium,

maximum,

minimumWithoutVendorSpecificExtension,

mediumWithoutVendorSpecificExtension,

maximumWithoutVendorSpecificExtension,

...,

minimumOnlyVendorSpecificTraceRecord,

mediumOnlyVendorSpecificTraceRecord,

maximumOnlyVendorSpecificTraceRecord

}

TrafficLoadReductionIndication ::= INTEGER (1..99)

TransportLayerAddress ::= BIT STRING (SIZE(1..160, ...))