3GPP TSG-RAN WG3 Meeting #127bis														R3-252369

Wuhan, China, 7th – 11th April 2025

Agenda item:	21.3

Source:	China Telecom, Nokia, Nokia Shanghai Bell, ZTE, Huawei, CMCC, Ericsson, Samsung

Title:	(TP to TS 38.415) enhancement for BSSize, TTNB and available data rate

Document for:	Discussion and Decision

1	Introduction

This contribution proposes TP to introduce BSSize and TTNB in DL PDU SET INFORMATION (PDU Type 0) frame.

The following were agreed:

For QoS Handling when Traffic Characteristics Change Dynamically

Introduce BSSize and TTNB in NG-U/F1-U/XN-U.

For Support of exposure of available data rate:

Introduce thresholds and direction to trigger NG-RAN monitor available data rate in NGAP/XnAP/F1AP/E1AP.

Introduce “available data rate” in TS 38.425 and TS 38.415.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

BSSize	Burst Size

DL	Downlink

MBS	Multicast Broadcast Service

MB-UPF	Multicast Broadcast User Plane Function

PDCP	Packet Data Convergence Protocol

PPI	Paging Policy Indicator

PPP	Paging Policy Presence

QFI	QoS Flow Identifier

RQA	Reflective QoS Attribute

RQI	Reflective QoS Indication

SN	Sequence Number

UL	Uplink

UP	User Plane

UPF	User Plane Function

EPDU	End PDU of the PDU Set

EDB	End of Data Burst

PSI	PDU Set Importance

PSSN	PDU Set Sequence Number

PSN	PDU Sequence Number within a PDU Set

PSSize	PDU Set Size

TTNB	Time To Next Burst

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

5.4.2	Transfer of UL PDU Session Information

5.4.2.1	Successful operation

The purpose of the Transfer of UL PDU Session Information procedure is to send control information elements related to the PDU Session from NG-RAN to UPF.

An UL PDU Session user plane instance making use of the Transfer of UL PDU Session Information procedure is associated to a single PDU Session. The Transfer of UL PDU Session Information procedure may be invoked whenever packets for that particular PDU Session need to be transferred across the related interface instance.

The UL PDU SESSION INFORMATION frame includes a QoS Flow Identifier (QFI) field associated with the transferred packet.

If QoS monitoring has been requested for the included QFI field, the UL PDU SESSION INFORMATION frame may include a QoS Monitoring Packet (QMP) field, a DL Sending Time Stamp Repeated field, a DL Receiving Time Stamp field, a UL Sending Time Stamp field, and/or Delay Result for UL or DL. If QoS monitoring with N3/N9 delay reporting has been requested for the included QFI field, the I-UPF may include in the UL PDU SESSION INFORMATION frame a N3/N9 Delay Ind. field, a N3/N9 Delay Result field and delay result for UL and DL if received from the RAN. The UPF shall, if supported, use this information to calculate UL, DL, or RTT delay as specified in TS 23.501 [5].

The UL PDU SESSION INFORMATION frame may also include a UL QFI Sequence Number field associated with the transferred packet. The UPF shall, if the QoS flow has been configured eligible for redundant transport bearer in TS 38.413 [6], use the received UL QFI Sequence Number field to determine and eliminate duplicated packets for a given QoS flow as specified in TS 23.501 [5].

If the ECN Marking or Congestion Information Reporting Status is active for a QoS flow indicated by the QFI field, the UL PDU SESSION INFORMATION frame should include an UL Congestion Information field and/or a DL Congestion Information field. The UPF shall, if supported, use this information to perform ECN marking at UPF or information exposure as specified in TS 23.501 [5].

If the Monitoring Request on Available Data Rate is requested for a QoS flow indicated by the QFI field, the UL PDU SESSION INFORMATION frame may include an UL Available Data Rate field and/or a DL Available Data Rate field. The UPF shall, if supported, use this information to perform available data rate exposure as specified in TS 23.501 [5].

Figure 5.4.2.1-1: Successful Transfer of UL PDU Session Information

5.4.2.2	Unsuccessful operation

Void.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

5.5.2.1	DL PDU SESSION INFORMATION (PDU Type 0)

This frame format is defined to allow the NG-RAN to receive some control information elements which are associated with the transfer of a packet over the interface.

The following shows the respective DL PDU SESSION INFORMATION frame.

| Bits |  |  |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 |  | 1 |  | 0 |  |
| PDU Type (=0) |  |  |  | QMP | SNP |  | MSNP |  | Spare | 1 |
| PPP | RQI | QoS Flow Identifier |  |  |  |  |  |  |  | 1 |
| PPI |  |  | Spare |  |  | BSSI |  | TTNBI |  | 0 or 1 |
| DL Sending Time Stamp |  |  |  |  |  |  |  |  |  | 0 or 8 |
| DL QFI Sequence Number |  |  |  |  |  |  |  |  |  | 0 or 3 |
| DL MBS QFI Sequence Number |  |  |  |  |  |  |  |  |  | 0 or 4 |
| BSSize |  |  |  |  |  |  |  |  |  | 0 or 3 |
| TTNB |  |  |  |  |  |  |  |  |  | 0 or 2 |
| Padding |  |  |  |  |  |  |  |  |  | 0-3 |

Figure 5.5.2.1-1: DL PDU SESSION INFORMATION (PDU Type 0) Format

5.5.2.2	UL PDU SESSION INFORMATION (PDU Type 1)

This frame format is defined to allow the UPF to receive some control information elements which are associated with the transfer of a packet over the interface.

The following shows the respective UL PDU SESSION INFORMATION frame.

| Bits |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| PDU Type (=1) |  |  |  | QMP | DL Delay Ind. | UL Delay Ind. | SNP | 1 |
| N3/N9 Delay Ind. | New IE Flag | QoS Flow Identifier |  |  |  |  |  | 1 |
| DL Sending Time Stamp Repeated |  |  |  |  |  |  |  | 0 or 8 |
| DL Received Time Stamp |  |  |  |  |  |  |  | 0 or 8 |
| UL Sending Time Stamp |  |  |  |  |  |  |  | 0 or 8 |
| DL Delay Result |  |  |  |  |  |  |  | 0 or 4 |
| UL Delay Result |  |  |  |  |  |  |  | 0 or 4 |
| UL QFI Sequence Number |  |  |  |  |  |  |  | 0 or 3 |
| N3/N9 Delay Result |  |  |  |  |  |  |  | 0 or 4 |
| New IE flag 7(E) | New IE Flag 6 | New IE Flag 5 | New IE Flag 4 | New IE Flag 3 | New IE Flag 2 | New IE Flag 1 | New IE Flag 0 | 0 or 1 New IE Flags Octet |
| Spare |  |  |  |  |  |  | D1 UL PDCPDelay Result Ind | 0 or 1 |
| UL Congestion Information |  |  |  |  |  |  |  | 0 or 2 |
| DL Congestion Information |  |  |  |  |  |  |  | 0 or 2 |
| UL Available Data Rate |  |  |  |  |  |  |  | FFS |
| DL Available Data Rate |  |  |  |  |  |  |  | FFS |
| Padding |  |  |  |  |  |  |  | 0-3 |

Figure 5.5.2.2-1: UL PDU SESSION INFORMATION (PDU Type 1) Format

The New IE Flag in bit 6 of 2nd octet in UL PDU SESSION INFORMATION (PDU Type 1) indicates if the first octet of New IE Flags Octet is present or not.

Bit 0 of New IE Flags Octet in UL PDU SESSION INFORMATION (PDU Type 1) indicates if the D1 UL PDCP Delay Result Ind is present (1) or not (0).

Bit 1 of New IE Flags Octet in UL PDU SESSION INFORMATION (PDU Type 1) indicates if the UL Congestion Information is present (1) or not (0).

Bit 2 of New IE Flags Octet in UL PDU SESSION INFORMATION (PDU Type 1) indicates if the DL Congestion Information is present (1) or not (0).

Bit 3 of New IE Flags Octet in UL PDU SESSION INFORMATION (PDU Type 1) indicates if the UL Available Data Rate is present (1) or not (0).

Bit 4 of New IE Flags Octet in UL PDU SESSION INFORMATION (PDU Type 1) indicates if the DL Available Data Rate is present (1) or not (0).

Editor’s Note: FFS on the field size.

5.5.3	Coding of information elements in frames

5.5.3.1	PDU Type

Description: The PDU Type indicates the structure of the PDU session UP frame. The field takes the value of the PDU Type it identifies; i.e. "0" for PDU Type 0. The PDU type is in bit 4 to bit 7 in the first octet of the frame.

Value range: {0= DL PDU SESSION INFORMATION, 1=UL PDU SESSION INFORMATION, 2-15=reserved for future PDU type extensions}.

Field length: 4 bits.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

5.5.3.26	UL Congestion Information

Description: For the cases of ECN marking at UPF request, this field indicates the percentage of UL IP packets up to two decimal points that should be ECN marked for a QoS flow.

For the case of congestion information request, this field should be interpreted as a percentage of congestion level in UL up to two decimal points for a QoS Flow.

As an example, value 9574 corresponds to a percentage of 95.74%

Value range: {0..10000}.

Field length: 2 octets.

5.5.3.a	UL Available Data Rate Information

Description: This parameter indicates the UL available data rate of the QoS flow. The unit is Mbps.

Value range: {FFS}.

Field length: FFS octets.

5.5.3.b	DL Available Data Rate Information

Description: This parameter indicates the DL available data rate of the QoS flow. The unit is Mbps.

Value range: {FFS}.

Field length: FFS.octets.

5.5.3.ca	BSSI (Burst Size Indicator)

Description: This parameter indicates the presence of Burst Size (BSSize).

Value range: {0= BSSize not present, 1= BSSize present}.

Field length: 1 bit.

5.5.3.db	Burst Size (BSSize)

Description: This parameter indicates the total size of the burst.

Value range: {0..224-1}.

Field length: 3 octets.

5.5.3.e	TTNBI (TTNB Indicator)

Description: This parameter indicates the presence of Time To Next Burst (TTNB).

Value range: {0= TTNB not present, 1= TTNB present}.

Field length: 1 bit.

5.5.5.f	Time To Next Burst (TTNB)

Description: This parameter indicates the the approximate time in tenth of milliseconds to the next burst.

Value range: {0..216-1}.

Field length: 2 octets.

Editor’s Note: FFS on whether TTNB and/or BSSize related information may be removed in this clause and introduced into other frame(e.g. DL PDU SET INFORMATION).

5.5.4	Timers

Void.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////