3GPP TSG-RAN WG3 Meeting #127	R3-250233

Athens, Greece, 17th -21st February,2025

Agenda Item:	12.2

Source:	Huawei

Title:	(TP for TS 38.401/38.455) Discussion on WAB related procedures

Document for:	Discussion

1	Introduction

At the previous meeting, RAN3 had an agreement on the support UE’s CN change during the mobility of WAB-node [1].

The “two logical gNB solution” can support UE’s AMF change during WAB-gNB mobility.

It is for further discussions which solution to adopt, if the single gNB solution, the “two gNB solution” or both. It is to be further checked whether a single gNB solution is feasible via an LS to SA2, if the LS is agreeable.

RAN3 sent the LS R3-247910 to SA2 to further check with the feasibility of the single gNB based solutions. And SA2’s reply LS in S2-2501336[2] states that “There is no consensus on the support of a single logical gNB solution in SA2”.

In addition, SA2 sent an LS to RAN3 in R3-250016/S2-2412625[3] to inform their progress on the Location service of UEs served by MWAB, some works involving RAN3 are needed to align with SA2’s conclusion. This contribution will further dicusses on the mobility, authorization, resource multiplexing, parameters configuration and the support of location services for WAB.

2	Discussion

2.1	WAB-node mobility support

2.1.1 Handling of WAB-gNB’s traffic during WAB-node mobility

Regarding the objective “Handling of WAB-gNB’s traffic during WAB-node mobility, including the case where the WAB-MT’s BH PDU session changes”, there are two possibilities. If the WAB-gNB’s traffic is protected by a tunnel between the WAB-node and another node which is deployed before or co-located with the termination of this traffic (e.g., IPsec tunnerl may be used between WAB-node and the SeGW, and tunnel is carried via the BH PDU session), the WAB-gNB’s IP address for the traffic is the inner IP address which is different from the IP address allocated to the WAB-MT during the PDU session management procedures. Then the BH PDU session change has no impact to the WAB-gNB’s traffic.

Another possibility is the WAB-gNB reuse the IP address allocated to the WAB-MT, if so, the WAB-gNB’s traffic should be redirected to the new IP address assigned to the WAB-MT during PDU session update. We think the redirection of the WAB-gNB’s traffic should follow the legacy procedures.

- During the mobility of WAB node, there is no impact on the WAB-gNB’s traffic if a tunnel carried via BH PDU session is used to protect the WAB-gNB’s traffic.

- If the WAB-gNB’s traffic reuse the WAB-MT’s IP address, the WAB-gNB’s traffic should be redirected to the new IP address using legacy procedures when the WAB-MT’s IP address is updated during mobility.

2.1.2 NG management for WAB-gNB during WAB-node mobility

Considering that the UE’s AMF may change during the WAB-node mobility, the NG removal between the WAB-gNB and the original AMF should be supported after the AMF change. It is worth noting that the NG removal is also approved to be supported for the NTN topic [4][5] [6], we think the general procedure should be designed for both NTN and WAB. Specifically, in the NTN topic, the agreed CRs for 38.413 [4] and 38.410 [5] do not limit the scenario of the NG removal procedure, thus the NG removal procedure in [4] and [5] can be directly reused for WAB; the agreed 38.300 CR [6] provides general descriptions for introducing NG removal in NTN, and we can capture similar descriptions in 38.401, as in Annex A.

- RAN3 to design unified NG removal procedure for WAB and NTN, and introduce stage 2 description of NG removal procedure for WAB in TS 38.401.

2.1.3 Xn update for WAB-gNB during WAB-node mobility

The WAB-node mobility will result in the dynamic change of the neighboring relationship for the WAB-gNB. Thus, it is essential for the WAB-gNB to update the Xn connection timely in order to support the UE mobility via the Xn connection.

Legacy ANR mechanism requires UE to be present in the overlap area of neighbour cells, and may be not be a time-effictive way in the WAB scenario where the cell neighboring relationship changes fast.

An alternative solution is that the BH-gNB can provides the information of neighboring gNBs (including the TNL address) to the WAB node, then the WAB-gNB can try to establish the Xn interface towards these neighboring gNBs directly. RAN3 can further investigate whether to use RRC signalling or Xn signalling for providing the neighboring gNBs’ information.

- The BH-gNB can provide the information of neighboring gNBs (including the TNL address) to the WAB node.

2.1.4 UE’s AMF change during WAB-node mobility

Regarding the UE’s AMF change, RAN3 agreed that the “two logical gNB solution” is feasible and sent LS in R3-250016 to further check SA2’s feedback on the feasibility of two single logical gNB based solutions. In the reply LS from SA2[2], SA2 responsed that “There is no consensus on the support of a single logical gNB solution in SA2”. So, RAN3 can first capture the two logical gNB based solution for the UE’s AMF change.

- RAN3 to capture the two logical gNB based solution for supporting the UE’s AMF change in stage 2.

2.2	WAB-node authorization

Since the authorization for the WAB-MT and WAB-gNB should be performed separately, we also need to consider two cases for the non-authorized handling, i.e., one case is WAB-MT becomes not authorized and another case is WAB-gNB is not authorized.

On one hand, the WAB-MT cannot continue to provide backhaul transmission for the WAB-gNB when the WAB-MT becomes non-authorized, so that the UP resources for the established PDU session(s) should be released. On the other hand, if the WAB-gNB is non-authorized, the WAB-gNB should not work as a gNB, e.g., the WAB-gNB needs to handover the UEs to other gNB, or release the NG/Xn connection. Under this circumstance, it is meaningless for the WAB-MT to maintain the UP resources because there is no CP/UP traffic from the WAB-gNB. Hence, the WAB-MT is required to inform the 5GC serving the WAB-MT of the de-authorization status of the WAB-gNB in time, then the UP resources for the established BH PDU session(s) of WAB-MT can be released.

- The UP resources for the established BH PDU session(s) should be released if the WAB-MT/WAB-gNB is non-authorized.

- The WAB-MT informs the 5GC serving the WAB-MT of the de-authorization status of the WAB-gNB in time.

2.3	Resource multiplexing for WAB-node

2.3.1 General case

For out-band mode, the WAB-gNB and WAB-MT use different frequency, so, there is no interference between the WAB-gNB and the WAB-MT. For in-band mode, to avoid interference between access link and backhaul link, the resources (e.g., frequency domain, time domain, spatial domain, etc.) to be used by access link and backhaul link must be coordinated.Specifically, the BH-RAN node should be upgraded to be aware of which WAB-gNB is co-located with a WAB-MT, and it also needs to know the multiplexing capability of the WAB-MT and its co-located WAB-gNB, as well as the resource configurations of the WAB-gNB. For the co-location discovery, the WAB-gNB sends collocated WAB-MT’s ID to the BH-gNB over Xn, or the WAB-MT sends collocated WAB-gNB’s ID to the BH-gNB over RRC. To reduce the RRC impact, we prefer the Xn-based solution. It is proposed:

- For in-band deployment, the resource multiplexing must be conducted between access link and backhaul link.

- For the co-location discovery on the BH-gNB, the WAB-gNB sends WAB-MT’s ID to the BH-gNB over Xn.

Similarly, in IAB, there is also some procedures related to the resouce multiplexing coordination over F1AP since R16. For the IAB-DU to donor-CU resource reporting, the IAB-DU includes the IAB Info IAB-DU IE which contains the multiplexing info and the IAB STC info. The multiplexing info represents the multiplexing capabilities between the IAB-DU’s cell and the cells configured on the co-located IAB-MT. The IAB STC info contains cell SSB Transmission Configurations (STC) information of the IAB-DU.

In addition, for the donor-CU to IAB-DU messages, e.g., the GNB-DU RESOURCE CONFIGURATION message, can provide the following information

- the resource configuration of the cells served by the IAB-DU;

- the child node’s cell resource configuration and multiplexing capabilities;

- the neighbour nodes’ cell resource configuration;

- the NA resource configuration of the serving cell of the co-located IAB-MT.

For the BH-gNB, it would be helpful if the BH-gNB can know the information of WAB-node to avoid scheduling confliction between WAB-MT’s backhaul link and the WAB-gNB’s access link, similar as the parent node to know the child IAB node’s configuration in IAB case. So, the WAB-gNB should report its cell resource configuration and multiplexing capabilities to the BH-gNB.

- The WAB-gNB sends the multiplexing capabilities and the resource configuration information of its served cells to the BH-gNB.

For the WAB-gNB, since it is a full gNB and the relation between WAB-gNB and BH-gNB is somehow different from the relation between IAB-DU and donor-CU, it is FFS whether the BH-gNB can configure the WAB-gNB’s cell resource like the donor-CU to IAB-DU resource configuration, which needs to be coordinated with RAN1. Furthermore, it would be helpful if the WAB-gNB is informed the neighbour nodes’ cell resource configuration, for the collision avoidance. FFS on whether it is informed by the BH-gNB or the neighbour node itself. For the child node and serving cell configurations, since the WAB-gNB should not have any child node, the “child node” related configurations in IAB is not needed for WAB; the “NA resource configuration of the serving cell of the co-located WAB-MT” is also not needed, since the parent node should not be an IAB-node, and there is no “NA resource” for a non-IAB-DU. In summary, it is proposed:

- The WAB-gNB is informed the neighbour nodes’ cell resource configuration. FFS on whether it is informed by the BH-gNB or the neighbour node itself.

- RAN3 to coordinate with RAN1 on whether the BH-gNB configures the resource of the WAB-gNB cells.

2.3.2 Resource multiplexing during WAB-node mobility

During WAB mobility, the WAB-node resource multiplexing by the target BH-gNB may be untimely, causing the performance degradation. This issue can be categorized into two cases:

- Case 1: If the Xn interface between the WAB-gNB and the target BH-gNB does not exist, the resource multiplexing related signalling cannot be exchanged between the WAB-gNB and the target BH-gNB.

- Case 2: Even if the Xn interface is established, the performance degradation may still occur in this case, since the WAB-node may only perform resource multiplexing coordination with the serving BH-gNB but not the candidate target BH-gNBs before the WAB-MT handover to the target gNB.

Therefore, before the WAB-MT handover execution, it will be beneficial if the target BH-gNB and the WAB-gNB can be aware of each other, and implement resource multiplexing coordination timely. If the Xn interface does not exist, the Xn interface setup can be triggered at that time, and then the resource multiplexing can be implemented. To address this issue, two solutions are provided here:

- Solution 1. The target BH-gNB trigger the resource multiplexing coordination towards the WAB-gNB during the co-located WAB-MT’s handover. This solution requires that the target BH-gNB can be aware of the co-location between the WAB-gNB and the WAB-MT, and establishes the Xn interface towards the WAB-gNB if needed. Then the resource multiplexing is implemented.

- Solution 2. The WAB-gNB trigger the resource multiplexing coordination towards the target BH-gNB during the co-located WAB-MT’s handover. This solution requires that the WAB-gNB knows the BH-gNB’s ID and can provide information for co-location discovery at the target BH-gNB. Then the resource multiplexing can be implemented.

It is proposed:

- For the timely resource multiplexing during mobility, the resource multiplexing coordination between target BH-gNB and the WAB-gNB can be conducted before the completion of WAB-MT’s handover.

2.4	Parameter configuration for WAB-node

The WID also includes the handling of PCI collision avoidance on WAB-gNBs. According to the “NOTE 1: For PCI collision avoidance and reconfiguration of TAC and RANAC on WAB-gNBs, follow the conclusion of mobile IAB”, it can be observed that the legacy PCI assignment which includes both the centralized way and distributed way should be applicable to the WAB-gNB directly. For mobile IAB, the PCI can be re-configured by donor CU, but there is no donor CU for WAB-gNB, so, the PCI of WAB-gNB can only be configured or re-configured by the OAM. And, as specified in TS 38.401, the PCI space can be partitioned between mobile IAB cells and stationary cells by implementation. So, it is straightforward to allow such implementation-based PCI space partition can also be supported for WAB cell. But there is no specification impact.

- The PCI of the WAB-gNB is (re-)configured by OAM, same as legacy gNB.

- PCI space partition for mobile IAB is also applicable for the PCI collision avoidance of WAB-node, but up to implementation.

2.5	Location Service involving WAB-node

In this meeting, RAN3 received an LS on Location service of UEs served by MWAB from SA2 [3]. The content from SA2’s LS is copied as below.

-------------Excerpted from S2-2412625-------------

While waiting for the response of S2-2405822 (LS on FS_VMR_Ph2 solution impacts to RAN) and S2-2407345 (LS on questions regarding FS_VMR_Ph2), SA2 would like to inform RAN3 that SA2 had progressed the work on location service of UEs served by MWAB in TS 23.273 v19.0.0 and TS 23.501 v19.1.0, and the related CRs are attached.

SA2 noted that such aspect may need the associating work in RAN3.

The potential impacts on RAN3 includes the following aspects as defined in clause 6.1.5 of TS 23.273:

-	TRP Information Exchange supported by MWAB-gNB in TS 38.455: "When the MWAB-gNB is integrated to the 5GS, the OAM triggers the LMF to perform TRP Information Exchange procedure. The LMF learns that a new integrated TRP at a MWAB-gNB is mobile and its UE ID (GPSI) of the MWAB-UE via a TRP information exchange towards the MWAB-gNB with the Cell ID of the TRP."

-	Additional ULI sent by the MWAB-gNB to the AMF in TS 38.413: when AMF is aware that the target UE is served by a MWAB based on the additional ULI, then "the AMF serving the target UE may indicate to the LMF that the serving cell is an MWAB (if applicable)".

-	Location and velocity information in TS 38.455: "If Network Assisted procedure is used, the MWAB-gNB may provide the MWAB-UE updated location and velocity information and the time obtained them to the LMF via NRPPa message as defined in TS 38.455."

And the following aspect as in the agreed CR 5687 of TS 23.501:

-	Additional ULI sent by the MWAB-gNB to the AMF in TS 38.413: "If serving cell change report is required by AMF, the MWAB-gNB shall report Additional ULI (and ULI) for the UE even when only serving cell of the MWAB-UE changes."

The design of additaionl ULI is analysed in our another paper [9]. Here, we provide an analysis of the impact on TS 38.455 for supporting the LCS enhancement involving WAB.

According to the description of the 5GC-MT-LR procedure involving MWAB in Clause 6.4.5 of TS 23.273, it is captured in step 5 that the LMF initiates a NRPPa TRP Information Exchange procedure to request the updated location information of the TRP(s) associated with MWAB(s) by directing the TPR information exchange message to the MWAB-gNB, for example, by setting the NPRRa TRP Information Type Item IE to “mobile TRP location info".  We observe that the “mobile TRP location info” was also used in the mobile IAB involved positioning. So, it seems the “mobile TRP” is general enough to cover both mobile IAB and WAB.

Obervation 1: The concept of “mobile TRP” is applicable to both mobile IAB and WAB.

During the NRPPa TRP Information Exchange procedure, the LMF should know the TRP is at a MWAB-gNB and the UE ID (GPSI) of the MWAB-UE via a TRP information reported by the MWAB-gNB.

Similarly, R18 mobile IAB introduced the Mobile IAB-MT UE ID IE in TRP Information IE, which is included in the condition that the TRP type is indicated as the mobile trp. One issue here is how to indicate the UE ID of the WAB-MT and the corresponding TRP type, i.e, in what degree it can reuse the IEs for mobile IAB.

Companies proposed to introduce a new mobile trp in wab-gnb TRP type, and a new WAB-MT UE ID IE for decoupling the scenarios of mobile IAB and WAB [10].

We provide two other alternatives here.

- Alt. 1, the Mobile IAB-MT UE ID IE could be reused directly for WAB, requiring the semantics description that “the UE ID of IAB-MT or the WAB-MT associated with mobile TRP”. In this option, the legacy mobile trp type indication is reused, and the specific TRP type, i.e., mobile IAB or WAB, are not differentiated in the message. Either the mobile IAB-MT UE ID or WAB-MT UE ID is included in this Mobile IAB-MT UE ID IE.

- Alt. 2, a new IE for WAB-MT (e.g. WAB-MT UE ID IE) could be introduced to indicate the UE ID of the WAB-MT associated with the mobile TRP, and if this IE is present, the legacy Mobile IAB-MT UE ID should be ignored.  this alternative also reuses the mobile trp type indication like option 1.

Obervation 2: Both Alt. 1 and Alt. 2 can reuse the current mobile trp type for WAB.

It is also captured that "If Network Assisted procedure is used, the MWAB-gNB may provide the MWAB-UE updated location and velocity information and the time obtained them to the LMF via NRPPa message as defined in TS 38.455". The Mobile TRP Location Information IE is defined in TS 38.455 as blow [11].

-------------Excerpted from TS 38.455-------------

9.2.88	Mobile TRP Location Information

This IE contains location information for one mobile TRP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Location Information | M |  | OCTET STRING | Location of the mobile TRP, Includes the locationEstimate IE as defined in TS 37.355 [14] |
| Velocity Information | O |  | OCTET STRING | Velocity of the mobile TRP, Includes the velocityEstimate IE as defined in TS 37.355 [14] |
| Location Time stamp | O |  | Time Stamp 9.2.42 | Indicates the time when the mobile TRP location information is generated. |

It is observed that the Mobile TRP Location Information IE contains the location and velocity information for the mobile TRP, and the elements includedare not limited to mobile IAB. Therefore, the Mobile TRP Location Information IE can be directly reused for WAB.

Obervation 3: It is possible to reuse directly the Mobile TRP Location Information IE for WAB.

- RAN3 to discuss the following alternatives for indicating the UE ID of the WAB-MT while reusing the mobile trp type in TRP information IE:

  - Option 1: Use the existing Mobile IAB-MT UE ID IE to indicate the UE ID of the IAB-MT or the WAB-MT associated with the mobile TRP.

  - Option 2: Introduce a new IE (e.g.WAB-MT UE ID IE) to indicate the UE ID of the WAB-MT associated with the mobile TRP.

The corresponding TPs accordingly are provided in Annex B, C based on the analysis in this paper.

3	Conclusion

This paper provides mobility and other issues for the WAB-node. The proposals are as follows:

WAB-node mobility support

- During the mobility of WAB node, there is no impact on the WAB-gNB’s traffic if a tunnel carried via BH PDU session is used to protect the WAB-gNB’s traffic.

- If the WAB-gNB’s traffic reuse the WAB-MT’s IP address, the WAB-gNB’s traffic should be redirected to the new IP address using legacy procedures when the WAB-MT’s IP address is updated during mobility.

- RAN3 to design unified NG removal procedure for WAB and NTN, and introduce stage 2 description of NG removal procedure for WAB in TS 38.401.

- The BH-gNB can provide the information of neighboring gNBs (including the TNL address) to the WAB node.

- RAN3 to capture the two logical gNB based solution for supporting the UE’s AMF change in stage 2.

WAB-node authorization

- The UP resources for the established BH PDU session(s) should be released if the WAB-MT/WAB-gNB is non-authorized.

- The WAB-MT informs the 5GC serving the WAB-MT of the de-authorization status of the WAB-gNB in time.

Resource multiplexing for WAB-node

- For in-band deployment, the resource multiplexing must be conducted between access link and backhaul link.

- For the co-location discovery on the BH-gNB, the WAB-gNB sends WAB-MT’s ID to the BH-gNB over Xn.

- The WAB-gNB sends the multiplexing capabilities and the resource configuration information of its served cells to the BH-gNB.

- The WAB-gNB is informed the neighbour nodes’ cell resource configuration. FFS on whether it is informed by the BH-gNB or the neighbour node itself.

- RAN3 to coordinate with RAN1 on whether the BH-gNB configures the resource of the WAB-gNB cells.

- For the timely resource multiplexing during mobility, the resource multiplexing coordination between target BH-gNB and the WAB-gNB can be conducted before the completion of WAB-MT’s handover.

Parameter configuration for WAB-node

- The PCI of the WAB-gNB is (re-)configured by OAM, same as legacy gNB.

- PCI space partition for mobile IAB is also applicable for the PCI collision avoidance of WAB-node, but up to implementation.

Location Service involving WAB-node

Obervation 1: The concept of “mobile TRP” is applicable to both mobile IAB and WAB.

Obervation 2: Both Alt. 1 and Alt. 2 can reuse the current mobile trp type for WAB.

Obervation 3: It is possible to reuse directly the Mobile TRP Location Information IE for WAB.

- RAN3 to discuss the following alternatives for indicating the UE ID of the WAB-MT while reusing the mobile trp type in TRP information IE:

  - Option 1: Use the existing Mobile IAB-MT UE ID IE to indicate the UE ID of the IAB-MT or the WAB-MT associated with the mobile TRP.

  - Option 2: Introduce a new IE (e.g.WAB-MT UE ID IE) to indicate the UE ID of the WAB-MT associated with the mobile TRP.

4. Reference

- RAN3 Chairnotes, RAN3#126.

- S2-2501336, Reply LS on FS_VMR_Ph2 solution impacts to RAN (MWAB mobility)

- S2-2412625, LS on Location service of UEs served by MWAB, SA2

- R3-247935, (BL CR to 38.413) Support for Regenerative Payload and MBS broadcast in NR NTN

- R3-247034, (BL CR to 38.410) Introduce NG Removal procedure

- R3-247934, (BL CR to 38.300) Support for Regenerative Payload and MBS broadcast in NR NTN

- TS 38.413, NG Application Protocol (NGAP), V18.2.0

- TS 38.410, NG general aspects and principles, V18.2.0

- R3-250232, Architecture, Access Control and Additional ULI for WAB, Huawei.

- R3-247198, (TP to 38.455) Support of Location Service Involving WAB-Nodes, ZTE

- TS 38.455, NR Positioning Protocol A (NRPPa), V18.4.0

Annex A——TP for BL CR of TS 38.401

Start of Change

X.2	Configuration of WAB-node

X.2.1	General

The following configurations of the WAB-node may need to be updated as the node moves:

-	The parameters that enable the WAB-gNB to select and connect to the AMF(s) that serve the UE(s).

-	The parameters that enable the WAB-gNB to connect to, and communicate with, the OAM system.

-	The parameters that the WAB-gNB should broadcast, e.g., the TAC(s), the cell ID(s), the RANAC(s).

A WAB-node may be provisioned with the parameters pertinent to different potential locations of the WAB-node. Alternatively, the OAM can provision configuration parameters to the WAB-node based on the location of the node. In that case, the continuity of OAM connectivity needs to be ensured as the WAB-node moves.

X.2.2	IP address configuration for the WAB-gNB

A WAB-MT obtains IP address(es) for the PDU sessions in the same manner as a legacy UE.

The IP address(es) of the WAB-MT can be used for the backhauling of WAB-gNB’s NG, Xn and OAM traffic. In case the WAB-gNB uses the IPsec tunnel mode to protect the OAM, NG and/or Xn traffic, the IP address(es) of the WAB-MT is used as outer IP address(es), and the allocation of the inner tunnel IP address(es) is outside of 3GPP scope.

X.2.3	TAC/RANAC (re-)configuration for a WAB-gNB’s cell

The TAC/RANAC of WAB-gNB’s cell is configured by the OAM, and it can be reconfigured by the OAM during the mobility of WAB-node. The TAC/RANAC of the WAB-gNB’s cell may be the same as, or different than, the TAC/RANAC of the co-located WAB-MT’s serving cell. The TAC/RANAC broadcast by the WAB-gNB’s cell can be changed in order to reflect the WAB-node’s physical location.

X.2.4	PCI (re-)configuration for a WAB-gNB’s cell

The PCI of WAB-gNB’s cell is configured by the OAM, and it can be reconfigured by the OAM. The PCI Optimization Function for non-split gNB as specified in TS 38.300 [2] is applicable to the WAB-gNB. The PCI space can be partitioned between WAB-gNB’s cells and stationary cells by implementation.

Next Change

## X.Y	WAB-node mobility

### X.Y.1	General

The following scenarios for WAB-node mobility are supported:

1.	The UE´s AMF/UPF remains unchanged as the WAB-gNB moves.

2.	The UE´s AMF/UPF changes as the WAB-gNB moves.

3.	The BH-UPF/BH-AMF remains unchanged as the WAB-MT moves inside a PLMN.

4.	The BH-UPF/BH-AMF changes as the WAB-MT moves inside a PLMN.

The WAB-gNB does not change PLMN while the WAB-node moves across the network.

### X.Y.2	WAB-MT mobility

#### X.Y.2.1 General

The WAB-MT reuses legacy mobility handover procedures for the UE as it moves throughout the BH-RAN.

#### X.Y.2.2 The BH-UPF remains unchanged as the WAB-MT moves inside a PLMN

In case the WAB-MT’s PSA UPF does not change during these mobility procedures, the IP addresses allocated for the WAB-MT’s PDU sessions do not change. Therefore, the OAM, NG and/or Xn traffic of the WAB-gNB carried over the BH PDU session(s) remain unaffected.

#### X.Y.2.3 The BH-UPF changes as the WAB-MT moves inside a PLMN

When the WAB-MT’s PSA UPF need to be changed due to WAB-MT’s mobility, new BH PDU session(s) need to be established reusing existing mechanisms defined in TS 23.502 [32]. In this case, the (outer) IP addresses used by the WAB-gNB for the transport of OAM, NG and/or Xn traffic of the WAB-gNB will change.

If the WAB-gNB’s traffic reuse the WAB-MT’s IP address, the WAB-gNB’s traffic should be redirected to the new IP address when the WAB-MT’s IP address is updated during mobility.

In case the WAB-gNB uses the IPsec tunnel mode to protect its traffic, the update of the outer IP address will not impact the WAB-gNB’s OAM, NG and/or Xn traffic.

### X.Y.3	WAB-gNB mobility

#### X.Y.3.1	General

During WAB-node movement, establishment and removal of the WAB-gNB’s NG and/or Xn connections may be needed.

Establishment of Xn connections of the WAB-gNB with BH-RAN nodes, as well as with surrounding RAN nodes, is supported, and it can follow legacy procedures.

#### X.Y.3.2	WAB-gNB mobility without change of UE’s AMF(s)

During WAB-node movement, radio configuration parameters of the WAB-gNB may be changed (e.g., cell ID, PCI and/or TAC) without the change of the UE’s AMF(s). This change of radio configuration parameters may require the UE handling by means of, e.g., intra-gNB handover and/or Mobility Registration Update as defined in TS 23.502 [7].

#### X.Y.3.3	WAB-gNB mobility with change of UE’s AMF(s)

Due to WAB-node movement, the change of UE’s AMF(s) may be needed, based on, e.g., WAB-node’s current location and/or additional criteria. The NG connection handling and WAB-gNB configuration update may affect the served UEs.

The steps for the solution with two logical WAB-gNBs are as follows:

1.	The WAB-node may obtain the configuration parameters needed to establish the connection to the UE’s new AMF(s).

2.	A new logical WAB-gNB is instantiated, and it establishes NG connection(s) towards one or more new AMF(s).

3.	The new logical WAB-gNB may activate one or more new cells, with new cell configuration parameters related to the WAB-gNB’s current location. The new cells may broadcast the radio parameters configured for the new AMF(s), e.g., TAC, etc. The old cell(s) remain(s) active.

4.	The UEs are handled as follows:

-	A UE in RRC_CONNECTED state is handed over between an old cell served by the old logical WAB-gNB and a new cell served by the new logical WAB-gNB via NG-based handover with AMF relocation, as defined in TS 23.502 [7]. When all UEs in RRC_CONNECTED state have been handed over, the old cell(s) are removed from service.

-	A UE in RRC_IDLE or RRC_INACTIVE state camping on the old cell(s) reselects a new cell, and legacy procedure (e.g., Mobility Registration Update procedure as defined in TS 23.502 [7]) is performed.

5.	The NG connection(s) between the old logical WAB-gNB and the initial AMF(s) are removed and the WAB-gNB is removed from service.

#### X.Y.3.4	NG Removal

For the case of WAB-gNB mobility, when disconnecting from an AMF is required, the WAB-gNB may request the removal of NG interface by inititing the NG Removal procedure toward the AMF.

End of Change

Annex B——Alternative 1 TP for TS 38.455

Start of Change

9.2.25	TRP Information

The TRP Information IE contains information for one TRP within an NG-RAN node.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| TRP ID | M |  | 9.2.24 |  | - |  |
| TRP Information Type |  | 1 .. <maxnoTRPInfoTypes> |  |  | - |  |
| >CHOICE TRP Information Item | M |  |  |  | - |  |
| >>NR PCI |  |  | INTEGER (0..1007) | NR Physical Cell ID |  |  |
| >>NR CGI |  |  | 9.2.9 |  |  |  |
| >>NR ARFCN |  |  | INTEGER (0..3279165) |  |  |  |
| >>PRS Configuration |  |  | 9.2.44 |  |  |  |
| >>SSB Information |  |  | 9.2.54 |  |  |  |
| >>SFN Initialisation Time |  |  | Relative Time 1900 9.2.36 |  |  |  |
| >>Spatial Direction Information |  |  | 9.2.45 |  |  |  |
| >>Geographical Coordinates |  |  | 9.2.46 |  |  |  |
| >>TRP type |  |  | ENUMERATED (prs-only-tp, srs-only-rp, tp, rp, trp, …, mobile trp) | TS 38.305 [18] | YES | reject |
| >>On-demand PRS TRP Information |  |  | 9.2.65 |  | YES | reject |
| >>TRP Tx TEG Association |  |  | 9.2.79 |  | YES | reject |
| >>TRP Beam Antenna Information |  |  | 9.2.82 |  | YES | reject |
| >>Mobile TRP Location Information | M |  | 9.2.88 |  | YES | ignore |
| >>Common TA Parameters | M |  | 9.2.89 |  | YES | reject |
| Mobile IAB-MT UE ID | C-ifMobileTRP |  | OCTET STRING | The UE ID of the IAB-MT or the WAB-MT associated with the mobile TRP. Includes the GPSI as defined in TS 29.571 [21] | YES | reject |

| Condition | Explanation |
| --- | --- |
| ifMobileTRP | This IE shall be present if the TRP type IE is set to the value "mobile trp" |

| Range bound | Explanation |
| --- | --- |
| maxnoTRPInfoTypes | Maximum no of TRP information types that can be requested and reported with one message. Value is 64. |

End of Change

Annex C——Alternative 2 TP for TS 38.455

Start of Change

8.2.8.2	Successful Operation

Figure 8.2.8.2-1: TRP Information Exchange procedure, successful operation

The LMF initiates the procedure by sending a TRP INFORMATION REQUEST message. The NG-RAN node responds with a TRP INFORMATION RESPONSE message that contains the requested TRP information.

If the TRP List IE is included in the TRP INFORMATION REQUEST message, the NG-RAN node should include in the TRP INFORMATION RESPONSE message, the requested information for all TRPs included in the TRP List IE.

If the TRP List IE is not included in the TRP INFORMATION REQUEST message, the NG-RAN node should include the requested information for all TRPs hosted by the NG-RAN node in the TRP INFORMATION RESPONSE message.

If the PRS Muting IE is included in the PRS Configuration IE in the TRP INFORMATION RESPONSE message, the LMF may take it into account as the muting information for the given PRS resource set.

If the QCL Info IE is included in the PRS Configuration IE in the TRP INFORMATION RESPONSE message, the LMF may take it into account for the given PRS resource list.

If the Aggregated PRS Resource Set List IE is included in the PRS Configuration IE in the TRP INFORMATION RESPONSE message, the LMF may take it into account as the PRS resource sets linked for PRS bandwidth aggregation.

If the DL-PRS Resource Coordinates IE is included in the Geographical Coordinates IE in the TRP Information IE in the TRP INFORMATION RESPONSE message, the LMF may take it into account as the DL PRS Resource Coordinates relative to the TRP coordinate.

If the Mobile IAB-MT UE ID IE is included in the TRP Information IE in the TRP INFORMATION RESPONSE message, the LMF shall, if supported, use this information to determine an updated location of the Mobile TRP as specified in TS 23.273 [20].

If the WAB-MT UE ID IE is included in the TRP Information IE in the TRP INFORMATION RESPONSE message, the LMF shall, if supported, use this information to determine an updated location of the Mobile TRP as specified in TS 23.273 [20].

If the TRP Information Type Item IE is set to 'mobile trp location info', the NG-RAN node shall, if supported, derive the location of the Mobile TRP as specified in TS 23.273 [20] and include the Mobile TRP Location Information in the TRP INFORMATION RESPONSE message.

Next Change

9.2.25	TRP Information

The TRP Information IE contains information for one TRP within an NG-RAN node.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| TRP ID | M |  | 9.2.24 |  | - |  |
| TRP Information Type |  | 1 .. <maxnoTRPInfoTypes> |  |  | - |  |
| >CHOICE TRP Information Item | M |  |  |  | - |  |
| >>NR PCI |  |  | INTEGER (0..1007) | NR Physical Cell ID |  |  |
| >>NR CGI |  |  | 9.2.9 |  |  |  |
| >>NR ARFCN |  |  | INTEGER (0..3279165) |  |  |  |
| >>PRS Configuration |  |  | 9.2.44 |  |  |  |
| >>SSB Information |  |  | 9.2.54 |  |  |  |
| >>SFN Initialisation Time |  |  | Relative Time 1900 9.2.36 |  |  |  |
| >>Spatial Direction Information |  |  | 9.2.45 |  |  |  |
| >>Geographical Coordinates |  |  | 9.2.46 |  |  |  |
| >>TRP type |  |  | ENUMERATED (prs-only-tp, srs-only-rp, tp, rp, trp, …, mobile trp) | TS 38.305 [18] | YES | reject |
| >>On-demand PRS TRP Information |  |  | 9.2.65 |  | YES | reject |
| >>TRP Tx TEG Association |  |  | 9.2.79 |  | YES | reject |
| >>TRP Beam Antenna Information |  |  | 9.2.82 |  | YES | reject |
| >>Mobile TRP Location Information | M |  | 9.2.88 |  | YES | ignore |
| >>Common TA Parameters | M |  | 9.2.89 |  | YES | reject |
| Mobile IAB-MT UE ID | C-ifMobileTRP |  | OCTET STRING | The UE ID of the IAB-MT associated with the mobile TRP. Includes the GPSI as defined in TS 29.571 [21] | YES | reject |
| WAB-MT UE ID | O |  | OCTET STRING | The UE ID of the IAB-MT or WAB-MT associated with the mobile TRP. Includes the GPSI as defined in TS 29.571 [21]. If this IE is included, the Mobile IAB-MT UE ID should be ignored. | YES | reject |

| Condition | Explanation |
| --- | --- |
| ifMobileTRP | This IE shall be present if the TRP type IE is set to the value "mobile trp" |

| Range bound | Explanation |
| --- | --- |
| maxnoTRPInfoTypes | Maximum no of TRP information types that can be requested and reported with one message. Value is 64. |

Next Change

    - 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NRPPA-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

>>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<<

id-NR-PCI,

id-SCS-SpecificCarrier,

id-MeasBasedOnAggregatedResources,

id-UEReportingInterval-milliseconds,

id-WAB-MT-UE-ID

>>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<<

#### -- T

>>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<<

TRPInformation ::= SEQUENCE {

tRP-ID							TRP-ID,

tRPInformationTypeResponseList	TRPInformationTypeResponseList,

iE-Extensions					ProtocolExtensionContainer { { TRPInformation-ExtIEs } }		OPTIONAL,

...

}

TRPInformation-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-Mobile-IAB-MT-UE-ID			CRITICALITY reject EXTENSION Mobile-IAB-MT-UE-ID		PRESENCE optional},

--This IE shall be present if the TRP type IE is set to the value "mobile trp"

{ ID id-WAB-MT-UE-ID			CRITICALITY reject EXTENSION Mobile-MT-UE-ID

PRESENCE optional},

...

}

>>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<<

WAB-MT-UE-ID ::= OCTET STRING

Next Change

    - 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

>>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<<

id-NR-PCI														ProtocolIE-ID ::= 155

id-SCS-SpecificCarrier											ProtocolIE-ID ::= 156

id-MeasBasedOnAggregatedResources								ProtocolIE-ID ::= 157

id-UEReportingInterval-milliseconds								ProtocolIE-ID ::= 158

id-WAB-MT-UE-ID												ProtocolIE-ID ::= XXX

END

-- ASN1STOP

End of Change