3GPP TSG-RAN WG3 Meeting #127 	R3-250266

# Athens, Greece, EU, 17. - 21.02.2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 1395 | rev | 2 | Current version: | 17.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of a wrong description of the S-NG-RAN node Counter Check procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2025-02-06 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Counter Check procedure that is used to request the UE to verify the amount of data sent/ received on each DRB. More specifically, the UE is requested to check if, for each DRB, the most significant bits of the COUNT match with the values indicated by the network. In case of DC, the UE is contacted by the MN, but the initiating node may be the SN, in case of the SN-terminated bearers. The SN uses a dedicated class-2 procedure towards the MN to request executing the Counter Check with the UE. In LTE DC, the procedure is the SeNB Counter Check procedure. Since the SeNB could host only own bearers, which were called in LTE “SCG bearers” (see TS 36.300, chapter 4.9.2), the description of the procedure declares that SeNB can initiate the procedure for “SCG bearers”. When EN-DC was created, another procedure was defined in X2AP: SgNB Counter Check procedure. In EN-DC, the en-gNB may host SCG bearers, MCG bearers or split bearers. Therefore, in the description of the SgNB Counter Check procedure, it is declared correctly that the procedure can be used for SN terminated bearers. In XnAP, there is only one procedure, the S-NG-RAN node Counter Check procedure. Unfortunately, when it was defined, the description must have been copied from the SeNB Counter Check procedure instead of using the SgNB Counter Check procedure. And thus, in the description of XnAP procedure, the “SCG bearers” are mentioned instead of “SN terminated bearers”. Since in NR DC options the “SCG bearers” has a meaning, different than intended here, the current description is an error and requires correction (not a clarification!). |  |  |  |  |  |  |  |  |
| Summary of change: |  | The wrong “SCG bearers” is replaced with the correct “SN terminated bearers”. Impact assessment towards the previous version of the specification (same release): The impact can be considered isolated because the change concerns only one IE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Since “SCG bearers” do exist in case of Xn-based DC options, the description may be interpreted literary. In this case, the issue may cause interoperability problems. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.8.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev.1: Resubmission to R3#126 Rev.2: Resubmission to R3#127 |  |  |  |  |  |  |  |  |

First change, ommited text not changed

### 8.3.8	S-NG-RAN node Counter Check

#### 8.3.8.1	General

This procedure is initiated by the S-NG-RAN node to request the M-NG-RAN node to execute a counter check procedure to verify the value of the PDCP COUNTs associated with SN terminated bearers established in the S-NG-RAN node.

The procedure uses UE-associated signalling.

Remaining text not changed