3GPP TSG-RAN WG3#128 	R3-253136

St. Julien’s, Malta, 19-23 May 2025

Agenda item:	13.2 Support for inter-CU LTM

Source:	Nokia

Title:	TP (BL CR TS 38.300, TS 38.401, TS 38.473, TS 38.423) Remaining issues on Inter-CU LTM procedure

Document for:	Discussion and Decision

# 1	Introduction

In regard to L1/L2 Triggered Mobility (LTM), a new Work Item has been approved (RP-250339 NR mobility enhancements Phase 4), including the following related objectives and scope impacting RAN3:

• Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] o Prioritize the case when CU is acting as MN when DC is not configured o When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases:  As secondary priority, support the case where CU is acting as SN and MN is unchanged  As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released o Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM  Coordination with SA3 needed with respect to security key handling o Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support • Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] o Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM o Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] o Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] o Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure.

This paper discusses the remaining issues for the basic required mobility procedures for inter-CU L1/L2 triggered mobility (LTM). These issues are grouped based on the Inter-CU LTM phases (preparation phase, early synchronization phase, execution phase, and completion phase).

# 2	Preparation Phase

With respect to the preparation phase, the following aspects require further consideration.

## 2.1	Retrieval of CSI-RS Configuration

To support CSI-RS based LTM, RAN3 has previously reached agreement for retrieving CSI-RS configuration over Xn and reflected it in TS 38.300 BLCR.

The source CU can request candidate CU to provide CSI-RS configuration in HANDOVER REQUEST message, and candidate CU signals the CSI-RS configuration in HANDOVER REQUEST ACKNOWLEDGEMENT message.

This mechanism is also required over F1 for both Intra-gNB LTM and Inter-gNB LTM scenarios where split architecture is used. Hence, Stage 2 changes to TS 38.401 describing retrieval CSI RS configuration from CU to DU and reporting back from DU to CU are also necessary.

Proposal 1: Add description in Stage 2 TS 38.401 for describing that CU can request Candidate DU to provide CSI-RS configuration in UE CONTEXT SETUP REQUEST message, and Candidate DU signals the CSI-RS configuration in UE CONTEXT SETUP RESPONSE message.

## 2.2	Semi-persistent CSI-RS Support for LTM

At prior meetings, RAN3 agreed to introduce new F1AP and XnAP procedures to support Semi Persistent CSI-RS activation/deactivation in candidate cell(s). However, RAN3 left open the detail on the required IEs to be introduced in the newly introduced procedures.

Introduce two new Class-1 UE-associated procedures over F1AP to support the source gNB-DU to trigger the activation/deactivation of the SP CSI-RS transmission in the candidate cell(s). Introduce a new class-1 UE-associated procedures over XnAP for the SP CSI-RS activation/deactivation.

The new procedures are understood to operate in the following sequence:

- Inter-DU case within same gNB

  - Source-DU to CU: 					F1AP DU-CU CSI-RS COORDINATION REQUEST

  - CU to Candidate-DU:				F1AP CU-DU CSI-RS COORDINATION REQUEST

  - Candidate-DU to CU: 				F1AP DU-CU CSI-RS COORDINATION RESPONSE

  - CU to Source-DU: 					F1AP DU-CU CSI-RS COORDINATION RESPONSE

- Inter-CU case across different gNBs

  - Source-DU to Source-CU: 			F1AP DU-CU CSI-RS COORDINATION REQUEST

  - Source-CU to Candidate-CU:		XnAP CSI-RS COORDINATION REQUEST

  - Candidate-CU to Candidate-DU:	F1AP CU-DU CSI-RS COORDINATION REQUEST

  - Candidate-DU to Candidate-CU:	F1AP DU-CU CSI-RS COORDINATION RESPONSE

  - Candidate-CU to Source-CU:		XnAP CSI-RS COORDINATION RESPONSE

  - Source-CU to Source-DU: 			F1AP DU-CU CSI-RS COORDINATION RESPONSE

In principle, the contents of the F1AP and XnAP messages will be common upon request from the source cell towards the target for both Intra-gNB and Inter-gNB scenarios, and should comprise of the following:

- A list containing

  - NR-CGI, indicating the target cell

  - SP CSI-RS transmission request type, indicating whether to activate or deactivate the SP CSI-RS transmission

  - CSI-RS Resources ID List comprising a list of SP CSI-RSs to activate or deactivate

  - TCI State/QCL-info List, indicating the associated TCI State for each CSI-RS being activated/deactivated

The TCI state/QCL-info is retrieved and informed to source cell during the LTM preparation phase utilizing existing LTM framework. For an SP CSI-RSs, the associated TCI state is selected and provided dynamically. This can be selected by the source cell based on latest SSB or CSI-RS measurements.

Proposal 2: The F1AP DU-CU CSI-RS COORDINATION REQUEST and CU-DU CSI-RS COORDINATION REQUEST messages include a list containing: NR-CGI, SP CSI-RS transmission request (activate/deactivate), CSI-RS Resources ID List, and TCI State/QCL-info List.

Proposal 3: The XnAP CSI-RS COORDINATION REQUEST message includes a list containing: NR-CGI, SP CSI-RS transmission request (activate/deactivate), CSI-RS Resources ID List, and TCI State/QCL-info List.

## 2.3	Security Considerations

RAN3 has reached the following agreements with respect to security. However, it was left open whether and how to update to Candidate gNBs. Below excerpt from RAN3#127bis chair notes.

For the initial inter-CU LTM preparation, the source gNB computes the KgNB*(s) (per candidate cell) for the candidate gNB, and forwards the {KgNB*, NCC} pair(s) to the candidate gNB via a Handover Request message as legacy. The new source gNB needs to provide the new KgNB*(s) to the corresponding candidate gNB via a LTM configuration update procedure. FFS on the procedure design either per cell or per gNB. Whether to define additional information from CU to DU to deliver NCC value.

In our understanding, the procedures for Rel-19 Inter-CU LTM use Rel-18 framework as basis and operate on a per cell basis. Therefore, also after Inter-CU cell switch, the new source CU will trigger LTM CONFIGURATION UPDATE procedures on a per cell level basis, and the KeygNB* will be signaled for each of the cells.

Proposal 4: For Inter-CU LTM, the security keys provided in LTM CONFIGURATION UPDATE procedure are on a per cell level basis.

Further, in Stage 2 draftCR from RAN3 in R3-253086, there is statement that security keys may be included.

20. The new source gNB (i.e., the target gNB) sends the LTM CONFIGURATION UPDATE message to the candidate gNBs. This message may include the new security key(s) to be used with the UE.

In our view this is incorrect, as the security keys need to be provided to all Candidate CUs after cell switch, since this is also required in case of LTM recovery, since the UE may attempt to access a different cell in a different CU than originally directed to. Therefore, the security key is required for this operation to succeed. Thus, from a Stage 2 perspective in 38.300, the optionality of providing keys should be removed and mandated instead.

Proposal 5: For Inter-CU LTM, the LTM CONFIGURATION UPDATE procedure provided from the new Source gNB to Candidate gNBs includes the security key. Thus, the “may” wording shall be removed from procedural text in TS 38.300.

## 2.4	Reference Configuration

RAN3 had earlier concluded to follow approach of single reference configuration. However, at prior last meetings it was debated whether the single reference configuration must be from a cell from the Source-CU, or whether it could also be retrieved from a cell a different Candidate-CU.

In our view, the single reference configuration to be used to generate the common configuration for all the LTM candidate cells should be maximized to address as much commonalities as possible. Therefore, the Source-CU, who is aware of all candidate LTM cells and their corresponding gNBs, is in best position to determine whether to generate the common configuration based on a cell of its own, or to request a reference configuration from one of the Candidate-CUs. Consider the following scenarios:

- Scenario 1 – Candidate-CU has majority of LTM cells being prepared: In this case, most commonalities would be achieved by using a reference from the Candidate-CU since it will host most LTM cells. E.g., such as if Source-CU is being configured two LTM cells, and in contrast the Candidate gNB-CU is being configured six LTM cells.

- Scenario 1bis – Source-CU has no LTM candidate cells: Similar to the previous case, most commonalities would be achieved from a reference from a Candidate-CU rather than a cell of the Source-CU, which in this scenario, is not hosting any LTM cells.

Furthermore, at prior meetings it was debated whether request for a reference from a candidate gNB would incur additional signaling to all cells. To this matter, given that the Source-gNB is aware of all potential candidate cells and to which Candidate-gNB they belong to, a single procedure is sufficient to request a reference from one of the Candidate-gNBs. Hence, it is not necessary to query each candidate for a reference respectively.

Proposal 6: Source-CU/Source-gNB can request a Candidate-CU/Candidate-gNB to provide a reference configuration.

Proposal 7: Remove FFS in Stage 3 with respect of request for reference configuration from candidate.

## 2.5	Subsequent LTM Update Information Mandatory vs. Optional:

In prior RAN3 discussions, it was left open whether Step 6-7 in for LTM CONFIGURATION UPDATE and LTM CONFIGURATION UPDATE ACKNOWLEDGE were mandatory or optional with a corresponding note in the BL CR. In line with Rel-18 and with flexibility for updating candidates at a flexible timing, we think that Steps 6-7 should be indicated as optional and the Note removed.

Further, these additional steps incur unnecessary delay in several scenarios, as the Source gNB-CU could be able to generate the RRC Reconfiguration message toward the UE without waiting for an additional updated response from all the Candidate gNB-CUs. Thus, carrying out any further updates toward the Candidate gNB-CUs at a later time (e.g., to update TNL related information). For example:

Single configuration: if the Source gNB-CU determines that a common LTM CSI-RS resource configuration is used, it could signal this decision to the Candidate gNB-CU(s) in the HANDOVER REQUEST message. Hence, removing the need to consolidate individual reports from each Candidate gNB-CUs prior to generating the RRC Reconfiguration message to the UE.

L3 inter-CU LTM: L3 based LTM is already agreed to be applicable to Inter-CU scenarios. Thus, another related case is when any given Candidate gNB-CU only supports L3 based LTM configuration. Such nodes need not generate a lower layer LTM configuration to be consolidated at the Source gNB-CU and could indicate it accordingly to the Source gNB-CU. The Source gNB-CU would in place proceed to generate the RRC Reconfiguration message to the UE without a second update procedure for these Candidates.

Subsequent LTM not required: Moreover, if subsequent LTM is determined not to be required after Inter-CU LTM operation either by the source or Candidate gNB-CU, these nodes could coordinate whether to restrict Subsequent Inter-CU LTM mobility via Xn signaling. In this case, Steps 6-7 would not be required entirely.

Proposal 8: Remove Note in TS 38.300 BL CR “Editor’s Note: FFS on whether step 6 and 7 are mandatory or optional.” And leave as optional procedure.

Proposal 9: RAN3 to study impact and required handling in Inter-CU LTM preparation phase when only L3 based Inter-CU LTM is supported at the Candidate gNB-CU(s).

Proposal 10: RAN3 to study means to simplify Inter-CU LTM preparation by support of using a common CSI-RS configuration, or by limiting execution of subsequent LTM.

# 3	Early Synchronization Phase

With respect to the early synchronization phase, the following aspects require further consideration.

## 3.1	TA Information Transfer

RAN3 has agreed to introduce a new non-UE associated procedure over Xn to transfer TA information.

Turn the WA into agreement: Introduce a new non-UE associated class 2 procedure on Xn, namely TA information Transfer message, to transfer the TA information from the candidate gNB-CU to the source gNB-CU. Include the following information in the TA information transfer message: Candidate Cell ID TA value Preamble index RA-RNTI FFS on the XnAP IDs.

The format of this message simply needs to match to existing F1AP messages introduced in Rel-18 for the same purpose, and adding an XnAP ID on top is not needed. Further, such addition would introduce a conflict with existing messages over F1.

Proposal 11: XnAP IDs are NOT needed in the XnAP TA INFORMATION TRANSFER message.

# 4	LTM Execution Phase

With respect to the LTM execution phase, the following aspects require further consideration.

## 4.1	Timing Advance Command (TAC)

Upon cell switch decision, the gNB-DU sends a MAC CE command to the UE which includes the TCI state indicating the UE which beams it should use to access the target cell (applicable for both RACH-less and RACH-based mobility), as well as a Timing Advance Command (TAC). Likewise, the value in the TAC indicates to the UE whether it should perform a RACH-less switch.

However, over F1AP and XnAP interfaces, the Cell Switch Notification messages although it includes the TCI State ID, lacks support for indicating whether RACH-less mobility was triggered to the UE. Therefore, the Candidate gNB-DU/Candidate gNB is unaware whether the upcoming UE access is RACH-less or RACH-based. Consequently, it can also not determine whether it should issue a Dynamic Grant (DG) for DG-based RACH-less cell switch, or whether it should activate a Configured Grant (CG) previously provided in the RRC Configuration for CG-based RACH-less switch. Thus, due to this uncertainty, the target cell is forced to issue a DG, even when the triggered switch by the source was RACH-based, leading to PDCCH and PUSCH resources being wasted. Similarly, to support CG-based RACH-less cell switch, the target cell is also forced to activate and monitor the CG, even for the RACH-based cell switch, leading to PUSCH resource waste. For these reasons, an IE from Source gNB-DU/gNB is required over F1 and Xn interfaces to indicate whether the cell switch triggered was RACH-less based correspondingly.

Proposal 12: Add a “Cell Switch Type (ENUMERATED(RACH-less, RACH-based))” in F1AP and XnAP CELL SWITCH NOTIFICATION messages.

## 4.2	Data Forwarding and TNL Information Update

To support data forwarding in case of inter-CU LTM, the Source gNB needs to transfer information regarding PDU Session Resource Information List of all other Candidate gNBs to the Target gNB to execute data forwarding for subsequent LTM. Similarly, RAN3 has agreed this information to be included within LTM CONFIGURATION UPDATE message with the exact timing left to implementation.

Reuse the LTM Configuration Update procedure to sync up configurations among candidate gNBs for subsequent LTM, including early sync configuration, configuration ID, and data forwarding addresses, etc. It is up to the network implementation when the sync up is performed: during the preparation step, or during the cell switch execution step, or after successful cell switch. Normal data forwarding may be initiated after the source gNB decides to trigger the LTM cell switch for the UE, and when exactly it is initiated is left to implementation. … Data Forwarding Information IE is a list including multiple data forwarding addresses from each candidate gNB(s).

Given the RAN3 agreement that timing is left to implementation, we believe it is useful to also optionally include the data forwarding related information in the LTM CELL SWITCH NOTIFICATION message. In this way the Source-gNB can utilize this existing signaling step to inform the data forwarding details to the new Source-gNB as well without having to wait for an additional procedure (i.e. a LTM CONFIGURATION UPDATE message afterwards).

Proposal 13: The Source gNB may also provide PDU Session Resource Information List of all Candidate gNBs via the LTM CELL SWITCH NOTIFICATION message.

## 4.3	TA Forwarding

In the Rel-18 DU-CU and CU-DU CELL SWITCH NOTIFICATION messages, the TA information list IE may be included to transfer obtained TA values to the target (i.e., new serving) cell. Similarly, the TA validity was left up to implementation in case of intra-CU LTM.

If TA forwarding is supported for Rel-19 Inter-CU LTM, we believe the TA validity cannot be left up to implementation as the Xn link delay needs to be accounted for. A possible approach which can be linked to UE signal experienced, is to provide information such as the RSRP which the UE experienced when TA was acquired. This would allow the Target-gNB/DU to better understand the signal conditions associated with a TA value being received, rather than a “time” value tied to the TA acquisition itself. Based on the received RSRP value along with TA value(s) and the new RSRP value, the new source DU can determine the validity of the TA value. Note that such scheme is already possible for SDT and should be considered for LTM.

Proposal 14: The source gNB(-DU) signals to target gNB(-DU) the RSRP measured during TA Acquisition along with the TA value within the F1AP DU-CU CELL SWITCH NOTIFICATION and XnAP CELL SWITCH NOTIFICATION. The target gNB-CU forwards the RSRP within the F1AP CU-DU CELL SWITCH NOTIFICATION message.

Figure 1: Execution Phase Considerations for Subsequent Inter-gNB-CU LTM

# 4	Conclusions

Preparation Phase

Proposal 1: Add description in Stage 2 TS 38.401 for describing that CU can request Candidate DU to provide CSI-RS configuration in UE CONTEXT SETUP REQUEST message, and Candidate DU signals the CSI-RS configuration in UE CONTEXT SETUP RESPONSE message.

Proposal 2: The F1AP DU-CU CSI-RS COORDINATION REQUEST and CU-DU CSI-RS COORDINATION REQUEST messages include a list containing: NR-CGI, SP CSI-RS transmission request (activate/deactivate), CSI-RS Resources ID List, and TCI State/QCL-info List.

Proposal 3: The XnAP CSI-RS COORDINATION REQUEST message includes a list containing: NR-CGI, SP CSI-RS transmission request (activate/deactivate), CSI-RS Resources ID List, and TCI State/QCL-info List.

Proposal 4: For Inter-CU LTM, the security keys provided in LTM CONFIGURATION UPDATE procedure are on a per cell level basis.

Proposal 5: For Inter-CU LTM, the LTM CONFIGURATION UPDATE procedure provided from the new Source gNB to Candidate gNBs includes the security key. Thus, the “may” wording shall be removed from procedural text in TS 38.300.

Proposal 6: Source-CU/Source-gNB can request a Candidate-CU/Candidate-gNB to provide a reference configuration.

Proposal 7: Remove FFS in Stage 3 with respect of request for reference configuration from candidate.

Proposal 8: Remove Note in TS 38.300 BL CR “Editor’s Note: FFS on whether step 6 and 7 are mandatory or optional.” And leave as optional procedure.

Proposal 9: RAN3 to study impact and required handling in Inter-CU LTM preparation phase when only L3 based Inter-CU LTM is supported at the Candidate gNB-CU(s).

Proposal 10: RAN3 to study means to simplify Inter-CU LTM preparation by support of using a common CSI-RS configuration, or by limiting execution of subsequent LTM.

Early Synchronization Phase

Proposal 11: XnAP IDs are NOT needed in the XnAP TA INFORMATION TRANSFER message.

LTM Execution Phase

Proposal 12: Add a “Cell Switch Type (ENUMERATED(RACH-less, RACH-based))” in F1AP and XnAP CELL SWITCH NOTIFICATION messages.

Proposal 13: The Source gNB may also provide PDU Session Resource Information List of all Candidate gNBs via the LTM CELL SWITCH NOTIFICATION message.

Proposal 14: The source gNB(-DU) signals to target gNB(-DU) the RSRP measured during TA Acquisition along with the TA value within the F1AP DU-CU CELL SWITCH NOTIFICATION and XnAP CELL SWITCH NOTIFICATION. The target gNB-CU forwards the RSRP within the F1AP CU-DU CELL SWITCH NOTIFICATION message.

# References

- 3GPP TS 38.401

- RP-250339 Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom

# Annex: Text Proposal for TS 38.300

<< First Change >>

##### 9.2.3.5.2	C-Plane Handling

Cell switch command is conveyed in a MAC CE, which contains the necessary information to perform the LTM cell switch.

The overall procedure for LTM is shown in Figure 9.2.3.5.2-1 below. Subsequent LTM is done by repeating the early synchronization, LTM cell switch execution, and LTM cell switch completion steps without the need to release, reconfigure or add other LTM candidate configurations after each LTM cell switch completion. The general procedure over the air interface is applicable to SCG LTM. Further details of SCG LTM can be found in TS 37.340 [21].

Figure 9.2.3.5.2-1. Signalling procedure for LTM

The procedure for LTM is as follows:

1.	The UE sends a MeasurementReport message (L3 measurement result) to the source gNB containing measurements of neighbouring cells.

2.	The source gNB decides to configure LTM.

3.	The source gNB requests LTM for one or more candidate cells belonging to the source gNB and/or one or more candidate gNB(s). For inter-gNB LTM, the source gNB initiates a HANDOVER REQUEST message per candidate cell containing one candidate cell ID and the CSI resource configuration for subsequent LTM. For both intra and inter-gNB LTM, the source gNB may request the candidate cell(s)/gNB(s) to provide the CSI resource configuration .

4.	Admission Control may be performed by the candidate cells(s)/gNB(s).

5.	The candidate prepares and provides the LTM configuration(s) to the source gNB. For inter-gNB LTM, the candidate gNB(s) respond(s) (with HO REQUEST ACKNOWLEDGE) to the source gNB including the generated RRC configurations for the accepted candidate cell. For both intra and inter-gNB LTM, the candidate may also include additional information related to the CSI configuration and early sync information upon request.

6.	The source gNB sends an LTM CONFIGURATION UPDATE message to the candidate gNB(s) to update the LTM configurations of candidate cell(s). The source gNB may include the common CSI resource configuration.

7.	The candidate gNB(s) sends the LTM CONFIGURATION UPDATE ACKNOWLEDGE message to the source gNB. The candidate gNB(s) may also provide the CSI report configuration.

NOTE :	Step 6 may also be triggered after step 14, or after step 17 by implementation for subsequent LTM.

8.	The source gNB sends an RRCReconfiguration message to the UE.

9.	The UE stores the LTM candidate configurations and sends an RRCReconfigurationComplete message to the source gNB.

9a	If early data forwarding is applied, the source gNB sends the EARLY STATUS TRANSFER message to the candidate gNB(s).

10/11. Early DL and UL synchronization to some LTM candidate cell(s) may be performed. The source gNB may activate or deactivate the TCI states of the candidate LTM cells.  The UE may perform UL synchronization with LTM candidate cell(s), by using UE-based TA measurement, if configured, and/or by transmitting a preamble towards the candidate cell, as triggered by the source gNB. With a NW triggered UL synchronization, a PDCCH order is received from the source cell to trigger CFRA to a candidate cell, the UE performs early TA acquisition by sending preamble towards the indicated candidate cell. In order to minimize the data interruption on the source cell due to CFRA towards the indicated candidate cell(s), the UE does not receive random access response from the network for the purpose of TA value acquisition. The candidate gNB(s) sends the TA INFORMATION TRANSFER message to the source gNB instead.

12.	 The UE performs L1 measurements on the configured LTM candidate cell(s) and transmits L1 measurement reports to the source gNB. L1 measurement should be performed as long as RRC reconfiguration (step 8) is applicable.

13.	The source gNB determines to initiate LTM. L3 measurement can also be used to determine this step.

14.	The source gNB decides to execute cell switch to a target cell and transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID which indicates the index of the candidate configuration, a beam indicated with a TCI state or beams indicated with DL and UL TCI states, and a TA command for the target cell. The UE switches to the target cell and applies the candidate configuration indicated by the target configuration ID.

NOTE :	Up to implementation, data forwarding and SN Status Transfer may be initiated once the source gNB triggers the inter-gNB LTM cell switch for the UE in Step 14.

15.	The source gNB sends the CELL SWITCH NOTIFICATION message to the target gNB to indicate the initiation of Cell Switch command to the UE.

16.	The UE performs the random access procedure towards the target cell, if UE does not have valid TA of the target cell as specified in clause 5.18.35 of TS 38.321[6].

17/18. The target gNB sends the HANDOVER SUCCESS message to the source gNB to inform that the UE has successfully accessed the target cell. In return, the source gNB sends the SN STATUS TRANSFER message following the principles described in step 7 of Intra-AMF/UPF Handover in clause 9.2.3.2.1.

NOTE :	Late data forwarding may be initiated as soon as the source gNB receives the HANDOVER SUCCESS message.

19.	The UE completes the LTM cell switch procedure by sending RRCReconfigurationComplete message to target cell. If the UE has performed a RA procedure in step 16 the UE considers that LTM cell switch execution is successfully completed when the random access procedure is successfully completed. For RACH-less LTM, the UE considers that LTM cell switch execution is successfully completed when the UE determines that the network has successfully received its first UL data

20. The new source gNB (i.e., the target gNB) sends the LTM CONFIGURATION UPDATE message to the candidate gNBs. This message includes the new security key(s) to be used with the UE.

21. The candidate gNB(s) responds to the LTM CONFIGURATION UPDATE ACKNOWLEDGE message to the new source gNB.

22. The new source gNB may send the UE CONTEXT RELEASE message to inform the old source gNB to release radio and C-plane related resources associated to the UE context if no LTM candidate cell(s) exist in the old source gNB. Any ongoing data forwarding may continue.

The steps 10-22 can be performed multiple times for subsequent LTM cell switch executions using the LTM candidate configuration(s) provided in step 8.

The procedure over the air interface described in Figure 9.2.3.5.2-1 is applicable to both intra-gNB-DU LTM , inter-gNB-DU LTM and inter-gNB-CU LTM. The overall LTM procedures over F1-C interface are captured in TS 38.401[4].

<< End of Changes >>

# Annex: Text Proposal for TS 38.401

<< Start of Changes >>

#### 8.2.1.5	Inter-gNB-DU LTM

This procedure is used for the case when the UE moves from one gNB-DU to another gNB-DU within the same gNB-CU during NR operation for LTM. Figure 8.2.1.5-1 shows the inter-gNB-DU LTM procedure for intra-NR.

Figure 8.2.1.5-1: Inter-gNB-DU LTM

1.	The UE sends a MeasurementReport message (L3 measurement result) to the source gNB-DU containing measurements of neighbouring cells. The source gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

2.	The gNB-CU determines to initiate LTM configuration.

3.	The gNB-CU sends a UE CONTEXT SETUP REQUEST message to the candidate gNB-DU(s) for each candidate cell, containing one candidate cell ID and the CSI resource configuration for subsequent LTM. The gNB-CU may provide the LTM configuration ID mapping list to the candidate gNB-DU(s). The gNB-CU may request PRACH resources from the candidate gNB-DU(s). The gNB-CU may request the candidate gNB-DU(s) to provide the lower layer configuration for the purpose of generating the reference configuration or provide the lower layer part of the reference configuration to the candidate gNB-DU(s). The gNB-CU may request the candidate gNB-DU(s) to provide the CSI-RS configuration.

4.	If the candidate gNB-DU accepts the request of LTM configuration, it responds with a UE CONTEXT SETUP RESPONSE message including the generated lower layer RRC configurations for the accepted target candidate cell. The candidate gNB(s) may also include the CSI-RS configuration upon request.

NOTE 1:	The CU-initiated UE Context Modification procedure may be initiated for preparing candidate cells in the source gNB-DU as specified in step 3 and 4 in 8.2.1.4 Intra-gNB-DU LTM.

5.	The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source gNB-DU including the information related to early sync and the LTM configuration ID mapping list for the accepted target candidate cell(s). The gNB-CU may send the updated CSI resource configuration to the source gNB-DU. The gNB-CU may inform the source gNB-DU about intra-DU L2 reset configuration.

6.	The source gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message which includes an updated lower layer configuration, e.g., containing the updated CSI report configuration of the source cell.

7.	The gNB-CU may send a UE CONTEXT MODIFICATION REQUEST message for each candidate cell accepted in the candidate gNB-DU(s), containing the information for subsequent LTM or for updating the configurations of candidate cells. The gNB-CU may also provide the lower layer part of the reference configuration to the candidate gNB-DU(s). The gNB-CU may inform the candidate gNB-DU(s) about intra-DU L2 reset configuration.

8.	The candidate gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message including the updated lower layer configuration, e.g., containing the updated CSI report configuration of the requested candidate cell.

NOTE 2: Step 7 may also be triggered after step 19, or after step 22 by implementation for subsequent LTM.

9.	The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the source gNB-DU, which includes the generated RRCReconfiguration message with the LTM configuration.

10.	The source gNB-DU forwards the received RRCReconfiguration message to the UE.

11.	The UE responds to the source gNB-DU with an RRCReconfigurationComplete message.

12.	The source gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

13. The source gNB-DU may send a DU-CU CSI-RS COORDINATION REQUEST message to the gNB-CU including the CSI-RS resource to be activated/deactivated of the candidate cell(s).

14. The gNB-CU then sends a CU-DU CSI-RS COORDINATION REQUEST message to the candidate gNB-DU including the CSI-RS resource to be activated/deactivated of the candidate cell(s).

15. The candidate gNB-DU responses a CU-DU COORDINATION REQUEST ACKNOWLEDGE message.

16. The gNB-CU responses a DU-CU COORDINATION REQUEST ACKNOWLEDGE message.

Editor’s Note: Details are FFS on step 13 to 16.

17.	Early TA acquisition to the candidate cell(s) may be performed as specified in TS 38.300 [2].

18.The candidate gNB-DU sends a DU-CU TA INFORMATION TRANSFER message to the gNB-CU, which includes the TA values, and the associated PRACH resource information.

19.	The gNB-CU forwards the TA value, and the associated PRACH resource information to the source gNB-DU in the CU-DU TA INFORMATION TRANSFER message.

20. The UE sends the L1 measurement result to the source gNB-DU.

21. The source gNB-DU decides to execute LTM to a target cell.

22. The source gNB-DU sends the Cell Switch Command to the UE.

23. The source gNB-DU sends the DU-CU CELL SWITCH NOTIFICATION message to the gNB-CU to indicate the initiation of the Cell Switch Command to the UE, including the target cell ID and the TCI state ID(s). The TA value(s) related information applicable for subsequent LTM may also be included.

24.	The gNB-CU forwards in the CU-DU CELL SWITCH NOTIFICATION message to the target gNB-DU the target cell ID, the TCI state ID(s), and the TA value(s) related information received in step 19.

25.	The target gNB-DU detects the UE access as specified in TS 38.300 [2].

26.	The target gNB-DU sends the ACCESS SUCCESS message to the gNB-CU with the target cell ID.

27.	The UE sends an RRCReconfigurationComplete message to the target gNB-DU.

28.	The target gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

29.	The gNB-CU may send the UE CONTEXT RELEASE COMMAND message to the source gNB-DU to release the resources of prepared cells.

30. The source gNB-DU responds with a UE CONTEXT RELEASE COMPLETE message.

<< Next Change >>

# Annex: Text Proposal for TS 38.473

<< Start of Changes >>

#### 9.2.2.x1	DU-CU CSI-RS COORDINATION REQUEST

This message is sent by the gNB-DU to request the gNB-CU e.g. to activate/deactivate the SP CSI-RS transmissions from specific cells.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| CSI-RS Resource Request List |  | 1 |  | Cell ID list to which the request applies. | YES | reject |
| >CSI-RS Resource Request Item |  | 1 .. <maxnoofCSIRSResourceCells> |  |  | – |  |
| >>NR CGI | M |  | 9.2.2.7 |  | – |  |
| >>Semi Persistent CSI-RS Transmission Request | M |  | ENUMERATED(activate, deactivate, …) |  | – |  |
| >>CSI-RS Resources ID List | O | 1 .. <maxnoofCSIRSResourceIDs> |  |  | – |  |
| >>>CSI-RS Resource ID | M |  | INTEGER (0..191) |  | – |  |
| >>TCI State ID List | O | 1 .. <maxnoofCSIRSResourceIDs> |  |  |  |  |
| >>>TCI State Index | M |  |  |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCSIRSResourceCells | Maximum no. of cells configured for CSI-RS resources allowed towards one UE, the maximum value is 8. |
| maxnoofCSIRSResourceIDs | Maximum number of CSI RS Resource IDs. Value is 192. |

#### 9.2.2.x2	DU-CU CSI-RS COORDINATION RESPONSE

This message is sent by the gNB-CU e.g. to inform the gNB-DU about the SP CSI-RS transmissions activation/deactivation result.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | ignore |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | ignore |
| CSI-RS Resource Response List |  | 1 |  | Cell ID list to which the response applies. | YES | ignore |
| >CSI-RS Resource Response Item |  | 1 .. <maxnoofCSIRSResourceCells> |  |  | – |  |
| >>NR CGI | M |  | 9.2.2.7 |  | – |  |
| >>Semi Persistent CSI-RS Transmission Response | M |  | ENUMERATED(success, failure, …) |  | – |  |
| >>CSI-RS Resources ID List | O | 1 .. <maxnoofCSIRSResourceIDs> |  |  | – |  |
| >>>CSI-RS Resource ID | M |  | INTEGER (0..191) |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCSIRSResourceCells | Maximum no. of cells configured for CSI-RS resources allowed towards one UE, the maximum value is 8. |
| maxnoofCSIRSResourceIDs | Maximum number of CSI RS Resource IDs. Value is 192. |

#### 9.2.2.y1	CU-DU CSI-RS COORDINATION REQUEST

This message is sent by the gNB-CU e.g. to request the gNB-DU to activate/deactivate the SP CSI-RS transmissions from specific cells.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| CSI-RS Resource Request List |  | 1 |  | Cell ID list to which the request applies. | YES | reject |
| >CSI-RS Resource Request Item |  | 1 .. <maxnoofCSIRSResourceCells> |  |  | – |  |
| >>NR CGI | M |  | 9.2.2.7 |  | – |  |
| >>Semi Persistent CSI-RS Transmission Request | M |  | ENUMERATED(activate, deactivate, …) |  | – |  |
| >>CSI-RS Resources ID List | O | 1 .. <maxnoofCSIRSResourceIDs> |  |  | – |  |
| >>>CSI-RS Resource ID | M |  | INTEGER (0..191) |  | – |  |
| >>TCI State ID List | O | 1 .. <maxnoofCSIRSResourceIDs> |  |  |  |  |
| >>>TCI State Index | M |  |  |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCSIRSResourceCells | Maximum no. of cells configured for CSI-RS resources allowed towards one UE, the maximum value is 8. |
| maxnoofCSIRSResourceIDs | Maximum number of CSI RS Resource IDs. Value is 192. |

#### 9.2.2.y2	CU-DU CSI-RS COORDINATION RESPONSE

This message is sent by the gNB-DU e.g. to inform the gNB-CU about the SP CSI-RS transmissions activation/deactivation result.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | ignore |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | ignore |
| CSI-RS Resource Response List |  | 1 |  | Cell ID list to which the response applies. | YES | ignore |
| >CSI-RS Resource Response Item |  | 1 .. <maxnoofCSIRSResourceCells> |  |  | – |  |
| >>NR CGI | M |  | 9.2.2.7 |  | – |  |
| >>Semi Persistent CSI-RS Transmission Response | M |  | ENUMERATED(success, failure, …) |  | – |  |
| >>CSI-RS Resources ID List | O | 1 .. <maxnoofCSIRSResourceIDs> |  |  | – |  |
| >>>CSI-RS Resource ID | M |  | INTEGER (0..191) |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCSIRSResourceCells | Maximum no. of cells configured for CSI-RS resources allowed towards one UE, the maximum value is 8. |
| maxnoofCSIRSResourceIDs | Maximum number of CSI RS Resource IDs. Value is 192. |

/*********************Next change***********************/

#### 9.2.2.15	DU-CU CELL SWITCH NOTIFICATION

This message is sent by the gNB-DU to inform the gNB-CU about the initiation of the cell switch  command to the UE.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cell ID | M |  | NR CGI 9.3.1.12 |  | YES | reject |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Includes the TCI-StateId IE, as defined in TS 38.331 [8]. | - |  |
| >UL TCI State ID | O |  | OCTET STRING | Includes the TCI-UL-StateId IE, as defined in TS 38.331 [8]. | - |  |
| >Cell Switch Type | O |  | ENUMERATED(RACH-less, RACH-based, …) | Indicates whether the cell switch is RACH-less or RACH-based | - |  |
| TA Information List |  | 0..1 |  |  | YES | ignore |
| >TA Information Item IEs |  | 1 .. <maxnoofTAList> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. | - |  |
| >>Tag ID Pointer | O |  | OCTET STRING | Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [8]. | YES | ignore |
| >>RSRP | O |  | INTEGER (0… 127) | Indicates the RSRP when the TA value was requested. Corresponds to information provided in the RSRP-Range IE as defined in TS 38.331 [10]. |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAList | Maximum no. of TA values to be sent, the maximum value is 8. |

/*********************Next change***********************/

#### 9.2.2.16	CU-DU CELL SWITCH NOTIFICATION

This message is sent by the gNB-CU to inform the gNB-DU about the initiation of the cell switch command to the UE.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cell ID | M |  | NR CGI 9.3.1.12 |  | YES | reject |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Includes the TCI-StateId IE, as defined in TS 38.331 [8]. | - |  |
| >UL TCI State ID | O |  | OCTET STRING | Includes the TCI-UL-StateId IE, as defined in TS 38.331 [8]. | - |  |
| >Cell Switch Type | O |  | ENUMERATED(RACH-less, RACH-based, …) | Indicates whether the cell switch is RACH-less or RACH-based | - |  |
| TA Information List |  | 0..1 |  |  | YES | ignore |
| >TA Information Item IEs |  | 1 .. <maxnoofTAList> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. | - |  |
| >>Tag ID Pointer | O |  | OCTET STRING | Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [8]. | YES | ignore |
| >>RSRP | O |  | INTEGER (0… 127) | Indicates the RSRP when the TA value was requested. Corresponds to information provided in the RSRP-Range IE as defined in TS 38.331 [10]. |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAList | Maximum no. of TA values to be sent, the maximum value is 8. |

# Annex: Text Proposal for TS 38.423

<< Start of Changes >>

#### 9.1.1.x1 CELL SWITCH NOTIFICATION

This message is sent by the source NG-RAN node to inform the target NG-RAN node about the initiation of the cell switch command to the UE.

Direction: source NG-RAN node  target NG-RAN node.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | reject |
| Target NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | YES | reject |
| Target Cell Global ID | M |  | NR CGI 9.2.2.7 |  | YES | reject |
| Data Forwarding Info | FFS |  |  |  | YES | ignore |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Corresponds to the TCI-StateId IE as defined in TS 38.331 [10]. | - |  |
| >UL TCI State ID | O |  | OCTET STRING | Corresponds to the TCI-UL-StateId IE as defined in TS 38.331 [10]. |  |  |
| >Cell Switch Type | O |  | ENUMERATED(RACH-less, RACH-based, …) | Indicates whether the cell switch is RACH-less or RACH-based | - |  |
| TA Information List |  | 0..1 |  |  | YES | ignore |
| >TA Information Item IEs |  | 1 .. <maxnoofTAList> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. | - |  |
| >>Tag ID Pointer | O |  | OCTET STRING | Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [8]. | YES | ignore |
| >>RSRP | O |  | INTEGER (0… 127) | Indicates the RSRP when the TA value was requested. Corresponds to information provided in the RSRP-Range IE as defined in TS 38.331 [10]. |  |  |

<< End of Changes >>