3GPP TSG-RAN WG3 Meeting #128	R3-253150

Malta, Malta, 19 – 23 May, 2025

Agenda item:	10.2

Source:	Nokia

Title:	(TP for SON BL CR for TS 38.473, TP for SON BL CR for TS 38.401, TP for SON BL CR for TS 38.300, TP for SON BL CR for TS 38.423) MRO Enhancements for LTM and CHO with Candidate SCG(s)

Document for:	Discussion

# 1	Introduction

This is a contribution related to MRO for LTM and CHO with candidate SCG(s) in Rel-19.

# 2	Discussion

## 2.1 MRO for LTM

### 2.1.1 Agreements achieved in RAN3 and RAN2

The agreements achieved in RAN3 #127bis [1]:

BFR shortly after successful LTM cell switch to the wrong beam

RAN3 supports network-based solution for the case of BFR shortly after a successful LTM cell switch.

The target DU identifies the BFR happened in UE shortly after successful LTM cell switch caused due to wrong beam.

In case that the source DU selects a wrong beam among candidate beam list, the source DU is responsible for MRO optimization

In case that the target DU provides a wrong candidate beam list, the target DU is responsible for MRO optimization

Target DU needs to send the recovery beam information to CU and CU forwards it to source DU.

CU does not need to send the old beam information to source DU.

FFS if/how the solution for case 1 could also be used for the case of LTM cell switch failure due to wrong beam (case 2)?

LTM failure due to outdated TA

MRO will cover the scenario that RACH-less LTM fails including outdate TA and UE performs RACH based LTM.

RAN3 will not consider the case of LTM cell switch failure due to outdated TA calculated by UE.

For PDCCH order triggered early TA acquisition, RAN3 consider the case of RACH-less LTM fails due to outdate TA and UE performs RACH-based LTM failure recovery or RRC re-establishment to the same cell.

Whether UE based solution or network based solution is used for LTM failure due to outdated TA?

Whether LTM failure due to outdated TA is defined as a new failure case or it can covered by existing failure case?

Network based or UE based solution for TA acquisition type

For inter-DU case, CU aware TA acquisition type without any signaling impact in RAN3.

FFS for intra-DU case.

The agreements achieved in RAN3 #127 [2]:

CU can adjust the candidate Cell list for LTM based on UHI.

DU should be able to resolve LTM ping-pong based on information over F1.

MRO will cover the scenario that RACH-less LTM fails including outdate TA and UE performs RACH based LTM?

Network based solution is used for LTM candidate cell list.

Network based or UE based solution for TA acquisition type?

Will TA acquisition type always be available in the CU or is there a need for the DU to store and retrieve (partial) UE Context?

The agreements achieved in RAN2 #129bis [3]:

We will not add support for including the unnecessary LTM configuration, e.g. LTM candidate cell(s) unused, in SON reports, for both failure and near failure cases.

Do not specify support for reporting difference between the TA value received from the LTM cell switch command and TA value measured by the UE.

Do not specify support for reporting of elapsed time between UE obtains the target cell TA and UE receives the cell switch command, or an indication to indicate whether the UE obtains TA before UE receives LTM cell switch command

For LTM-related RA reports, the UE sets the cellId to the CGI if available, otherwise sets it based on ltm-CandidatePCI and ssb-Frequency in corresponding LTM-Candidate.

### 2.1.2 Connection Failure Monitoring for MCG LTM

In the context of RAN3 # 126 meeting the solution for TA acquisition type in RLF report was discussed. Majority of companies agreed with network-based solution as source gNB can store the Early UL sync configuration and UE-based TA configuration and can determine whether the UE had determined the TA via UE-based TA measurement or gNB indicated TA value in Cell Switch Command by “retrieving” the UE context upon receiving the RLF Report.

During the RAN3 #127 the discussion continued. Some of the companies prefer supporting network-based solution in case if TA acquisition type is always available in CU to avoid some extra effort to retrieve this information from the DU. Therefore, the following was marked to be clarified in the next meeting:

Will TA acquisition type always be available in the CU or is there a need for the DU to store and retrieve (partial) UE Context?

The discussion continued in RAN3 # 127bis meeting with the following to be clarified in the next meeting:

For inter-DU case, CU aware TA acquisition type without any signaling impact in RAN3. FFS for intra-DU case.

In case of inter DU LTM cell switch the TA Information List as part of the DU-CU CELL SWITCH NOTIFICATION sent from source DU to CU as specified in TS 38.473 contains the list of candidate cell IDs together with the acquired TA values. The target cell, i.e. the cell the UE will attempt as the first one in target DU may also be a candidate cell so may be part of this list together with the acquired TA value (if any) which was also communicated to UE via Cell Switch Command message. Based on presence of the target cell together with the TA value in the TA Information List the CU shall deduce the gNB indicated TA. However, in case the target cell is not included in the TA Information List but the RLF report contains an indicator about RACH-less LTM cell switch, the CU Is not able to distinguish between the gNB indicated and UE based TA.

Observation 1-1: In case of inter DU LTM cell switch the TA acquisition type may not be always available in the CU.

In case of intra DU LTM cell switch TA Information List is not part of the DU-CU CELL SWITCH NOTIFICATION sent from DU to CU as specified in TS 38.473. The CU does not have any info about acquiring the TA in the DU.

Observation 1-2: In case of intra DU LTM cell switch the TA acquisition type is not available in the CU.

Proposal 1-1: RAN3 to discuss the network-based solution for TA acquisition type for intra and inter DU LTM cell switch.

To solve this problem and make the TA type available in CU it is proposed to include new binary indicator into Cell ID IE (representing target cell) of the DU-CU CELL SWITCH NOTIFICATION message (TS 38.473) informing CU about gNB indicated TA.

The new binary indicator “gNB indicated TA” is set to “YES” if TA value obtained via Early TA acquisition procedure was added into Cell Switch Command and sent from gNB to UE. Otherwise, it is set to “No”.

Proposal 1-2: RAN3 agrees to add new binary indicator into Cell ID IE (representing target cell) of the DU-CU CELL SWITCH NOTIFICATION message (TS 38.473) informing CU about gNB indicated TA.

Proposal 1-3: RAN3 agrees TP for BL CR 38.473 and 38.401 in Annex 1 and 2, respectively.

In RAN3 # 126 meeting the following was agreed:

For TP to BL CR to TS 38.473:

- In ACCESS AND MOBILITY INDICATION message, the presence of the C-RNTI IE is kept optional. FFS on stage2 or stage3 to define the condition to include this optional C-RNTI.

The reasoning why the C-RTNI is added shall be part of stage 3. It is therefore proposed to update the Semantics description for the C-RNTI in TS 38.473 as proposed in the attached TP.

Proposal 1-4: RAN3 agrees the attached TP for TS 38.473 in Annex 3 to update the Semantics description for the C-RNTI.

There may be a plurality of problems causing the RLF. Besides the wrong cell selection at cell switch and wrong cell switch timing the problem may be related to wrong or missing TA in the Cell switch command.  In general, it may be related to an issue in Early TA Acquisition procedure which oversees source DU. Despite the TA was included in Cell Switch Command it may become outdated because the UE significantly moved between the time of the Early TA Acquisition and the time of the LTM execution. The UE then fails to attempt the target LTM cell and had to switch RACH based access via LTM recovery/re-establishment procedure when new TA value is provided from network in RAR message. We think that reporting of the information about the difference between the TA reported in the Cell switch command for which the RA less LTM failed, and TA received consequently in RAR for recovery/re-establishment cell may help to optimize the wrong cell switch timing.

In RAN3 #127bis the issue was confirmed with the following to be clarified in the next meeting:

Whether UE based solution or network based solution is used for LTM failure due to outdated TA?

Some of companies prefer network-based solution as the difference between the TA values may be easily obtained in network side. On the other hand, some of companies pointed out that first the TA outdated issue must be identified to differentiate this case from failure due to wrong beam where also UE will do LTM recovery or re-establishment to the same cell and TA value obtained in RAR may differ from the one provided in Cell switch Command.  On the other hand, in case of outdated TA the beam UE did the LTM recovery or re-establishment may be differ from the one UE did initial RACH less attempt. To identify the RACH less attempt failed due to outdated TA means to identify the UL synchronization is lost which means that each attempt (first transmission and consequent re-transmissions) to transmit RRCReconfigurationComplete to network failed even upon radio conditions when unsuccessful transmission caused by radio conditions is unexpected. In NR asynchronous HARQ is used also for PUSCH with no HARQ ACK/NACK communicated from network to UE. As per TS 38.321, instead the network in DCI for UL grant of the PDSCH for the given HARQ process with toggled or not toggled NDI indicates the previous transmission was successful or not, respectively.  Thus, considering the radio conditions in principle both UE and network may identify the outdated TA. Network based solution is complicated as requires correlation between the TA difference and RLF report and forwarding the TA value which was sent via Cell Switch command from source to target DU. The network also may not have actual measurements to properly distinguish between outdated TA and wrong beam failure. Second thing is that there is already RLF report reported by UE so why not just to add the TA difference into this report.

- Proposal 1-5: RAN3 to discuss the following information to be included in the RLF report:

- TA difference between the TA provided by network in Cell Switch Command for which the RA less LTM failed due to outdated TA value, and TA received in RAR for recovery/re-establishment in the same cell.

In RAN3 #127bis also the following was marked as to be clarified in the next meeting:

Whether LTM failure due to outdated TA is defined as a new failure case or it can covered by existing failure case?

- In case of outdated TA, the UE may re-connect or re-establish into the same target LTM cell to which original access failed. Even such failure case is related to MRO it neither fits to too late, not too early nor to LTM to wrong cell. It may therefore be desirable to introduce new failure cause “Outdated TA” in stage 2.

- Proposal 1-6: RAN3 agrees on new failure cause “Outdated TA” definition in stage 2.

- In RAN3 #127bis meeting we discussed also the LS R3-251514 where RAN2 respectfully asks RAN3 to consider defining a network-based solution for the issue of BFR shortly after successful LTM cell switch (“near failure”). Furthermore, RAN2 asks RAN3 to see if/how the solution (if specified) could also be used for the case of LTM cell switch failure due to wrong beam.

Regarding the first issue the following was agreed:

RAN3 supports network-based solution for the case of BFR shortly after a successful LTM cell switch.

The target DU identifies the BFR happened in UE shortly after successful LTM cell switch caused due to wrong beam.

In case that the source DU selects a wrong beam among candidate beam list, the source DU is responsible for MRO optimization

In case that the target DU provides a wrong candidate beam list, the target DU is responsible for MRO optimization

Target DU needs to send the recovery beam information to CU and CU forwards it to source DU.

CU does not need to send the old beam information to source DU.

- In order not to delay the transferring the recovery beam information from target DU to CU and to source DU the UE associated signalling is preferred.

- Proposal 1-7: RAN3 agrees on UE associated signalling to transfer the recovery beam information from target DU to CU and to source DU.

## 2.2 MRO for CHO with Candidate SCG(s)

### 2.2.1 Agreements achieved in RAN3

The agreements achieved in RAN3 #127bis [1]:

No additional information is needed in handover report message.

Check stage 3 TP on the Target cell CGI related in handover report message.

One SCGFailureInfomation is included in SCG Failure Indication message, no need aggregation.

Respective MN in stage2 is at least a candidate MN.

Information needed in the SCG Failure Indication message: The Xn UE AP ID in the receiving node.

Additional information reported from the UE in SCGFailureInformation: The Identifier of candidate PCell(s) or PSCell(s) that fulfilled execution conditions before the SCG failure is encountered.

No support of near failure case in R19.

Whether Handover Report message is needed for case 7a?

In addition to SCGFailureInformation, information needed in the SCG Failure Indication message?

- CPC failure type (CPC failure type is needed in case of SCG Failure Indication is sent from the candidate to the source?)

- Suitable PCell ID

The agreements achieved in RAN3 #127 [2]:

In case of too late CHO execution, reusing Handover Report message from the last serving MN to the relevant candidate MN.

FFS on whether Handover Report message is needed for case 7a.

FFS on the additional information included in handover report.

New message is defined for the serving MN to forward the SCGFailureInformation to the respective MN which should perform the optimization.

When to send the message? Immediately or not?

Check whether respective MN is only a candidate?

In addition to SCGFailureInformation, information needed in the new message?

- CPC failure type

The agreements achieved in RAN2 #129bis [3]:

No new triggering conditions such as time gap between the first met condition (CHO or CPAC) and the second met condition (CPAC or CHO) is above a threshold, can be considered for SHR/SPR procedure.

No new triggering conditions for SHR/SPR procedure for CHO with candidate SCG.

UE includes the target PSCell ID in SHR for successful CHO with candidate SCGs.

UE includes the target PCell ID in SPR for successful CHO with candidate SCG.

In general, and where applicable, agreements valid for SHR, RLF reports and SCG failure info applies also to SPR.

The agreements achieved in RAN2 #129 [4]:

Enhance RLF report for CHO with candidate SCGs to include the information for each CHO, i.e., first fulfilled event and time duration between two events fulfilled, if any.

Enhance RLF report for CHO with candidate SCGs to include the associations between CHO and CPAC.

Enhance RLF report for CHO with candidate SCGs to include at least the following information:

-	Identifier of candidate PCell(s) which met the configured CHO execution conditions when the RLF is encountered;

-	Identifier of candidate PSCell(s) which met the configured CPAC execution conditions when the RLF is encountered;

-	The Identifier of candidate PCell(s) or PSCell(s) that fulfilled execution conditions before the RLF is encountered.

Enhance SCGFailureInformation for CHO with candidate SCGs to include the information for each CHO, i.e., first fulfilled event and time duration between two events fulfilled, if any.

We should avoid specifying the procedure in a way that the UE sends redundant information

### 2.2.3 Connection Failure Monitoring for CHO with Candidate SCG(s)

Despite of a discussion in RAN3 #127bis the following was proposed still clarified in the next meeting:

FFS on whether Handover Report message is needed for case 7a.

There may be a need to forward the RLF report also in case which is not relevant to too late CHO execution. Case 7a when CHO execution conditions were fulfilled first then with some delay the CPA/CPC execution conditions are also fulfilled consequently the CHO execution is unsuccessful while CPA/CPC execution is successful, which may be caused by delayed fulfilment of CPA/CPC execution conditions which may require optimization on CPA/CPC related part. The RLF report received by source (serving) MN may also need to be forwarded to candidate target MN.

As per request of some companies in RAN3 # 127bit meeting we demonstrate the need for forwarding the Handover report message also for case 7a in an example. Fig. 1 show a an example when for UE configured with CHO with Candidate SCG(s) and connected in PCell1 and PSCell1,  the CHO execution conditions have been fulfilled to PCell2 and while waiting for fulfilment of the CPC execution conditions to PSCell2 the UE moved out of coverage area of PCell1 (T310 started), and also of coverage area of PCell2 at the point in time when CPC execution conditions have been fulfilled to PSCell2. As T310 has not yet expired the CHO with candidate SCG(s) execution starts to PCell2 and PSCell2 however, as stated, UE is already out of coverage area of PCell2 it leads to RLF and HO failure. Possible optimization without touching the CPC execution conditions is shown in Fig. 2 where the PCell3 (let’s consider PCell 1 and PCell 3 belong to different MNs, MN1 and MN2) is added as a candidate target PCell which is done via sending HO request with target PCell3 from MN1 to MN2. Let’s consider that in HO Request ACK message the MN2 confirms the PSCell2 as PSCell counterpart for PCell3.  With the modified CHO execution conditions, the CHO with candidate SCG(s) execution is successful to PCell3 and PSCell2. However, the interruption in providing the service in UL direction represented via elapsed T310 value remains unchanged. Fig. 3 shows the situation when in addition to CHO optimization there was also modification of CPC execution conditions which allows significant reduction of elapsed T310 value and thus interruption in providing the service in UL.

Let’s consider another example illustrated in Fig. 4 which similarly as example in Fig.1 results into HO failure however compared to Fig. 1 the CPC execution conditions are set too aggressively. As known, CPC execution condition for CHO with Candidate SCG(s) are based on A4 measurement, i.e. as absolute value independent of radio conditions in source PSCell. Therefore, the CPC execution condition setting may mean their fulfilment slightly out of the intersection area between the PScell1 and 2 as illustrated in Fig.4.  Possible optimization may be followed with tuning the CPC execution conditions without any CHO optimization as shown in Fig.5.

Observation 2-1: Based on examples in Fig. 1 -5, the last serving MN may need to send HO report message to the candidate MN(s) which may need CPC/CPA optimization also for case 7a.

Proposal 2-1: RAN3 agrees the attached TP for TS 38.300 in Annex 4 to update the scenario when the last serving MN may need to send message to the candidate MN(s) which may need optimization also for handover to wrong cell.

Proposal 2-2: RAN3 agrees the attached TP for TS 38.423 in Annex 5 to add Handover to wrong cell.

Figure 1

Figure 2

Figure 3

Figure 4

Figure 5

Note: In the example represented via Fig. 1-5 it is considered that coverage area of the PSCell2 overlaps with the PCell2 and 3 for the UE trajectory in example.

In RAN3 #127bis the following was proposed still clarified in the next meeting:

In addition to SCGFailureInformation, information needed in the SCG Failure Indication message?

- CPC failure type (CPC failure type is needed in case of SCG Failure Indication is sent from the candidate to the source?)

- Suitable PCell ID

In our understanding the agreement related to forwarding the SCGFailureInformation to the respective MN applies to case 3b and 9b when CHO execution conditions are fulfilled and during waiting for CPC/CPA execution conditions fulfilment an RLF is declared on SCG. UE sends the SCGFailureInformation to the serving MN (source MN), while optimization on CPC/CPA related part is needed which requires forwarding the SCGFailureInformation to respective candidate MN(s). For case 1b when no one of the execution conditions is fulfilled the serving MN based on SCGFailureInformation may decide some optimization is needed also for CPC/CPA part and may forward the SCGFailureInformation to the candidate MN(s). For case 2b when CPC/CPAC execution conditions are fulfilled but due to waiting for the CHO execution conditions fulfilment there is RLF on SCG when issue may be in the CHO related part which is configured by source (serving) MN there may be no need to forward the SCGFailureInformation to candidate MN(s). Concerning case 7b when CPC/CPA execution conditions were fulfilled first then with some delay the CHO execution conditions were also fulfilled but CPC/CPA execution is unsuccessful while CHO execution is successful, which may be caused by delayed fulfilment of CHO execution conditions which may require optimization in CHO related part. The SCGFailureInformation received by target (serving) MN may need to be forwarded to source MN. Considering the same case 7b but with CHO execution conditions fulfilled first then with some delay the CPA/CPC execution conditions but CPC/CPA execution is unsuccessful while CHO execution is successful, which may point to CPA/CPC issue configured by target (serving) MN and thus non need to forward the SCGFailureInformation. Regarding case 8b when CPC/CPA execution conditions were fulfilled first then with some delay the CHO execution conditions were fulfilled which was followed consequently with successful CHO and CPC/CPA execution, but shortly there is SCG failure which may be caused by delayed fulfilment of CHO execution conditions which may require optimization in CHO related part. The SCGFailureInformation received by target (serving) MN may need to be forwarded to source MN.  Considering the same case 8b but with CHO execution conditions fulfilled first then with some delay the CPA/CPC execution conditions and consequent successful CHO and CPC/CPA execution, but shortly there is SCG failure which may point of CPA/CPC issue configured by target (serving) MN and thus non need to forward the SCGFailureInformation.

Observation 2-2: The agreement that the serving MN forward the SCGFailureInformation to the respective MN which should perform the optimization if needed applies for case 1b, 3b, 7b, 8b, 9b.

Observation 2-3: The respective MN is candidate MN in case 1b, 3b, 9b while the respective MN is source MN in case 7b (CPA/CPC execution condition fulfilled first then with some delay CHO execution conditions) and 8b (CPA/CPC execution condition fulfilled first then with some delay CHO execution conditions).

Proposal 2-3: RAN3 agrees the attached TP for TS 38.300 in Annex 4 to add stage 2 text about the SCGFailureInformation forwarding respecting the fact that respective MN may by candidate or source MN.

During the RAN3 # 127 meeting the new message SCG Failure Indication was agreed to forward the SCGFailureInformation from service to respective MN. Discussion started and continued in RAN3 #127bis on whether CPAC failure type shall be added as a new IE into this message. The CPAC failure type may be obtained only in candidate MN node which means that the SCGFailureInformation would have to be reported by UE to the candidate MN node which means that the respective MN is source MN. Based on the Observation 2-3 such a scenario applies only for case 7b (CPA/CPC execution condition fulfilled first then with some delay CHO execution conditions) and 8b (CPA/CPC execution condition fulfilled first then with some delay CHO execution conditions).

Observation 2-4: Sending the CPAC failure type is only applicable from candidate to source MN which is only possible in case 7b (CPA/CPC execution condition fulfilled first then with some delay CHO execution conditions) and 8b (CPA/CPC execution condition fulfilled first then with some delay CHO execution conditions).

Proposal 2-4: RAN3 discuss and analyses the suitability of including the CPAC Failure type into SCG Failure Indication due to its limited applicability only for case 7b (CPA/CPC execution condition fulfilled first then with some delay CHO execution conditions) and 8b (CPA/CPC execution condition fulfilled first then with some delay CHO execution conditions).

# 3	Conclusions

We have the following observations and proposals to enhance MRO for LTM:

Observation 1-1: In case of inter DU LTM cell switch the TA acquisition type may not be always available in the CU.

Observation 1-2: In case of intra DU LTM cell switch the TA acquisition type is not available in the CU.

Proposal 1-1: RAN3 to discuss the network-based solution for TA acquisition type for intra and inter DU LTM cell switch.

Proposal 1-2: RAN3 agrees to add new binary indicator into Cell ID IE (representing target cell) of the DU-CU CELL SWITCH NOTIFICATION message (TS 38.473) informing CU about gNB indicated TA.

Proposal 1-3: RAN3 agrees TP for BL CR 38.473 and 38.401 in Annex 1 and 2, respectively.

Proposal 1-4: RAN3 agrees the attached TP for TS 38.473 in Annex 3 to update the Semantics description for the C-RNTI.

- Proposal 1-5: RAN3 to discuss the following information to be included in the RLF report:

- TA difference between the TA provided by network in Cell Switch Command for which the RA less LTM failed due to outdated TA value, and TA received in RAR for recovery/re-establishment in the same cell.

- Proposal 1-6: RAN3 agrees on new failure cause “Outdated TA” definition in stage 2.

- Proposal 1-7: RAN3 agrees on UE associated signalling to transfer the recovery beam information from target DU to CU and to source DU.

To support MRO for CHO with candidate SCG(s), we propose what follows:

Observation 2-1: Based on examples in Fig. 1 -5, the last serving MN may need to send HO report message to the candidate MN(s) which may need optimization also for case 7a.

Proposal 2-1: RAN3 agrees the attached TP for TS 38.300 in Annex 4 to update the scenario when the last serving MN may need to send message to the candidate MN(s) which may need optimization also for handover to wrong cell.

Proposal 2-2: RAN3 agrees the attached TP for TS 38.423 in Annex 5 to add Handover to wrong cell.

Observation 2-2: The agreement that the serving MN forward the SCGFailureInformation to the respective MN which should perform the optimization if needed applies for case 1b, 3b, 7b, 8b, 9b.

Observation 2-3: The respective MN is candidate MN in case 1b, 3b, 9b while the respective MN is source MN in case 7 (CPA/CPC execution condition fulfilled first then with some delay CHO execution conditions) and 8b (CPA/CPC execution condition fulfilled first then with some delay CHO execution conditions).

Proposal 2-3: RAN3 agrees the attached TP for TS 38.300 in Annex 2 to add stage 2 text about the SCGFailureInformation forwarding respecting the fact that respective MN may by candidate or source MN.

Observation 2-4: Sending the CPAC failure type is only applicable from candidate to source MN which is only possible in case 7b (CPA/CPC execution condition fulfilled first then with some delay CHO execution conditions) and 8b (CPA/CPC execution condition fulfilled first then with some delay CHO execution conditions).

Proposal 2-4: RAN3 discuss and analyses the suitability of including the CPAC Failure type into SCG Failure Indication due to its limited applicability only for case 7b (CPA/CPC execution condition fulfilled first then with some delay CHO execution conditions) and 8b (CPA/CPC execution condition fulfilled first then with some delay CHO execution conditions).

# References

- Report of 3GPP TSG RAN3 meeting #127bis

- Report of 3GPP TSG RAN3 meeting #127

- Report of 3GPP TSG RAN2 meeting #129bis

- Report of 3GPP TSG RAN2 meeting #129

# Annex 1: Text proposal to TS 38.473 (MRO for LTM)

First change, ommited text not changed

### 8.3.9	DU-CU Cell Switch Notification

#### 8.3.9.1	General

The purpose of the DU-CU Cell Switch Notification procedure is to enable the gNB-DU to inform the gNB-CU about the initiation of the cell switch command to the UE. This procedure is also used to transfer the selected TCI state from the gNB-DU to the gNB-CU. The procedure uses UE-associated signalling.

#### 8.3.9.2	Successful Operation

Figure 8.3.9.2-1: DU-CU Cell Switch Notification procedure. Successful operation.

The gNB-DU initiates the procedure by sending a DU-CU CELL SWITCH NOTIFICATION message.

Upon reception of the DU-CU CELL SWITCH NOTIFICATION message, the gNB-CU shall, if supported, consider that a cell switch command was sent to the UE where the target cell is indicated by the included Cell ID IE.

Interaction with CU-DU Cell Switch Notification procedure:

If the LTM Cell Switch Information IE is included in the DU-CU CELL SWITCH NOTIFICATION message, the gNB-CU shall, if supported, forward it to the target gNB-DU in the CU-DU CELL SWITCH NOTIFICATION message.

If the TA Information List IE is included in the DU-CU CELL SWITCH NOTIFICATION message, the gNB-CU shall, if supported, forward it to the target gNB-DU in the CU-DU CELL SWITCH NOTIFICATION message to transfer the TA value(s).

If the gNB Indicated TA is included in the DU-CU CELL SWITCH NOTIFICATION message, the gNB-CU shall react as specified in TS 38.401.

#### 8.3.9.3	Unsuccessful Operation

Not applicable.

#### 8.3.9.4	Abnormal Conditions

Not applicable.

Next change, ommited text not changed

#### 9.2.2.15	DU-CU CELL SWITCH NOTIFICATION

This message is sent by the gNB-DU to inform the gNB-CU about the initiation of the cell switch  command to the UE.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cell ID | M |  | NR CGI 9.3.1.12 |  | YES | reject |
| >gNB Indicated TA | O |  | ENUMERATED (YES, NO) |  | YES | ignore |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Includes the TCI-StateId IE, as defined in TS 38.331 [8]. | - |  |
| >UL TCI State ID | O |  | OCTET STRING | Includes the TCI-UL-StateId IE, as defined in TS 38.331 [8]. | - |  |
| TA Information List |  | 0..1 |  |  | YES | ignore |
| >TA Information Item IEs |  | 1 .. <maxnoofTAList> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. | - |  |
| >>Tag ID Pointer | O |  | OCTET STRING | Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [8]. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAList | Maximum no. of TA values to be sent, the maximum value is 8. |

Remaining text not changed

# Annex 2: Text proposal to TS 38.401 (MRO for LTM)

First change, ommited text not changed

## 7.X	MRO support for LTM

The gNB-CU receives a RLF report associated to an LTM mobility event from the UE or via the Failure indication message over Xn. The gNB-CU performs initial analysis and in case of failure due to inappropriate cell switch triggering and related to gNB-DU indicated TA or RACH-based access may forward the RLF report to the last serving gNB-DU in case of too late LTM, or to the source gNB-DU in case of too early LTM or LTM to wrong cell.

Remaining text not changed

# Annex 3: Text proposal to TS 38.473 (MRO for LTM)

First change, ommited text not changed

# 9	Elements for F1AP Communication

[snip]

## 9.2	Message Functional Definition and Content

[snip]

### 9.2.10	Self Optimisation Support Messages

#### 9.2.10.1	ACCESS AND MOBILITY INDICATION

This message is sent by gNB-CU to gNB-DU to provide access and mobility information to the gNB-DU.

Direction: gNB-CU ® gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| RA Report List |  | 0..1 |  |  | YES | ignore |
| >RA Report Item |  | 1 .. <maxnoofRAReports> |  |  | - |  |
| >>RA Report Container | M |  | OCTET STRING | Includes the RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| RLF Report Information List |  | 0..1 |  |  | YES | ignore |
| >RLF Report Information Item |  | 1 .. <maxnoofRLFReports> |  |  | - |  |
| >>NR UE RLF Report Container | M |  | OCTET STRING | Includes the nr-RLF-Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| >>C-RNTI | O |  | 9.3.1.32 | C-RNTI allocated at the source gNB-DU added in case the gNB-DU responsible for the LTM failure is not the gNB-DU serving the UE at the time of LTM failure. | YES | ignore |
| >>RLF Report Failure Type | O |  | ENUMERATED (too late LTM, too early LTM, LTM to wrong cell,...) |  | YES | ignore |
| Successful HO Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful HO Report Information Item |  | 1 .. <maxnoofSuccessfulHOReports> |  |  | - |  |
| >>Successful HO Report Container | M |  | OCTET STRING | Includes the SuccessHO-Report IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| Successful PSCell Change Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful PSCell Change Report information Item |  | 1..<maxnoofSuccessfulPSCellChangeReports> |  |  | - |  |
| >>Successful PSCell Change Report Container | M |  | OCTET STRING | Includes the SuccessPSCell-Report IE as defined in TS 38.331 [8]. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofRAReports | Maximum no. of RA Reports, the maximum value is 64. |
| maxnoofRLFReports | Maximum no. of RLF Reports, the maximum value is 64. |
| maxnoofSuccessfulHOReports | Maximum no. of Successful HO Reports, the maximum value is 64. |
| maxnoofSuccessfulPSCellChangeReports | Maximum no. of Successful PSCell Change Reports. Value is 64. |

Remaining text not changed

# Annex 4: Text proposal to TS 38.300 (MRO for CHO with Candidate SCG(s))

First change, ommited text not changed

##### 15.5.2.2.2	Connection failure due to intra-system mobility

One of the functions of Mobility Robustness Optimization is to detect connection failures that occur due to Too Early or Too Late Handovers, or Handover to Wrong Cell. These problems are defined as follows:

-	Intra-system Too Late Handover: an RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a different cell.

-	Intra-system Too Early Handover: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell.

-	Intra-system Handover to Wrong Cell: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell.

In the definition above, the "successful handover" refers to the UE state, namely the successful completion of the RA procedure.

In case of LTM cell switch, the "successful handover" in the definitions above refers to a successful completion of the RA procedure in case of RACH-based LTM or the successful reception of RRCReconfigurationComplete in case of RACH-less LTM.

In case of CHO, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell.

In case of LTM Cell switch, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late LTM cell switch, Too Early LTM cell switch and LTM cell switch to Wrong Cell.

In case of CHO with candidate SCG(s), the Too Late Handover in the definition above means Too Late CHO with candidate SCG(s).

Detection mechanism

A failure indication may be initiated after a UE attempts to re-establish the radio link connection at NG-RAN node B after a failure at NG-RAN node A. NG-RAN node B may initiate the Failure Indication procedure towards multiple NG-RAN nodes if they control cells which use the PCI signalled by the UE during the re-establishment procedure. The NG-RAN node receiving this selects the UE context that matches the received Failure Cell ID and C-RNTI, and, if available, uses the shortMAC-I to confirm this identification, by calculating the shortMAC-I and comparing it to the received IE.

A failure indication may also be sent to the node last serving the UE when the NG-RAN node fetches the RLF REPORT from UE by triggering:

-	The Failure Indication procedure over Xn;

-	The Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer procedure over NG.

The detailed detection mechanisms for too late handover, too early handover and handover to wrong cell are carried out through the following in the NG-RAN node that served the UE before the reported connection failure:

-	Intra-system Too Late Handover: there is no recent handover for the UE prior to the connection failure e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt), or if CHO or CHO with candidate SCG(s) is configured but the CHO execution is not initiated for the UE prior to the connection failure, e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt).

-	Intra-system Too Early Handover: there is a recent handover for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/the successful re-connect cell is the cell that served the UE at the last handover initialisation or fall back to the source cell configuration in case of DAPS HO.

-	Intra-system Handover to Wrong Cell: there is a recent handover for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/ the cell UE attempts to re-connect/the cell UE attempts CHO recovery is neither the cell that served the UE at the last handover initialisation nor the cell that served the UE where the RLF happened or the cell that the handover was initialized toward.

The "UE reported timer" above indicates the time elapsed since the last handover initialisation until connection failure or the time elapsed since the CHO execution until connection failure.

In case of Too Early Handover or Handover to Wrong Cell, the NG-RAN node receiving the failure indication may inform the NG-RAN node controlling the cell where the mobility configuration caused the failure by means of the Handover Report procedure over Xn or the Uplink RAN Configuration Transfer procedure over NG. This may include the RLF report.

In case of Too Late Handover or Handover to wrong cell when UE is configured with CHO with candidate SCG(s), the NG-RAN node receiving the failure indication may send a Handover Report over Xn to the candidate NG-RAN node(s) controlling the candidate PSCell(s) whose mobility configuration caused the failure .

For MRO analysis, in case of RLFs or handover failures, the source gNB may take into account the information regarding the LBT failures occurred in UL and DL for a specific UE. For UL, the information may be collected and provided by the UE to the network in the RLF report. For DL, the information may be collected at the source and target gNBs. The target gNB may provide the information to the source gNB via the Access and Mobility Indication procedure over Xn, if requested by the source gNB during the handover preparation.

Retrieval of information needed for problem analysis

In order to retrieve relevant information collected at the network side as part of the UE context, the UE provides C-RNTI used in the last serving cell. If the cause for the failure is identified as a "Too Early HO" or a "HO to Wrong Cell", the NG-RAN node controlling the last serving cell shall, include in the HANDOVER REPORT message the C-RNTI used in the source cell of the last completed handover before the failure. If the NG RAN node controlling that source cell provided the Mobility Information, it is also included in the HANDOVER REPORT message. If used, the Mobility Information is prepared at the source NG RAN node of a handover and may refer to or identify any handover-related data at this NG RAN node. In the Handover Preparation procedure, the source gNB can request the target gNB to provide information on DL LBT failures at the target gNB during handover execution.

Handling multiple reports from a single failure event

In case the RRC re-establishment fails and the RRC connection setup succeeds, MRO evaluation of intra-RAT mobility connection failures may be triggered twice for the same failure event. In this case, only one failure event should be counted.

Next change, ommited text not changed

#### 15.5.2.6	PSCell addition/change failure

For analysis of PSCell addition/change failures, the UE makes the SCG Failure Information available to the MN. If the MN can perform an initial analysis, it transfers the SCG Failure Information together with the analysis results to the relevant SN as defined in TS 37.340 [21].

In case there is an SCG failure when UE is configured with CHO with candidate SCG(s), the serving M-NG-RAN node receiving SCGFailureInformation from the UE performs initial analysis. If the SCG failure is broughthas been caused by anothera respective M- NG-RAN node configuration, the serving M-NG-RAN node transfers the SCG Failure Information to thate respective M-NG-RAN node where the mobility configuration caused the failure.

Remaining text not changed

# Annex 5: Text proposal to TS 38.423 (MRO for CHO with Candidate SCG(s))

First change, ommited text not changed

# TP for TS38.423 on CHO with candidate SCG(s)

### 8.4.8	Handover Report

#### 8.4.8.1	General

The purpose of the Handover Report procedure is to transfer mobility related information between NG-RAN nodes.

The procedure uses non UE-associated signalling.

#### 8.4.8.2	Successful Operation

Figure 8.4.8.2-1: Handover Report, successful operation

NG-RAN node1 initiates the procedure by sending the HANDOVER REPORT message to NG-RAN node2. When receiving the message NG-RAN node2 shall assume that a mobility-related problem was detected.

If the Handover Report Type IE is set to "HO too early" or "HO to wrong cell", then NG-RAN node1 indicates to NG-RAN node2 that, following a successful handover from a cell of NG-RAN node2 to a cell of NG-RAN node1, a radio link failure occurred and the UE attempted RRC Re-establishment or re-connected either at the original cell of NG-RAN node2 (Handover Too Early), or at another cell (Handover to Wrong Cell). The detection of Handover Too Early and Handover to Wrong Cell events is made according to TS 38.300 [9].

If the Handover Report Type IE is set to "HO to wrong cell for CHO with candidate SCG(s)", then NG-RAN node1 indicates to NG-RAN node2 that, a radio link failure occurred during the handover from a cell of NG-RAN node1 to a cell of NG-RAN node2, and the UE attempted RRC Re-establishment or re-connected at another cell which may be due to incorrect CPAC configuration. The detection of HO to wrong cell for CHO with candidate SCG(s) is based on detection of Handover to Wrong Cell event which is made according to TS 38.300 [9].

If the Handover Report Type IE is set to "Too Late CHO with candidate SCG", then NG-RAN node2 shall deduce that a too late CHO with candidate SCG from a cell of NG-RAN node1 to a suitable cell of NG-RAN node2 occurred. The detection of Too Late CHO with candidate SCG event is made according to TS 38.300 [9].

The HANDOVER REPORT message may include:

-	the Mobility Information IE, if the Mobility Information IE was sent for this handover from NG-RAN node2 (in case the NG-RAN node2 provided it more than once, the most recent Mobility Information IE is included in the HANDOVER REPORT message);

-	the Source cell C-RNTI IE.

-	the CHO Configuration IE, if the CHO Configuration IE was sent for this handover from NG-RAN node2.

If received, NG-RAN node2 uses the above information according to TS 38.300 [9].

If the Handover Report Type IE is set to "Inter-system ping-pong", then NG-RAN node2 shall deduce that a completed handover from a cell of NG-RAN node2 to a cell in another system might have resulted in an inter-system ping-pong and the UE was successfully handed over to a cell of NG-RAN node1 (indicated with Target cell CGI IE).

If the Target cell C-RNTI IE and the Time Since Failure IE are received, NG-RAN node2 uses the information as specified in TS 38.300 [9].

Interaction with the Failure Indication procedure:

If NG-RAN node1 receives a UE RLF Report from an NG-RAN node via the FAILURE INDICATION message, as described in TS 38.300 [9], NG-RAN node1 may also include it in the UE RLF Report Container IE included in the HANDOVER REPORT message.

Next change, ommited text not changed

#### 9.1.3.17	HANDOVER REPORT

This message is sent by NG-RAN node1 to NG-RAN node2 to report a handover failure event, or other critical mobility problem.

Direction: NG-RAN node 1  NG-RAN node 2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| Handover Report Type | M |  | ENUMERATED (HO too early, HO to wrong cell, Inter-system ping-pong. …,Too Late CHO with candidate SCG, HO to wrong cell for CHO with candidate SCG(s)) |  | YES | ignore |
| Handover Cause | M |  | Cause 9.2.3.2 | Indicates handover cause employed for handover from NG-RAN node 2 | YES | ignore |
| Source cell CGI | M |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN CGI of source cell for handover procedure (in NG-RAN node 2) | YES | ignore |
| Target cell CGI | M |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN CGI of target cell for handover procedure (in NG-RAN node 1). If the Handover Report Type is set to “Inter-system ping-pong”, it contains the target cell of the inter system handover from the other system to NG-RAN node 1 cellIf the Handover Report Type is set to “Too Late CHO with candidate SCG”, it is the suitable cell in the NG-RAN node 2. | YES | ignore |
| Re-establishment cell CGI | C- ifHandoverReportType HoToWrongCell |  | Global Cell Identity 9.2.2.73 | CGI of cell where UE attempted re-establishment or where UE successfully re- connected after the failure | YES | ignore |
| Target cell in E-UTRAN | C- ifHandoverReportType Intersystempingpong |  | OCTET STRING | Encoded according to Global Cell ID in the Last Visited E-UTRAN Cell Information IE, as defined in TS 36.413 [31] | YES | ignore |
| Source cell C-RNTI | O |  | BIT STRING (SIZE (16)) | C-RNTI allocated at the source NG-RAN node (in NG-RAN node 2) | YES | ignore |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information provided in the HANDOVER REQUEST message or in the SN STATUS TRANSFER message from NG-RAN node 2. | YES | ignore |
| UE RLF Report Container | O |  | UE RLF Report 9.2.2.59 | The UE RLF Report Container IE received in the FAILURE INDICATION message. | YES | ignore |
| CHO Configuration | O |  | 9.2.2.76 |  | YES | ignore |
| Target cell C-RNTI | O |  | BIT STRING (SIZE (16)) | C-RNTI allocated at the target NG-RAN node. | YES | ignore |
| Time Since Failure | O |  | INTEGER (0.. 172800, ...) | Corresponds to the TimeSinceFailure IE received from the UE in RLF Report as defined in TS 36.331 [14]. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifHandoverReportType HoToWrongCell | This IE shall be present if the Handover Report Type IE is set to the value "HO to wrong cell" |
| ifHandoverReportType Intersystempingpong | This IE shall be present if the Handover Report Type IE is set to the value "Inter-system ping-pong" |

Remaining text not changed