3GPP TSG-RAN WG3 Meeting #128	R3-253152

# Malta, MT, 19 – 23 May 2025

Agenda Item:	11.2

Source:	ZTE Corporation

Title:	Discussion on AI/ML assisted Network Slicing

Document for:	Discussions & Approval

- Introduction

In the last meeting, following is the agreement reached:

For Slice UE performance request, take Opt2: Extend Existing Report Characteristics Bitmap.

Extending the existing Node Measurement Failed Report Characteristics IE to include slice-level UE performance metrics for indicating the failed result for slice-level UE performance metrics.

Add Slice UE performance IE in the DATA COLLECTION UPDATE message.

Per S-NSSAI per UE performance metrics is reported as part of the Slice UE Performance IE in the Data Collection Update message. An FFS on how the metrics are calculated will be added in the TP. If no SA5 definition on how to calculate the metrics is available, RAN3 to decide the metrics are derived by an average or weighted average measurement.

Introduce the measured CCCV within the predicted slice available capacity.

In this contribution, we further to discuss the FFS part related to AI/ML assisted Network Slicing and provide the corresponding stage 2 & stage 3 TP to finalize the normative work.

- Discussion

According to the current content in the BLCR, the normative work on AI/ML-assisted Network Slicing is largely complete, with the introduction of key information such as Predicted Slice Available Capacity, Predicted Slice Radio Resource Status, and Slice UE Performance, among others. However, there are still a few open FFS items remaining:

9.2.3.x2	Slice UE Performance

This IE indicates per Slice UE performance measurements metrics.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Slice UE Performance |  | 1..< maxnoofBPLMNs > |  |  |
| >PLMN Identity | M |  | 9.2.2.4 | Broadcast PLMN |
| >S-NSSAI UE Performance List |  | 1 |  | Indicates the UE performance per network slice (details are FFS) |
| >>S-NSSAI UE Performance Item |  | 1 .. < maxnoofSliceItems> |  |  |
| >>>S-NSSAI | M |  | 9.2.3.21 |  |
| >>>Slice Based UE Performance | M |  | UE Performance 9.2.3.179 | [It is FFS how the slice based UE performance metrics are derived] [It is FFS whether and how the Slice Average Packet Loss DL IE is defined] |

For the FFS on whether and how the Slice Average Packet Loss DL IE should be defined, it depends on the clarification regarding the feasibility of calculating average packet loss at the PDCP level. Until the reply LS is received, this FFS needs to be retained.

With respect to the FFS how the slice based UE performance metrics are derived, companies would like to make confirmation with SA5 colleagues internally, and decide the feasibility of slice based UE performance. In this meeting, we received the LS from SA5 about the clarification on UE level Average throughput, we think the similar understanding can apply to slice UE performance.

Firstly, with respect to the average UL/DL UE throughput specified in TS 28.558:

6.3.1.4 UE throughput 6.3.1.4.1 Average DL UE throughput in gNB a) This measurement provides the average UE throughput in downlink. This measurement is intended for data bursts that are large enough to require transmissions to be split across multiple slots. The UE data volume refers to the total volume scheduled for each UE regardless of if using only primary- or also supplemental aggregated carriers. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in EN-DC architecture [16]) and subcounters per supported S-NSSAI. This measurement is also referred to as DL M5 in TS 37.320 [9]. b) DER(N=1) *******Skipped******* d) Each measurement is a real value representing the throughput in kbit per second. The number of measurements is equal to one. If the optional QoS level subcounter and S-NSSAI subcounter are performed, the number of measurements is equal to the number of mapped 5QIs and the number of supported S-NSSAIs. e) The measurement name has the form DRB.UEThpDl, or optionally DRB.UEThpDl.QoS, where QoS identifies the target quality of service class, and DRB.UEThpDl.SNSSAI, where SNSSAI identifies the S-NSSAI. f) NRCellDU g) 5G-S-TMSI h) One usage of this measurement is to support ML training and performance evaluation. 6.3.1.4.2 Average UL UE throughput in gNB a) This measurement provides the average UE throughput in uplink. This measurement is intended for data bursts that are large enough to require transmissions to be split across multiple slots. The UE data volume refers to the total volume scheduled for each UE regardless of using only primary- or also supplemental aggregated carriers. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in EN-DC architecture [16]) and subcounters per supported S-NSSAI. This measurement is also referred to as UL M5 in TS 37.320[9]. B) DER(N=1) *******Skipped******* d) Each measurement is a real value representing the throughput in kbit per second. The number of measurements is equal to one. If the optional QoS level subcounter and S-NSSAI subcounter are performed, the number of measurements is equal to the number of mapped 5QIs and the number of supported S-NSSAIs. e) The measurement name has the form DRB.UEThpUl, or optionally DRB.UEThpUl.QoS, where QoS identifies the target quality of service class, and DRB.UEThpUl.SNSSAI, where SNSSAI identifies the S-NSSAI. f) NRCellDU g) 5G-S-TMSI h) One usage of this measurement is to support ML training and performance evaluation.

It can be clearly observed that Average UL/DL UE throughput can be measured/calculated per S-NSSAI level. SA5 mentioned in the LS that the rationale for using the "DRB." prefix in the measurement names, as listed in field (e), is to indicate that the measurements originate from data transmissions occurring at the DRB level. Therefore, the Average UL/DL UE throughput per S-NSSAI level can also be aggregated based on the measurements taken per DRB.

- Based on the TS28.558 and clarification from SA5, Average UL/DL throughput per S-NSSAI can be supported through the aggregated data across DRBs for each slice.

Similarly, packet delay and packet loss specified in TS 28.558 also can be achieved through the aggregated data across DRBs for each slices.

6.3.1.1 Packet delay 6.3.1.1.1 Average delay DL air-interface e) The measurement name has the form DRB.AirIfDelayDlUe.Filter, Where Filter is a combination of QoS level and S-NSSAI, QoS level represents the mapped 5QI or QCI level, and SNSSAI represents S-NSSAI. *******Skipped******* 6.3.1.1.2 Average delay DL in gNB-DU e) The measurement name has the form DRB.RlcSduDelayDlUe.Filter, Where Filter is a combination of QoS level and S-NSSAI, QoS level represents the mapped 5QI or QCI level, and SNSSAI represents S-NSSAI. *******Skipped******* 6.3.1.1.3 Average delay DL on F1-U e) The measurement name has the form DRB.PdcpF1DelayDlUe.Filter, Where Filter is a combination of QoS level and S-NSSAI, QoS level represents the mapped 5QI or QCI level, and SNSSAI represents S-NSSAI. *******Skipped******* 6.3.1.1.4 Average delay DL in CU-UP e) The measurement name has the form DRB.PdcpSduDelayDlUe.Filter, Where Filter is a combination of QoS level and S-NSSAI, QoS level represents the mapped 5QI or QCI level, and SNSSAI represents S-NSSAI. *******Skipped******* 6.3.1.1.5 UL PDCP packet average delay e) The measurement name has the form DRB.PdcpDelayUlUe.Filter, Where Filter is a combination of QoS level and S-NSSAI, QoS level represents the mapped 5QI or QCI level, and SNSSAI represents S-NSSAI. 6.3.1.2.1 DL Packet Loss Rate on Uu e) DRB.PacketLossRateUuDlUe.Filter, Where Filter is a combination of QoS level and S-NSSAI, QoS level represents the mapped 5QI or QCI level, and SNSSAI represents S-NSSAI.

- Based on the TS28.558 and clarification from SA5, Average UL/DL packet delay and Average DL packet loss per S-NSSAI can be supported through the aggregated data across DRBs for each slice.

Therefore, we propose to remove the FFS regarding how the slice-based UE performance metrics are derived. In addition, the UE Performance IE (9.2.3.179) represents per-UE level performance metrics, which are defined differently from Slice UE performance metrics. Hence, we suggest introducing a new IE to represent Slice UE performance metrics, rather than reusing the legacy UE Performance IE.

- Introduce a separate IE to represent Slice UE performance metrics instead of reusing the legacy UE Performance IE.

The criticality of Slice UE performance IE in the tabular should be “ignore” to align with the ASN.1.

- Modify the criticality of the Slice UE Performance IE in the tabular to "ignore".

The criticality of the Slice Measurement Initiation Result IE should be aligned with the Cell Measurement Initiation Result List IE and the Node Measurement Initiation Result List IE, which is "reject".

- Modify the criticality of the Slice Measurement Initiation Result IE to "reject".

Corresponding TPs to 38.300 & 38.423 to reflect the proposals above is provided in [1][2].

3	Conclusion

Following is our proposals and observations:

- Based on the TS28.558 and clarification from SA5, Average UL/DL throughput per S-NSSAI can be supported through the aggregated data across DRBs for each slice.

- Based on the TS28.558 and clarification from SA5, Average UL/DL packet delay and Average DL packet loss per S-NSSAI can be supported through the aggregated data across DRBs for each slice.

- Introduce a separate IE to represent Slice UE performance metrics instead of reusing the legacy UE Performance IE.

- Modify the criticality of the Slice UE Performance IE in the tabular to "ignore".

- Modify the criticality of the Slice Measurement Initiation Result IE to "reject".

4	Reference

[1] R3-253154 [TP to 38.423] AI/ML assisted Network Slicing ZTE Corporation, Qualcomm, China Unicom

[2] R3-253153 [TP to 38.300] AI/ML assisted Network Slicing ZTE Corporation, Lenovo, Samsung