3GPP TSG-RAN WG3 Meeting #128	R3-253161

Malta, MT, 19-23, May, 2025

Agenda Item:	8.1

Source:	ZTE Corporation

Title:	Discussion on per-UE UE performance metrics

Document for:	Discussions & Approval

- Introduction

During the previous meetings, RAN3 has discussed how to derive UE level performance metrics which is transferred between gNBs to support AI/ML for NG-RAN function. The LS [1] was sent to SA5 to seek clarification on the interpretation of Average UE Throughput DL and Average UE Throughput UL measurements defined in TS 28.558.

In this meeting, RAN3 received the reply LS [2] from SA5 and SA5 provides the clear clarification on per-UE UE throughput UL/DL. Based on the reply LS, we present our understanding of how to specify UE performance metrics in the RAN3 specifications.

- Discussion

With respect to the UE performance metrics for AI/ML for NG-RAN feature, RAN3 has reached the following common understanding: UE level performance is derived as the average value of per DRB level measurements of the UE. Specifically, the metrics can be categorized into three key aspects: Average UE Throughput, Average UE Packet Delay, Average UE Packet Loss.

Regarding UE throughput, RAN3 identified a discrepancy between the current common understanding within RAN3 and the definition of per-UE throughput specified by SA5 in TS 28.558. To address this inconsistency and align the understanding, RAN3 has sent the LS to SA5 requesting clarification. SA5 now replied the LS below:

SA5 confirms that the formulas specified in fields (c) of clauses 6.3.1.4.1 and 6.3.1.4.2 in TS 28.558 calculate average UE throughput based on per DRB level measurement for a UE. The formulas aggregate data across DRBs for each UE, ensuring that the reported throughput measurement represents the overall throughput per UE rather than being limited to individual DRBs. The rationale for using the "DRB." prefix in the measurement names, as listed in field (e), is to indicate that the measurements originate from data transmissions occurring at the DRB level. However, these measurements are aggregated to provide a per-UE throughput metric. Specifically: - The formulas include a summation over DRBs, ensuring that the result represents the total throughput per UE rather than individual DRB-level values. - The "DRB." prefix in field (e) indicates that the contributing measurements originate from DRBs, but they are aggregated to reflect per-UE throughput. - The definitions in the tables in field (c) in clauses 6.3.1.4.1 and 6.3.1.4.2 are based on DRB level measurements, but the summation ensures that the final metric is at the UE level. In summary, although the measurement inputs are taken per DRB, they are aggregated to provide throughput metrics at the UE level.

Based on the provided explanations, it can be concluded that per-UE throughput metrics can indeed be calculated by aggregating per-DRB measurement inputs. Although the individual throughput measurements originate at the DRB level (as indicated by the "DRB." prefix), the calculation explicitly includes summation across all DRBs for a given UE. Take parallel transmission and serial transmission as examples, respectively to explain how the formulation to represents the overall throughput per UE.

Example 1 (Serial transmission):

# DRB 1: Data Volume 1 = 10Mbit, and the time period to transmit such data volume T1 =100ms.

# DRB 2: Data Volume 2 = 20Mbit, and time period to transmit such data volume T2 = 50ms.

Based on the formulation in TS28.558:

If ThpTimeUl>0, ThpVolUlThpTimeUl×1000 [kbit/s]

If ThpTimeUl=0, 0 [kbit/s]

The respective average throughput per DRB level can be calculated below:

AvgThpDRB1= 10Mbit10ms=1bit/s                         AvgThpDRB2= 20Mbit5ms=4bit/s

And the average throughput per UE level can be aggregated by summation of measurement inputs across all DRBs. Thus, average throughput per UE level can be calculated:

AvgThpUE= 10Mbit+20Mbit10ms+5ms=2bit/s

Example 2 (Parallel transmission):

# DRB 1: Data Volume 1 = 10Mbit, and the time period to transmit such data volume T1 =100ms.

# DRB 2: Data Volume 2 = 20Mbit, and time period to transmit such dat volume T2 = 50ms.

And the average throughput at the UE level can be derived by aggregating the measurement inputs across all DRBs. According to the definition in field (e) of clauses 6.3.1.4.1 and 6.3.1.4.2 in TS 28.558, ThpTime refers to the time taken to transmit a data burst, excluding the time associated with the slot when the buffer is emptied. This implies that, in the case of parallel transmission scenarios, the transmission time should be taken as the longest transmission time among the parallel paths, rather than the sum of the individual transmission times.

| ThpTimeDl | The time to transmit a data burst excluding the data transmitted in the slot when the buffer is emptied. A sample of "ThpTimeDl" for each time the DL buffer for one Data Radio Bearer (DRB) is emptied. |
| --- | --- |

Thus, average throughput per UE level can be calculated:

AvgThpUE= 10Mbit+20Mbit10ms=3bit/s

The numerical examples above illustrate how to calculate average UE throughput at the per-UE level, based on the clarification provided by SA5. Most importantly, it confirms that the approach defined in TS 28.558 for average UE throughput per UE level is supported by SA5.

- Average UL/DL UE throughput per UE level is supported by SA5 based on the formulation defined in TS 28.558.

According to Observation 1, the common understanding in RAN3 regarding Average UL/DL UE Throughput should be revised. Since SA5 supports measuring and calculating average UL/DL throughput at the per-UE level through the aggregation of measurements at the DRB level, we propose to directly adopt this definition in the RAN3 specification.

- Directly refer to the definition in TS 28.558 for Average UL/DL UE Throughput over XnAP.

The average packet delay and average packet loss are supported at the per-DRB level, rather than at the per-UE level, in TS 28.558. Therefore, the previous common understanding in RAN3 can be reused for these metrics. The revised common understanding can be stated as follows:

UE-level Packet Loss and UE-level Packet Delay performance metrics are derived as the average of the corresponding per-DRB level measurements for the UE.

- Revise the previous common understanding stated as “UE-level Packet Loss and UE-level Packet Delay performance metrics are derived as the average of the corresponding per-DRB level measurements for the UE.”

The definition and understanding of Average UL/DL UE Throughput, Average UL/DL UE Packet Delay, and Average DL UE Packet Loss should be  reflected in the semantic description of UE performance IE.

- Agree the CR to reflect the understanding of each UE performance metrics in the semantic description of UE performance IE.

- Conclusion

Following is our proposals and observations:

- Average UL/DL UE throughput per UE level is supported by SA5 based on the formulation defined in TS 28.558.

- Directly refer to the definition in TS 28.558 for Average UL/DL UE Throughput over XnAP.

- Revise the previous common understanding stated as “UE-level Packet Loss and UE-level Packet Delay performance metrics are derived as the average of the corresponding per-DRB level measurements for the UE.”

- Agree the CR to reflect the understanding of each UE performance metrics in the semantic description of UE performance IE.

- Reference

[1]	R3-247775, LS on per-UE UE performance metrics, RAN3

[2]	S5-252056, Reply LS on per-UE UE performance metrics, SA5