3GPP TSG-RAN WG3 Meeting #128	R3-253177

Malta, MT, 19-23 May, 2025

Title:	Consideration on leftovers for Inventory and Command

Agenda Item:	16.2

Source:	Huawei

Document for:	Discussion

# 1	Introduction

In this contribution, we further discuss the leftovers for Inventory and Command, and provide the corresponding TPs to baseline CRs.

# 2	Discussion

## 2.1	About Inventory

- 2.1.1		Correlation Identifier IE

In last meeting, the following agreements were captured in chairman notes:

General - Turn previous “WA: Introduce a NGAP Class 1 Inventory Request procedure.” and “WA: Introduce a NGAP Class 1 Command Request procedure” to agreements. - Define the Reader Index as INTEGER (1..65536, …). - Introduce a new “A-IoT Area”. The AIOTF may indicate the Requested Service Area as a list of “A-IoT Areas” and/or a list of readers in the Inventory Request message. - The AIOTF is aware of the area within which the gNB supports the provision of A-IoT services by means of OAM. This area may be represented as “A-IoT Areas” and/or the readers supported by the gNB. About Inventory - Introduce a “Correlation ID” in the respective NGAP inventory at least inside the transfer containers. - WA: Include the Correlation Identifier IE outside of the Inventory related Transfer IEs in all the Inventory related messages. - Include the AIOTF Identifier IE outside of the Inventory related Transfer IEs in all the Inventory related messages. - The Inventory Request Transfer IE, also includes the following: - Device Identification for Paging (to enable paging for single device, a group of devices, all devices) - Requested Service Area Information (A-IoT Area list and/or Reader list) - Inventory Assistance Information (Approximates Number of Target A-IoT Devices). - Reader indexes and the gNB ID are mandatory included in Inventory Report Transfer IE.

For the “WA: Include the Correlation Identifier IE outside of the Inventory related Transfer IEs in all the Inventory related messages.”, to allow parallel NGAP Inventory Request procedures between gNB and AMF, in addition to the AIOTF Identifier, we also need to include the Correlation identifier IE outside of the containers in all the Inventory related messages. Therefore, the WA should be turned into an agreement.

Proposal 1-1:	Turn the “WA: Include the Correlation Identifier IE outside of the Inventory related Transfer IEs in all the Inventory related messages.” to agreement.

- 2.1.2		Inventory Assistacne Information IE in the Inventory Request Transfer IE

About Inventory Assistance Information IE in the Inventory Request Transfer IE, the following FFS was made in last meeting:

- It is FFS whether the Inventory Assistance Information includes the “Estimate of Expected D2R Message Size”

In RAN2#129bis meeting, there is following agreement:

RAN2#129bis agreements on segmentation: … 4 For inventory response, RAN2 assumes that segmentation is not applied. RAN2 assumes that the reader can avoid segmentation by reader being aware of inventory response size. Notify SA2 about this assumption.

And in the up-to-date SA2 TS 23.369-030 [1], there are also the following decriptions about assistance information provided to gNB in Inventory related procedure:

SA2 TS 23. 369-030: ## 5.4 Assistance information provided to NG-RAN node The AIOTF provides the following assistance information to the NG-RAN together with the service operation requests: a) AIoT service type (e.g. Inventory, Command) . b) Optionally, approximate number of AIoT devices based on AF request. c) Optionally, approximate D2R message size based on AF request. d) Optionally, time interval as AIoT aggregation assistance information. ## 6.2.2 Inventory Procedure 7. The AIOTF sends the Inventory Request message including the Correlation ID, the AIoT Identification Information to be included in the paging message, and assistance information to the selected NG-RAN. …

Although SA2 TS says “optionally”, in RAN2 LS (R2-2503199), RAN2 informs SA2 and RAN3 that RAN2 assumes that the inventory response size can always be provided by the core network to the reader.”. To support the RAN2 requirement, in NGAP BL CR, the “Expected D2R Message Size” should be mandatorily included in the Inventory Request Transfer IE, and it is an accuratre value, instead of an estimated/approximiate value.

Proposal 1-2:	Mandatorily include the “Expected D2R Message Size” in the Inventory Request Transfer IE.

In the latest TS 23.369, the following can be found:

The following lengths are supported for the Identification Information in an AIoT Device Permanent Identifier: 96 bits, and 128 bits.

Therefore, for the Expected D2R Messag Size IE to be provided in the Inventory Request Transfer IE, we can define it as INTEGER (1..255, …) unit: bit.

Proposal 1-3	Define the Expected D2R Message Size IE in the Inventory Request Transfer IE as INTEGER (1..255, …) unit: bit.

Proposal 1-4	Send Reply LS to RAN2 (reply to R2-2503199), inform them that RAN3 mandatorily include the “Expected D2R Message Size” defined as INTEGER (1..255, …) in the Inventory Request Transfer IE.

- 2.1.3		Reader Selection

The following agreements were achieved in RAN3#127 meeting:

- Upon receiving only the area in Inventory Request, the gNB selects readers within the indicated area. - Upon receiving neither the area nor the reader list in Inventory Request, the gNB selects all the served readers. - WA: Upon receiving only the reader list in Inventory Request, the gNB selects the readers indicated by the reader list. - FFS: Upon receiving both the area and the reader list in Inventory Request, the gNB selects the readers within the indicated area and the readers within the reader list.

For the “WA: Upon receiving only the reader list in Inventory Request, the gNB selects the readers indicated by the reader list.”, upon receiving only the reader list in the Inventory Request, the gNB should select the readers indicated by the reader list, the gNB should not intend to add other readers or remove some indicated readers. Therefore, the WA should be turned to agreement. And the last FFS will be solved by combing of the bullets 1 and 3.

Proposal 1-5:	Turn the “WA: Upon receiving only the reader list in Inventory Request, the gNB selects the readers indicated by the reader list” to agreement.

Proposal 1-6:	Turn the “FFS: Upon receiving both the area and the reader list in Inventory Request, the gNB selects the readers within the indicated area and the readers within the reader list.” to agreement.

## 2.2	About Command after Inventory

- 2.2.1		Command after inventory procedures

In last meeting, the following agreements were captured in chairman notes:

General - Turn previous “WA: Introduce a NGAP Class 1 Inventory Request procedure.” and “WA: Introduce a NGAP Class 1 Command Request procedure” to agreements. - Define the Reader Index as INTEGER (1..65536, …). - Introduce a new “A-IoT Area”. The AIOTF may indicate the Requested Service Area as a list of “A-IoT Areas” and/or a list of readers in the Inventory Request message. - The AIOTF is aware of the area within which the gNB supports the provision of A-IoT services by means of OAM. This area may be represented as “A-IoT Areas” and/or the readers supported by the gNB. About Command after inventory - Introduce a follow-on command indication in Inventory Request Transfer IE, in case of Command after inventory. - For command related NGAP messages, the Correlation Identifier IE is the same one as in the related Inventory procedure. - WA: NGAP: Command Request procedure is a per single device procedure, and no need to have a Command Report procedure. - WA: Include the Correlation Identifier IE in both inside and outside of the Command related Transfer IEs in all the Command related messages. - Include the AIOTF Identifier IE outside of the Command related Transfer IEs in all the command related messages. - The Command Request Transfer IE, also includes the following: - An A-IoT NAS PDU - Command Assistance Information: (Estimate of Expected D2R Message Size) - It is FFS whether the Command Assistance Information includes the Command Type (read, write, disable…) - Introduce per-session per-device Device Association (“RAN NGAP Device ID”, and “(FFS) CN NGAP Device ID” pair) between gNB and AIOTF, in case of Command after inventory. - In case of command after inventory, the gNB allocates and provides a “RAN NGAP Device ID” in the Inventory Report Transfer IE for each device. - The Device Association is included in the Command Request Transfer IE and Command Response Transfer IE. It is FFS whether to introduce the Device Association or a transaction ID, outside of the Transfer IEs, to allow parallel Command Request procedures for different devices between gNB and AMF.

For the “WA: NGAP: Command Request procedure is a per single device procedure, and no need to have a Command Report procedure.”, according to the decriptions about A-IoT Command service captured in SA2 TS 23.369-030, Command request procedure is a per single device procedure.

SA2 TS 23.369-030: ### 5.2.2 AIoT Command service Three types of Command service operations are supported: Read, Write and Disable. An AF uses the Command service Read operation to retrieve information from a specific AIoT device. An AF uses the Command service Write operation to configure information into a specific AIoT device. An AF uses the Command service Disable operation to permanently disable the capability of the AIoT device(s) to transmit RF signals

Furthermore, in case of multiple devices Inventory, we should allow the network to perform command transmission for one inventoried device before the final completion of the Inventory operation for all devices, if the NGAP command procedure is defined as a procedure related to multiple devices, multiple command procedures over NG interface are still needed, from this perspective, it is also preferred to define the command procedure as a per device procedure, and there is no need to additionally introduce a Command Report procedure.

Proposal 2-1:	Turn the “WA: NGAP: Command Request procedure is a per single device procedure, and no need to have a Command Report procedure.” to agreement.

- 2.2.2		Correlation Identifier IE

For the “WA: Include the Correlation Identifier IE in both inside and outside of the Command related Transfer IEs in all the Command related messages.”, according to the up-to-date SA2 TS 23.369-030, the Correlation Identifier IE is included in all the NGAP Command related messages. we follow SA2 TS to include a Correlation Identifier provided by the AIOTF in all command related messages.

In order to allow parallel NGAP Command Request procedures between gNB and AMF, in addition to the AIOTF Identifier, we also need to include the Correlation identifier IE outside of the containers in all the Command related messages. Therefore, the WA should be turned into agreement.

SA2 TS 23.369-030: ### 6.2.3 Command Procedure 8. For each successful Inventory response received, the AIOTF sends Command Request message (Correlation ID, [Reader ID], NAS Command Request, [Approximate D2R message size]) to the NG-RAN directly or as an AIoT Transfer Container via an AMF as specified in clause 6.2.4. The NAS Command Request message includes the AIoT data. 11. The NG-RAN responds with a Command Response message (Correlation ID, Reader ID, NAS Command Response) to the AIOTF directly or as an AIoT Transfer Container via an AMF as specified in clause 6.2.4.

Proposal 2-2:	Turn the “WA: Include the Correlation Identifier IE in both inside and outside of the Command related Transfer IEs in all the Command related messages.” to agreement.

- 2.2.3		Command Assistance Information IE

About Command Assistance Information IE included in Command Request Transfer IE, the following FFS was made last meeting:

- It is FFS whether the Command Assistance Information includes the Command Type (read, write, disable…)

According to SA2 TS 23.369-030 [1], there are also the following assistance information provided to gNB in Command related procedure:

SA2 TS 23. 369-030: ## 5.4 Assistance information provided to NG-RAN node The AIOTF provides the following assistance information to the NG-RAN together with the service operation requests: a) AIoT service type (e.g. Inventory, Command) . b) Optionally, approximate number of AIoT devices based on AF request. c) Optionally, approximate D2R message size based on AF request. d) Optionally, time interval as AIoT aggregation assistance information. If not provided by the AF, bullets b) and c) in the above assistance information provided by the AIOTF may be based on local configuration based on SLA between the AIoT service provider represented by an AF and the operator. ### 7.2.3 Naiotf_AIoT_Command service operation Service operation name: Naiotf_AIoT_Command. Description: The NF consumer requests a command operation for one or multiple AIoT Device(s). Inputs, Required: 1) AF ID. 2) At least one of the following parameters are included: - Target area information for the command operation. - Information about the target AIoT Device(s): - either the AIoT Device ID(s) or the filtering information for multiple AIoT Devices. 4) Command Type: Read, Write or Permenant Disable. 5) Command specific parameters: Editor’s note: The AIoT Data specific for the Read/Write/Disable command is FFS. Editor’s note: The Command specific parameters for Read/Write/Disable command is FFS. Inputs, Optional: 1) Information to be used for resource allocation: - Approximate number of AIoT Devices. - Approximate message size from the AIoT Device for Read Operation. Editor’s note: It is FFS whether the Approximate message size from the AIoT Device can also be provided by the AF for the other command operations. Outputs, Required: Transaction ID, Result indication (Success or Failure), Failure Cause in case of Failure. Outputs, Optional: None. ### 7.4.3 Nnef_AIoT_Command service operation Service operation name: Nnef_AIoT_Command Description: The consumer requests to perform a command operation for an AIoT Device or multiple AIoT Devices. Input, Required: AF ID, Target area information or AIoT Device ID(s) or AIoT Device ID filter information for the command operation, Command type (Read, Write, or Disable). Editor’s note: The AIoT Data specific for the Read/Write/Disable command is FFS. Input, Optional: Approximate number of AIoT Devices, Approximate message size from the AIoT Device for Read command type. Editor’s note: It is FFS whether the Approximate message size from the AIoT Device can also apply for the other command types. Output, Required: AF Transaction ID, Result indication, Failure cause in case of Failure. Output, Optional: None. .…

According to SA2 TS, the “approximate D2R message size” is optional in AIOTF services, in NEF services and from AIOTF to the gNB, which means that it may be absent in COMMAND REQUEST message. BUT the Command type (Read, Write, or Disable) is mandatory in both in AIOTF services and in NEF services, therefore, it is needed and easily to include Command Type (read, write, disable, …) in Command Assistance Information IE in the Command Request Transfer IE, so that the gNB can differentiate command type and take it into account for scheduling.

Proposal 2-3:	Include the Command Type (read, write, disable, …) in Command Assistance Information IE in the Command Request Transfer IE.

It is noticed that SA2 sends LS S2-2504294 asks RAN whether there is any concern if SA2 agrees to remove “Service Type” (i.e., Inventory or Command) from “Assistance Information”. We can reply to them that there is no concern on that, and inform them about RAN3 agreement on including the Command Type.

Proposal 2-4:	Send Reply LS to SA2 (reply to S2-2504294), inform them that RAN3 has no concern on their removal of “Service Type” (i.e., Inventory or Command) information, and inform them that RAN3 agreed to include the Command Type (read, write, disable, …) as one of the Command Assistance Information.

- 2.2.4		Device association management

About Device association management, the following FFSs were made last meeting:

- Introduce per-session per-device Device Association (“RAN NGAP Device ID”, and “(FFS) CN NGAP Device ID” pair) between gNB and AIOTF, in case of Command after inventory. - The Device Association is included in the Command Request Transfer IE and Command Response Transfer IE. It is FFS whether to introduce the Device Association or a transaction ID, outside of the Transfer IEs, to allow parallel Command Request procedures for different devices between gNB and AMF.

And in the LS R3-250905, the related decriptions about Device association can be founded:

LS R3-250905: - RAN3 agreed to introduce per-session per-device device association (“RAN NGAP Device ID”). - Upon receiving the “follow-on command indication”, the gNB allocates for each device a “RAN NGAP Device ID” and provides it in the AIOTF information container in the Inventory Report message. - The “RAN NGAP Device ID” is included in the AIOTF information container of each Command related NGAP message.

For the FFS “Introduce per-session per-device Device Association (“RAN NGAP Device ID”, and “(FFS) CN NGAP Device ID” pair) between gNB and AIOTF, in case of Command after inventory.”, if there is no CN NGAP Device ID, the AIOTF needs to record both RAN NGAP Device ID and the Global gNB ID to uniquely identify a Device, it can work, but not as efficient as using the pair of CN/RAN NGAP Device ID, like we always use a pair of NGAP UE ID in UE associated messages. Therefore, it is preferred to introduce “CN NGAP Device ID” allocated by the AIOTF.

More specifically, in command after inventory procedure, the AIOTF allocates CN NGAP Device ID and includes the RAN/CN device ID pair in the COMMAND REQUEST message for one device, after the AIOTF receives the INVENTORY REPORT message including A-IoT NAS PDU (Device Identifier) and RAN NGAP Device ID.

Proposal 2-5:	Introduce “CN NGAP Device ID” allocated by the AIOTF, in case of Command after inventory.

- 2.2.5		Parallel command procedures for different devices between gNB and AMF

For the “It is FFS whether to introduce the Device Association or a transaction ID, outside of the Transfer IEs, to allow parallel Command Request procedures for different devices between gNB and AMF.”, in our view, comparing with using an additionally defined transaction ID, since the RAN NGAP Device ID has already been introduced, it is easier to include the RAN NGAP Device ID outside of the Command related Transfer IEs in all the Command related messages.

Proposal 2-6:	Include the CN/RAN NGAP Device ID pair outside of the Command related Transfer IEs, to allow parallel Command Request procedures for different devices between gNB and AMF.

## 2.3	IE Definition Details

### 2.3.1		A-IoT Correlation Identifier

#### 9.3.3.xx1 A-IoT Correlation Identifier This IE uniquely identifies an A-IoT session within an AIOTF. IE/Group Name; Presence; Range; IE type and reference; Semantics description. A-IoT Correlation Identifier; M; FFS; ..

Based on TS 23.369, this ID is generated by the AIOTF.

And in MBS, both the TMGI and Associated Session ID are defined as OCTET STRING. And in TS 29.571, the notifyCorrelationId is defined as string.

Taking all these into account, maybe we can also define this A-IoT Correlation Identifier as an OCTET STRING.

Proposal 3-1:	Define the A-IoT Correlation Identifier IE as OCTET STRING.

### 2.3.2		AIOTF Identifier

#### 9.3.3.xx2 AIOTF Identifier This IE uniquely identifies the AIOTF. IE/Group Name; Presence; Range; IE type and reference; Semantics description. AIOTF Identifier; M; FFS; ..

As defined in TS 29.571, a NF instance is uniquely identified by an NfInstanceId (except AMF), and the NfInstanceId uses string format:

| NfInstanceId | string | String uniquely identifying a NF instance. The format of the NF Instance ID shall be a Universally Unique Identifier (UUID) version 4, as described in IETF RFC 4122 [15]. The hexadecimal letters should be formatted as lower-case characters by the sender, and they shall be handled as case-insensitive by the receiver. Example: "4ace9d34-2c69-4f99-92d5-a73a3fe8e23b" (NOTE 3) |
| --- | --- | --- |

From our understanding, AIOTF Identifier can also be defined as OCTET STRING, and refers to the NfInstanceId IE defined in TS 29.571.

Proposal 3-2:	Define the AIOTF Identifier IE as OCTET STRING, refer to the NfInstanceId IE defined in TS 29.571.

### 2.3.3		A-IoT Area

Chairman notes: Introduce a new “A-IoT Area”. The AIOTF may indicate the Requested Service Area as a list of “A-IoT Areas” and/or a list of readers in the Inventory Request message. The AIOTF is aware of the area within which the gNB supports the provision of A-IoT services by means of OAM. This area may be represented as “A-IoT Areas” and/or the readers supported by the gNB. It is FFS whether the new A-IoT Area is encoded as GNSS info or new A-IoT Area ID.

NGAP BL CR #### 9.3.3.xx3 Requested Service Area Information This IE includes the Requested Service Area Information for the A-IoT inventory. IE/Group Name; Presence; Range; IE type and reference; Semantics description. …. >>A-IoT Area; M; FFS.

It was mentioned in offline session that in NTN, the CN can indicate a list of area in GNSS info format and send to the gNB. The following can be found in NTN NGAP BL CR (R3-251562):

In case GNSS Info is used

Probably the coverage of one reader may belong to different A-IoT Areas.

Upon receiving the requested A-IoT Areas in Inventory Request, the gNB has to calculate which readers belong to the indicated areas.

In case new A-IoT Area ID is used:

The A-IoT ID can be defined similar as TAI, e.g.:

A-IoT Area ID =PLMN ID + A-IoT Area Code = OCTET STRING (SIZE(3)) + OCTET STRING (SIZE(3))

It is easy to map one reader only to one A-IoT Area.

Upon receiving the requested A-IoT Areas, the gNB just choose the readers belongs to the indicated A-IoT Areas.

Considering that NTN scenarios are quite special, encode A-IoT Area as new ID is simpler from both NGAP signalling and gNB handling point of view.

Proposal 3-3:	Introduce new A-IoT Area ID to represent the A-IoT Area, defined as follows:

A-IoT Area ID =PLMN ID + A-IoT Area Code = OCTET STRING (SIZE(3)) + OCTET STRING (SIZE(3))

It is noticed that there is a reply LS S2-2504288 from SA2 on Ambient IoT progress of RAN3, there are two questions from SA2:

- SA2 would ask whether the ‘supported Area’ is Tracking Area or a new type of area.

- SA2 would like to ask whether one reader is only part of one supported Area or can be part of multiple supported Areas.

RAN3 has already answered the first question in the LS R3-252481 send in RAN3#127.

For the second question, in our view, one reader should only belong to one A-IoT Area. And it is also better to clarify that one reader only belongs to one gNB, one A-IoT Area may cover readers belongs to the same or different gNBs.

Proposal 3-4:	One reader only belongs to one A-IoT Area. One reader only belongs to one gNB. One A-IoT Area may cover readers belong to the same or different gNBs. One gNB may serve multiple readers which belong to the same or different A-IoT Areas.

with the discussion above, we can reply to SA2 to clarify the things, and we can also inform them about the detailed encoding of the A-IoT Area ID.

Proposal 3-5:	Send Reply LS to SA2 (reply to S2-2504288), inform the agreements on proposal 3-3 and 3-4.

### 2.3.4		A-IoT Device Identification Requested

#### 9.3.3.xx5 A-IoT Device Identification Requested This IE represents the Requested Identity of the A-IoT device(s). IE/Group Name; Presence; Range; IE type and reference; Semantics description. FFS.

Editor’s Note: this IE enables identification of a single A-IoT device, a group of A-IoT devices, all A-IoT devices, detailed coding FFS.

RAN TR 38.769 has captured that A-IoT Device Identification can be included in INVENTORY REQUEST message. And according to RAN2#129 agreements, reader needs to know whether the inventory is for a single device or multiple devices. When we take this into account with R19 RAN WID, RAN2#129 agreements and TS 23.369 on paging ID, for the “multiple devices” inventory, there should be one filtering information, or no identifier.

- R19 RAN WID: - RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. - Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - …

RAN2#129 agreements on paging ID - The “one identifier” in the paging message includes both the case of “one single device identifier” and “one group identifier”/”filtering criteria”, while the exact format of latter is supposed to be designed by SA2. - The current assumption is that the paging identifier is transparent to the A-IoT MAC Layer and carried by upper layer. FFS if there is really a need for visibility in the MAC layer

RAN2#129 agreements RAN2 understands that the service type of A-IoT (e.g. inventory, command) and whether the service is targeted for a single or multiple devices can always be provided. The approximate number of target devices can be provided if available.

TS 23.369 The AIoT Identification Information to be provided to NG-RAN can include Filtering Information, as defined in clause 5.8, or a single AIoT Device Identifier.

- Therefore, the Device Identification for Paging IE should be able to include a single device ID for single device inventory, or to include a filtering information for a group of devices inventory, or no identifier for all device inventory. This can be achieved by including three branches in a CHOICE type IE.

Proposal 3-6:	Define the Device Identification for Paging IE as CHOICE type, with three branches, one with single device Identifier for single device inventory, one with a filtering information for a group devices inventory, and one NULL branch for all devices inventory.

### 2.3.5		Estimate of Expected D2R Message Size (command)

#### 9.3.3.yy3 Command Assistance Information This IE includes the assistance information for the A-IoT Command Service. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Estimate of Expected D2R Message Size; O; FFS.

In TR 38.769, the following can be found:

-	In both R2D and D2R directions, there is no lower bound on the minimum TB size, and a maximum TB size of around 1000 bits can be supported.

In RAN1 April meeting, the following WA was made:

Working assumption
For indicating the payload size (i.e. TBS-like) for PDRCH transmission with variable size:
• 7 bits for byte-level D2R payload size indication

Considering that 27=128, 1000bits/8 = 125, the 1000 bits maximum size assumption is still there in RAN1 discussion. Therefore we can define this IE as INTEGER (1..1024, …) unit: bit.

Proposal 3-7:	Define the Estimate of Expected D2R Message Size IE in the Command Assistance Information IE as INTEGER (1..1024, …) unit: bit.

### 2.3.7		Location of Reader

AIOTF may be aware of the location of reader via OAM configuration. The details of the location of reader (e.g., room number, GNSS info) are FFS.

As summarized in the last meeting sod R3-252297, there are several proposals on the details of the Reader Location:

1)	Civic Address defined in TS 29.572

2)	NG-RAN Access Point Position defined in TS 38.455

3)	NG-RAN High Accuracy Access Point Position IE defined in TS 38.455

4)	Geographical Coordinates specified in TS 38.455

4) is specified as TRP location, it is not proper to be reused directly, as in A-IoT discussion, “Reader” is used instead of “TRP”.

Proposal 3-8:	The Reader Location can be indicated by Civic Address defined in TS 29.572, or NG-RAN Access Point Position or NG-RAN High Accuracy Access Point Position defined in TS 38.455.

## 2.4	Other small FFSs

It is noticed that there are two FFSs on the presence of Cause IE in the INVENTORY FAILURE message and the COMMAND FAILURE messages:

These two FFSs were added because there is a mandatory Cause IE included in the Inventory/Command Failure Transfer IEs, and therefore the Cause IE outside of the container seems redundant in case of direct connectivity. Considering that the current design principle is try to have unified handling at gNB side (note that the AIOTF Identifier IE is added outside of the container as mandatory IE although it is not useful in case of direct connectivity), it is better to also include the Cause IE as mandatory IE in these two messages.

Proposal 4:	Remove the FFSs on the presence of Cause IE in the INVENTORY FAILURE message and the COMMAND FAILURE message.

# 3	Conclusion

In this contribution, we further discussed the leftovers for Inventory and Command over NG interface, and got the following proposals:

About Inventory

Proposal 1-1:	Turn the “WA: Include the Correlation Identifier IE outside of the Inventory related Transfer IEs in all the Inventory related messages.” to agreement.

Proposal 1-2:	Mandatorily include the “Expected D2R Message Size” in the Inventory Request Transfer IE.

Proposal 1-3:	Define the Expected D2R Message Size IE in the Inventory Request Transfer IE as INTEGER (1..255, …) unit: bit.

Proposal 1-5:	Turn the “WA: Upon receiving only the reader list in Inventory Request, the gNB selects the readers indicated by the reader list” to agreement.

Proposal 1-6:	Turn the “FFS: Upon receiving both the area and the reader list in Inventory Request, the gNB selects the readers within the indicated area and the readers within the reader list.” to agreement.

About Command

Proposal 2-1:	Turn the “WA: NGAP: Command Request procedure is a per single device procedure, and no need to have a Command Report procedure.” to agreement.

Proposal 2-2:	Turn the “WA: Include the Correlation Identifier IE in both inside and outside of the Command related Transfer IEs in all the Command related messages.” to agreement.

Proposal 2-3:	Include the Command Type (read, write, disable, …) in Command Assistance Information IE in the Command Request Transfer IE.

Proposal 2-5:	Introduce “CN NGAP Device ID” allocated by the AIOTF, in case of Command after inventory.

Proposal 2-6:	Include the CN/RAN NGAP Device ID pair outside of the Command related Transfer IEs, to allow parallel Command Request procedures for different devices between gNB and AMF.

About Detailed IE Definition

Proposal 3-1:	Define the A-IoT Correlation Identifier IE as OCTET STRING.

Proposal 3-2:	Define the AIOTF Identifier IE as OCTET STRING, refer to the NfInstanceId IE defined in TS 29.571.

Proposal 3-3:	Introduce new A-IoT Area ID to represent the A-IoT Area, defined as follows:

A-IoT Area ID =PLMN ID + A-IoT Area Code = OCTET STRING (SIZE(3)) + OCTET STRING (SIZE(3))

Proposal 3-4:	One reader only belongs to one A-IoT Area. One reader only belongs to one gNB. One A-IoT Area may cover readers belong to the same or different gNBs. One gNB may serve multiple readers which belong to the same or different A-IoT Areas.

Proposal 3-6:	Define the Device Identification for Paging IE as CHOICE type, with three branches, one with single device Identifier for single device inventory, one with a filtering information for a group devices inventory, and one NULL branch for all devices inventory.

Proposal 3-7:	Define the Estimate of Expected D2R Message Size IE in the Command Assistance Information IE as INTEGER (1..1024, …) unit: bit.

Proposal 3-8:	The Reader Location can be indicated by Civic Address defined in TS 29.572, or NG-RAN Access Point Position or NG-RAN High Accuracy Access Point Position defined in TS 38.455.

Other small FFSs

Proposal 4:	Remove the FFSs on the presence of Cause IE in the INVENTORY FAILURE message and the COMMAND FAILURE message.

About Reply LS to other WGs

Proposal 1-4	Send Reply LS to RAN2 (reply to R2-2503199), inform them that RAN3 mandatorily include the “Expected D2R Message Size” defined as INTEGER (1..255, …) in the Inventory Request Transfer IE.

Proposal 2-4:	Send Reply LS to SA2 (reply to S2-2504294), inform them that RAN3 has no concern on their removal of “Service Type” (i.e., Inventory or Command) information, and inform them that RAN3 agreed to include the Command Type (read, write, disable, …) as one of the Command Assistance Information.

Proposal 3-5:	Send Reply LS to SA2 (reply to S2-2504288), inform the agreements on proposal 3-3 and 3-4.

Based on all these proposals, the corresponding TPs are provided in R3-253178 [2], and in the following sections, we also provide draft replies on the incoming LSs.

# Reference

- [1] 3GPP TS 23.369, Architecture support for Ambient power-enabled Internet of Things (Release 19)

[2] R3-253178 (TPs for TS 38.300 38.401 38.413 BL CRs) Leftovers for Inventory and Command, Huawei

# Draft Reply LS to SA2 S2-2504288

3GPP TSG-RAN WG3 Meeting #128	TDoc <TDoc#>

Malta, MT, 19 – 23 May, 2025

Title:	[DRAFT] Reply LS on Ambient IoT progress of RAN3

Response to:	LS S2-2504288 on Reply on Ambient IoT progress of RAN3 from SA2

Release:	Release-19

Work Item:	AmbientIoT-ARC, Ambient_IoT_solutions-Core

Source:	xxx [will be RAN3]

To:	SA2

Cc:	SA5

Contact person:	<name_of_tdoc_requestor>

<email_of_tdoc_requestor>

<phone_of_tdoc_requestor>

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	N/A

# 1	Overall description

RAN3 thanks SA2 for the Reply on Ambient IoT progress of RAN3. About the questions from SA2, RAN3 would like to provide answers as follows:

- SA2 would ask whether the ‘supported Area’ is Tracking Area or a new type of area.

RAN3 answer:

- RAN3 agreed to introduce new A-IoT Area, details have already been provided in R3-252481. And RAN3 further agreed to use new A-IoT Area ID to represent an A-IoT Area.

- AIoT Area ID =PLMN ID + A-IoT Area Code = OCTET STRING (SIZE(3)) + OCTET STRING (SIZE(3))

- SA2 would like to ask whether one reader is only part of one supported Area or can be part of multiple supported Areas.

RAN3 answer:

- A gNB serves one or more readers. One reader only belongs to one gNB.

- An A-IoT Area may cover readers belong to the same or different gNBs. One reader only belongs to one A-IoT Area.

- A gNB may serve multiple readers which belong to the same or different A-IoT Areas.

# 2	Actions

To SA2

ACTION: 	RAN3 kindly asks SA2 to take above answsers into account in their discussion.

# 3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#129		2025-08-25 - 2025-08-29			Bangalore, IN

RAN3#129-bis	2025-10-13 - 2025-10-17			Prague, CZ

RAN3#130		2025-11-17 - 2025-11-21			Dallas, US

# Draft Reply LS to RAN2 R2-2503199

3GPP TSG-RAN WG3 Meeting #128	TDoc <TDoc#>

Malta, MT, 19 – 23 May, 2025

Title:	[DRAFT] Reply LS on D2R message size for inventory response

Response to:	LS R2-2503199 on D2R message size for inventory response from RAN2

Release:	Release-19

Work Item:	Ambient_IoT_solutions

Source:	xxx [will be RAN3]

To:	RAN2, SA2

Cc:

Contact person:	<name_of_tdoc_requestor>

<email_of_tdoc_requestor>

<phone_of_tdoc_requestor>

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	N/A

# 1	Overall description

RAN3 thanks RAN2 for the LS on D2R message size for inventory response.

RAN3 would like to inform RAN2 and SA2 that, based on RAN2 requirement, RAN3 mandatorily included an Expected D2R Message Size IE (INTEGER (1..255, …) unit: bit) in the Inventory Request Transfer IE in the NGAP: INVENTORY REQUEST message.

# 2	Actions

To RAN2, SA2

ACTION: 	RAN3 kindly asks RAN2 and SA2 to take above into account in their discussion.

# 3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#129		2025-08-25 - 2025-08-29			Bangalore, IN

RAN3#129-bis	2025-10-13 - 2025-10-17			Prague, CZ

RAN3#130		2025-11-17 - 2025-11-21			Dallas, US

# Draft Reply LS to SA2 S2-2504294

3GPP TSG-RAN WG3 Meeting #128	TDoc <TDoc#>

Malta, MT, 19 – 23 May, 2025

Title:	[DRAFT] Reply LS on the removal of service type information

Response to:	LS S2-2504294 on on the removal of service type information SA2

Release:	Release-19

Work Item:	AmbientIoT-ARC, Ambient_IoT_solutions

Source:	xxx [will be RAN3]

To:	SA2, RAN2

Cc:	RAN1

Contact person:	<name_of_tdoc_requestor>

<email_of_tdoc_requestor>

<phone_of_tdoc_requestor>

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	N/A

# 1	Overall description

RAN3 thanks RAN2 for the LS on the removal of service type information.

RAN3 do not have concern on the removal of the service type information.

And further, RAN3 agreed to include the Command Type (read, write, disable) in the AIOTF information container in the Command Request message as one of the command assistance information.

# 2	Actions

To SA2, RAN2

ACTION: 	RAN3 kindly asks SA2 and RAN2 to take above into account in their discussion.

# 3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#129		2025-08-25 - 2025-08-29			Bangalore, IN

RAN3#129-bis	2025-10-13 - 2025-10-17			Prague, CZ

RAN3#130		2025-11-17 - 2025-11-21			Dallas, US