3GPP TSG-RAN WG3 #128	R3-253189

# St Julian’s, Malta, 19-23 May 2025

Agenda Item:	10.3.1

Source:	Ericsson

Title: 	SON/MDT enhancements for NTN

Document for:	Discussion

# 1	Introduction

RAN plenary decided during RAN#103 to support further data collection for Self-organizing Networks (SON). One of the topics that was endorsed was the SON enhancements for non-terrestrial networks.

This paper builds on the previous submissions by discussing SON for NTN on a range of different topics such as RLF enhancements and MDT area scope related enhancements.

# 2	Discussion

## 2.1 MDT related enhancements for NTN

An agreement regarding logged MDT Area Scope was reached in RAN3#126 as follows:

RAN3 assumes both geographical area and the mapped cell ID as Area Scope of logged MDT for NTN over NGAP are feasible and confirm with RAN2 whether the geographical area defined for MBS NTN can be reused for MDT NTN.

In NTN, the mapping between the coverage of an NTN cell and the geographical area covered by that cell changes over time. To enable measuring the NTN service coverage in a given geographical area, it is straightforward to extend the MDT Area Scope to support geographical areas with smaller granularity.

The mapping between the mapped cell IDs and geographical areas is configured in the RAN and Core Network. The gNB is responsible for constructing the mapped cell ID based on the UE location info received from the UE, if available. The mapping may be pre-configured (e.g., up to operator's policy) or up to implementation. UE has no idea of the mapping between the mapped cell ID and the geographical areas. The NG-RAN node needs to translate the geographical fixed areas of the mapped cell ID(s) and send this information to the UE, which is redundant and not efficient.

In addition, the mapped cell ID is normally a large geographical area, thus the use case of using mapped cell ID for the MDT area scope is limited to cases when the MDT measurement is configured in a large area, e.g. within a whole NTN cell. As the purpose of the MDT area scope enhancement is to enable collection of MDT measurement in smaller geographical areas and/or enabling finer resolution when defining the geographical areas, mapped cell ID for area scope of MDT is not an optimal option.

On the other hand, using the geographical area allows an operator to collect MDT more precisely, for example around a lake, mountain, instead of the whole NTN cell which is big. Moreover, the definition of geographical area over NGAP and Uu interface can be identical, which means that the area scope in the NGAP can be signaled directly to the UE, which does not need an extra translation in the gNB side.

Proposal 1: RAN3 to support only geographical area for logged MDT Area Scope.

The MDT area scope enhancement for logged MDT over NGAP can also be applied to immediate MDT using the same IE without any extra effort.

Proposal 2: The same MDT Area Scope enhancement applies to both logged and immediate MDT over NGAP.

The Area Scope of MDT needs specific enhancements to indicate to the UE, that the MDT has to be collected in NTN networks. To do so, we need to add an NTN wide indicator in the Area Scope of MDT. This will allow the network to indicate to the UE to collect MDT only in NTN. There is currently no method in the specifications to indicate that MDT must be collected in a specific type of network, NTN in this case. This will allow an operator to build a coverage map of NTN network and detect problems specific to that type of RAT.

Other options such as indicating the CGI of the cell can be used, however they have an upper limit on the number of identifiers and as such is unsuitable for this scenario.

Proposal 3: Include NTN wide indication in the MDT Area Scope to enable MDT collection.

## 2.2 RLF related enhancements for NTN

Coarse location information

In the current RLF report, the cell and beam level measurements associated to SSB/CSI-RS of both serving cell and neighboring cells in the form of RSRP, RSRQ, SINR and/or RSSI are included. These reported measurement results at the time of connection failure are essential important for the network to understand the cause of the failure. The reason behind this is that in terrestrial networks, the RRM measurement, e.g. RSRP has a clear difference in a cell edge compared to cell center. However, in NTN deployments, there is typically a small difference in signal strength between cell center and cell edge. Therefore, based on the RRM measurements, the network may not be able to find the root cause for the connection failure as the UE may experience a very small difference in signal strength between two beams(cells) in a region of overlap.

Observation 1: The network may not be able to detect the root cause of failure based on the RRM measurements in RLF report as in TN deployment.

In case of location-based CHO, with user consent, the location information included in the RLF report can be used to detect and optimize the CHO configurations. However, without user consent, there will be no location information reported, which makes the reported RLF report meaningless to tune the location-based CHO configuration.

Similarly, in case of non-CHO handover, even when Event D1/D2 measurement report is configured, without user consent, there will be no location information available in the RLF report when failure occurs.

The coarse location of the UE when failure occurs can be considered. The coarse location information requires no user consent and with a 2km accuracy. The network can roughly detect the failure cause based on the coarse location information received in the RLF report and adjust the handover configurations accordingly both for conditional handover and non-conditional handover.

Proposal 4: RAN3 to suggest to RAN2 to include the coarse location information when failure occurs in the RLF report.

Satellite switch with re-sync

One of these additions would be related to the satellite switch with re-sync procedure, as defined in rel-18. It would be beneficial for the network to understand that this procedure was performed and failed. This could be done by e.g. reusing the “hof” codepoint as connection failure type in the RLF report, and by adding a new indication (explicit or implicit) that the mobility procedure as failed. A new a new value for the lastHO-Type field (e.g. satSwitchWithResync) could also be added.

Proposal 5: RAN3 to suggest to RAN2 to enhance the RLF report with indication that a satellite switch with re-sync procedure was performed and failed.

## 2.3 RACH-less HO related enhancements for NTN

During the RAN3 #127bis, MRO for RACH-less handover for NTN was discussed and the following two scenarios were reached:

Divide the MRO for RACH-less handover case into two scenarios: - RACH-less handover was attempted but failed; - UE was configured RACH-less handover but did not attempt RACH-less handover because RSRP is below threshold

In RAN2 #128 MRO for LTM discussion, it was agreed that “No need to include the specific access type (i.e. RACH-based or RACH-less) in the RLF report explicitly. It can be known implicitly. This can be revisited if RAN2 concludes that there are issues.” Similar with LTM, the NW can implicitly figure out whether the HOF pertains to a RACH-less failure or a RACH-based failure for NTN. Therefore, for scenario 1, the NW can understand if RACH-less handover was attempted but failed. Unlike LTM, the timing advance for RACH-less HO in NTN is always configured with 0 or the same timing advance with source cell. When scenario1 happens, NW can analyze that the SSB for dynamic grant configuration or the SSB and related configuration for CG may not be optimized.

Scenario 2 is the fallback scenario, when uplink grant for RACH-less handover is configured, the UE will initiate RACH-less HO or Random-Access procedure for HO by checking if there is SSB corresponding to the configured uplink grant with SS-RSRP above a RSRP threshold is available. Therefore, as NW knows all the CG RACH-less configuration, when there is RA information reported in the RLF report, NW can figure out that RACH-less fallback happens for the failed handover. However, when RACH-less fallback successes, the NW may not be able to know if it is a RACH-less fallback or a RACH-based handover via the RA Report.

For the failure case, the NW can differentiate scenario 1 and 2. But the NW is lack of necessary information to accurately tune the RACH-less HO configuration, e.g. the information of the measured RSRP for the SSB beam(s) for the configured grant can be used for the NW to optimize the configured uplink grant for RACH-less handover configuration. Similar with the RACH optimization for 2-Step RA in Rel-17, dlPathlossRSRP IE was introduced which can be used by NW to tune the RSRP threshold for selection between 2-step RA type and 4-step RA type. It is therefore proposed to include the measured RSRP information in SON reports to help the NW to optimize the RSRP threshold configured for SSB selection for the CG RACH-less handover.

Proposal 6: UE logs the information of the measured RSRP for the SSB beam(s) corresponding to the configured grant for RACH-less HO.

## 2.4 UHI related enhancements for NTN

The final aspect we would like to introduce in the paper is the collection of UE history information (UHI) for NTN. UE history information is network side collection of history of cells that the UE has visited. Initially, this was used to detect and avoid ping-pong. However, the application of UHI has been expanded to other use-cases such as trajectory prediction and so on.

Upon successful HO of a UE, the network records the CGI, time stayed, cell type and HO cause value. This UHI needs to be enhanced to indicate to the network that the cell type the UE was in, was an NTN cell. The obvious way of doing this is to enhance the Cell Type IE in UHI to denote that the cell is an NTN cell (e.g. LEO, GEO), in addition to the legacy cell size.

Proposal 7: Add new NTN cell types (e.g. LEO, GEO) to Cell Type IE for indicating the NTN cells in UHI.

The cells in a non-terrestrial network may be moving as opposed to the generally static deployment of a terrestrial network. Thus, one of the new handovers causes that might occur is because the cell moved away from the UE, as opposed to the opposite in a terrestrial network. This needs to be added as a cause value in UHI. Additionally, satellite switch with re-sync is a special mobility procedure because both the PCI and the carrier frequency remains the same, in order to identify the satellite switch with re-sync mobility, it should also be considered in the enhancement of cause value in the UHI.

Proposal 8: Enhance Cause value to reflect cell mobility in a non-terrestrial network.

# 3	Summary

In this discussion paper, NTN SON/MDT enhancements were discussed, and the following observations and proposals were made:

Proposal 1: RAN3 to support only geographical area for logged MDT Area Scope.

Proposal 2: The same MDT Area Scope enhancement applies to both logged and immediate MDT over NGAP.

Proposal 3: Include NTN wide indication in the MDT Area Scope to enable MDT collection.

Observation 1: The network may not be able to detect the root cause of failure based on the RRM measurements in RLF report as in TN deployment.

Proposal 4: RAN3 to suggest to RAN2 to include the coarse location information when failure occurs in the RLF report.

Proposal 5: RAN3 to suggest to RAN2 to enhance the RLF report with indication that a satellite switch with re-sync procedure was performed and failed.

Proposal 6: UE logs the information of the measured RSRP for the SSB beam(s) corresponding to the configured grant for RACH-less HO.

Proposal 7: Add new NTN cell types (e.g. LEO, GEO) to Cell Type IE for indicating the NTN cells in UHI.

Proposal 8: Enhance Cause value to reflect cell mobility in a non-terrestrial network.

# 4	References

- RP-234038, “New WID: Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity) Phase 4”, RAN#102, Edinburgh, Scotland, December 2023.