3GPP TSG-RAN WG3 Meeting #128	R3-253193

# St Julian, Malta, 19th – 23rd May, 2025

Title: 	MDT Enhancements for slice-focused measurement collection

Source: 	Ericsson, Orange, InterDigital, Deutsche Telekom, Jio Platforms (JPL), FiberCop

Agenda item:	10.3.2

Document Type:	Discussion and decision

# 1	Introduction

The release 19 work item for Data Collection for SON/MDT in NR standalone and MR-DC focuses on enhancements for Rel-17/Rel-18 SON/MDT features such as MRO enhancements for LTM, CHO with candidate SCGs and subsequent CPAC, as well as on SON/MDT enhancements for Intra NTN mobility and Network Slicing [1].

As part of this work item, both RAN3 and SA5 have agreed to enhance the functionality of management-based immediate MDT for network slicing, by enabling defining the area scope for MDT measurements using a list of S-NSSAIs.

Despite these slice specific extensions of MDT area scope, the current MDT framework does not enable collecting the MDT measurements for a slice only when the slice is actively used by the UEs, i.e., the UE from which the MDT measurements are collected has at least one active PDU session on the slice. Instead, the specified MDT area scope extensions allow for UEs to be selected if the cells that the UE is connected to supports the S-NSSAIs in the slice area scope. Furthermore, to keep the measurement overhead to a minimum it is important to have the functionality to collect MDT measurements only when the slice that is specified as part of the area scope is experiencing certain load conditions.

In this contribution, we share our views on the above-mentioned MDT enhancements for Network slicing, where we present the motivation and benefits of MDT measurement collection/reporting subject to certain conditions on slice utilization and obtaining MDT measurements from UEs that have active PDU sessions on the slice of interest.

# 2	Discussion

## Shortcomings of the existing MDT framework

In management-based immediate MDT, the OAM configures individual RAN nodes with MDT measurement configuration and the RAN nodes perform UE selection and collection of MDT measurements. The collected measurements are then sent back to the OAM. The UE selection process is based on the configuration provided by the OAM, i.e., based on the provided area scope, and after taking considerations of UE capability, consent, etc. However, the MDT measurement/report collection starts immediately upon selection of a suitable UE, regardless of the the slice-level conditions such as load, SLA fulfilment, etc., and/or the services/slices in use at the UE when the measurements are collected/reported.

Similarly, in signalling-based immediate MDT, the OAM and the Core Network tasks the RAN to configure MDT measurements on a specific UE, and the UE is configured immediately. The measurement in this case too commences immediately at the UE and continues regardless of the services/slices in use at the UE. The results are then signalled back to the OAM.

Observation 1: In signalling-based and management-based immediate MDT, the measurement session/report collection starts immediately after the UE has been selected and upon it receiving an MDT configuration. The measurement collection is independent of the services/slices in use at the selected UE. The current MDT framework does not enable restricting the collection/reporting of MDT measurements only to the slice(s) which are specified as the slice area scope in the MDT measurement configuration.

Depending on the existing slice-level conditions during the period over which the OAM configures the RAN to report MDT measurements, it may happen that the utilization of a certain network slice for which measurements are collected fluctuates from low to high. The reported performance of UEs may be good, e.g., during non-peak utilization of the slice, while it may have degraded when the slice utilization was high.

In the context of Network Slicing, where each slice is a logically isolated network on top of a shared physical network, observing the performance of each of the slices to ensure that the slice-related QoS/QoE/SLA requirements are fulfilled is vital for the delivered services. However, the delivered service characteristics on a certain slice may depend on the underlying network/hardware/load-related conditions which may need to be carefully correlated with the data collected to monitor SLA guarantee/QoS monitoring.

Thus, to increase slice specific observability and collection/exchange of pertinent data, i.e., data that is collected when the necessary pre-conditions are met, it is important to have the capability to collect MDT measurements based on slice-specific KPIs available in the RAN. For example, if the RAN nodes are provided with criterion to collect MDT measurements for a slice only when the slice specific load is very high, it would be able to observe if there is a degradation in the slice performance under specific preconditions. When the slice specific load is low, there may be no incentive to obtain detailed measurements, e.g., MDT, and the SLA/QoS monitoring may be performed via existing mechanisms.

Observation 2: Conditional observability into slice-level performance will enable the network to perform slice- focussed data collection and can significantly reduce the amount of monitoring required to guarantee SLA/QoS requirements.

## Enhancements for slice-focused MDT measurements

### MDT measurements collection/reporting from UE(s) actively using the slice

In the current framework for both signalling and management-based MDT, once a UE has been selected for MDT measurements collection by the gNB (in case of management-based MDT) or by the OAM/Core Network (in case of signalling-based MDT), the measurements are collected from the selected UEs regardless of the slice being actively used by the UE. In the context of increasing the slice observability, is very important to ensure that measurements for the selected UE(s) are collected only while the UE(s) is actively using the slice(s) under observation.

Proposal 1: RAN3 to agree enhancements to Signalling-based and Management-based Immediate MDT measurements, where the MDT measurement collection/reporting is conditional on UE having one or more active PDU sessions on the given slice(s).

### Conditionally triggered management-based immediate MDT

In the context of management-based immediate MDT, the existing MDT framework allows the OAM to specify the Area Scope as a UE selection criterion. While this enables physically identifying and selecting UEs in certain cells, TAs, etc., the selection criteria does not determine when the measurements are done, or which of the collected measurements are signalled to the OAM. In the current MDT framework, measurements are always collected and transmitted to the OAM after a UE has been selected (as described in observation 1).

The above described, existing, MDT functionality is especially expensive when considering NTN and other deployment scenarios where the cost associated with transmitting additional information to/from the network is high. Indeed, in case of NTN all the measurements collected by the gNB and reported to the OAM are sent over the satellite to ground link which makes the process very expensive. If not all the collected measurements are relevant (no filtering is done at gNB level before reporting the measurements to the OAM), a good amount of the satellite to ground link bandwidth is wasted at a high cost on transferring irrelevant information. Currently the only trade-off available is to configure such MDT measurements, although not all of the collected information may be relevant to the analysis that may be planned with the collected data; or not configure such measurements at all. In this context, conditionally triggered MDT measurements can significantly decrease the demands on the UE and the network and facilitate collection of MDT measurements in situations where the additional collected information can be put to good use.

The points made in this paper identify and motivate for measurement/reporting criteria, where for a UE that has previously been selected (in accordance with a selection criteria), the measurements are collected/reported to the OAM only upon fulfilment of the measurement/reporting criteria, which may be defined in terms of thresholds onslice-level utilisation values.

Proposal 2: RAN3 to agree sending an LS to SA5 to request enhancements to Management-based Immediate MDT measurements, where the MDT measurement collection/reporting is conditional on slice-level utilization and/or on the selected UE having one or more active PDU sessions on the given slice(s).

### Conditionally triggered signalling-based immediate MDT

The above-mentioned discussion points, in terms of the need for slice-specific observability and collection of data only when certain conditions are met, are also true in the context of signalling-based immediate MDT, where the gNB knows the UE for which MDT measurements are to be collected. Furthermore, compared to management-based immediate MDT, signalling-based immediate MDT is less scalable as the number of UEs selected increases. With the proposed conditional MDT enhancements, the scalability of signalling based immediate MDT can also be improved, with the additional benefit of obtaining more focussed and granular measurements only when the specified pre-conditions are met.

Proposal 3: RAN3 to agree enhancements to Signalling-based Immediate MDT measurements, where the MDT measurement collection/reporting is conditional on slice utilization.

# Conclusion

In the previous sections we made the following observations and proposals:

Observation 1: In signalling-based and management-based immediate MDT, the measurement session/report collection starts immediately after the UE has been selected and upon it receiving an MDT configuration. The measurement collection is independent of the services/slices in use at the selected UE. The current MDT framework does not enable restricting the collection/reporting of MDT measurements only to the slice(s) which are specified as the slice area scope in the MDT measurement configuration.

Observation 2: Conditional observability into slice-level performance will enable the network to perform slice- focussed data collection and can significantly reduce the amount of monitoring required to guarantee SLA/QoS requirements.

Proposal 1: RAN3 to agree enhancements to Signalling-based and Management-based Immediate MDT measurements collection, where the MDT measurement collection/reporting is conditional on the selected UE(s) having one or more active PDU sessions on the given slice(s).

Proposal 2: RAN3 to agree sending an LS to SA5 to request enhancements to Management-based Immediate MDT measurements, where the MDT measurement collection/reporting is conditional on slice-level utilization and/or on the selected UE(s) having at least one active PDU sessions on the given slice.

Proposal 3: RAN3 to agree enhancements to Signalling-based Immediate MDT measurements, where the MDT measurement collection/reporting is conditional on slice utilization.

Proposal 4: RAN3 to agree to the TP to TS 38.413 (R3-253194) and TP to TS 38.423 (R3-253195), which mirror the proposals discussed in this paper.

# References

[1] RP-234038: WID: Data collection for SON/MDT in NR standalone and MR-DC

# Appendix: [DRAFT] LS to SA5

3GPP TSG RAN WG3 Meeting #128	R3-25xxxx

St Julian, Malta, 19 - 23 May 2025

Title:	LS on Deferred MDT measurements

Release:	Rel-19

Work Item:	NR_ENDC_SON_MDT_Ph4-Core

Source:	Ericsson (to be RAN3)

To:	SA5

Cc:

Contact person:	Serban Purge

serban.purge@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	R3-25xxxx, R3-25xxxx

# 1	Overall description

RAN3 has discussed the topic of deferred MDT measurements, where the reporting/collection of MDT measurements may be subject to existing slice-level conditions (e.g. slice available capacity under a given threshold) and/or on the selected UE having at least one active PDU sessions on the given slice.

RAN3 would like to ask SA5 to discuss and agree implementing the above enhancement for management-based MDT measurements. Further description of the enhancements and implementation details are available in the attached discussion paper and TPs to 38.413 and 38.423.

# 2	Actions

To SA5 group:

ACTION: 	RAN3 would like to ask SA5 to discuss and agreed MDT enhancements for deferred MDT support and to update the SA5 specification accordingly.

# 3	Dates of next TSG-RAN WG3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

TSG-RAN3 Meeting #129		August 25-28, 2025			Bangalore, India

TSG-RAN3 Meeting #129-bis		October 13-17, 2025			Prague, Czech Republic