3GPP TSG-RAN WG3 Meeting #128	R3-253197

# Malta, MT, 19 - 23 May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR | Draft | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (BL CR to 37.340) Support of XR enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Nokia, Nokia Shanghai Bell, Qualcomm Inc, Lenovo, Huawei, Ericsson, CATT, CMCC, Samsung, Xiaomi, NEC, China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-04-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Add support of NR-DC related enhancements for XR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | RAN3#123bis: - Add the support for XR, and ECN Marking and congestion information exposure in NR-DC. RAN3#124: - Add the support for Burst Arrival Time and End of Data Burst Indication in NR-DC. RAN3#125: - For DL, MN/SN notifies SN/MN whether the DL PSI based discard is configured or not via XnAP signaling. - Update the description for forwarding end indication of data burst in 13.x.2. RAN3#127bis: - Capture stage 2 description for SN initiated ECN marking/congestion reporting status change procedure. - Capture stage 2 description to support alternative PDU Set QoS in MRDC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Enhancement on NR-DC for XR is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.13.2, 13.x (new), 13.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.300 CR TS 37.483 CR 0171 TS 38.413 CR 1282 TS 38.423 CR 1269 TS 38.473 CR 1485 TS 38.415 CR 0056 TS 38.425 CR 0156 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: Resubmit R3-242192 Rev 2: Revision of R3-243104, capture the agreed TPs R3-243897 and R3-243899 Rev 3: Resubmit to RAN3#125. Rev 4: Revision of R3-244032, capture the agreed TP R3-244846. Rev 5: Resubmit to RAN3#125bis. Rev 6: Resubmit to RAN3#127bis. Rev 7: Revision of 251577, capture the agreed TP R3-252350, R3-252367. Rev 8: Resubmit to RAN3#128. Rev 9: Capture the description in R3-252350 in clause 13.y. |  |  |  |  |  |  |  |  |

-----------------Start of the Changes-------------------

### 10.13.2	MR-DC with 5GC

The Notification Control Indication procedure may be initiated either by the MN or by the SN and is used to indicate that GFBR/PSER/PSDB for one or several QoS flows cannot be fulfilled any more or can be fulfilled again by the reporting node.

Figure 10.13.2-1: Notification Control Indication procedure

Figure 10.13.2-1 shows an example signalling flow for the Notification Control Indication procedure.

1.	The MN may, for an SN terminated bearer, indicate, that the GFBR/PSER/PSDB requested from the MN cannot be fulfilled anymore.
In case the SN terminated bearer is configured as a split bearer, the SN may decide to increase the share provided by the SN or it may decide to notify the MN that resources requested for the SN terminated bearer cannot fulfill the GFBR/PSER/PSDB any more.

2.	Continuing the example message flow from step 1, the SN informs the MN that the GFBR/PSER/PSDB for an SN terminated bearer cannot be fulfilled any more.

3.	The MN decides to inform the 5GC that NG-RAN cannot fulfill the GFBR/PSER/PSDB for a GBR QoS flow any more.

Editor Note: FFS whether other parameters are necessary in this clause.

-----------------Next Change-------------------

## 13.x	eXtended Reality Services

### 13.x.1	Overview

The eXtended Reality Services as described in TS 38.300 [3] is extended to address the NR-DC operation.

### 13.x.2	Awareness

During the SN Addition Preparation procedure and the MN initiated SN modification procedure, the MN may send the PDU Set QoS Parameters as part of the QoS profile to the SN to enable PDU Set based QoS handling.

The UE may report uplink assistance information (jitter range, burst arrival time, UL data burst periodicity) per QoS flow via UE Assistance Information to the MN or the SN as configured. If the SN receives the burst arrival time, the SN may use it by considering the SFN offset of the MN.

When the PDCP hosting gNB receives the End of Data Burst Indication from the UPF, it always provides the End of Data Burst Indication to the corresponding node when data transmission from the PDCP hosting node to the corresponding node is needed for the Data Burst.

### 13.x.3	PSI based Discarding

For MN terminated SCG bearer and split bearer, the MN notifies SN whether the UL/DL PSI based SDU discarding is (re)configured/released for uplink/downlink discarding via XnAP signaling.

For SN terminated MCG bearer and split bearer, the SN notifies MN whether the UL/DL PSI based SDU discarding is (re)configured/released for uplink/downlink discarding via XnAP signaling.

### 13.y	ECN marking for L4S and congestion information exposure

The ECN marking for L4S and congestion information exposure as described in TS 38.300 [3] is extended to address the NR-DC operation.

For ECN marking for L4S at gNB or UPF, during the Xn SN Addition Preparation procedure or MN initiated SN Modification Preparation procedure, the MN provides the ECN marking request at gNB or ECN marking request at UPF to SN and SN reports the status indication back to the MN. The SN may change the status and report the new status indication to the MN during the SN initiated SN Modification procedure.

For congestion reporting from gNB to UPF, during the Xn SN Addition Preparation procedure or MN initiated SN Modification Preparation procedure, the MN provides the congestion information request to SN and SN reports the status indication back to the MN. The SN may change the status and report the new status indication to the MN during the SN initiated SN Modification procedure.

-----------------End of the Change-------------------