3GPP TSG-RAN WG3 Meeting #128	R3-253202

St Julian’s, Malta, 19th – 23rd May 2025

Agenda Item:		11.2

Source:		NEC, ZTE, Qualcomm, Samsung, CMCC

Title:	(TP for BLCR for TS38.300) Support of enhancements on AI/ML for NG-RAN

Document for:		Discussion and decision

# Introduction

This contribution provides the TP to AIML BLCR TS38.300 on support of enhancements on AI/ML for NG-RAN, including the following changes in order to reflect all agreements made in Rel-19:

- Add the two new use cases specified in Rel-19, i.e. Slicing and CCO.

- Add the new information that can be configured to be reported by an NG-RAN node for slicing and CCO.

# TP to BLCR TS38.300

<Start of change>

## 16.20	Support of AI/ML for NG-RAN

### 16.20.1	General

Support of AI/ML for NG-RAN, as a RAN function, is used to facilitate Artificial Intelligence (AI) and Machine Learning (ML) techniques in NG-RAN.

The objective of AI/ML for NG-RAN is to improve network performance and user experience, through analysing the data collected and autonomously processed by the NG-RAN, which can yield further insights, e.g., for Network Energy Saving, Load Balancing, Mobility Optimization, Network Slicing, Coverage and Capacity Optimization.

### 16.20.2	Principles

Support of AI/ML for NG-RAN requires inputs from neighbour NG-RAN nodes (e.g., predicted information, feedback information, measurements) and/or UEs (e.g., measurement results).

Signalling procedures used for the exchange of information to support AI/ML for NG-RAN, are use case and data type agnostic, which means that the intended usage (e.g., input, output, feedback) of the data exchanged via these procedures is not indicated.

AI/ML algorithms and models are out of 3GPP scope, and the details of model performance feedback are also out of 3GPP scope.

Support of AI/ML for NG-RAN does not apply to ng-eNB.

For the deployment of AI/ML for NG-RAN the following scenarios may be supported:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the NG-RAN node;

-	AI/ML Model Training and AI/ML Model Inference are both located in the NG-RAN node.

AI/ML Model Training follows the definition of the "ML model training" as specified in clause 3.1 of TS 28.105 [64].

AI/ML Model Inference follows the definition of the "AI/ML inference" as defined in clause 3.1 of TS 28.105 [64].

### 16.20.3	Data Collection and Reporting

The following information can be configured to be reported by an NG-RAN node:

-	Predicted resource status information, including radio resource status per slice;

-	UE performance feedback;

-	Measured UE trajectory;

-	Energy Cost (EC);

-	Predicted slice available capacity;

The collection and reporting of the above information are configured through the Data Collection Reporting Initiation procedure, while the actual reporting is performed through the Data Collection Reporting procedure.

The collection of measured UE trajectory and UE performance feedback is triggered at successful Handover.

Cell-based UE trajectory prediction, which can be used, e.g., for the Mobility Optimization use case, is transferred to the target NG-RAN node via the Handover Preparation procedure to provide information for, e.g., subsequent mobility decisions. Cell-based UE trajectory prediction is limited to the first-hop target NG-RAN node.

Future Coverage State and the associated Coverage Modification Cause can be reported by an NG-RAN node through the NG-RAN NODE CONFIGURATION UPDATE procedure.

<End of change>