3GPP TSG-RAN WG3 #128	R3-253221

St. Julian's, Malta, 19 - 23 May 2025

Agenda Item:	10.3.1

Source:	NEC

Title: 	Discussion on SON/MDT enhancements for NTN

Document for:	Discussion

1	Introduction

A new WID on Data collection for SON/MDT in NR standalone and MR-DC Phase 4 has been approved at the RAN#102 meeting [1]. In the WID, “Support of SON/MDT enhancements for Intra-NTN mobility” is included.

The objective of this work item is to specify data collection enhancement in NR standalone and MR-DC for SON/MDT purpose. The specific objectives of this work item are: - MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs, subsequent CPAC [RAN3, RAN2]: - Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3] - Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2] - Support of SON/MDT enhancements for [RAN3, RAN2]: - Intra-NTN mobility - Network Slicing - Support of the leftovers in Rel-18 SON/MDT [RAN3, RAN2]: - RACH optimization for SDT - MHI Enhancement for SCG Deactivation/Activation - MRO for MR-DC SCG failure If needed, co-operate with RAN1, SA2, SA5, CT4.

In this contribution, we describe our view for SON/MDT for NTN.

2	Discussion

2.1 MRO for location-based CHO

In the last RAN3#127bis meeting, the RLF report for location-based trigger condition was discussed and following question was sent to RAN2 by LS.[2]

RAN3 discussed the MRO enhancement for location-based CHO in NTN for earth moving scenario. In order to optimize the distance threshold for location-based CHO, RAN3 thinks it is beneficial to support either option provided below. - Option 1: to introduce the absolute UTC time when RLF/HOF happens, based on which the network computes the distance. - Option 2: to introduce measured distance from UE to moving reference location of serving cell, and measured distance from UE to moving reference location of candidate cell when RLF/HOF happens. RAN3 would like to check with RAN2 which option to be adopted in the UE RLF report.

We understand that both options can solve the problem in NTN for earth moving scenario technically, and its delta is the complexity in UE implementation. Therefore, RAN3 sent LS to RAN2 to ask which implementation will work efficiently from interface with UE point of view. Then, RAN3 needs to wait for RAN2 progress which IE (i.e., absolute UTC   or measured distance) will be introduced in stage3 spec.

Proposal 1: Wait for RAN2 discussion for further details (i.e., stage-3) about MRO for location-based CHO.

2.2  MRO for RACH-less handover

In the last meeting, the MRO for RACH-less HO was discussed and following agreements were made.[3]

MRO for RACH-less Handover for NTN: Divide the MRO for RACH-less handover case into two scenarios: - RACH-less handover was attempted but failed; - UE was configured RACH-less handover but did not attempt RACH-less handover because RSRP is below threshold

We understood that whether fallback from RACH-less handover to RACH-based handover has occurred can be known from the existing mechanism because the target node configures RACH-less HO. Therefore, it is not necessary to introduce a new mechanism to notify the fallback to RACH-based HO.

Proposal 2: It is not necessary to introduce a new mechanism to notify the fallback from RACH-less to RACH-based HO.

We would like to discuss whether to report the measured RSRP of SSB in the SON report, referencing the following two scenarios.

- RACH-less handover was attempted but failed;

- UE was configured RACH-less handover but did not attempt RACH-less handover because RSRP is below threshold

In the figure above, the horizontal axis shows the RSRP of SSB measured by the UE. To determine the RSRP threshold for RACH-less fallback to RACH-based HO, the ideal value to differentiate "RACH-less HO succeeds/fails" is shown with a broken line, and the actual set value is shown with a green solid line.

Case (a) shows when the configured value is larger than the ideal value, leading to an event where RACH-based HO fallback occurs despite RACH-less HO being possible (i.e., 2. UE was configured RACH-less handover but did not attempt RACH-less handover because RSRP is below threshold). However, it is difficult to distinguish from cases where fallback should occur, making it challenging to optimize parameters even if the measured RSRP is notified.

On the other hand, case (b) shows when the configured value is smaller than the ideal value, resulting in attempts at RACH-less HO that fail (i.e., 1. RACH-less handover was attempted but failed). In this situation, the Configured RSRP threshold needs to be increased, and setting this threshold larger than the measured RSRP when HO fails can assist in parameter optimization.

Therefore, we believe that including the measured RSRP information in the SON report only for scenario 1 would be beneficial.

Proposal 3: For scenario 1 (i.e., RACH-less handover was attempted but failed), measured RSRP of SSB can be introduced in SON report.

Proposal 4: For scenario 2 (i.e., UE was configured RACH-less handover but did not attempt RACH-less handover because RSRP is below threshold), there is no need to be introduced in SON report.

2.3  Logged MDT enhancement

In RAN3 #126 meeting, following agreement was made. Then, RAN3 sent LS to RAN2.

RAN3 assumes both geographical area and the mapped cell ID as Area Scope of logged MDT for NTN over NGAP are feasible and confirm with RAN2 whether the geographical area defined for MBS NTN can be reused for MDT NTN.

In this RAN3#128 meeting, RAN3 received RAN2 response as follows in the reply LS[4]

A: RAN2 confirms that the geographical area defined for MBS NTN can be used as a reference for Area Scope of logged MDT over Uu for NTN, including both reference location/radius and polygon-based area indication. RAN2 also agreed that only geographical area scope is used to indicate applicable logging area to the UE over Uu, i.e., no mapped cell ID(s).

We understand that RAN3 confirmed that two options for the area scope of logged MDT for NTN are feasible from the RAN3 perspective and sought RAN2's opinion on whether to adopt either or both options. Therefore, RAN3 should respect RAN2's agreement and adopt only the geographical area defined for MBS NTN as the area scope of logged MDT for NTN.

Proposal 5: RAN3 follow RAN2 agreement which introduces the geographical area defined for MBS NTN is introduced as the area scope for the logged MDT for NTN over NGAP.

3	Summary

In this discussion paper, SON for NTN enhancements were discussed and the following observations and proposals were made:

Proposal 1: Wait for RAN2 discussion for further details (i.e., stage-3) about MRO for location-based CHO.

Proposal 2: It is not necessary to introduce a new mechanism to notify the fallback from RACH-less to RACH-based HO.

Proposal 3: For scenario 1 (RACH-less handover was attempted but failed), measured RSRP of SSB can be introduced in SON report.

Proposal 4: For scenario 2 (i.e., UE was configured RACH-less handover but did not attempt RACH-less handover because RSRP is below threshold), there is no need to be introduced in SON report.

Proposal 5: RAN3 follow RAN2 agreement which introduces the geographical area defined for MBS NTN is introduced as the area scope for the logged MDT for NTN over NGAP.

4	References

- RP-234038, New WID: Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity) Phase 4, RAN#102, CMCC

- R3-252382, LS on SONMDT for NTN, RAN3#127bis, RAN3(CMCC)

- Draft meeting report RAN3#127-bis, Apr15

- R2-2500026/R3-247908, Relay LS on MDT for NTN, RAN2#129bis/RAN3#128, RAN2