3GPP TSG-RAN WG3 Meeting #128	R3-253235

Malta, MT, 19th - 23th May 2025

Agenda Item:	14.3

Source:	ZTE Corporation

Title:	Remaining issues for NR NTN

Document for:	Discussion

1  Introduction

In previous meetings, the discussion on support of regenerative payload was discussed, and the following agreements were reached[1].

NG Interface management Issue: Option 2 NG Removal/Setup can be used for Regenerative Payload NG interface management. No new NGAP suspend/resume procedures in R19. Inactive UE support: RAN3 assumes that current mechanisms to support UEs performing RRC connection resume and RRC connection re-establishment may be reused in regenerative payload architecture, subject to implementation and deployment. Support RRC_INACTIVE UE in NTN by implementation in R19. Feeder link switch issue: RAN3 assumes that during the feeder link switch, AMF is not changed for the UEs. Whether there has any standard impact needs to be further checked. The current mechanisms including, e.g. multiple TNLA association, are sufficient for TNL management during feeder link switch. In case of the TNL addresses are changed due to feeder link switch over, existing procedures may be used. Further check the functionality: in case of hard feeder link switch over, the 5GC suspends or resumes the DL signalling/data transmission(s) to the gNB based on the new indicator in RAN configuration update, FFS on the relationship with respect to NG suspend/resume (if any). Supported TAI list of the on-board gNB: The supported TAIs of an on-board gNB could be provided to AMF by OAM/pre-configuration. Others: No enhancements on location based CHO for NTN in R19.

In this contribution, we provide our further views on the remaining issues for NTN in Rel-19.

2  Discussion

2.1  Correction on NG Removal procedure

In Rel-19, the NG Removal procedure has been introduced to support the NG interface management for regenerative payload. Meanwhile, this procedure has also been introduced in the R19 WAB topic. In the NTN BL CR 38.300 [2], there is one Editor’s Note to indicate the current description of NG Removal procedure may need further discussion to consider WAB, as given below.

16.14.x NG Removal For the case of regenerative payload, when disconnecting from an AMF is required due to e.g. the satellite movement in a LEO constellation, the gNB on board the satellite may request the removal of the NG interface by initiating the NG Removal procedure toward the AMF. Editor’s Note: NG Removal procedure may also be used for WAB; how to capture this for WAB may need further discussion (pending WAB discussion).

- In the WAB BL CR 38.401 [3], the NG Removal procedure has been captured in the chapter of NG connection management and the corresponding description is similar as that in NR NTN WI.

## X.3 NG connection management Based on the OAM configuration, the WAB-gNB can set up NG interface with an AMF. When disconnecting from an AMF is required, due to inter-AMF mobility of a WAB-gNB, or when the authorization status of the WAB-gNB changes from “authorized” to “not authorized”, the WAB-gNB may request the removal of the NG interface by triggering the NG Removal procedure toward the AMF.

- Since there has been detailed description of NG Removal procedure in 38.401, the potential description of NG Removal procedure for WAB function in 38.300 can simply refer to the 38.401 without details. In addition, the chapter of NG Removal in NTN BL CR is 16.14.x, which is only used for NR NTN function, therefore, there is no need to consider the WAB function in this chapter, and the corresponding Editor’s Note should be removed.

Proposal 1: Remove the Editor’s Note in the chapter of NG Removal in BL CR 38.300.

2.2  NG connection interruption

In last meeting, the NG connection interruption due to hard FLSO issue was discussed, and the following progress is given as below.

NG Connection interruption due to hard feeder link switch over Proposal 3bis, Further check the functionality: in case of hard feeder link switch over, the 5GC suspends or resumes the DL signalling/data transmission(s) to the gNB based on the new indicator in RAN configuration update, FFS on the relationship with respect to NG suspend/resume (if any).

The “Downlink NG Transmission Control” IE was introduced over NG to support the hard feeder link switchover. However, in this case, in addition to the DL data/ signalling which should be kept and suspended, the UE Context should be also kept in the AMF. Therefore, it is better to change the newly added IE name as “NG Connection Control” to make it more general.

Proposal 2: Regarding the NG connection interruption, rename the IE as “NG Connection Control” to make it more general.

In addition, currently, the above indication is only used for hard FLSO scenario, while, as we discussed before, this indication can be used to suspend the NG connection, not only for suspending and resuming data and signalling procedures over NG. Meanwhile, the applicable scenario should not be limited to the hard FLSO, and it could be used as the complementation to the NG Removal to help resume the NG interface quickly.

Proposal 3: Regarding the NG connection interruption, remove the restriction on the usage of hard FLSO.

3  Conclusion

Proposal 1: Remove the Editor’s Note in the chapter of NG Removal in BL CR 38.300.

Proposal 2: Regarding the NG connection interruption due to FLSO, rename the IE as “NG Connection Control” to make it more general.

Proposal 3: Regarding the NG connection interruption, remove the restriction on the usage of hard FLSO.

The stage 2 TP on NG Removal is given in [4].

The stage 2 and stage 3 TP on NG connection interruption are given in [5] and [6] respectively.

References

- RAN3#127bis Chairman Notes

- R3-253093, (Support for Regenerative Payload and MBS broadcast in NR NTN, Ericsson, Thales, Deutsche Telekom, Nokia, ESA, CATT, ZTE, Sateliot, Huawei, Dish Networks, Echostar, Eutelsat Group, Xiaomi, Samsung, CMCC, LG Electronics, NEC, Lenovo, ETRI

- R3-253078, (BL CR to 38.401 for WAB) Support for Wireless Access Backhaul, Ericsson, ZTE, Nokia, Nokia Shanghai Bell, Huawei, Samsung, Lenovo, Qualcomm, Jio Platforms

- R3-253236, (TP to BL CR for 38.300) Clarification on NG Removal, ZTE

- R3-253292, (TP to BL CR for 38.300) Support for Regenerative Payload, CATT, ZTE, Samsung, CMCC, NEC

- R3-253237, (TP to BL CR for 38.300) Support of regenerative payload, ZTE Corporation, CATT, Samsung, CMCC, NEC