# 3GPP TSG-RAN WG3 Meeting #128	R3-253245

# Malta, MT, 19th – 23th May 2025

Title: 	Discussion on Leftover issues on AIoT

Source: 	ZTE Corporation, China Telecom

Agenda item:	16.2

Document for:	Discussion and Decision

- Introduction

In the RAN3 #127bis meeting, we have achieved the following progress.

Turn the WA to agreement, i.e., Including AIOTF information containers in NGAP.

Enhancing the existing NGAP interface management procedures.

Introducing an AIoT indicator in the NGAP Setup Request message. The detail of this indicator can be further discussed.

Introduce a “Correlation ID” in the respective NGAP inventory at least inside the transfer containers.

General:

- Turn previous “WA: Introduce a NGAP Class 1 Inventory Request procedure.” and “WA: Introduce a NGAP Class 1 Command Request procedure” to agreements.

- Define the Reader Index as INTEGER (1..65536, …).

- Introduce a new “A-IoT Area”. The AIOTF may indicate the Requested Service Area as a list of “A-IoT Areas” and/or a list of readers in the Inventory Request message.

- The AIOTF is aware of the area within which the gNB supports the provision of A-IoT services by means of OAM. This area may be represented as “A-IoT Areas” and/or the readers supported by the gNB.

- It is FFS whether the new A-IoT Area is encoded as GNSS info or new A-IoT Area ID.

About Inventory:

- WA: Include the Correlation Identifier IE outside of the Inventory related Transfer IEs in all the Inventory related messages.

- Include the AIOTF Identifier IE outside of the Inventory related Transfer IEs in all the Inventory related messages.

- The Inventory Request Transfer IE, also includes the following:

  - Device Identification for Paging (to enable paging for single device, a group of devices, all devices)

  - Requested Service Area Information (A-IoT Area list and/or Reader list)

  - Inventory Assistance Information (Approximates Number of Target A-IoT Devices).

- It is FFS whether the Inventory Assistance Information includes the “Estimate of Expected D2R Message Size”

About Command after inventory:

- Introduce a follow-on command indication in Inventory Request Transfer IE, in case of Command after inventory.

- For command related NGAP messages, the Correlation Identifier IE is the same one as in the related Inventory procedure.

- WA: NGAP: Command Request procedure is a per single device procedure, and no need to have a Command Report procedure.

- WA: Include the Correlation Identifier IE in both inside and outside of the Command related Transfer IEs in all the Command related messages.

- Include the AIOTF Identifier IE outside of the Command related Transfer IEs in all the command related messages.

- The Command Request Transfer IE, also includes the following:

  - An A-IoT NAS PDU

  - Command Assistance Information: (Estimate of Expected D2R Message Size)

  - It is FFS whether the Command Assistance Information includes the Command Type (read, write, disable…)

- Introduce per-session per-device Device Association (“RAN NGAP Device ID”, and “(FFS) CN NGAP Device ID” pair) between gNB and AIOTF, in case of Command after inventory.

- In case of command after inventory, the gNB allocates and provides a “RAN NGAP Device ID” in the Inventory Report Transfer IE for each device.

- The Device Association is included in the Command Request Transfer IE and Command Response Transfer IE.

- It is FFS whether to introduce the Device Association or a transaction ID, outside of the Transfer IEs, to allow parallel Command Request procedures for different devices between gNB and AMF.

FFS on the Release/Cancel/End related procedure(s)/IE(s) for the A-IoT service/inventory/command.

FFS on the NGAP signaling support to provide the A-IoT RAN Information (Reader List, Reader location, Supported A-IoT Areas) to the AIOTF.

This paper further discusses the AIoT function based on the above progress.

- Discussion

  - Definition of the Supported A-IoT Area

- The Inventory Request Transfer IE, also includes the following:

  - Requested Service Area Information (A-IoT Area list and/or Reader list)

- Introduce a new “A-IoT Area”. The AIOTF may indicate the Requested Service Area as a list of “A-IoT Areas” and/or a list of readers in the Inventory Request message.

- The AIOTF is aware of the area within which the gNB supports the provision of A-IoT services by means of OAM. This area may be represented as “A-IoT Areas” and/or the readers supported by the gNB.

- It is FFS whether the new A-IoT Area is encoded as GNSS info or new A-IoT Area ID.

FFS on the NGAP signaling support to provide the A-IoT RAN Information (Reader List, Reader location, Supported A-IoT Areas) to the AIOTF.

SA2 LS: R3-253015 1. Overall Description: SA2 thanks RAN3 for the LS about the RAN3 progress related to Architecture and OAM aspects. SA2 would like to reply as following: - AIOTF obtains the A-IoT RAN information (supported Area, served reader ID list) via OAM configuration, FFS on RAN3 signalling for the A-IoT RAN information. [SA2 reply]: - SA2 would ask whether the ‘supported Area’ is Tracking Area or a new type of area. - SA2 would like to ask whether one reader is only part of one supported Area or can be part of multiple supported Areas.

According to the previous discussion, the definition of supported A-IoT area has the following options.

- Option 1: Extend TAC value to include the supported A-IoT area, i.e., the Supported TA List includes both existing TAC for NR service and new TAC for AIoT service.

- Option 2: Define a new Supported AIoT area, i.e., define the Supported AIoT area as below.

9.3.3.xx	A-IoT Service Area Code

This IE is used to uniquely identify an A-IoT area Code.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| AIOT service Area Code | M |  | OCTET STRING (SIZE(3)) |  |

- Option 3: Reader list

In R19, the reader is static, the option 3 is feasible. However, in R20, the UE as reader is movable. So, for the future proof, the option 3 is not always feasible.

Proposal 1: Option 3 (i.e., the supported A-IoT area is defined by reader list) is excluded.

Both option 1 and option 2 are similar to define a large number code values to include the supported A-IoT area. Moreover, the option 2 is much easier to extend the code values than option 1.

Proposal 2: Define the supported AIoT area by a new type of area.

Similar to NR service area, the supported A-IoT area can be large or small. For the future proof, reader can be both TRP and UE, the device can both type 1 and type 2, one reader can be part of one supported AIoT area or multiple supported A-IoT areas.

Proposal 3: One reader can be part of one supported AIoT area or multiple supported A-IoT areas.

  - Introduce the Transaction ID

- It is FFS whether to introduce the Device Association or a transaction ID, outside of the Transfer IEs, to allow parallel Command Request procedures for different devices between gNB and AMF.

[BLCR to TS38.413] 9.2.x.5	COMMAND REQUEST

This message is sent by the A-IoT CN node to the gNB to trigger the command transmission over the A-IoT radio.

Direction: A-IoT CN node  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AIOTF Identifier | M |  | 9.3.3.xx2 |  | YES | reject |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  | YES | reject |
| Command Request Transfer | M |  | OCTET STRING | Containing the Command Request Transfer IE specified in subclause 9.3.6.5. | YES | reject |

According to the current BLCR, parallel command procedures are configure with the same Correlation identifier. Then, if one command procedure is unsuccessful, both AMF and RAN node cannot differentiate it.

Proposal 4: Introduce a transaction ID outside of the Transfer IEs to differentiate parallel command procedure.

Moreover, for the parallel Inventory report messages, we think the transaction ID is also needed.

Observation 1: The parallel Inventory report procedures for different devices between gNB and AMF shall be supported.

Meanwhile, if the AIOTF sends parallel Inventory request messages to one gNB including multiple readers, the transaction ID is also needed.

Observation 2: The parallel Inventory request procedures for different readers within the same gNB between gNB and AMF shall be supported.

Proposal 5: Introduce a transaction ID outside of the Transfer IEs to differentiate parallel Inventory report procedures.

Proposal 6: Introduce a transaction ID outside of the Transfer IEs to differentiate parallel Inventory request procedures.

  - Other small issues

For the following FFS, we provide our view directly.

- It is FFS whether the new A-IoT Area is encoded as GNSS info or new A-IoT Area ID.

Proposal 7: No need to encode A-IoT area as GNSS info.

It is FFS whether the Inventory Assistance Information includes the “Estimate of Expected D2R Message Size”

Proposal 8: It is out of RAN3 scope, pending input from other groups.

- It is FFS whether the Command Assistance Information includes the Command Type (read, write, disable…)

Proposal 9: It is out of RAN3 scope, pending input from other groups.

- Introduce per-session per-device Device Association (“RAN NGAP Device ID”, and “(FFS) CN NGAP Device ID” pair)

Proposal 10: No need to introduce CN NGAP Device ID” pair

  - Reader selection

Architecture aspects:

- AIoT RAN node is an aggregated gNB. And this gNB may or may not only provide the A-IoT radio.

- AIOTF may be aware of the location of reader via OAM configuration.

- FFS: The details of the location of reader (e.g., room number, GNSS info) are FFS, and it is FFS on signaling support.

- FFS: To support selection of RAN node by AIOTF, define a new A-IoT Area, or use Tracking Area dedicated for A-IoT?

- Upon receiving only the area in Inventory Request, the gNB selects readers within the indicated area.

- Upon receiving neither the area nor the reader list in Inventory Request, the gNB selects all the served readers.

- WA: Upon receiving only the reader list in Inventory Request, the gNB selects the readers indicated by the reader list.

- FFS: Upon receiving both the area and the reader list in Inventory Request, the gNB selects the readers within the indicated area and the readers within the reader list.

According to above cases, if the AIoT CN does not provide AIoT area in the Inventory request message, the AIoT area can be seen as the gNB’s serving area. If the AIoT CN provides area, the AIoT areas shall be full or part of the gNB’s serving area. On the contrary, if AIoT CN does not provide AIoT area, the gNB will use its serving area as AIoT area by default.

Proposal 11: If not providing area from AIoT CN, the gNB will use its serving area as AIoT area.

For instance, 4 readers (reader 1, 2, 3, 4) are deployed in the gNB serving area (grey color), 3 readers (reader 1, 2, 3) are deployed in the area provided by AIoT CN (green color), and 2 readers (reader 1 and reader 2) are suggested by AIoT CN.

- Case 1: Neither providing the area nor providing reader list

It is evident and agreed that the gNB can select reader(s) from 4 readers within the gNB serving area.

- Case 2: Providing the area but not providing reader list

It is evident and agreed that the gNB can select reader(s) from 3 readers within the area provided by AIoT CN.

- Case 3: Not providing the area but providing reader list (4 readers in the area, but 2 readers suggested)

The gNB will use its serving area as AIoT area, i.e., 4 readers in the area. However, it is strange if AIoT CN suggests 2 from 4 readers.

- Case 4: Providing both the area and reader list (3 readers in the area, but 2 readers suggested)

It is strange why AIoT CN indicates 2 from 3 readers other than all of 3 readers, perhaps due to the latest association between reader and devices in the last Inventory report.

For above four cases, it is clear for both Case 1 and Case 2. However, it is worth discussing how the gNB selects readers within the reader list in Case 3 and Case 4.

In our view, the intention of providing suggested reader(s) from AIoT CN is to help gNB to select suitable reader performing Inventory procedure with devices. Accurately, the AIoT CN mainly focuses on the Inventory result but is not care of which reader to provide it.

Observation 3: AIoT CN focuses on the Inventory result other than which reader to provide it.

From gNB side, upon receiving Inventory request message, it shall select suitable reader within either the area indicated by AIoT CN or gNB serving area by default. If receiving reader list provided by AIoT CN, the gNB can use it as suggested readers, it means that the gNB can based on the radio resources, select other reader out of reader list but within the area.

Proposal 12: Upon receiving reader list in Inventory Request message, no matter whether receiving the area in this message, the gNB uses the reader list as reference, i.e., it can select reader either in or out of the reader list within the area.

  - Removal of service type information

The following is abstracted from SA2 incoming LS “R3-253016”.

1 Overall description SA2 has been discussing whether “Service Type” (i.e., Inventory or Command) information in “Assistance Information” provided by the AIOTF to the NG-RAN is still needed, considering the fact that RAN3 has agreed to use separate NGAP procedures for Inventory and Command respectively. SA2 would like to ask RAN2 and RAN3 whether there is any concern if SA2 agrees to remove “Service Type” from “Assistance Information”. 2 Actions To RAN2 and RAN3: ACTION: SA2 kindly asks RAN2 and RAN3 to provide feedback.

As indicated that “RAN3 has agreed to use separate NGAP procedures for Inventory and Command respectively”, from RAN3 point of view, there is no issue left.

Proposal 13: RAN3 has no concern that SA2 agrees to remove “Service Type” from “Assistance Information”.

- Conclusion

Based on the above analysis, we provide the following observation and proposals.\

Proposal 1: Option 3 (i.e., the supported A-IoT area is defined by reader list) is excluded.

Proposal 2: Define the supported AIoT area by a new type of area.

Proposal 3: One reader can be part of one supported AIoT area or multiple supported A-IoT areas.

Proposal 4: Introduce a transaction ID outside of the Transfer IEs to differentiate parallel command procedure.

Observation 1: The parallel Inventory report procedures for different devices between gNB and AMF shall be supported.

Observation 2: The parallel Inventory request procedures for different readers within the same gNB between gNB and AMF shall be supported.

Proposal 5: Introduce a transaction ID outside of the Transfer IEs to differentiate parallel Inventory report procedures.

Proposal 6: Introduce a transaction ID outside of the Transfer IEs to differentiate parallel Inventory request procedures.

- It is FFS whether the new A-IoT Area is encoded as GNSS info or new A-IoT Area ID.

Proposal 7: No need to encode A-IoT area as GNSS info.

It is FFS whether the Inventory Assistance Information includes the “Estimate of Expected D2R Message Size”

Proposal 8: It is out of RAN3 scope, pending input from other groups.

- It is FFS whether the Command Assistance Information includes the Command Type (read, write, disable…)

Proposal 9: It is out of RAN3 scope, pending input from other groups.

- Introduce per-session per-device Device Association (“RAN NGAP Device ID”, and “(FFS) CN NGAP Device ID” pair)

Proposal 10: No need to introduce CN NGAP Device ID” pair

Proposal 11: If not providing area from AIoT CN, the gNB will use its serving area as AIoT area.

Observation 3: AIoT CN focuses on the Inventory result other than which reader to provide it.

Proposal 12: Upon receiving reader list in Inventory Request message, no matter whether receiving the area in this message, the gNB uses the reader list as reference, i.e., it can select reader either in or out of the reader list within the area.

Proposal 13: RAN3 has no concern that SA2 agrees to remove “Service Type” from “Assistance Information”.

We also provide the corresponding TPs and draft reply LS (to R3-253016) in the corresponding paper R3-253246.