# 3GPP TSG-RAN WG3 Meeting #128	R3-25xxxx

# Malta, MT, 19th – 23th May 2025

Title: 	(TP for BL CR 38.413) New A-IoT procedures

Source: 	ZTE Corporation, China Telecom

Agenda item:	16.2

Document for:	Discussion and Decision

- Introduction

In the RAN3 #127bis meeting, we have achieved much progress.

This paper addresses the following issue.

FFS on the Release/Cancel/End related procedure(s)/IE(s) for the A-IoT service/inventory/command.

- Discussion

  - A-IoT service cancel procedure

RAN2 has agreed that a certain device can perform Inventory procedures with multiple readers.

5. RAN2 acknowledges that multi-reader scenario may exist but we will not specify something specific for this purpose.  We can rely on transaction ID and implementation to handle it.

For example, if A-IoT CN is not sure that a group of devices are associated with which reader, it can send multiple Inventory request messages to the several readers in parallel or in sequence. If A-IoT CN receives Inventory report message including inventoried devices from one reader, it acknowledges that this group of devices are served by this reader, then it can cancel other ongoing Inventory procedures with other readers.

For the Inventory and command procedure, for the similar reason, if A-IoT CN receives Inventory report message from one reader, it can cancel other Inventory and command procedure with other readers.

Proposal 1: Introduce A-IoT service cancel procedure to cancel ongoing Inventory or command procedure.

For an ongoing Inventory procedure, if the reader receives Inventory cancel request message from A-IoT CN, it shall stop sending A-IoT paging message and release A-IoT resources for this Inventory procedure.

Since there may be multiple ongoing Inventory/command procedures, the A-IoT CN shall include the indicated Correlation ID for the Inventory procedure to be cancelled.

Proposal 2: A-IoT service cancel procedure shall include the same correlation ID with the to-be cancelled Inventory/command procedure.

Because both reader and gNB are allowed to aggregate multiple inventoried devices into one Inventory report, upon reception of Inventory cancel request message from A-IoT CN, the gNB can also send the Inventory report message including inventoried device IDs to the A-IoT CN.

Proposal 3: The gNB can send Inventory report message if any, even if the corresponding Inventory procedure is cancelled.

For the Inventory and command procedure, the A-IoT CN can cancel follow-on command procedure to cancel one or more inventoried devices.

Proposal 4: The A-IoT service cancel procedure shall include the RAN A-IoT Device NGAP ID list to indicate that the command procedure for these inventoried devices will be cancelled.

  - A-IoT reader Status Reporting procedure

SA2 spec has capture the following content.

The AIOTF selects AIoT RAN node(s) and, optionally, a list of the RAN readers. The AIOTF selects the AIoT RAN node(s) based on the internal area information.

The AIOTF may have AIoT RAN information and the optional RAN reader information. For direct path, the AIOTF obtains the information from AIoT RAN by the NGAP or OAM configuration.

It means that the AIOTF can provide reader list to the RAN node. So far, AIOTF provides reader list according to the stored association between reader and devices by receiving Inventory report message. Besides, AIOTF may acquire other assistant information from RAN node, for reader selection.

RAN node can provide reader status reporting to the AIOTF, e.g., workable or not workable, load status.

Proposal 5: A-IoT CN can request RAN node to provide reader status information.

Proposal 6: Introduce A-IoT reader Status Reporting procedure.

- Conclusion

Based on the above analysis, we provide the following proposals.

Proposal 1: Introduce A-IoT service cancel procedure to cancel ongoing Inventory or command procedure.

Proposal 2: A-IoT service cancel procedure shall include the same correlation ID with the to-be cancelled Inventory/command procedure.

Proposal 3: The gNB can send Inventory report message if any, even if the corresponding Inventory procedure is cancelled.

Proposal 4: The A-IoT service cancel procedure shall include the RAN A-IoT Device NGAP ID list to indicate that the command procedure for these inventoried devices will be cancelled.

Proposal 5: A-IoT CN can request RAN node to provide reader status information.

Proposal 6: Introduce A-IoT reader Status Reporting procedure..

- TP to TS38.413

-----------------Start of the Change-----------------

8.1	List of NGAP Elementary Procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 8.1-1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| AMF Configuration Update | AMF CONFIGURATION UPDATE | AMF CONFIGURATION UPDATE ACKNOWLEDGE | AMF CONFIGURATION UPDATE FAILURE |
| RAN Configuration Update | RAN CONFIGURATION UPDATE | RAN CONFIGURATION UPDATE ACKNOWLEDGE | RAN CONFIGURATION UPDATE FAILURE |
| Handover Cancellation | HANDOVER CANCEL | HANDOVER CANCEL ACKNOWLEDGE |  |
| Handover Preparation | HANDOVER REQUIRED | HANDOVER COMMAND | HANDOVER PREPARATION FAILURE |
| Handover Resource Allocation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER FAILURE |
| Initial Context Setup | INITIAL CONTEXT SETUP REQUEST | INITIAL CONTEXT SETUP RESPONSE | INITIAL CONTEXT SETUP FAILURE |
| NG Reset | NG RESET | NG RESET ACKNOWLEDGE |  |
| NG Setup | NG SETUP REQUEST | NG SETUP RESPONSE | NG SETUP FAILURE |
| Path Switch Request | PATH SWITCH REQUEST | PATH SWITCH REQUEST ACKNOWLEDGE | PATH SWITCH REQUEST FAILURE |
| PDU Session Resource Modify | PDU SESSION RESOURCE MODIFY REQUEST | PDU SESSION RESOURCE MODIFY RESPONSE |  |
| PDU Session Resource Modify Indication | PDU SESSION RESOURCE MODIFY INDICATION | PDU SESSION RESOURCE MODIFY CONFIRM |  |
| PDU Session Resource Release | PDU SESSION RESOURCE RELEASE COMMAND | PDU SESSION RESOURCE RELEASE RESPONSE |  |
| PDU Session Resource Setup | PDU SESSION RESOURCE SETUP REQUEST | PDU SESSION RESOURCE SETUP RESPONSE |  |
| UE Context Modification | UE CONTEXT MODIFICATION REQUEST | UE CONTEXT MODIFICATION RESPONSE | UE CONTEXT MODIFICATION FAILURE |
| UE Context Release | UE CONTEXT RELEASE COMMAND | UE CONTEXT RELEASE COMPLETE |  |
| Write-Replace Warning | WRITE-REPLACE WARNING REQUEST | WRITE-REPLACE WARNING RESPONSE |  |
| PWS Cancel | PWS CANCEL REQUEST | PWS CANCEL RESPONSE |  |
| UE Radio Capability Check | UE RADIO CAPABILITY CHECK REQUEST | UE RADIO CAPABILITY CHECK RESPONSE |  |
| UE Context Suspend | UE CONTEXT SUSPEND REQUEST | UE CONTEXT SUSPEND RESPONSE | UE CONTEXT SUSPEND FAILURE |
| UE Context Resume | UE CONTEXT RESUME REQUEST | UE CONTEXT RESUME RESPONSE | UE CONTEXT RESUME FAILURE |
| UE Radio Capability ID Mapping | UE RADIO CAPABILITY ID MAPPING REQUEST | UE RADIO CAPABILITY ID MAPPING RESPONSE |  |
| Broadcast Session Setup | BROADCAST SESSION SETUP REQUEST | BROADCAST SESSION SETUP RESPONSE | BROADCAST SESSION SETUP FAILURE |
| Broadcast Session Modification | BROADCAST SESSION MODIFICATION REQUEST | BROADCAST SESSION MODIFICATION RESPONSE | BROADCAST SESSION MODIFICATION FAILURE |
| Broadcast Session Release | BROADCAST SESSION RELEASE REQUEST | BROADCAST SESSION RELEASE RESPONSE |  |
| Distribution Setup | DISTRIBUTION SETUP REQUEST | DISTRIBUTION SETUP RESPONSE | DISTRIBUTION SETUP FAILURE |
| Distribution Release | DISTRIBUTION RELEASE REQUEST | DISTRIBUTION RELEASE RESPONSE |  |
| Multicast Session Activation | MULTICAST SESSION ACTIVATION REQUEST | MULTICAST SESSION ACTIVATION RESPONSE | MULTICAST SESSION ACTIVATION FAILURE |
| Multicast Session Deactivation | MULTICAST SESSION DEACTIVATION REQUEST | MULTICAST SESSION DEACTIVATION RESPONSE |  |
| Multicast Session Update | MULTICAST SESSION UPDATE REQUEST | MULTICAST SESSION UPDATE RESPONSE | MULTICAST SESSION UPDATE FAILURE |
| Timing Synchronisation Status | TIMING SYNCHRONISATION STATUS REQUEST | TIMING SYNCHRONISATION STATUS RESPONSE | TIMING SYNCHRONISATION STATUS FAILURE |
| MT Communication Handling | MT COMMUNICATION HANDLING REQUEST | MT COMMUNICATION HANDLING RESPONSE | MT COMMUNICATION HANDLING FAILURE |
| Broadcast Session Transport | BROADCAST SESSION TRANSPORT REQUEST | BROADCAST SESSION TRANSPORT RESPONSE | BROADCAST SESSION TRANSPORT FAILURE |
| Inventory Request | INVENTORY REQUEST | INVENTORY RESPONSE | INVENTORY FAILURE |
| Command Request | COMMAND REQUEST | COMMAND RESPONSE | COMMAND FAILURE |
| A-IoT Service Cancellation | A-IOT SERVICE CANCEL | A-IOT SERVICE CANCEL ACKNOWLEDGE |  |
| A-IoT reader Status | A-IOT READER STATUS REQUEST | A-IOT READER STATUS RESPONSE | A-IOT READER STATUS FAILURE |

Table 8.1-2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Downlink RAN Configuration Transfer | DOWNLINK RAN CONFIGURATION TRANSFER |
| Downlink RAN Status Transfer | DOWNLINK RAN STATUS TRANSFER |
| Downlink NAS Transport | DOWNLINK NAS TRANSPORT |
| Error Indication | ERROR INDICATION |
| Uplink RAN Configuration Transfer | UPLINK RAN CONFIGURATION TRANSFER |
| Uplink RAN Status Transfer | UPLINK RAN STATUS TRANSFER |
| Handover Notification | HANDOVER NOTIFY |
| Initial UE Message | INITIAL UE MESSAGE |
| NAS Non Delivery Indication | NAS NON DELIVERY INDICATION |
| Paging | PAGING |
| PDU Session Resource Notify | PDU SESSION RESOURCE NOTIFY |
| Reroute NAS Request | REROUTE NAS REQUEST |
| UE Context Release Request | UE CONTEXT RELEASE REQUEST |
| Uplink NAS Transport | UPLINK NAS TRANSPORT |
| AMF Status Indication | AMF STATUS INDICATION |
| PWS Restart Indication | PWS RESTART INDICATION |
| PWS Failure Indication | PWS FAILURE INDICATION |
| Downlink UE Associated NRPPa Transport | DOWNLINK UE ASSOCIATED NRPPA TRANSPORT |
| Uplink UE Associated NRPPa Transport | UPLINK UE ASSOCIATED NRPPA TRANSPORT |
| Downlink Non UE Associated NRPPa Transport | DOWNLINK NON UE ASSOCIATED NRPPA TRANSPORT |
| Uplink Non UE Associated NRPPa Transport | UPLINK NON UE ASSOCIATED NRPPA TRANSPORT |
| Trace Start | TRACE START |
| Trace Failure Indication | TRACE FAILURE INDICATION |
| Deactivate Trace | DEACTIVATE TRACE |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| Location Reporting Control | LOCATION REPORTING CONTROL |
| Location Reporting Failure Indication | LOCATION REPORTING FAILURE INDICATION |
| Location Report | LOCATION REPORT |
| UE TNLA Binding Release | UE TNLA BINDING RELEASE REQUEST |
| UE Radio Capability Info Indication | UE RADIO CAPABILITY INFO INDICATION |
| RRC Inactive Transition Report | RRC INACTIVE TRANSITION REPORT |
| Overload Start | OVERLOAD START |
| Overload Stop | OVERLOAD STOP |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Uplink RIM Information Transfer | UPLINK RIM INFORMATION TRANSFER |
| Downlink RIM Information Transfer | DOWNLINK RIM INFORMATION TRANSFER |
| Retrieve UE Information | RETRIEVE UE INFORMATION |
| UE Information Transfer | UE INFORMATION TRANSFER |
| RAN CP Relocation Indication | RAN CP RELOCATION INDICATION |
| Connection Establishment Indication | CONNECTION ESTABLISHMENT INDICATION |
| AMF CP Relocation Indication | AMF CP RELOCATION INDICATION |
| Handover Success | HANDOVER SUCCESS |
| Uplink RAN Early Status Transfer | UPLINK RAN EARLY STATUS TRANSFER |
| Downlink RAN Early Status Transfer | DOWNLINK RAN EARLY STATUS TRANSFER |
| Multicast Group Paging | MULTICAST GROUP PAGING |
| Broadcast Session Release Required | BROADCAST SESSION RELEASE REQUIRED |
| Timing Synchronisation Status Report | TIMING SYNCHRONISATION STATUS REPORT |
| RAN Paging Request | RAN PAGING REQUEST |
| Inventory Report | INVENTORY REPORT |
| A-IoT reader Status Report | A-IOT READER STATUS REPORT |

-----------------Start of the Next  Change-----------------

8.xx.a	A-IoT Service Cancellation

8.xx.a.1	General

The purpose of the A-IoT Service Cancellation procedure is to enable an A-IoT CN to cancel an ongoing A-IoT Service procedure. The procedure uses non-UE associated signalling.

8.xx.a.2	Successful Operation

Figure 8.xx.a.2-1: A-IoT cancel: successful operation

The A-IoT CN initiates the procedure by sending an A-IOT SERVICE CANCEL message to the NG-RAN node.

8.xx.a.3	Unsuccessful Operation

Not applicable.

8.xx.a..4	Abnormal Conditions

Void.

-----------------Start of the Next  Change-----------------

8.xx.b	A-IoT reader Status

8.xx.b.1	General

The purpose of the A-IoT reader Status procedure is to enable the A-IoT CN to request the NG-RAN node to start or stop reporting of A-IoT reader status information as specified in TS 23.501 [9] and TS 23.502 [10]. The procedure uses non-UE associated signalling.

8.xx.b.2	Successful Operation

Figure 8.xx.b.2-1: A-IOT READER status procedure: successful operation

The A-IoT CN initiates the procedure by sending an A-IOT READER STATUS REQUEST message to the NG-RAN node.

8.xx.b.3	Unsuccessful Operation

Figure 8.xx.b.3-1: Timing synchronisation status procedure: unsuccessful operation

If the NG-RAN node is not able to report A-IoT reader status, it shall consider the procedure as failed and reply with the A-IOT READER STATUS FAILURE message.

8.xx.b.4	Abnormal Conditions

Void.

8.xx.c	A-IoT reader Status Report

8.xx.c.1	General

The purpose of the A-IoT reader Status Report procedure is to enable the NG-RAN node to provide A-IoT reader status information to the A-IoT CN as specified in TS 23.501 [9] and TS 23.502 [10]. The procedure uses non-UE associated signalling.

8.xx.c.2	Successful Operation

Figure 8.xx.c.2-1: Timing synchronisation status report

The NG-RAN node initiates the procedure by sending an A-IOT READER STATUS REPORT message to the A-IoT CN.

8.xx.c.3	Abnormal Conditions

Void.

-----------------Start of the Next  Change-----------------

9.2.xx.a	A-IoT SERVICE CANCEL

This message is sent by the A-IoT CN to the NG-RAN node to request the cancellation of an ongoing A-IoT service procedure.

Direction: A-IoT CN  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AIOTF Identifier | M |  | 9.3.3.xx2 |  | YES | reject |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  | YES | reject |
| A-IoT Service Cancel Transfer | M |  | OCTET STRING | Containing the A-IoT Service Cancel Transfer IE specified in subclause 9.3.x.x | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |

9.2.xx.b	A-IoT SERVICE CANCEL ACKNOWLEDGE

This message is sent by the NG-RAN node to the A-IoT CN to confirm that the ongoing A-IoT service procedure was cancelled.

Direction: AMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AIOTF Identifier | M |  | 9.3.3.xx2 |  | YES | reject |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  | YES | reject |
| A-IoT Service Cancel Acknowledge Transfer | M |  | OCTET STRING | Containing the A-IoT Service Cancel Acknowledge Transfer IE specified in subclause 9.3.x.x | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

9.2.xx.c	A-IOT READER STATUS REQUEST

This message is sent by the A-IoT CN to request the NG-RAN node to start or stop reporting of A-IoT reader status information.

Direction: A-IoT CN  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AIOTF Identifier | M |  | 9.3.3.xx2 |  | YES | reject |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  | YES | reject |
| Reader Status Request Transfer | M |  | OCTET STRING | Containing the Reader Status Request Transfer IE specified in subclause 9.3.x.x | YES | reject |

9.2.xx.d	A-IOT READER STATUS RESPONSE

This message is sent by the NG-RAN node to confirm the request to start or stop reporting of A-IoT reader status information.

Direction: NG-RAN node  A-IoT CN

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| AIOTF Identifier | M |  | 9.3.3.xx2 |  | YES | reject |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  | YES | reject |
| Reader Status Response Transfer | M |  | OCTET STRING | Containing the Reader Status Response Transfer IE specified in subclause 9.3.x.x | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

9.2.xx.e	A-IOT READER STATUS FAILURE

This message is sent by the NG-RAN node to indicate that reporting of A-IoT reader status information cannot be initiated.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| AIOTF Identifier | M |  | 9.3.3.xx2 |  | YES | reject |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  | YES | reject |
| Reader Status Failure Transfer | M |  | OCTET STRING | Containing the Reader Status Failure Transfer IE specified in subclause 9.3.x.x | YES | ignore |
| Cause | M[FFS] |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

9.2.xx.f	A-IOT READER STATUS REPORT

This message is sent by the NG-RAN node to report previously requested A-IoT reader status information.

Direction: NG-RAN node  A-IoT CN

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| AIOTF Identifier | M |  | 9.3.3.xx2 |  | YES | reject |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  | YES | reject |
| Reader Status Report Transfer | M |  | OCTET STRING | Containing the Reader Status Report Transfer IE specified in subclause 9.3.x.x | YES | reject |

-----------------Start of the Next  Change-----------------

9.3.x.xx	A-IoT Service Cancel Transfer

This IE provides the A-IoT Service Cancel related information from the AIOTF to the NG-RAN node.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  | YES | reject |
| A-IoT Service type | M |  | ENUMERATED (Inventory, Command, ...) |  | YES | reject |
| Device List |  | 0..1 |  | This IE is present if A-IoT Service type is Command. |  |  |
| >Device Item |  | 1..<maxnoofDevices> |  |  |  |  |
| >>RAN A-IoT Device NGAP ID | O |  | 9.3.3.yy1 |  |  |  |

-----------------END of the Change-----------------