# 3GPP TSG-RAN WG3 Meeting #128	R3-253249

# Malta, MT, 19th – 23th May 2025

# Title: 	(TP for 38.423, 38.473, 38.401, 38.300) Inter-CU LTM

# Source: 	ZTE Corporation

Agenda item:	13.2

# Document for:	Discussion and Decision

- Introduction

In the RAN3 #127bis meeting, we have achieved the following progress.

Reuse the LTM configuration update procedure to transfer UE’s 5G security capabilities and/or UE’s UP security policy from the new serving gNB to the other candidate gNBs in subsequent LTM. The new NCC value needs to be sent from gNB-CU to the gNB-DU via UE context modification procedure. Introduce two new Class-1 UE-associated procedures over F1AP to support the source gNB-DU to trigger the activation/deactivation of the SP CSI-RS transmission in the candidate cell(s). Introduce a new class-1 UE-associated procedures over XnAP for the SP CSI-RS activation/deactivation. For the initial inter-CU LTM preparation, the source gNB computes the KgNB*(s) (per candidate cell) for the candidate gNB, and forwards the {KgNB*, NCC} pair(s) to the candidate gNB via a Handover Request message as legacy. Turn the WA into agreement: Introduce a new non-UE associated class 2 procedure on Xn, namely TA information Transfer message, to transfer the TA information from the candidate gNB-CU to the source gNB-CU. Include the following information in the TA information transfer message: Candidate Cell ID TA value Preamble index RA-RNTI FFS on the XnAP IDs. Turn the WA into agreement: Use a single UE association (e.g. identified by a pair of {Source NG-RAN node UE XnAP ID IE and Target NG-RAN node UE XnAP ID IE}) for multiple LTM handover request to the same candidate gNB. For subsequent LTM, the candidate gNB uses the old target UE XnAP ID (the target UE AP ID it previously allocated between it and the old serving gNB) to identify the UE context when receiving the LTM Configuration Update message from the new serving gNB. FFS on how to deliver the old target UE XnAP ID(s) to the new serving gNB. Data Forwarding Information IE is a list including multiple data forwarding addresses from each candidate gNB(s). Include the following information in SN Change Required (Source SN to MN) message: LTM request indication (explicit way) Suggested PScell list (implicit way in INM, FFS on explicit way) Measurement results (implicit way in INM) Upper limit number of PScell(s) for each candidate SN (FFS) CSI Resource configuration request (explicit way) Early sync information request (explicit way) Include the following information in SN Addition Request (MN to Candidate SN) message: LTM request indication (explicit way) Suggested PScell list (implicit way in INM, FFS on explicit way) Measurement results (implicit way in INM) Upper limit number of PScell(s) for the candidate SN (explicit way) CSI Resource configuration request (explicit way) Early sync information request (explicit way) It is the source SN to generate the LTM configuration ID mapping list for the accepted candidate PSCell(s), and send it to the MN for subsequent LTM via SN modification request acknowledge message. Then the MN forwards the LTM configuration ID mapping list to the candidate SN via SN Modification Request message. For SN-initiated inter-CU SCG LTM, the source SN can provide the SCG reference configuration via SN Change Required message. If the source SN does not provide the SCG reference configuration, MN can request SCG reference configuration from one of the candidate SN(s), FFS on whether MN can request SCG reference configuration from the source SN. The TA information transfer message can be reused and enhanced to transfer the TA information from the candidate SN to the source SN via the MN. Include the target PSCell ID and corresponding TCI State ID(s) in the Cell Switch Notification message. Continue to work on the solutions comparison and details…

RAN2 also has achieved the following progress in RAN2 #129bis meeting.

Agreements on inter-CU LTM

- For security key update in inter-CU LTM, RAN2 agree to include actual NCC value in the LTM cell switch command MAC CE.

- NCC is included in the LTM cell switch command MAC CE if the Rel-19 set ID is different between the target cell and source cell. Conversely, if the Rel-19 set ID is same for both cells, the NCC will not be included.

- NW configures the corresponding sk-Counter in all LTM candidate configurations, and UE uses the configured value for generating the SN key when security key update is performed in MCG.

- From RAN2 point of view, UE-based TA measurement can be also applicable for the synchronized inter-CU LTM scenario, and no additional RAN2 specification impact is expected. Final decision is up to RAN4.

- MIMO 2TA can be configured with inter-CU LTM using PDCCH-order based TA measurement.

- If UE-based TA measurement is supported for inter-CU LTM, it cannot be configured if candidate cell is configured with MIMO 2TA.

- An LTM target cell will not apply cell DTX/DRX until LTM cell switch is completed. RAN2 will not consider any specification impacts needed for LTM RACH-less cell switch to an NES candidate cell.

This paper further discusses the LTM based on the above progress.

- Discussion

  - Semi-Persistent CSI-RS

Introduce two new Class-1 UE-associated procedures over F1AP to support the source gNB-DU to trigger the activation/deactivation of the SP CSI-RS transmission in the candidate cell(s).

Introduce a new class-1 UE-associated procedures over XnAP for the SP CSI-RS activation/deactivation.

Currently, RAN2 is still discussing the detail of SP CSI-RS activation/deactivation.

Proposal 1: Postpone the detail of SP CSI-RS activation/deactivation.

  - Security Key change

Reuse the LTM configuration update procedure to transfer UE’s 5G security capabilities and/or UE’s UP security policy from the new serving gNB to the other candidate gNBs in subsequent LTM.

The new NCC value needs to be sent from gNB-CU to the gNB-DU via UE context modification procedure.

For the initial inter-CU LTM preparation, the source gNB computes the KgNB*(s) (per candidate cell) for the candidate gNB, and forwards the {KgNB*, NCC} pair(s) to the candidate gNB via a Handover Request message as legacy.

The new source gNB needs to provide the new KgNB*(s) to the corresponding candidate gNB via a LTM configuration update procedure. FFS on the procedure design either per cell or per gNB.

Whether to define additional information from CU to DU to deliver NCC value.

In legacy RRC based HO, the Security generation and transfer is per cell, so that, LTM procedure shall reuse the same mechanism.

Proposal 2: The new source gNB needs to provide per cell new KgNB*(s) to the corresponding candidate gNB via a LTM configuration update procedure.

RAN2 also sent an LS “LS on RAN2 agreements for security key handling in inter-CU LTM” to RAN3 in R3-253009, as below.

1 Overall description Regarding the security key handling in Rel-19 Inter-CU LTM, RAN2 made the following agreements at RAN2#129bis meeting: Agreements: - For security key update in inter-CU LTM, RAN2 agree to include actual NCC value in the LTM cell switch command MAC CE. - NCC is included in the LTM cell switch command MAC CE if the Rel-19 set ID is different between the target cell and source cell. Conversely, if the Rel-19 set ID is same for both cells, the NCC will not be included. - NW configures the corresponding sk-Counter in all LTM candidate configurations, and UE uses the configured value for generating the SN key when security key update is performed in MCG. - A list of sk-counters is linked to a Rel-19 set ID configured by the SN. - At RLF and reconfiguration with sync failure: - If RLF: - If the selected candidate cell has the same Rel-19 set ID as source (no security key change), the UE performs fast failure recovery (same as in Rel-18). - If reconfiguration failure (inter-CU LTM): - If the selected candidate cell has the same Rel-19 set ID as target, the UE performs fast failure recovery. FFS if fast failure recovery with different Rel-19 set IDs is allowed. - The indication on whether to allow or not the SN to configure an inter-SN candidate is included in the inter-node RRC message.. The term “Rel-19 set ID” refers to the “LTM no security change ID” configured to the UE for serving cell and candidate cells separately. RAN2 assumes that even for intra-CU LTM, the network is allowed to configure different Rel-19 set IDs for different cells (including serving and/or intra-CU candidate cells) if the network wants to trigger security key update. From UE perspective, security key update is performed when the Rel-19 set IDs are different between the serving cell and the target cell, whether it is intra-CU or inter-CU LTM can be transparent to the UE.

According to the RAN2 LS in R3-253009, in the candidate gNB, due to e.g., CU-UP split case, some candidate cells are belonged to the same LTM set in which LTM no security is changed at these candidate cells, some candidate cell are belonged to the different LTM sets.

It is the candidate gNB-CU to configure the candidate cells into the same or different LTM set.

Observation 1: In one candidate gNB, candidate cells can be belonged to the same or different LTM set, in which LTM no security is changed at these candidate cells.

Observation 2: The candidate cells do not need to change security Key, if they are belonged to the same LTM set.

Figure 1: Common Rel-19 set ID list generation and transfer

In this figure 1, in step 1/2, source gNB prepares multiple candidate gNBs, each candidate gNB includes multiple candidate cells. Candidate PCell-1 and PCell-2 are belonged to one LTE set ID 1 and candidate PCell-3 is belonged to another LTE set ID 2.

Proposal 3: The candidate gNB sends per cell LTM set ID to the source gNB, via Handover request ACK message.

In the RAN 2 LS R3-253009, the term “Rel-19 set ID” refers to the “LTM no security change ID” configured to the UE for serving cell and candidate cells separately. So, the gNB-CU shall generate a common Rel-19 set ID list for serving cell and all of candidate cells.

In this figure 1, in step 3/4, after receiving HO request messages from multiple candidate gNB, the source gNB-CU generates the common Rel-19 set ID list based on the received gNB level’s LTM set ID.

Common Rel-19 set ID list:

Rel-19 set ID 1 {Pcell-1, Pcell-2}, Candidate gNB 1

Rel-19 set ID 2 {Pcell-3}, Candidate gNB 1

Rel-19 set ID 3 {PCell-4, PCell-5, Pcell-6}}, Candidate gNB 2

Rel-19 set ID 4 {…}, Candidate gNB 0 (i.e., source gNB)

…

Proposal 4: The source gNB-CU sends the Rel-19 set ID list and NCC to the source gNB-DU via F1: UE context modification request message, each Rel-19 set ID includes serving cell and/or candidate cells.

In step 5/6/7, during source gNB-DU performs LTM switch, the source gNB-CU shall sends the target PCell ID and the common Rel-19 set ID list to the target gNB by Xn: Cell Switch Notification message.

Proposal 5: The source gNB sends Rel-19 set ID list to the target gNB via Xn: Cell Switch Notification message.

In step 8, the target gNB-CU sends F1: CU-DU Cell Switch Notification message to the target gNB-DU, including Target Cell ID.

In step 9/10, after Path Switch procedure, the target gNB-CU receives new {NH, NCC}, then it sends new NCC and common Rel-19 set ID list to the target gNB-DU.

Proposal 6: The target gNB-CU sends Rel-19 set ID list and NCC to the target gNB-DU via F1: UE context modification request message, each Rel-19 set ID.

  - TA information transfer

Turn the WA into agreement: Introduce a new non-UE associated class 2 procedure on Xn, namely TA information Transfer message, to transfer the TA information from the candidate gNB-CU to the source gNB-CU.

Include the following information in the TA information transfer message:

Candidate Cell ID

TA value

Preamble index

RA-RNTI

FFS on the XnAP IDs.

Since we have agreed to use non-UE associated class 2 procedure, meanwhile, it is difficult for candidate cell to identify which UE performing early synch, so that we propose to remove the XnAP IDs.

Proposal 7: Remove the “FFS on the XnAP IDs” from Xn: TA information transfer message.

  - UE context Transfer

For subsequent LTM, the candidate gNB uses the old target UE XnAP ID (the target UE AP ID it previously allocated between it and the old serving gNB) to identify the UE context when receiving the LTM Configuration Update message from the new serving gNB. FFS on how to deliver the old target UE XnAP ID(s) to the new serving gNB.

Figure 2: UE context reference/UE context transfer

In step 1, for each of target nodes, the source node initiates the Handover procedure by sending the HANDOVER REQUEST message including IE “source node UE XnAP ID reference” to the target node.

If Handover preparation procedure is successful operation, the target node responds HANDOVER REQUEST ACKNOWLEDGE message including a pair of IEs, i.e., {source node UE XnAP ID, target node UE XnAP ID} to the source node, both source node and target node shall allocate and store a dedicated pair of {source node UE XnAP ID, Target node UE XnAP ID}. Both source node and target node shall and shall establish UE context, and source node may store the established UE context of this UE, the target node shall store the established UE context of this UE.

Observation 3: During LTM preparation procedure, the source node and target node establishes a pair of {source node UE XnAP ID, target node UE XnAP ID}.

Observation 4: During LTM preparation procedure, the source node and target node establishes UE context.

In the TS37.340, in 10.7	Inter-Master Node handover with/without Secondary Node change

1.	The source MN starts the handover procedure by initiating the Xn Handover Preparation procedure including both MCG and SCG configuration. The source MN includes the source SN UE XnAP ID, SN ID and the UE context in the source SN in the Handover Request message.

NOTE 2:	The source MN may trigger the MN-initiated SN Modification procedure (to the source SN) to retrieve the current SCG configuration and the QMC configuration information managed by the SN and to allow provision of data forwarding related information before step 1.

2.	If the target MN decides to keep the UE context in source SN, the target MN sends SN Addition Request to the SN including the SN UE XnAP ID as a reference to the UE context in the SN that was established by the source MN. If the target MN decides to change the SN allowing delta configuration, the target MN sends the SN Addition Request to the target SN including the UE context in the source SN that was established by the source MN. Otherwise, the target MN may send the SN Addition Request to the target SN including neither the SN UE XnAP ID nor the UE context in the source SN that was established by the source MN.

In case that the source node transfers the UE context reference/UE context to the selected target node, the above mechanism can be reused.

Figure 3: Reuse the same mechanism with Inter MN HO without SN change

So, in step 2, if source node decides to cell switch to the target node A, it sends Xn: cell switch notification message which may include UE context reference/UE context in target B and target C. Refer to legacy, the UE context can be also sent from source node to the selected target node via Inter-node RRC container.

Source node -> selected target node A: Xn Cell Switch Notification message, including:

UE context reference/UE context for target node B

UE context reference/UE context for target node C

Proposal 8: The source node can send UE context reference/UE context at other candidate nodes to the selected target node by Xn: Cell Switch Notification message. The UE context reference is indicated by a pair list of {Global target NG-RAN ID, target NG-RAN UE XnAP ID}, and the UE context is transferred by Xn: Inter-node RRC container.

In order to support subsequent LTM, after initial LMT execution, the selected target node becomes to new source node. The new source node shall initiate LTM configuration procedures with each of candidate nodes.

In case that the new source node transfers the UE context reference/UE context to candidate node, the inter-MN HO with/without SN change mechanism can be reused.

For subsequent LTM, the candidate gNB uses the old target UE XnAP ID (the target UE AP ID it previously allocated between it and the old serving gNB) to identify the UE context when receiving the LTM Configuration Update message from the new serving gNB.

Proposal 9: For subsequent LTM, the new source gNB can send UE context to each of candidate gNB via LTM configuration update message.

Proposal 10: For subsequent LTM, if new source gNB decides to keep the UE context previous stored in candidate gNB, it send LTM configuration update message including the Target NG-RAN UE XnAP.

Proposal 11: For subsequent LTM, if new source gNB decides to change the UE context, it sends LTM configuration update message including UE context.

  - Early RACH configuration

According to RAN2#129 meeting, it is agreed that How to support shared RACH resources for inter-CU LTM is left to RAN3. And during RAN3#127 meeting, RAN3 had discussed that in order to avoid explicitly exposing the gNB internal architecture, a new ID of early RACH configuration resource or other new name can be used to request early RACH configuration.

However, even if a new name, in case the number of provided early RACH resource request ID(s) for one gNB ID is larger than one, the the source gNB internal architecture is actually exposed regardless of a new name.

Observation 3: The number of early RACH resource request ID(s) for one gNB ID will expose the gNB internal architecture regardless of a new name.

On the other hand, some companies may think it is not a big deal to expose gNB internal architecture, which means it is acceptable to provide gNB-DU ID in the early RACH request.

Therefore, there is no need to introduce a new name of gNB ID for early RACH configuration resource.

Proposal 12: For inter-CU LTM, reuse gNB DU ID for early RACH resource request.

However, for a UE configured with inter-CU LTM, if any of the configured candidate LTM cells belongs to other gNB, the source gNB is not aware of the associated gNB-DU ID or RACH resource request ID(s) so that it is unable to provide the gNB-DU ID or RACH resource request ID(s) for the candidate LTM cell in the early RACH request.

Observation 4: The source gNB-CU is unable to provide either the gNB-DU ID(s) or RACH resource request ID(s)  in the early RACH request since it is not aware of the associated gNB-DU ID or RACH resource request ID(s)  of the candidate LTM cell(s) which belongs to other gNB-CU.

So during LTM preparation phase, the candidate gNB-CU shall provide the gNB-DU ID or RACH resource request ID(s) of the accepted candidate LTM cell(s) to the source gNB-CU. Then the source gNB can further inform this mapping to other candidate gNB(s).

Proposal 13: During LTM preparation phase, the candidate gNB shall provide the gNB-DU ID(s) or RACH resource request ID(s) of the accepted candidate LTM cell to the source gNB.

  - CFRA resource delivery

In F1 interface of legacy inter-DU LTM, candidate DU will generate the CFRA resource for source DU to construct LTM cell switch command MAC CE, as shown in below:

If the LTM CFRA Resource Config List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it for the LTM cell switch command as specified in TS 38.321 [16].

| LTM CFRA Resource Config List |  | 0..1 |  |  | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >LTM CFRA Resource Config Item IEs |  | 1 .. <maxnoofLTMCells> |  |  | EACH | ignore |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>LTM CFRA Resource Configuration | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. | - |  |
| >>LTM CFRA Resource Configuration for SUL | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. This IE applies for SUL carrier. | - |  |

However, corresponding signaling is still missing in Inter-DU LTM scenario. Therefore it is suggested to also introduce the CFRA resource in Xn interface.

Proposal 14: In Xn interface, candidate gNB provides the LTM CFRA Resource Configuration of each candidate cell to source gNB for LTM cell switch command generation.

According to current MAC specification, source gNB can indicate the message1 repetition number to UE in the LTM cell switch command.  The messge1 repetition number is to make UE to re-transmit the preamble for the coverage enhancement.

Additionally, UE will receive the RA response in response reception window which is started at the first from the end of ALL repetitions of the Random Access Preamble transmission in case message1 repetition is enabled, and target gNB or target gNB-DU also needs to know which message1 repetition number is used by UE and transmit the RA response to UE wihtin correct time window.

During L3 HO, target gNB or target gNB-DU will configure one message1 repetition number (2, or 4, or 8) for the preambles included in RACH-ConfigDedicated, so that target gNB can identify which message 1 repetition number is used via the RA preamble, then figure out corresponding RAR window. The specific repetition number is configured based UE’s L3 measurement report included in HO request. A high repetition number will be used for low Uu link quality to increase the possibility of successful preamble transmission.

In LTM, as clarified in above, CFRA resource, i.e. RACH-ConfigDedicated, is the baseline information for gNB to generate LTM cell switch command MAC CE.

As shown in below, RACH-ConfigDedicated include more than one SSBs and preambles, and one repetition number.

RACH-ConfigDedicated ::= SEQUENCE { cfra CFRA OPTIONAL, -- Need S ra-Prioritization RA-Prioritization OPTIONAL, -- Need N ..., [[ ra-PrioritizationTwoStep-r16 RA-Prioritization OPTIONAL, -- Need N cfra-TwoStep-r16 CFRA-TwoStep-r16 OPTIONAL -- Need S ]] } CFRA ::= SEQUENCE { occasions SEQUENCE { rach-ConfigGeneric RACH-ConfigGeneric, ssb-perRACH-Occasion ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen} OPTIONAL -- Cond Mandatory } OPTIONAL, -- Need S resources CHOICE { ssb SEQUENCE { ssb-ResourceList SEQUENCE (SIZE(1..maxRA-SSB-Resources)) OF CFRA-SSB-Resource, ra-ssb-OccasionMaskIndex INTEGER (0..15) }, csirs SEQUENCE { csirs-ResourceList SEQUENCE (SIZE(1..maxRA-CSIRS-Resources)) OF CFRA-CSIRS-Resource, rsrp-ThresholdCSI-RS RSRP-Range } }, ..., [[ totalNumberOfRA-Preambles INTEGER (1..63) OPTIONAL -- Cond Occasions ]], [[ msg1-RepetitionNum-r18 ENUMERATED {n2, n4, n8, spare1} OPTIONAL -- Cond 4StepCFRArep ]] }

Regarding SSB and preamble, the source gNB or source gNB-DU can indicate specific SSB and preamble to be used by UE via LTM cell switch command MAC CE based on L1 measurement report. However, since only one repetition number is provided by candidate gNB, when generating the LTM cell switch command MAC CE, source gNB can only set same repetition number as in RACH-ConfigDedicated.

Observation 5: More than one SSBs and preambles are provided by candidate gNB. The source gNB or source gNB-DU can indicate specific SSB and preamble to be used by UE via LTM cell switch command MAC CE based on L1 measurement report.

Observation 6: Only one repetition number is provided by candidate gNB, source gNB cannot adjust the repetition number.

The L1 measurement report for LTM execution and L3 measurement report maybe totally different since there may be a long time gap between these two measurement reports. Therefore, the message1 repetition number should be set by source gNB or source gNB-DU according to the latest UE measurement report.

To allow source gNB or source gNB-DU to adjust the message1 repetition number, it is suggested to allow source gNB sets the message1 repetition number different from the one indicated in CFRA resource. And since candidate gNB needs to identify which message1 repetition number is used by UE, if repetition number different from the one indicated in CFRA resource is used, source gNB needs to notify the corresponding repetition number to target gNB. The whole procedure can be illustrated below:

Figure 4: Repetition number adjustment

Proposal 15: Allow source gNB to set the message1 repetition number different from that indicated in CFRA resource.

Proposal 16: The message1 repetition number and corresponding preamble used by UE should be included in Xn CELL SWITCH NOTIFICATION message, F1 DU-CU CELL SWITCH NOTIFICATION message and F1 CU-DU CELL SWITCH NOTIFICATION message.

- Conclusion

Based on the above analysis, we provide the following observations and proposals.

Proposal 1: Postpone the detail of SP CSI-RS activation/deactivation.

Proposal 2: The new source gNB needs to provide per cell new KgNB*(s) to the corresponding candidate gNB via a LTM configuration update procedure.

Observation 1: In one candidate gNB, candidate cells can be belonged to the same or different LTM set, in which LTM no security is changed at these candidate cells.

Observation 2: The candidate cells do not need to change security Key, if they are belonged to the same LTM set.

Proposal 3: The candidate gNB sends per cell LTM set ID to the source gNB, via Handover request ACK message.

Proposal 4: The source gNB-CU sends the Rel-19 set ID list and NCC to the source gNB-DU via F1: UE context modification request message, each Rel-19 set ID includes serving cell and/or candidate cells.

Proposal 5: The source gNB sends Rel-19 set ID list to the target gNB via Xn: Cell Switch Notification message.

Proposal 6: The target gNB-CU sends Rel-19 set ID list and NCC to the target gNB-DU via F1: UE context modification request message, each Rel-19 set ID.

Proposal 7: Remove the “FFS on the XnAP IDs” from Xn: TA information transfer message.

Observation 3: During LTM preparation procedure, the source node and target node establishes a pair of {source node UE XnAP ID, target node UE XnAP ID}.

Observation 4: During LTM preparation procedure, the source node and target node establishes UE context.

Proposal 8: The source node can send UE context reference/UE context at other candidate nodes to the selected target node by Xn: Cell Switch Notification message. The UE context reference is indicated by a pair list of {Global target NG-RAN ID, target NG-RAN UE XnAP ID}, and the UE context is transferred by Xn: Inter-node RRC container.

Proposal 9: For subsequent LTM, the new source gNB can send UE context to each of candidate gNB via LTM configuration update message.

Proposal 10: For subsequent LTM, if new source gNB decides to keep the UE context previous stored in candidate gNB, it send LTM configuration update message including the Target NG-RAN UE XnAP.

Proposal 11: For subsequent LTM, if new source gNB decides to change the UE context, it sends LTM configuration update message including UE context.

Observation 3: The number of early RACH resource request ID(s) for one gNB ID will expose the gNB internal architecture regardless of a new name.

Proposal 12: For inter-CU LTM, reuse gNB DU ID for early RACH resource request.

Observation 4: The source gNB-CU is unable to provide either the gNB-DU ID(s) or RACH resource request ID(s)  in the early RACH request since it is not aware of the associated gNB-DU ID or RACH resource request ID(s)  of the candidate LTM cell(s) which belongs to other gNB-CU.

Proposal 13: During LTM preparation phase, the candidate gNB shall provide the gNB-DU ID(s) or RACH resource request ID(s) of the accepted candidate LTM cell to the source gNB.

Proposal 14: In Xn interface, candidate gNB provides the LTM CFRA Resource Configuration of each candidate cell to source gNB for LTM cell switch command generation.

Observation 5: More than one SSBs and preambles are provided by candidate gNB. The source gNB or source gNB-DU can indicate specific SSB and preamble to be used by UE via LTM cell switch command MAC CE based on L1 measurement report.

Observation 6: Only one repetition number is provided by candidate gNB, source gNB cannot adjust the repetition number.

Proposal 15: Allow source gNB to set the message1 repetition number different from that indicated in CFRA resource.

Proposal 16: The message1 repetition number and corresponding preamble used by UE should be included in Xn CELL SWITCH NOTIFICATION message, F1 DU-CU CELL SWITCH NOTIFICATION message and F1 CU-DU CELL SWITCH NOTIFICATION message.

- TP to 38.423

---------------------------Start of the change--------------------------------------------

9.1.1.2	HANDOVER REQUEST ACKNOWLEDGE

This message is sent by the target NG-RAN node to inform the source NG-RAN node about the prepared resources at the target.

Direction: target NG-RAN node  source NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | ignore |
| Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | YES | ignore |
| PDU Session Resources Admitted List | M |  | 9.2.1.2 | Ignored, if the CHO-CPAC Configuration Indicator IE is included within the CHO-CPAC Information IE and set to "cho-only-not-prepared". | YES | ignore |
| PDU Session Resources Not Admitted List | O |  | 9.2.1.3 |  | YES | ignore |
| Target NG-RAN node To Source NG-RAN node Transparent Container | M |  | OCTET STRING | Either includes the HandoverCommand message as defined in subclause 10.2.2 of TS 36.331 [14], if the target NG-RAN node is an ng-eNB, or the HandoverCommand message as defined in subclause 11.2.2 of TS 38.331 [10], if the target NG-RAN node is a gNB. Ignored if the CHO-CPAC Configuration Indicator IE is included within the CHO-CPAC Information IE and set to "cho-only-not-prepared". | YES | ignore |
| UE Context Kept Indicator | O |  | 9.2.3.68 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| DRBs transferred to MN | O |  | DRB List 9.2.1.29 | In case of DC, indicates that SN Status is needed for the listed DRBs from the S-NG-RAN node. | YES | ignore |
| DAPS Response Information | O |  | 9.2.1.34 |  | YES | reject |
| Conditional Handover Information Acknowledge | O |  |  |  | YES | reject |
| >Requested Target Cell ID | M |  | Target Cell Global ID 9.2.3.25 | Target cell indicated in the corresponding HANDOVER REQUEST message | – |  |
| >Maximum Number of CHO Preparations | O |  | 9.2.3.101 |  | – |  |
| >CHO-CPAC Information | O |  | 9.2.3.202 |  | YES | reject |
| MBS Session Information Response List | O |  | 9.2.1.38 |  | YES | ignore |
| RRC Config Indication | O |  | 9.2.3.72 |  | YES | ignore |
| PDU Set based Handling Indicator | O |  | 9.2.3.206 |  | YES | ignore |
| LTM Handover Information Response | O |  |  |  | YES | ignore |
| >LTM Information Response | O |  | 9.2.1.xx2 |  | – |  |
| Early Sync Information Response | O |  | 9.2.1.xx4 |  | YES | ignore |
| LTM Set ID | O |  | INTEGER(0..7,…) | Indicates the LTM no security change between candidate cells within the same candidate gNB | YES | ignore |

--------------------------- Next of the change --------------------------------------------

9.1.1.x1 CELL SWITCH NOTIFICATION

This message is sent by the source NG-RAN node to inform the target NG-RAN node about the initiation of the cell switch command to the UE.

Direction: source NG-RAN node  target NG-RAN node.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | reject |
| Target NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | YES | reject |
| Target Cell Global ID | M |  | NR CGI 9.2.2.7 |  | YES | reject |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Corresponds to the TCI-StateId IE as defined in TS 38.331 [10]. | - |  |
| >UL TCI State ID | O |  | OCTET STRING | Corresponds to the TCI-UL-StateId IE as defined in TS 38.331 [10]. |  |  |
| >preamble | O |  | INTEGER (0..63) |  |  |  |
| >message1 repetition number | O |  | ENUMERATED {n2, n4, n8, spare1} |  |  |  |
| Rel-19 set ID List |  | 0..1 |  |  |  |  |
| > Rel-19 set ID Item |  | 1 .. <maxnoofTargetcells>> |  |  |  |  |
| >>Target Cell Global ID | M |  | NR CGI 9.2.2.7 |  | YES | reject |
| >> Rel-19 set ID | M |  | OCTET STRING | Includes the ltm-NoSecurityChangeID IE as defined in TS 38.331 [8], for the LTM candidate cell identified by the Cell ID IE. Details FFS pending RAN2. | YES | reject |
| UE Context Reference at the candidate NG-RAN nodes List |  | 0..1 |  |  |  |  |
| >UE Context Reference at the candidate NG-RAN nodes Item |  | 1 .. <maxnoofTargetnodes>> |  |  |  |  |
| >>Global NG-RAN Node ID |  |  | 9.2.2.3 |  |  |  |
| >>Target NG-RAN node UE XnAP ID |  |  | NG-RAN node UE XnAP ID 9.2.3.16 |  |  |  |

--------------------------- Next of the change --------------------------------------------

9.1.1.x3		LTM CONFIGURATION UPDATE

This message is sent by the NG-RAN node1 to update LTM configuration data.

Direction: NG-RAN node1  NG-RAN node2.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node 1 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 1 | YES | reject |
| NG-RAN node 2 UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 2 | YES | reject |
| Target Cell Global ID (FFS) | M |  | NR CGI 9.2.2.7 |  | YES | reject |
| LTM Information Request | O |  | 9.2.1.xx1 |  | YES | ignore |
| Early Sync Information Request | O |  | 9.2.1.xx3 |  | YES | ignore |
| Early Sync Information | O |  | 9.2.1.xx6 |  | YES | ignore |
| Early Sync Information for SUL | O |  | 9.2.1.xx6 |  | YES | Ignore |
| Data Forwarding Info | FFS |  |  |  | YES | ignore |
| Key NG-RAN Star | O |  | BIT STRING (256) | KNG-RAN* defined in TS 33.501 [28]. | YES | ignore |
| Security Information for LTM (FFS on the name) | O |  | 9.2.1.x12 |  | YES | ignore |
| Target NG-RAN node UE XnAP ID |  |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | YES | ignore |

Editor’s note: The Data Forwarding Info IE is a list, the granularity is FFS.

9.1.1.x4	 LTM CONFIGURATION UPDATE ACKNOWLEDGE

This message is sent by the NG-RAN node2 to inform the NG-RAN node1 to acknowledge update of information associated to an LTM configuration.

Direction: NG-RAN node2  NG-RAN node1.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node 1 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 1. | YES | reject |
| NG-RAN node 2 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 2. | YES | reject |
| Target Cell Global ID (FFS) | M |  | NR CGI 9.2.2.7 |  | YES | reject |
| LTM Information Response | O |  | 9.2.1.xx2 |  | YES | ignore |
| Early Sync Information Response | O |  | 9.2.1.xx4 |  | YES | ignore |
| RRC Config Indication | O |  | 9.2.3.72 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

9.2.1.xx2	LTM Information Response

This IE contains LTM related information to be provided in response to information provided in the LTM Information Request IE.

Editor’s note: Details on configurations need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CSI Resource Configuration | O |  | 9.2.1.xx8 |  |
| LTM Candidate Configuration | M |  | OCTET STRING | Contains ltm-CandidateConfig as defined in TS 38.331 [10]. |
| Reference Configuration (FFS) | O |  | OCTET STRING | Contains the ltm-ReferenceConfiguration as defined in TS 38.331 [10]. |
| Complete Candidate Configuration Indicator | O |  | ENUMERATED (complete, ...) |  |
| Early RACH Resources Requester ID | M |  | 9.2.1.xx9 | Identifies the entity requesting Early RACH resources. |
| LTM CFRA Resource Config List |  | 0..1 |  |  |
| >LTM CFRA Resource Config Item IEs |  | 1 .. <maxnoofLTMCells> |  |  |
| >>Cell ID | M |  | NR CGI 9.2.2.7 |  |
| >>LTM CFRA Resource Configuration | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. |
| >>LTM CFRA Resource Configuration for SUL | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. This IE applies for SUL carrier. |

--------------------------- End of the change --------------------------------------------

- TP to 38.473

---------------------------Start of the change--------------------------------------------

9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| SpCell ID | O |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | ignore |
| <skip unchanged part> |  |  |  |  |  |  |
| LTM Information Modify |  | 0..1 |  |  | YES | reject |
| >LTM Indicator | M |  | ENUMERATED (true, …) |  | - |  |
| >Reference Configuration | O |  | 9.3.1.292 |  | - |  |
| >CSI Resource Configuration | O |  | 9.3.1.330 |  | - |  |
| LTM CFRA Resource Config List |  | 0..1 |  |  | YES | ignore |
| >LTM CFRA Resource Config Item IEs |  | 1 .. <maxnoofLTMCells> |  |  | EACH | ignore |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>LTM CFRA Resource Configuration | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. | - |  |
| >>LTM CFRA Resource Configuration for SUL | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. This IE applies for SUL carrier. | - |  |
| LTM Configuration ID Mapping List | O |  | 9.3.1.294 |  | YES | reject |
| Early Sync Information Request |  | 0..1 |  |  | YES | ignore |
| >Request for RACH Configuration | M |  | ENUMERATED (true, …) |  | - |  |
| >LTM gNB-DUs ID List |  | 1 |  | This IE contains the IDs of the source gNB-DU and candidate gNB-DU(s). | YES | reject |
| >>LTM gNB-DUs Item IEs |  | 1..< maxnoofLTMgNBDUs> |  |  | - |  |
| >>>LTM gNB-DU ID | M |  | gNB-DU ID 9.3.1.9 |  | - |  |
| >>>LTM gNB ID | O |  | Global gNB ID 9.3.1.305 |  | - |  |
| Early Sync Candidate Cell Information List |  | 0..1 |  |  | YES | ignore |
| >Early Sync Candidate Cell Information Item IEs |  | 1 .. <maxnoofLTMCells> |  |  | EACH | ignore |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TCI States Configurations List | O |  | OCTET STRING | Includes the LTM-TCI-Info IE, as defined in TS 38.331 [8]. | - |  |
| >>Early UL Sync Configuration | O |  | 9.3.1.328 |  | - |  |
| >>Early UL Sync Configuration for SUL | O |  | Early UL Sync Configuration 9.3.1.328 | This IE applies for SUL carrier. | - |  |
| >>TA Assistance Information | O |  | ENUMERATED (zero, …) | The value "zero" corresponds to TA value of the cell being equal to zero. | - |  |
| >>UE Based TA Measurement Configuration | O |  | OCTET STRING | Includes the ltm-UE-MeasuredTA-ID contained in the LTM-Candidate IE, as defined in TS 38.331 [8], for the LTM candidate cell identified by the Cell ID IE. | - |  |
| >>SSB Positions In Burst | C-ifEarlyUL |  | 9.3.1.138 | This IE applies to early TA acquisition. | YES | ignore |
| Early Sync Serving Cell Information |  | 0..1 |  |  | YES | ignore |
| >UE Based TA Measurement Configuration | O |  | OCTET STRING | Includes the ltm-ServingCellUE-MeasuredTA-ID contained in the LTM-Config IE, as defined in TS 38.331 [8], for the current serving cell. | - |  |
| LTM Cells To Be Released List | O |  | 9.3.1.291 |  | YES | reject |
| Path Addition Information | O |  | 9.3.1.296 |  | YES | reject |
| NR A2X Services Authorized | O |  | 9.3.1.323 |  | YES | ignore |
| LTE A2X Services Authorized | O |  | 9.3.1.324 |  | YES | ignore |
| NR UE Sidelink Aggregate Maximum Bit Rate for A2X | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.119 | This IE applies only if the UE is authorized for NR A2X services. | YES | ignore |
| LTE UE Sidelink Aggregate Maximum Bit Rate for A2X | O |  | LTE UE Sidelink Aggregate Maximum Bit Rate 9.3.1.118 | This IE applies only if the UE is authorized for LTE A2X services. | YES | ignore |
| DL LBT Failure Information Request | O |  | ENUMERATED (inquiry, …) |  | YES | ignore |
| Ranging and Sidelink Positioning Service Information | O |  | 9.3.1.331 | This IE applies only if the UE is authorized for NR V2X services and/or 5G ProSe services. | YES | ignore |
| Non-Integer DRX Cycle | O |  | 9.3.1.344 |  | YES | ignore |
| LTM Reset Information | O |  | 9.3.1.346 |  | YES | ignore |
| LTM Security Information | O |  | 9.3.1.XX |  | YES | reject |
| Rel-19 set ID List |  | 0..1 |  |  | YES | reject |
| >Rel-19 set ID Item |  | 1 .. <maxnoofTargetcells>> |  |  |  |  |
| >>Target Cell Global ID | M |  | NR CGI 9.3.1.12 |  | - | - |
| >> Rel-19 set ID | M |  | OCTET STRING | Includes the ltm-NoSecurityChangeID IE as defined in TS 38.331 [8], for the LTM candidate cell identified by the Cell ID IE. Details FFS pending RAN2. | - | - |

9.2.2.15	DU-CU CELL SWITCH NOTIFICATION

This message is sent by the gNB-DU to inform the gNB-CU about the initiation of the cell switch  command to the UE.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cell ID | M |  | NR CGI 9.3.1.12 |  | YES | reject |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Includes the TCI-StateId IE, as defined in TS 38.331 [8]. | - |  |
| >preamble | O |  | INTEGER (0..63) |  |  |  |
| >message1 repetition number | O |  | ENUMERATED {n2, n4, n8, spare1} |  |  |  |
| >UL TCI State ID | O |  | OCTET STRING | Includes the TCI-UL-StateId IE, as defined in TS 38.331 [8]. | - |  |
| TA Information List |  | 0..1 |  |  | YES | ignore |
| >TA Information Item IEs |  | 1 .. <maxnoofTAList> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. | - |  |
| >>Tag ID Pointer | O |  | OCTET STRING | Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [8]. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAList | Maximum no. of TA values to be sent, the maximum value is 8. |

9.2.2.16	CU-DU CELL SWITCH NOTIFICATION

This message is sent by the gNB-CU to inform the gNB-DU about the initiation of the cell switch command to the UE.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cell ID | M |  | NR CGI 9.3.1.12 |  | YES | reject |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Includes the TCI-StateId IE, as defined in TS 38.331 [8]. | - |  |
| >preamble | O |  | INTEGER (0..63) |  |  |  |
| >message1 repetition number | O |  | ENUMERATED {n2, n4, n8, spare1} |  |  |  |
| >UL TCI State ID | O |  | OCTET STRING | Includes the TCI-UL-StateId IE, as defined in TS 38.331 [8]. | - |  |
| TA Information List |  | 0..1 |  |  | YES | ignore |
| >TA Information Item IEs |  | 1 .. <maxnoofTAList> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. | - |  |
| >>Tag ID Pointer | O |  | OCTET STRING | Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [8]. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAList | Maximum no. of TA values to be sent, the maximum value is 8. |

--------------------------- End of the change --------------------------------------------

- TP to 38.401

#### 8.2.1.5	Inter-gNB-DU LTM

This procedure is used for the case when the UE moves from one gNB-DU to another gNB-DU within the same gNB-CU during NR operation for LTM. Figure 8.2.1.5-1 shows the inter-gNB-DU LTM procedure for intra-NR.

Figure 8.2.1.5-1: Inter-gNB-DU LTM

1.	The UE sends a MeasurementReport message (L3 measurement result) to the source gNB-DU containing measurements of neighbouring cells. The source gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

2.	The gNB-CU determines to initiate LTM configuration.

3.	The gNB-CU sends a UE CONTEXT SETUP REQUEST message to the candidate gNB-DU(s) for each candidate cell, containing one candidate cell ID and the CSI resource configuration for subsequent LTM. The gNB-CU may provide the LTM configuration ID mapping list to the candidate gNB-DU(s). The gNB-CU may request PRACH resources from the candidate gNB-DU(s). The gNB-CU may request the candidate gNB-DU(s) to provide the lower layer configuration for the purpose of generating the reference configuration or provide the lower layer part of the reference configuration to the candidate gNB-DU(s).

4.	If the candidate gNB-DU accepts the request of LTM configuration, it responds with a UE CONTEXT SETUP RESPONSE message including the generated lower layer RRC configurations for the accepted target candidate cell.

NOTE 1:	The CU-initiated UE Context Modification procedure may be initiated for preparing candidate cells in the source gNB-DU as specified in step 3 and 4 in 8.2.1.4 Intra-gNB-DU LTM.

5.	The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source gNB-DU including the information related to early sync and the LTM configuration ID mapping list for the accepted target candidate cell(s). The gNB-CU may send the updated CSI resource configuration to the source gNB-DU. The gNB-CU may inform the source gNB-DU about intra-DU L2 reset configuration. The gNB-CU may generate the global Rel-19 set ID list and send it to the source gNB-DU.

6.	The source gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message which includes an updated lower layer configuration, e.g., containing the updated CSI report configuration of the source cell.

7.	The gNB-CU may send a UE CONTEXT MODIFICATION REQUEST message for each candidate cell accepted in the candidate gNB-DU(s), containing the information for subsequent LTM or for updating the configurations of candidate cells. The gNB-CU may also provide the lower layer part of the reference configuration to the candidate gNB-DU(s). The gNB-CU may inform the candidate gNB-DU(s) about intra-DU L2 reset configuration.

<Skip unchanged part>

--------------------------- End of the change --------------------------------------------

- TP to 38.300

--------------------------- Start of the change --------------------------------------------

9.2.3.5.X C-Plane Handling of inter-gNB L1/L2 Triggered Mobility

This procedure is used for the case when the UE moves from one gNB to another gNB during NR operation for LTM. Figure 9.2.3.5.X-1 shows the inter-gNB LTM procedure for intra-NR.

Figure 9.2.3.5.X-1: Signalling procedure for inter-gNB LTM

The procedure for inter-gNB LTM is as follows:

1.	The UE sends a MeasurementReport message (L3 measurement result) to the source gNB containing measurements of neighbouring cells.

2.	The source gNB decides to configure inter-gNB LTM.

3.	The source gNB requests inter-gNB LTM for one or more candidate cells belonging to one or more candidate gNB(s). The source gNB initiates a HANDOVER REQUEST message per candidate cell containing one candidate cell ID and the CSI resource configuration for subsequent LTM. The source gNB may request the candidate gNB(s) to provide the CSI-RS configuration.

4.	Admission Control may be performed by the candidate gNB(s).

5.	The candidate gNB(s) prepares the LTM configuration(s) and sends inter-gNB response (HO REQUEST ACKNOWLEDGE) to the source gNB including the generated RRC configurations for the accepted candidate cell. The candidate gNB(s) may also include the CSI-RS configuration upon request and may include the LTM set IDs for each candidate cells.

6.	The source gNB sends an LTM CONFIGURATION UPDATE to the candidate gNB(s) to update the LTM configurations of candidate cell(s). The source gNB may include the common CSI-RS resource configuration.

7.	The candidate gNB(s) sends the LTM CONFIGURATION UPDATE ACKNOWLEDGE message to the source gNB. The candidate gNB(s) may also provide the CSI-RS report configuration.

Editor’s Note: FFS on whether step 6 and 7 are mandatory or optional.

NOTE :	Step 6 may also be triggered after step 14, or after step 17 by implementation for subsequent LTM.

8.	The source gNB sends an RRCReconfiguration message to the UE.

9.	The UE stores the LTM candidate configurations and sends an RRCReconfigurationComplete message to the source gNB.

9a	If early data forwarding is applied, the source gNB sends the EARLY STATUS TRANSFER message.

10/11. Early synchronization to some LTM candidate cell(s) belonging to the candidate gNB(s) may be performed. The candidate gNB(s) sends the TA INFORMATION TRANSFER message to the source gNB if early TA acquisition is performed to the candidate gNB(s).

12.	 The UE performs L1 measurements on the configured LTM candidate cell(s) and transmits L1 measurement reports to the source gNB. L1 measurement should be performed as long as RRC reconfiguration (step 8) is applicable.

13.	The source gNB determines to initiate inter-gNB LTM.

14.	The source gNB decides to execute cell switch to a target cell and transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID which indicates the index of the candidate configuration of the target cell, a beam indicated with a TCI state or beams indicated with DL and UL TCI states, and a timing advance command for the target cell, if available. The UE switches to the target cell and applies the candidate configuration indicated by the target configuration ID.

NOTE :	Up to implementation, data forwarding and SN Status Transfer may be initiated once the source gNB triggers the inter-gNB LTM cell switch for the UE in Step 14.

15.	The source gNB sends the CELL SWITCH NOTIFICATION message to the target gNB to indicate the initiation of Cell Switch command to the UE, to provide the Rel-19 set ID list, and to provide the UE context reference and/or UE context for other candidate gNBs.

16.	The target gNB detects the UE access. The UE performs the random access procedure towards the target cell, if UE does not have valid TA of the target cell as specified in clause 5.18.35 of TS 38.321[6].

17/18. The target gNB sends the HANDOVER SUCCESS message to the source gNB to inform that the UE has successfully accessed the target cell. In return, the source gNB sends the SN STATUS TRANSFER message following the principles described in step 7 of Intra-AMF/UPF Handover in clause 9.2.3.2.1.

NOTE :	Late data forwarding may be initiated as soon as the source gNB receives the HANDOVER SUCCESS message.

19.	The UE sends an RRCReconfigurationComplete message to the target gNB.

20. The new source gNB (i.e., the target gNB) sends the LTM CONFIGURATION UPDATE message to the candidate gNB(s). This message may include the new security key(s) to be used with the UE. If the new source gNB decides to keep the UE context in candidate gNB(s), it sends Target gNB UE XnAP ID as a reference to the UE context in the candidate gNB that was established by the source gNB. If the new source gNB decides to change the UE context allowing delta configuration, it sends UE context in the candidate gNB that was established by the source gNB. Otherwise, the new source gNB may send neither Target gNB UE XnAP ID nor UE context in the candidate node that was established by the source node.

21. The candidate gNB(s) responds the LTM CONFIGURATION UPDATE ACKNOWLEDGE message to the new source gNB.

22. The new source gNB may send the UE CONTEXT RELEASE message to inform the old source gNB to release radio and C-plane related resources associated to the UE context if no LTM candidate cell(s) exist in the old source gNB. Any ongoing data forwarding may continue.

--------------------------- End of the change --------------------------------------------