3GPP TSG-RAN WG3 Meeting #128		R3-253264

St. Julian’s, Malta, 19th – 23rd May 2025

Source:	vivo

Title:	Discussion on XR rate control

Agenda Item:	21.3

Document for:	Discussion and Decision

- Introduction

In last RAN3 meeting, UL rate control for XR has been discussed and achieved the following agreements. An LS [1] was also sent to RAN2/SA2 to inform these agreements.

Introduce an indication in NGAP, to allow the SMF to inform the gNB which QoS flow(s) are subject to uplink rate control. Introduce the same indication in XnAP to inform target gNB.

SA2 also discussed the related issue and a response LS [2] was sent in last meeting.

SA2 Feedback:

- SA2 confirms that the CN can indicate to the gNB that a QoS flow is subject to uplink rate control.

- SA2 would like to inform RAN WGs that SA2 has not decided yet on whether the existing or a new mechanism can be reused.

- SA2 would like RAN2 to confirm that the RAN2 LS on UL rate control applies to both GBR and non-GBR.

Besides, there are some open issues on UL rate control on per QoS flow indication or per DRB indication over F1AP and the coordination between CN and gNB on rate control.

In this contribution, we will discuss these open issues related to UL XR rate control.

- Discussion

  - Discussion on F1 impact on per QoS flow or per DRB indication for XR rate control

As confirmed by SA2 and RAN3, gNB receives QoS flow information from the CN, specifying which QoS flows are subject to uplink rate control. In case of CU-DU split architecture, gNB-CU will indicate such information via “QoS flow level QoS information” in F1AP signaling directly in case of per QoS flow level rate control. While for per DRB level rate control, either QoS flow information or mapped DRB information need to be indicated to gNB-DU via F1AP signaling.

In case of one QoS flow to one DRB mapping, both DRB based and QoS flow based rate control could be easily performed. In case of more than one QoS flows to one DRB mapping, per DRB based rate control can not provide proper recommended bit rate to UE for the adjustable QoS flow so that such mechanism may not achieve congestion relief target. gNB-DU can hardly choose proper target bit rate by its implementation, since specific UE behavior can not be defined when a DRB based rate control MAC CE be received. As a result, the rate control result will still be unpredictable.

Observation 1: Per DRB based rate control can hardly achieve expected rate adjustment from gNB perspective.

In case of more than one QoS flows to one DRB mapping, for QoS flow based rate control, gNB-DU could identify each QoS flow and choose a proper bit rate for the QoS flow. Another approach is to calculate a percentage of the adjustable bit rate for given QoS flow within one DRB by its implementation. Then sending the calculated bit rate to UE via MAC CE and achieve predictable rate adjustment.

In last RAN2 meeting, it was confirmed that from RAN2 point of view per QoS flow indication in MAC CE is preferred. Since both solutions have F1AP signaling impact and both are feasible to implement, it is proposed that:

Proposal 1: It is up to RAN2 to decide the XR rate control indication in MAC CE is per DRB or per QoS flow basis. And send an LS to inform RAN2 the agreement.

  - Discussion on CN coordination on XR rate control

In SA2 reply LS, a question raised regarding service type for rate control. It is foreseen different XR services may map on GBR or non-GBR. In congestion scenario, the high data rate for non-GBR services could also burden the radio. As a result, rate control for both GBR and non-GBR should be considered to either relieve the congested radio or guarantee the service transmission. Since CN has already defined ECN marking for L4S to expose congestion information so that application could adjust its codec accordingly. Once radio congestion happened, gNB based UL rate control would send to UE to adjust application codec rate. CN may also need to be informed so that it can decide to adjust related function. For example, CN may de-active ECN marking for L4S UL to avoid duplicated operation.

Proposal 2: Rate control for both GBR and non-GBR should be considered.

Proposal 3: gNB may inform SMF/UPF when UL rate control MAC CE send to UE in order to coordinate with CN for congestion handling.

  - Discussion on XR rate control in DC configuration

In Rel-18, we have discussed the PSI-based SDU discard activation in case DC is configured, and agreed that NG RAN nodes will not perform co-ordination and UE will always follow the latest activation or deactivation command via MAC CE. However, in case of XR rate control, the indication from gNB-DU is used to suggest the maximum bit rate according to related radio from one NG RAN node. UE can not easily perform rate adaptation according to rate control MAC CEs send from two NG RAN node separately. For example, if UE receive XR rate control bit rate from one NG-RAN node, whether it should wait for the bit rate from the other NG RAN node or consider the value as the suggest bit rate in total. As a result, it is suggested M-gNB and S-gNB perform bit rate co-ordination and send the overall value to UE for simplicity. One detailed example would be S-gNB-DU send its suggested bit rate per QoS flow to M-gNB-DU via F1AP and XnAP message. And only M-gNB DU is allowed to send the XR rate control MAC CE to affect UE. Furthermore, user plane GTP-U header could be also used to send such suggested bit rate information.

Proposal 4: In case of DC configuration, S-NG RAN node and M-NG RAN node need to coordinate the suggested bit rate for XR rate control and one rate control MAC CE would be sent to UE. Detailed would be discussed in the CR. An LS is sent to RAN2 with the conclusion.

- Conclusion

In this contribution, we discuss the RAN3 impacts and open issues of XR rate control. Based on the discussion, we have the following observations and proposals:

Observation 1: Per DRB based rate control can hardly achieve expected rate adjustment result from gNB perspective.

Proposal 1: It is up to RAN2 to decide the XR rate control indication in MAC CE is per DRB or per QoS flow basis. And send an LS to inform RAN2 the agreement.

Proposal 2: Rate control for both GBR and non-GBR should be considered.

Proposal 3: gNB may inform SMF/UPF when UL rate control MAC CE send to UE in order to coordinate with CN for congestion handling.

Proposal 4: In case of DC configuration, S-NG RAN node and M-NG RAN node need to coordinate the suggested bit rate for XR rate control and one rate control MAC CE would be sent to UE. Detailed would be discussed in the CR. An LS is sent to RAN2 with the conclusion.

- References

- R3-252491	      Reply LS on uplink rate control

- S2-2504252      LS reply on uplink rate control

- Annex - draft reply LS

Title:	[Draft] LS on XR rate control

Response to:

Release:	Rel-19

Work Item:	NR_XR_Ph3-Core

Source:	TBD [to be RAN3]

To:	RAN2

Cc: 	SA2

# Contact Person:

# Name:	              TBD

E-mail Address:	 TBD

# 1. Overall Description:

RAN3 has discussed the related issues on XR rate control, and concluded the following responses [To be updated based on the discussion conclusion]:

1. Both per QoS flow and per DRB XR rate control are feasible and have F1AP impacts, thus it is up to RAN2 to the XR rate control whether it is per DRB or per QoS flow basis. 2. In case of DC configuration, S-NG RAN node and M-NG RAN node need to coordinate the suggested bit rate for XR rate control and one rate control MAC CE would be sent to UE.

# 2. Actions:

# To RAN WG 2:

# RAN3 kindly request RAN2 to take the above information into account during the future work, and provide feedback, if any.

# 3. Date of Next RAN3 Meetings:

TSG RAN WG3 Meeting #129		25th Aug. – 29th Aug. 2025		India

TSG RAN WG3 Meeting #129bis	13rd Oct. – 17th Oct. 2025		Prague