3GPP TSG-RAN3 Meeting #128	R3-253268

Malta, MT, 19 – 23 May, 2025

Agenda Item:	11.2

Source: 	Qualcomm Incorporated

Title: 	Discussion on remaining issues for NG-RAN AI/ML for Network Slicing

Document for:	Discussion/Decision

# Background

In RAN3#127bis most of the open issues in Slicing were closed except UE Performance feedback. Though it is agreed to report UE Performance feedback at slice level, how to calculate it at PDCP layer is not readily available in TS 28.552.

RAN3#127bis For Slice UE performance request, take Opt2: Extend Existing Report Characteristics Bitmap. Extending the existing Node Measurement Failed Report Characteristics IE to include slice-level UE performance metrics for indicating the failed result for slice-level UE performance metrics. Add Slice UE performance IE in the DATA COLLECTION UPDATE message. Per S-NSSAI per UE performance metrics is reported as part of the Slice UE Performance IE in the Data Collection Update message. An FFS on how the metrics are calculated will be added in the TP. If no SA5 definition on how to calculate the metrics is available, RAN3 to decide the metrics are derived by an average or weighted average measurement. Introduce the measured CCCV within the predicted slice available capacity.

In this paper we discuss our views on the remaining open issues on UE performance reporting.

# Discussion

## Slice Average UE Throughput

It was agreed to support Average UE Throughput at slice level in the previous RAN3 meeting. Currently TS 28.558 only supports average DL UE throughput per slice at RLC layer Clause 6.3.1.4.1, UL in Clause 6.3.1.4.2 and not in PDCP layer.

TS 28.558 ##### 6.3.1.4.1 Average DL UE throughput in gNB a) This measurement provides the average UE throughput in downlink. This measurement is intended for data bursts that are large enough to require transmissions to be split across multiple slots. The UE data volume refers to the total volume scheduled for each UE regardless of if using only primary- or also supplemental aggregated carriers. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in EN-DC architecture [16]) and subcounters per supported S-NSSAI. This measurement is also referred to as DL M5 in TS 37.320 [9].

TS 28.558 supports UE Throughput per Slice at RLC layer in Clause 6.3.1.4.1 (DL) and Clause 6.3.1.4.2 (UL). Average DL UE Throughput at PDCP layer is supported in TS 28.552 Clause 5.1.1.3.7, but it does not support per Slice. Also Average UL UE Throughput at PDCP layer is not available in TS 28.552 or in TS 28.558.

In Split Architecture topic, a working assumption was agreed to support UE throughput at PDCP layer.

WA: In case of both split architecture and non-split architecture, UE throughput UL/DL at PDCP level can be considered for the AI/ML for NG-RAN function in Rel-19.

LS was sent to SA5 from RAN3 requesting them to provide UE Performance metrics at PDCP layer in the last meeting in R3-252436. It was also agreed that RAN3 needs to decide on UE Performance metrics if no SA5 definition is available.

If no SA5 definition on how to calculate the metrics is available, RAN3 to decide the metrics are derived by an average or weighted average measurement.

SA5 has clarified in their LS response R3-253023 that the "DRB." prefix in field (e) indicates that the contributing measurements originate from DRBs, but they are aggregated to reflect per-UE throughput

R3-253023 SA5 confirms that the formulas specified in fields (c) of clauses 6.3.1.4.1 and 6.3.1.4.2 in TS 28.558 calculate average UE throughput based on per DRB level measurement for a UE. The formulas aggregate data across DRBs for each UE, ensuring that the reported throughput measurement represents the overall throughput per UE rather than being limited to individual DRBs. - The "DRB." prefix in field (e) indicates that the contributing measurements originate from DRBs, but they are aggregated to reflect per-UE throughput.

Based on the above information, we think it is easier to refer to Average DL UE throughput per slice at RLC layer defined in TS28.558 Clause 6.3.1.4.1 and UL in Clause 6.3.1.4.2 for timely completion of the AI/ML enabled Slicing WI, as this information is readily available.

If SA5 agrees to define Average UE throughput per slice at PDCP layer as a response to RAN3 LS R3-252436, it can be considered as an enhancement in the subsequent release.

RAN3 to agree to refer to Average DL UE throughput per slice defined in TS28.558 Clause 6.3.1.4.1 and Average UL UE throughput per slice defined in Clause 6.3.1.4.2 for timely completion of the AI/ML enabled Slicing WI, as this information is readily available.

## Slice Average UE Packet Delay

Rel18 UE Packet Delay refers to TS 38.314. However, 38.314 provide Packet delay at per DRB per UE level. Average Packet delay at per UE per slice is not available in TS 38.314.

#### TS 38.314 #### 4.2.1.2 Packet delay ##### 4.2.1.2.1 General Packet delay includes RAN part of delay and CN part of delay. The RAN part of DL packet delay measurement comprises: - D1 (DL delay in over-the-air interface), referring to Average delay DL air-interface in TS 28.552 [2] 5.1.1.1.1. - D2 (DL delay on gNB-DU), referring to Average delay in RLC sublayer of gNB-DU in TS 28.552 [2] 5.1.3.3.3. - D3 (DL delay on F1-U), referring to Average delay on F1-U in TS 28.552 [2] 5.1.3.3.2. - D4 (DL delay in CU-UP), referring to Average delay DL in CU-UP in TS 28.552 [2] 5.1.3.3.1. The DL packet delay measurements, i.e. D1 (the DL delay in over-the-air interface ), D2 (the DL delay in gNB-DU), D3 (the DL delay on F1-U) and D4 (the DL delay in CU-UP), should be measured per DRB per UE. NOTE: The delay measurements D1, D2 and D4 are also applicable for EUTRA in case of EN-DC related DL delay measurements on the MN side. The RAN part (including UE) of UL packet delay measurement comprises: - D1 (UL PDCP packet average delay, as defined in clause 4.3.1.1). - D2.1 (average over-the-air interface packet delay, as defined in 4.2.1.2.2). - D2.2 (average RLC packet delay, as defined in 4.2.1.2.3). - D2.3 (average delay UL on F1-U, it is measured using the same metric as the average delay DL on F1-U defined in TS 28.552 [2] clause 5.1.3.3.2). - D2.4 (average PDCP re-ordering delay, as defined in 4.2.1.2.4). The UL packet delay measurements, i.e. D1(UL PDCP packet average delay), D2.1(average over-the-air interface packet delay), D2.2(average RLC packet delay), D2.3(average delay UL on F1-U) and D2.4(average PDCP re-ordering delay), should be measured per DRB per UE. The unit of D1, D2.1, D2.2, D2.3 and D2.4 is 0.1ms.

TS 38.314 support DL and UL Packet Delay at per DRB per UE level. But per Slice level is not supported.

TS 28.558 Clause 6.3.1.1 provides Average Packet Delay at DL and UL at per UE per Slice level.

#### TS 28.558 Clause 6.3.1.1 #### Average DL Packet Delay = #### TS 28.558 Clause 6.3.1.1.1 Average delay DL air-interface (per supported S-NSSAI) D1, #### TS 28.558 Clause 6.3.1.1.2 Average delay DL in gNB-DU (per supported S-NSSAI) D2, #### TS 28.558 Clause 6.3.1.1.3 Average delay DL on F1-U (per supported S-NSSAI) D3, #### TS 28.558 Clause 6.3.1.1.4 Average delay DL in CU-UP (per supported S-NSSAI) D4 #### Average UL Packet Delay = #### TS 28.558 Clause 6.3.1.1.5 UL PDCP packet average delay (per supported S-NSSAI) D1 #### TS 28.558 Clause 6.3.1.1.6 Average delay UL on over-the-air interface (per supported S-NSSAI) D2.1 #### TS 28.558 Clause 6.3.1.1.7 Average RLC packet delay in the UL (per supported S-NSSAI) D2.2 #### TS 28.558 Clause 6.3.1.1.3 Average delay DL on F1-U (per supported S-NSSAI) D2.3 (Average delay UL on F1-U, it is measured using the same metric as the average delay DL on F1-U) #### TS 28.558 Clause 6.3.1.1.8 Average PDCP re-ordering delay in the UL (per supported S-NSSAI) D2.4

TS 28.558 Clause 6.3.1.1 supports DL and UL Packet Delay at per DRB per Slice per UE level.

RAN3 to agree to refer to Average DL and UL Packet Delay per slice defined in TS28.558 Clause 6.3.1.1 for timely completion of the AI/ML enabled Slicing WI, as this information is readily available.

## Slice UE Packet Loss

Unfortunately, Average UE Packet Loss at both UL and DL is not well defined in both TS 28.558 and TS 38.314. DL UE Packet Loss Rate in Uu interface is available in TS 28.558 Clause 6.3.1.2.1 at Slice level. Whereas similar UL UE Packet Loss Rate in Uu interface is not available. Instead, UL PDCP SDU Loss Rate is available in TS 28.558 Clause 6.3.1.3.1 which provides combined Packet loss including any packet losses in the air interface, in the gNB-CU and on the F1-U interface. Since UL and DL Packet Loss is not available at the same layer, we propose to report only DL UE Packet Loss Rate in Uu interface for Rel19 at per Slice level.

#### TS 28.558 ##### 6.3.1.2.1 DL Packet Loss Rate on Uu a) This measurement provides the DL Packet (i.e., RLC SDU) Loss rate on Uu interface. The measurement is split into subcounters per QoS level (mapped 5QI or QCI in EN-DC architecture [16]) and per supported S-NSSAI. This measurement is also referred to as DL M7 in TS 37.320 [9].

TS 28.558 Clause 6.3.1.2.1 supports DL Packet Loss Rate on Uu interface at per DRB per Slice per UE level. Similar UL Packet Loss Rate on Uu interface is not available. Instead, UL PDCP SDU Loss Rate is available in TS 28.558 Clause 6.3.1.3.1 which provides combined Packet loss including any packet losses in the air interface, in the gNB-CU and on the F1-U interface

Since UL and DL Packet Loss Rate is not available at the same layer, RAN3 to agree to report only DL UE Packet Loss in Uu interface from TS 28.558 Clause 6.3.1.2.1 for Rel19 at per Slice level and wait for SA5 LS response for UL UE Packet Loss in Uu interface. Averaging of DL UE Packet Loss in Uu interface is not needed.

# Summary

Based on the discussion above, we have following observations:

Observation 1.	TS 28.558 supports UE Throughput per Slice at RLC layer in Clause 6.3.1.4.1 (DL) and Clause 6.3.1.4.2 (UL). Average DL UE Throughput at PDCP layer is supported in TS 28.552 Clause 5.1.1.3.7, but it does not support per Slice. Also Average UL UE Throughput at PDCP layer is not available in TS 28.552 or in TS 28.558.

Observation 2.	TS 38.314 support DL and UL Packet Delay at per DRB per UE level. But per Slice level is not supported.

Observation 3.	TS 28.558 Clause 6.3.1.1 supports DL and UL Packet Delay at per DRB per Slice per UE level.

Observation 4.	TS 28.558 Clause 6.3.1.2.1 supports DL Packet Loss Rate on Uu interface at per DRB per Slice per UE level. Similar UL Packet Loss Rate on Uu interface is not available. Instead, UL PDCP SDU Loss Rate is available in TS 28.558 Clause 6.3.1.3.1 which provides combined Packet loss including any packet losses in the air interface, in the gNB-CU and on the F1-U interface

Based on these observations and discussion above, we have the following proposals:

Proposal 1:	RAN3 to agree to refer to Average DL UE throughput per slice defined in TS28.558 Clause 6.3.1.4.1 and Average UL UE throughput per slice defined in Clause 6.3.1.4.2 for timely completion of the AI/ML enabled Slicing WI, as this information is readily available.

Proposal 2:	RAN3 to agree to refer to Average DL and UL Packet Delay per slice defined in TS28.558 Clause 6.3.1.1 for timely completion of the AI/ML enabled Slicing WI, as this information is readily available.

Proposal 3:	Since UL and DL Packet Loss Rate is not available at the same layer, RAN3 to agree to report only DL UE Packet Loss in Uu interface from TS 28.558 Clause 6.3.1.2.1 for Rel19 at per Slice level and wait for SA5 LS response for UL UE Packet Loss in Uu interface. Averaging of DL UE Packet Loss in Uu interface is not needed.