3GPP TSG-RAN3 Meeting #128	R3-253271

Malta, MT, 19 – 23 May, 2025

Agenda Item:	11.4

Source: 	Qualcomm Incorporated, Nokia, Nokia Shanghai Bell

Title: 	Discussion on LS Response for Continuous MDT

Document for:	Discussion/Decision

# Background

RAN3 sent an LS R3-250891 on Continuous MDT requirements to SA5 requesting SA5 to provide a solution. SA5 has responded back to RAN3 LS in R3-253024 with further question and suggestions. In this paper we discuss the LS response to SA5.

R3-253024 SA5 would like to thank RAN3 for their LS (R3-250891), on Continuous Management Based MDT. SA5 has been asked to provide guidance on possible solutions for Continuous Management Based MDT. For Measurement Continuity Problem, regarding RAN3 agreements: RAN3: “- The OAM provides to the RAN means to identify that the Management Based MDT is for Continuous MDT, e.g. reusing existing Management Based MDT configuration information.” Answer from SA5: In SA5 point of view, it is feasible for OAM to provide the configuration mechanisms to RAN to identify Management based MDT for continuous MDT. SA5 expects that RAN3 could provide the configuration details as OAM requirements. RAN3: “- RAN3 will derive solutions for the Measurement Continuity Problem across RRC_Connected mobility. SA5 is requested to focus on the Measurement Continuity Problem across different RRC states.” Answer from SA5: SA5 would like RAN3 to provide some clarifications on what problem is referred by “the Measurement Continuity Problem across different RRC states”. RAN3: “- A solution should minimise impacts on the current Management Based MDT framework.” Answer from SA5: SA5 agrees that the solution should minimise impacts on the current Management Based MDT framework. For Trace Correlation Problem, RAN3: “To solve the Trace Correlation Problem, namely to enable correlation of MDT measurements generated by the same UE, it is required to use a temporary identifier for the Continuous MDT, not revealing the UE identity. Such identifier should be made available at the TCE to enable correlation of the management-based MDT reports collected during the Continuous MDT, in a way that minimises the impacts on the Management Based MDT solution.” Answer from SA5: SA5 agrees that at least TR and TRSR (which does not reveal UE identity) could be used for the correlation of the management-based MDT reports at TCE. SA5 would like RAN3 to elaborate the details of how to avoid overlapping

# Discussion

## Question 1 on Configuration Details for Continuous MDT

RAN3: “- The OAM provides to the RAN means to identify that the Management Based MDT is for Continuous MDT, e.g. reusing existing Management Based MDT configuration information.”

Answer from SA5: In SA5 point of view, it is feasible for OAM to provide the configuration mechanisms to RAN to identify Management based MDT for continuous MDT. SA5 expects that RAN3 could provide the configuration details as OAM requirements.

There are different options in which OAM can enable RAN to understand that a particular Management Based MDT configuration is for Continuous MDT

Option 1: Allocating same TR/TRSR across both Immediate and Logged MDT sessions

OAM needs to provide both Immediate and Logged MDT configurations to the gNB with the same TR and TRSR. This way gNB can understand that Management based MDT configuration is for continuous MDT. gNB can use the same TR/TRSR for reporting the MDT reports for MDT report Correlation purposes.

Observation 1: The following pros and cons are observed when using the same TR/TRSR across both Immediate and Logged MDT sessions to identify Continuous MDT.

Pros:

- Re-uses the legacy configuration IEs and not much impact to both OAM and RAN

Cons:

- Security concern as same identifier (TR/TRSR) will be used over Uu interface across different RRC idle state transitions, which can enable tracking of the UE.

Option 2: Allocating a specific range of TR/TRSR

In this option OAM shall reserve a specific range of TR/TRSR values to identify Continuous MDT configuration. Different values of TR/TRSR can be used for Immediate and Logged MDT sessions. However with this option Trace Correlation of MDT reports in OAM is not possible as immediate and logged MDT sessions will have different TR/TRSR. OAM needs to maintain a mapping of TR/TRSR allocated for each immediate and logged MDT sessions for a Continuous MDT. However, it is not possible to see a time series of reporting for a single UE across RRC state transitions with this option.

Observation 2: The following pros and cons are observed when using a specific range of TR/TRSR to identify Continuous MDT configuration and using different values for Immediate and Logged MDT sessions.

Pros:

- Re-uses legacy configuration IEs and not much impact to both OAM and RAN

- Security concern with Option 1 is addressed here as logged and immediate MDT sessions have different TR/TRSR values

Cons:

- OAM needs to maintain a mapping of TR/TRSR configured for logged and immediate MDT sessions for a particular Continuous MDT for Trace Correlation.

- It is not possible to see a time series of reporting for a single UE across RRC state transitions with this option

Option 3: Providing a new Continuous MDT indicator with legacy TR/TRSR mechanism

OAM can include a new flag in the MDT configuration to indicate Continuous MDT with the legacy TR/TRSR mechanism. RAN can use a temporary identifier with assistance from AMF for trace correlation across RRC states.

Observation 3: The following pros and cons are observed when using a Continuous MDT indicator with legacy TR/TRSR mechanism.

Pros:

- All requirements of Continuous MDT are met with this option

Cons:

- Additional impact to OAM, RAN and AMF

Proposal 1: OAM shall indicate via (e.g flag) to RAN that a particular MDT configuration is for Continuous MDT and RAN handles the requirement on Measurement continuity and trace correlation with assistance from AMF.

## Question 2 on Measurement Continuity Problem across RRC_Connected mobility

RAN3: “- RAN3 will derive solutions for the Measurement Continuity Problem across RRC_Connected mobility. SA5 is requested to focus on the Measurement Continuity Problem across different RRC states.”

Answer from SA5: SA5 would like RAN3 to provide some clarifications on what problem is referred by “the Measurement Continuity Problem across different RRC states”.

For Continuous MDT, same UE should be configured with immediate MDT when the UE is handed over from source to target. For Connected Mode mobility RAN can provide the Continuous MDT configuration of the UE to the target in the Handover Request to maintain Measurement Continuity. The Target can configure the UE with Continuous MDT once the UE is in the target to ensure Measurement Continuity

Observation 4: For Continuous MDT, UE should be configured with immediate MDT when it is handed over from source to target. For Connected Mode mobility RAN can provide the Continuous MDT configuration of the UE to the target in the Handover Request to maintain Measurement Continuity.

Proposal 2: RAN3 can handle the requirement on Measurement Continuity Problem across RRC_Connected mobility and no action to SA5 on this.

## Question 3 on Trace Correlation using TR/TRSR

RAN3: “To solve the Trace Correlation Problem, namely to enable correlation of MDT measurements generated by the same UE, it is required to use a temporary identifier for the Continuous MDT, not revealing the UE identity. Such identifier should be made available at the TCE to enable correlation of the management-based MDT reports collected during the Continuous MDT, in a way that minimises the impacts on the Management Based MDT solution.”

Answer from SA5: SA5 agrees that at least TR and TRSR (which does not reveal UE identity) could be used for the correlation of the management-based MDT reports at TCE. SA5 would like RAN3 to elaborate the details of how to avoid overlapping

Observation 5: The issue with using TR/TRSR for Trace Correlation is detailed in Question 1.

Proposal 3: RAN3 to consider the above observations and proposals in the LS response to SA5.

# Summary

Based on the discussion above, we have following observations and proposals:

Option 1: Allocating same TR/TRSR across both Immediate and Logged MDT sessions

Observation 1: The following pros and cons are observed when using the same TR/TRSR across both Immediate and Logged MDT sessions to identify Continuous MDT.

Pros:

- Re-uses the legacy configuration IEs and not much impact to both OAM and RAN

Cons:

- Security concern as same identifier (TR/TRSR) will be used over Uu interface across different RRC idle state transitions, which can enable tracking of the UE.

Option 2: Allocating a specific range of TR/TRSR

Observation 2: The following pros and cons are observed when using a specific range of TR/TRSR to identify Continuous MDT configuration and using different values for Immediate and Logged MDT sessions.

Pros:

- Re-uses legacy configuration IEs and not much impact to both OAM and RAN

- Security concern with Option 1 is addressed here as logged and immediate MDT sessions have different TR/TRSR values

Cons:

- OAM needs to maintain a mapping of TR/TRSR configured for logged and immediate MDT sessions for a particular Continuous MDT for Trace Correlation.

- It is not possible to see a time series of reporting for a single UE across RRC state transitions with this option

Option 3: Providing a new Continuous MDT indicator with legacy TR/TRSR mechanism

Observation 3: The following pros and cons are observed when using a Continuous MDT indicator with legacy TR/TRSR mechanism.

Pros:

- All requirements of Continuous MDT are met with this option

Cons:

- Additional impact to OAM, RAN and AMF

Proposal 1: OAM shall indicate via (e.g flag) to RAN that a particular MDT configuration is for Continuous MDT and RAN handles the requirement on Measurement continuity and trace correlation with assistance from AMF.

Observation 4: For Continuous MDT, UE should be configured with immediate MDT when it is handed over from source to target. For Connected Mode mobility RAN can provide the Continuous MDT configuration of the UE to the target in the Handover Request to maintain Measurement Continuity.

Proposal 2: RAN3 can handle the requirement on Measurement Continuity Problem across RRC_Connected mobility and no action to SA5 on this.

Observation 5: The issue with using TR/TRSR for Trace Correlation is detailed in Question 1.

Proposal 3: RAN3 to consider the above observations and proposals in the LS response to SA5.