3GPP TSG RAN WG3 #128		  R3-253278

Malta, MT, 19th - 23th May 2025

Title:	(TP for SON BL CR for TS 38.473) Discussion on LTM MRO

Agenda item:	10.2

Source:	Google

Document for:	Discussion and Decision

# Introduction

In RAN3#127bis meeting, there were some agreements and FFSs for LTM MRO.

BFR shortly after successful LTM cell switch to the wrong beam

RAN3 supports network-based solution for the case of BFR shortly after a successful LTM cell switch.

The target DU identifies the BFR happened in UE shortly after successful LTM cell switch caused due to wrong beam.

In case that the source DU selects a wrong beam among candidate beam list, the source DU is responsible for MRO optimization

In case that the target DU provides a wrong candidate beam list, the target DU is responsible for MRO optimization

Target DU needs to send the recovery beam information to CU and CU forwards it to source DU.

CU does not need to send the old beam information to source DU.

FFS if/how the solution for case 1 could also be used for the case of LTM cell switch failure due to wrong beam (case 2)?

LTM failure due to outdated TA

MRO will cover the scenario that RACH-less LTM fails including outdate TA and UE performs RACH based LTM.

RAN3 will not consider the case of LTM cell switch failure due to outdated TA calculated by UE.

For PDCCH order triggered early TA acquisition, RAN3 consider the case of RACH-less LTM fails due to outdate TA and UE performs RACH-based LTM failure recovery or RRC re-establishment to the same cell.

Whether UE based solution or network based solution is used for LTM failure due to outdated TA?

Whether LTM failure due to outdated TA is defined as a new failure case or it can covered by existing failure case?

Network based or UE based solution for TA acquisition type

For inter-DU case, CU aware TA acquisition type without any signaling impact in RAN3.

FFS for intra-DU case.

In RAN3#126 meeting, there were some agreements and FFSs for LTM MRO.

For TP to BL CR to TS 38.473:

- In ACCESS AND MOBILITY INDICATION message, the presence of the C-RNTI IE is kept optional. FFS on stage2 or stage3 to define the condition to include this optional C-RNTI.

For TP to BL CR to TS 38.401:

- For failure due to inappropriate cell switch triggering, CU forwards RLF Report to the failure DU in case of too late LTM.

- For failure due to inappropriate cell switch triggering, CU forwards RLF Report to the source DU in case of too early LTM, or LTM to wrong cell.

Whether network based solution is needed? What’s the UE context to be stored in RAN side?

In RAN3#125bis meeting, there were some agreements and FFSs for LTM MRO.

MRO for LTM:

Add failure type to F1AP ACCESS AND MOBILITY INDICATION message.

CU does not forward the RLF Report to DU if failure is due to wrong selection of prepared LTM cell.

What additional information need to be added to F1AP Access and Mobility Indication message?

- C-RNTI?

Agreements - No need to include the specific access type (i.e. RACH-based or RACH-less) in the RLF report explicitly. It can be known implicitly. This can be revisited if RAN2 concludes that there are issues. - Reuse the existing ra-InformationCommon for the RA-based LTM failure. - If RAN3 does not address this meaning that we need a RAN2 solution, add a list indicating which LTM candidates the UE had at RLF.

In RAN2#128 meeting, there were some agreements on the reusing of the existing ra-InformationCommon for LTM MRO.

In this paper, we further discuss the remaining issues of LTM MRO.

# Discussion

## Discussion on Case 2 in RAN2 LS

There is an LS [1] from RAN2 on new cases of LTM MRO and to ask RAN3 for the solutions on the new cases. The details are depicted below.

RAN2 have discussed the issues of:

- BFR shortly after successful LTM cell switch to the wrong beam (“near failure”), and

- LTM cell switch failure due to wrong beam

On the former issue, RAN2 have acknowledged the issue but agreed not to define a UE-based solution. RAN2 respectfully asks RAN3 to consider defining a network-based solution for this.

Furthermore, RAN2 asks RAN3 to see if/how the solution (if specified) could also be used for the latter issue.

As shown in the agreements in RAN3#127bis, RAN3 has agreed NW-based solution to fix the issue of case 1.

For the issue of case 2, according to the description in [2], there is a case that a target LTM cell wrongly configured beam information for LTM cell switch. The UE executed LTM cell switch with the configured beam but detected connection failure with the target LTM cell. Then, the UE performs cell selection to the same target LTM cell and successfully completes LTM cell switch/reestablishment with new beam(s) different from previously configured one(s).

The procedure of case 2 is similar as the one of case 1 where the UE selects new serving beam(s) with the same target cell after successful LTM cell switch. The target DU can record the wrong beam information and the new beam information during reestablishment/recovery with the same target cell.

Proposal 1: The solution for case 1 can be used for the case of LTM cell switch failure due to wrong beam.

## TA Issues during LTM procedure

In last RAN3 meeting, it was agree to consider the outdated NW-configured TA case during LTM procedure. When the UE receives the TA configured by the target NW, there may be two cases for outdated TA issue:

Case a: TA outdate before LTM execution

In this case, the UE realizes that the current TA becomes outdate when executing LTM procedure. The UE fallbacks to RACH-based LTM with the target cell. This brings additional HO interruption time compared to normal LTM procedure.

Here we assume that there is only TA outdated issue. The configured target LTM cell and related beams are good and suitable. From the UE perspective, this is a successful LTM cell switch without any connection failure. Additionally, it was agreed in RAN2#127 meeting:

RA-based access will not be a new triggering condition for the SHR report.

Therefore, the UE will not generate an RLF report or SHR. It is noticed that UE-based solution cannot work if neither RLF report nor SHR is generated. Conversely, the target DU can identify that the unwanted RACH-based LTM occurs after it configured a TA for RACH-less LTM to the UE.

Observation 1: If the configured TA is outdated before LTM execution, there is no RLF report or SHR.

Case b: TA outdate during LTM execution

For case b, the UE fails to detect the outdate TA issue and preforms RACH-less LTM with the outdate TA. This results in handover failure with the target LTM cell. After cell selection, the UE selects the target LTM cell and performs RACH-based LTM. The outdate TA issue degrades the handover performance. Upon the HOF, the UE generates RLF report and log the cell information involved in the LTM procedure. In the RLF report, both the target cell and the recovery/reestablishment cell are set to the target LTM cell.

When the CU receives the RLF report, for inter-DU LTM cell switch case, the CU has been notified of the configured TA and the CU can identify there was an outdated TA issue. As indicated in the above section, there was RACH-less LTM handover failure. Thus, there is no RACH information in the RLF report. Concurrently, the target DU can store all the RACH-less and RACH-based configurations, including the configured TA and the TA used for successful recovery/reestablishment. Therefore, the CU doesn’t need to forward the RLF report to the target DU. The target DU can optimize the TA issue based on the stored information. For inter-DU LTM cell switch, the CU can discard the RLF report caused by an outdated TA issue.

Observation 2: If the configured TA is outdated during LTM execution, the CU can identify the outdated TA issue for inter-DU case.

In the other case, if the CU finds that it was an intra-DU LTM cell switch case, the DU can store the configurations during the LTM cell switch to fix outdated TA issue or wrong beam issue. Similarly, the CU can discard the RLF report.

Based on the above analysis, the outdated TA issue is different from the wrong beam one, which doesn’t involve the source DU. We can rely on the target DU implementation to fix the outdated TA issues.

Proposal 2: For LTM failures due to outdated TA, a network-based solution is proposed and up to target DU implementation.

## Delivery of LTM RLF report

In last RAN2#128 meeting, it was agreed to reuse the existing ra-InformationCommon for RA-based LTM failure.

- No need to include the specific access type (i.e. RACH-based or RACH-less) in the RLF report explicitly. It can be known implicitly. This can be revisited if RAN2 concludes that there are issues.

- Reuse the existing ra-InformationCommon for the RA-based LTM failure.

In legacy scheme, the UE includes the field ra-InformationCommon in case of HOF with LTM target cell. This will indicate the random-access related information and used for random-access configuration optimization.

1>	if connectionFailureType is hof and if the failed handover is an intra-RAT handover:

2>	set the ra-InformationCommon to include the random-access related information as described in clause 5.7.10.5;

Observation 3: RAN2 agreed to reuse the existing ra-InformationCommon for the RA-based LTM failure.

According to the agreements in RAN3#126 meeting, it was agreed that:

- For failure due to inappropriate cell switch triggering, CU forwards RLF Report to the failure DU in case of too late LTM.

- For failure due to inappropriate cell switch triggering, CU forwards RLF Report to the source DU in case of too early LTM, or LTM to wrong cell.

Base on the above agreements, the CU will always forward RLF report to the source DU though the UE provides random-access information for RA-based LTM failure.

The random-access information for RA-based LTM failure provides the access parameters between UE and LTM target cell. Technically speaking, the LTM target cell can perform root cause analysis on the random-access configuration provided to the UE and optimize the related configuration to improve the LTM robustness in the future.

Therefore, if there is random access information and the connection failure type is set to HOF in the RLF report, the CU can further delivery the RLF report to the target DU.

Proposal 3: CU forwards RLF report to target DU if random access information exists in RLF report for HOF case.

Accordingly, the CU should set the C-RNTI to the target one when forwarding RLF report to target DU.

Proposal 4: If P3 is agreed, the C-RNTI in the ACCESS MOBILITY INDICATION message is set to target one.

## Additional Information for RLF Report Forwarding

What additional information need to be added to F1AP Access and Mobility Indication message?

- C-RNTI?

To analyze the potential additional information, we prefer to classify the LTM MRO into two cases. In case 1, the UE provides the RLF report. In case 2, the UE may not provide the RLF report. We will give our considerations on the additional information provided from CU to DU for the two cases.

### Additional Information for RLF Report Forwarding

Case 1: MRO with RLF report

The field description of the C-RNTI information provided in the RLF report are quoted from the TS 38.331.

RLF-Report-r16 ::=                   CHOICE {

nr-RLF-Report-r16                    SEQUENCE {

measResultLastServCell-r16           MeasResultRLFNR-r16,

measResultNeighCells-r16             SEQUENCE {

measResultListNR-r16                 MeasResultList2NR-r16       OPTIONAL,

measResultListEUTRA-r16              MeasResultList2EUTRA-r16    OPTIONAL

}                                                OPTIONAL,

c-RNTI-r16                           RNTI-Value,

previousPCellId-r16                  CHOICE {

nrPreviousCell-r16                   CGI-Info-Logging-r16,

eutraPreviousCell-r16                CGI-InfoEUTRALogging

}                                                                    OPTIONAL,

failedPCellId-r16                    CHOICE {

nrFailedPCellId-r16                  CHOICE {

cellGlobalId-r16                     CGI-Info-Logging-r16,

pci-arfcn-r16                        PCI-ARFCN-NR-r16

},

eutraFailedPCellId-r16           CHOICE {

cellGlobalId-r16                 CGI-InfoEUTRALogging,

pci-arfcn-r16                    PCI-ARFCN-EUTRA-r16

}

},

reconnectCellId-r16                  CHOICE {

nrReconnectCellId-r16                CGI-Info-Logging-r16,

eutraReconnectCellId-r16             CGI-InfoEUTRALogging

}                                                                                        OPTIONAL,

RLF-Report field descriptions

[……]

c-RNTI This field indicates the C-RNTI used in the PCell upon detecting radio link failure or the C-RNTI used in the source PCell upon handover failure.

It is noticed that there is only one C-RNTI provided in the RLF report. The C-RNTI will indicate different information depending on the connection failure type. For HOF case, the UE provides the source C-RNTI used in source PCell where the UE was connected before the HOF. For RLF case, the UE provides the C-RNTI used in the PCell upon detecting RLF. For example, the UE provides the C-RNTI used in the failure cell (aka source cell) upon detecting RLF in case of too late LTM. The UE provides the target C-RNTI used in the target cell upon detecting RLF in case of too early LTM or LTM to wrong cell.

Observation 4: The C-RNTI in the RLF report will be set to:

- source C-RNTI for:

  - RLF in case of too late LTM; or

  - HOF in case of too early LTM or LTM to wrong cell

- target C-RNTI for RLF in case of too early LTM or LTM to wrong cell (if forwarding RLF report to target DU is agreed)

In the agreed TP [3] on TS 38.473, the CU can forward the RLF Report Failure Type and source C-RNTI along with the RLF report to the DU.

9.2.10.1	ACCESS AND MOBILITY INDICATION

This message is sent by gNB-CU to gNB-DU to provide access and mobility information to the gNB-DU.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| RA Report List |  | 0..1 |  |  | YES | ignore |
| >RA Report Item |  | 1 .. <maxnoofRAReports> |  |  | - |  |
| >>RA Report Container | M |  | OCTET STRING | Includes the RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| RLF Report Information List |  | 0..1 |  |  | YES | ignore |
| >RLF Report Information Item |  | 1 .. <maxnoofRLFReports> |  |  | - |  |
| >>NR UE RLF Report Container | M |  | OCTET STRING | Includes the nr-RLF-Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| >>C-RNTI | O |  | 9.3.1.32 | C-RNTI allocated at the source gNB-DU | YES | ignore |
| >>RLF Report Failure Type | O |  | ENUMERATED (too late LTM, too early LTM, LTM to wrong cell,...) |  | YES | ignore |

As analyzed in the previous section, it is worth noting that the UE will provide the source C-RNTI in RLF report for too late LTM, or HOF in case of too early LTM/LTM to wrong cell. Therefore, sending this information separately over the F1 interface to the source DU would be redundant.

However, for the RLF in case of too early LTM or LTM to wrong cell, there is only target C-RNTI in the RLF report. This means the source DU, which initiated the LTM switch, lacks the source C-RNTI to effectively identify the UE context and analyze the failure.

To address the UE context retrieval at source DU and optimize information exchange, it is recommended for the CU to provide the source C-RNTI when the source C-RNTI is not provided in the RLF report. Correspondingly, new conditions to include the source C-RNTI as additional information should be introduced to address the related case.

Proposal 5: The CU provides the source C-RNTI to the source DU:

- if the connection Failure type IE in RLF report is set to “RLF”; and

- if RLF Report Failure Type IE is set to value “too early LTM” or “LTM to wrong cell”.

If RAN3 agrees that the CU will forward the RLF report to target DU, the CU will check whether the forwarding conditions can be satisfied as described in P5 (e.g., the ra-InformationCommon exits and the connection Failure type is set to HOF).

As mentioned in the above section, UE includes source C-RNTI in the RLF report for HOF case. To facilitate UE identification and UE context retrieval at target DU, it is beneficial for the CU to provide target C-RNTI along with RLF report. The conditions for the presence of target CU are identical to the RLF report forwarding at CU side. Therefore, the conditions for including the target C-RNTI can be omitted, implicitly referencing the CU’s forwarding decision.

Proposal 6: The CU provides target C-RNTI when forwarding RLF report to the target DU.

In the table below, a small summary is depicted about possible cases to include the optional C-RNTI in ACCESS AND MOBILITY INDICATION message from CU to source/target DU.

| Cases | C-RNTI in RLF report | C-RNTI (CU->source DU) | C-RNTI (CU->target DU) |
| --- | --- | --- | --- |
| Case 1: Too late LTM | Source C-RNTI | C-RNTI is absent | no forwarding |
| Case 2: HOF for too early LTM/LTM to wrong cell | Source C-RNTI | C-RNTI is absent | no forwarding |
|  |  |  | present and set to target C-RNTI |
| Case 3: RLF for too early LTM/LTM to wrong cell | Target C-RNTI | present and set to source C-RNTI | no forwarding |

### Additional Information without RLF Report

During LTM procedure, UE detects connection failure with the source or target LTM cell belonging to the CU. The UE behaviors are described in the TS 38.331:

1>	if the cell selection is triggered by detecting radio link failure of the MCG or re-configuration with sync failure of the MCG for an LTM cell switch procedure triggered upon the indication by lower layers as specified in clause 5.3.5.18.6; and

1>	if attemptLTM-Switch is configured; and

1>	if the selected cell is one of the LTM candidate cells in the LTM-Candidate IE within ltm-Config associated with the MCG:

2>	perform the LTM cell switch procedure for the selected LTM candidate cell according to the actions specified in 5.3.5.18.6;

1>	else:

[……]

2>	initiate transmission of the RRCReestablishmentRequest message in accordance with 5.3.7.4;

Case 2a: Reestablishment without RLF report

It is worth noting that the UE performs LTM recovery with the selected cell if the selected cell is one of the LTM candidate cells and the attemptLTM-Switch is configured. The field attemptLTM-Switch is optional present for LTM candidate configuration. If the field attemptLTM-Switch is absent, the UE falls back to legacy reestablishment procedure with the selected LTM candidate cell.

Observation 5: If the LTM attempt is not configured, the UE performs legacy reestablishment procedure with the selected LTM candidate cell after LTM failure.

For Rel-18 LTM, it is introduced for intra-CU case. The source cell and all the candidate cells belong to the same CU. When the UE attempts to re-establish the radio link connection with the selected candidate cell, the UE may not provide the RLF report to the CU. For example, if the UE is a pre-Rel-18 one, or if the UE is a Rel-18 one but doesn’t support LTM SON feature, the UE cannot generate LTM RLF report.

Despite the lack of RLF report, the CU can still perform initial analysis, e.g., based on the information gathered in the reestablishment attempt, including failure cell PCI, failure C-RNTI and reestablishment cell CGI. For the case that the reestablishment cell is one of the LTM candidates, the CU determines that the LTM failure is not due to inappropriate LTM candidate cell preparation at CU side. Subsequently, the CU forwards the failure related additional information to the source DU.

Observation 6: For reestablishment without RLF report, the CU can perform initial analysis and identify whether the LTM failure is due to wrong selection of prepared LTM cell.

For reestablishment case without RLF report in existing MRO scheme for L3 handover, the RAN node receiving the reestablishment request may initiate a failure indication to the failure RAN node where the connection failure was detected. Correspondingly, the failure RAN node performs UE verification and perform a thorough analysis of the failure.

The failure indication for reestablishment without RLF report defined in TS38.423 [4] can include the failure information as shown below:

9.1.3.16	FAILURE INDICATION

This message is sent by NG-RAN node2 to indicate an RRC re-establishment attempt or a reception of an RLF Report from a UE that suffered a connection failure at NG-RAN node1.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| CHOICE Initiating condition | M |  |  |  | YES | reject |
| >RRC Reestab |  |  |  |  |  |  |
| >>CHOICE RRC Reestab Initiated Reporting | M |  |  |  | – |  |
| >>>RRC Reestab Reporting without RLF Report |  |  |  |  |  |  |
| >>>>Failure cell PCI | M |  | NG-RAN Cell PCI 9.2.2.10 | Physical Cell Identifier | – |  |
| >>>>Re-establishment cell CGI | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>>>C-RNTI | M |  | BIT STRING (SIZE (16)) | Corresponds to information provided in the c-RNTI contained either in the RRCReestablishmentRequest message (TS 38.331 [10]) or in the RRCConnectionReestablishmentRequest message (TS 36.331 [14]) | – |  |
| >>>>ShortMAC-I | M |  | BIT STRING (SIZE (16)) | Corresponds to information provided in the shortMAC-I contained either in the RRCReestablishmentRequest message (TS 38.331 [10]) or in the RRCConnectionReestablishmentRequest message (TS 36.331 [14]) | – |  |
| >>>>RRC Conn Reestab Indicator | O |  | ENUMERATED (reconfigurationFailure, handoverFailure, otherFailure, ...) | Corresponds to information provided in the reestablishmentCause contained in the RRCReestablishmentRequest message as defined in TS 38.331 [10] or in the RRCConnectionReestablishmentRequest message as defined in TS 36.331 [14]. | YES | ignore |

Therefore, for reestablishment case without RLF report for LTM MRO, when the RAN node (i.e., the CU) receives reestablishment attempt from the UE, it first verifies the UE and determines the failure type. If needed, the CU can forward the source cell, target cell, reestablishment cell, source C-RNTI and RLF Report Failure Type to the source DU for root cause analysis to optimize the LTM related parameters.

Proposal 7: For reestablishment without RLF report, the CU forwards to the source DU the following additional information:

- Source cell information

- Source C-RNTI

- Target cell information in case of too early LTM or LTM to wrong cell

- Reestablishment cell information

- RLF Report Failure Type

Case 2b: LTM Recovery without RLF Report

For CHO and LTM mobility, when the UE detects connection failure and selects a CHO or LTM candidate cell with attempt configured, the UE can perform CHO or LTM recovery with the selected candidate cell.

For CHO, the UE decides the CHO target cell if the CHO conditions are satisfied and executes CHO with the target cell. This is UE-based decision. If connection failure occurs, the UE selects a CHO cell and initiates CHO recovery. The UE sends the RRC Reconfiguration Complete message to the selected candidate cell. If the UE does not provide the RLF report to the CHO recovery cell, the CHO recovery cell is unable to identify whether the access is a normal CHO execution or a CHO recovery from CHO failure. Consequently, the CHO recovery cell will not initiate a failure indication to the failure RAN node where the connection failure was detected.

Observation 7: Without RLF report, the CHO recovery RAN node cannot identify whether there was a connection failure prior to the CHO access.

The situation changes for LTM. Although the CU configures the candidate LTM cells to UE, the LTM cell switch decision is still initiated by the source DU based on the L1 measurement results. This is NW-driven decision. The source DU selects the target LTM cell and informs the CU of the target LTM cell information via DU-CU CELL SWITCH NOTIFICATION message. After a successful LTM cell switch, the target LTM cell sends an ACCESS SUCCESS message to CU, indicating the (first) access cell where the UE has successfully accessed for LTM. If LTM procedure fails and the UE performs LTM recovery with a candidate LTM cell, the LTM recovery cell also sends an ACCESS SUCCESS message to the CU to indicate the (second) access cell.

For example, for too late LTM, the DU didn’t initiate the LTM cell switch command. The CU will not receive DU-CU CELL SWITCH NOTIFICATION message. The UE performs LTM recovery after the RLF in source cell. Subsequently, the CU will receive ACCESS SUCCESS message from the LTM recovery cell. The possible flow is illustrated in figure 1.

Fig.1 Too late LTM procedure

For too early LTM or LTM to wrong cell, the DU initiated LTM cell switch command. The CU received DU-CU CELL SWITCH NOTIFICATION message, e.g., including the LTM target cell 1.

In case of HOF, the UE fails to complete LTM procedure with target LTM cell and performs LTM recovery with a candidate cell, e.g., the source cell 0 or a different cell 2. The CU receives an ACCESS SUCCESS message including access cell (recovery one). The CU will determine the LTM failure type based on the LTM recovery cell and realize there was a HOF during LTM procedure. The possible flow is illustrated in figure 2.

Fig.2 procedures of HOF case for too early LTM/LTM to wrong cell

In case of RLF, the UE successfully completed LTM procedure with the target LTM cell. The CU will receive an ACCESS SUCCESS message from the target one. If the UE detects RLF with target LTM cell shortly, the UE performs LTM recovery. Correspondingly, the CU receives a second ACCESS SUCCESS message from the recovery cell. The CU can determine the LTM failure type and understand there was an RLF in LTM target cell during the LTM recovery procedure. The possible flow is illustrated in figure 3.

Fig.3 procedures of RLF case for too early LTM/LTM to wrong cell

As shown in the previous figures, the CU receives different messages and acquires different cell information for different failure cases. Consequently, if RLF report is not provide after successful LTM recovery, the CU still has the ability to detect the connection failure during LTM procedure.

Observation 8: For LTM recovery without RLF report, the CU can perform initial analysis and identify whether there was a connection failure during LTM procedure.

The additional information for LTM recovery without RLF report can follow the principle for reestablishment without RLF report case. For LTM recovery case, there may be recovery cell during the procedure. Based on the UE context at CU side and the information provided by the source/target/recovery DU, the CU can derive source cell information, LTM target cell information, LTM recovery cell information, source C-RNTI and RLF Report Failure Type.

To assist source DU for LTM performance optimization, it is highly expected for the CU to forward the additional information to the source DU.

Proposal 8: For LTM recovery without RLF report, the CU forwards to the source DU the following additional information:

- Source cell information

- Source C-RNTI

- Target cell information in case of too early LTM or LTM to wrong cell

- Recovery cell information

- RLF Report Failure Type

Based on the previous discussion, a TP for LTM MRO on TS 38.473 is provided in the annex.

# Conclusion

In this contribution, we discussed the potential solutions for LTM MRO. The related observations and proposals are summarized below:

Observation 1: If the configured TA is outdated before LTM execution, there is no RLF report or SHR.

Observation 2: If the configured TA is outdated during LTM execution, the CU can identify the outdated TA issue for inter-DU case.

Observation 3: RAN2 agreed to reuse the existing ra-InformationCommon for the RA-based LTM failure.

Observation 4: The C-RNTI in the RLF report will be set to:

- source C-RNTI for:

  - RLF in case of too late LTM; or

  - HOF in case of too early LTM or LTM to wrong cell

- target C-RNTI for:

  - RLF in case of too early LTM or LTM to wrong cell

Observation 5: If the LTM attempt is not configured, the UE performs legacy reestablishment procedure with the selected LTM candidate cell after LTM failure.

Observation 6: For reestablishment without RLF report, the CU can perform initial analysis and identify whether the LTM failure is due to wrong selection of prepared LTM cell.

Observation 7: Without RLF report, the CHO recovery RAN node cannot identify whether there was a connection failure prior to the CHO access.

Observation 8: For LTM recovery without RLF report, the CU can perform initial analysis and identify whether there was a connection failure during LTM procedure.

Proposal 1: The solution for case 1 can be used for the case of LTM cell switch failure due to wrong beam.

Proposal 2: For LTM failures due to outdated TA, a network-based solution is proposed and up to target DU implementation.

Proposal 3: CU forwards RLF report to target DU if random access information exists in RLF report for HOF case.

Proposal 4: If P3 is agreed, the C-RNTI in the ACCESS MOBILITY INDICATION message is set to target one.

Proposal 5: The CU provides the source C-RNTI to the source DU:

- if the connection Failure type IE in RLF report is set to “RLF”; and

- if RLF Report Failure Type IE is set to value “too early LTM” or “LTM to wrong cell”.

Proposal 6: The CU provides target C-RNTI when forwarding RLF report to the target DU.

Proposal 7: For reestablishment without RLF report, the CU forwards to the source DU the following additional information:

- Source cell information

- Source C-RNTI

- Target cell information in case of too early LTM or LTM to wrong cell

- Reestablishment cell information

- RLF Report Failure Type

Proposal 8: For LTM recovery without RLF report, the CU forwards to the source DU the following additional information:

- Source cell information

- Source C-RNTI

- Target cell information in case of too early LTM or LTM to wrong cell

- Recovery cell information

- RLF Report Failure Type

# Reference

- R2-2501378, LS on SON for LTM

- R2-2500831, MRO for LTM, ZTE Corporation, Sanechips

- R3-250039, (TP for BLCR for SON for 38.473) MRO enhancement for R18 mobility mechanisms

- TS 38.423

# Annex - TP for LTM MRO for TS 38.473

Start of Change

#### 9.2.10.1	ACCESS AND MOBILITY INDICATION

This message is sent by gNB-CU to gNB-DU to provide access and mobility information to the gNB-DU.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| RA Report List |  | 0..1 |  |  | YES | ignore |
| >RA Report Item |  | 1 .. <maxnoofRAReports> |  |  | - |  |
| >>RA Report Container | M |  | OCTET STRING | Includes the RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| RLF Report Information List |  | 0..1 |  |  | YES | ignore |
| >RLF Report Information Item |  | 1 .. <maxnoofRLFReports> |  |  | - |  |
| >>NR UE RLF Report Container | M |  | OCTET STRING | Includes the nr-RLF-Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| >>C-RNTI | O |  | 9.3.1.32 | C-RNTI allocated at the source gNB-DU if the connection Failure type IE in RLF report is set to “RLF” and RLF Report Failure Type IE is set to value “too early LTM” or “LTM to wrong cell”; or C-RNTI allocated at the target gNB-DU | YES | ignore |
| >>RLF Report Failure Type | O |  | ENUMERATED (too late LTM, too early LTM, LTM to wrong cell,...) |  | YES | ignore |
| RRC Reestab/Recovery Reporting without RLF report |  |  |  |  |  |  |
| >Source cell PCI | M |  | INTEGER (0..1007) | Physical Cell Identifier | - |  |
| >Target cell PCI | O |  | INTEGER (0..1007) | Physical Cell Identifier | - |  |
| >Re-establishment or recovery cell CGI | M |  | 9.3.1.12 |  | - |  |
| >C-RNTI | O |  | 9.3.1.32 | C-RNTI allocated at the source gNB-DU | - |  |
| >RLF Report Failure Type | O |  | ENUMERATED (too late LTM, too early LTM, LTM to wrong cell,...) |  | YES | ignore |
| Successful HO Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful HO Report Information Item |  | 1 .. <maxnoofSuccessfulHOReports> |  |  | - |  |
| >>Successful HO Report Container | M |  | OCTET STRING | Includes the SuccessHO-Report IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| [……] |  |  |  |  |  |  |

End of Change