3GPP TSG-RAN WG3 Meeting #128	R3-253285

Malta, MT, 19-23, May, 2025

Agenda item:	15.2

Source: 	Samsung

Title: 	Further discussion on support of regenerative payload for IoT NTN

Document for:	Discussion and decision

Introduction

RAN3#127 meeting discussed the support of regenerative payload for IoT NTN, and the following agreement is achieved,

Introduce stage 2 description of Store and Forward operation.

Introduce stage 2 description of full eNB as regenerative payload.

Introduce the S1 Removal procedure.

Check progress in NR NTN and any further impact on IoT NTN…

In this contribution, we continue to discuss the support of regenerative payload for IoT NTN.

Discussion

In RAN3#127, the support of regenerative payload for IoT NTN was continued to be discussed, and the main open issue is captured as follows,

Check progress in NR NTN and any further impact on IoT NTN…

And in our understanding, the above open issue applies to the regenerative payload with eNB-only onboard (i.e. without S&F satellite operation).

The first open issues is whether to support multiple TNL associations in S1AP in order to support soft FLSO for eNB-only onboard regenerative payload, considering that current BL CR to 38.300 captures the following text,

Both hard and soft feeder link switchover are supported in transparent or regenerative NTN payloads.

Considering that this updated sentence is not thoroughly discussed previously, and the main reason to update the above text is to follow the progress of NR NTN, we need to re-visit this sentence,

- If the above text is finally supported, the S1AP interface management procedures need to be enhanced to support managing multiple TNLAs.

- If we’d like to keep a single TNLA over S1AP, which means that the soft feeder link switch over is not supported for eNB-only onboard regenerative payload, the above text is needed to be further revised.

In fact, introducing multiple TNLAs over S1AP has big spec impact, and there’s no clear motivation to support soft feeder link switch over for regenerative IoT NTN payload. We prefer to keep the S1AP as it is, and revise the BL CR to 38.300 further.

Proposal 1: Soft feeder link switchover for regenerative IoT NTN payloads is not supported.

Another open issue to discuss is based on the progress of NR NTN.

According to the progress of NR NTN achieved last meeting, companies agreed to capture an indicator in NGAP RAN CONFIGURATION UPDATE message in the latest BL CR to 38.413 and 38.300 in R3-252525 and R3-252523 separately as follows,

BL CR to 38.413 in RAN CONFIGURATION UPDATE message:

| Downlink NG Transmission Control | O |  | ENUMERATED (suspend, resume, …) | Indicates the CN to suspend or resume the downlink NG Transmission | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

Editor’s Note: FFS whether this IE is needed.

BL CR to 38.300:

For regenerative payload, an indication to suspend and resume data and signaling procedures over NG interface may be sent to AMF from gNB in the RAN Configuration Update procedure when the hard feeder link switch over occurs.

Editor’s Note: FFS whether this text is needed.

It can be observed that, the BL CRs capture the suspend/resume indicator mainly for the purpose of avoiding DL transmission loss in case of hard feeder link switch over, which in our understanding is a valid case to introduce the suspend/resume indicator in RAN CONFIGURATION UPDATE message.

For IoT NTN, when the hard feeder link switch over happens, the similar issue is also possibly to be happened over S1-C and S1-U, so we think it is also needed to introduce a suspend/resume indicator in S1AP ENB CONFIGURATION UPDATE message to avoid DL packet loss in case of hard feeder link switchover.

In addition, similarly, whether a suspend/resume indicator is needed for S1 Suspend/Resume can also be discussed, and in our understanding we can also adopt a suspend/resume indicator in ENB CONFIGURATION UPDATE message to support S1 Suspend/Resume. The detailed reason can be found in our contribution submitted to NR NTN WI.

Briefly speaking, the benefit of introducing suspend/resume indicator to support S1 Suspend/Resume is clear: The TA supported list in S1 SETUP REQUEST message is mandatory while optional in ENB CONFIGURATION UPDATE message, which means S1 SETUP REQUEST message has to send TA supported list; under such case, the signalling overhead is saved on TA supported list if the eNB sends ENB CONFIGURATION UPDATE message with one resume indicator.

As a result, from saving signalling overhead point of view, it is beneficial to introduce one suspend/resume indicator in ENB CONFIGURATION UPDATE message.

Proposal 2: Agree to introduce a suspend/resume indicator in ENB CONFIGURATION UPDATE message, for the following purposes,

- S1 Suspend/Resume

- Avoid DL packet loss in case of hard FLSO.

If we want to adopt such indicator for multiple purposes, the IE name should be general enough.

Conclusion

In this paper, we provide primary discussion on support of regenerative payload for IoT NTN. The following proposals are provided,

Proposal 1: Soft feeder link switchover for regenerative IoT NTN payloads is not supported.

Proposal 2: Agree to introduce a suspend/resume indicator in ENB CONFIGURATION UPDATE message, for the following purposes,

- S1 Suspend/Resume

- Avoid DL packet loss in case of hard FLSO.

References