3GPP TSG-RAN WG3 Meeting #128	R3-253289

Malta, MT, 19-23, May, 2025

Agenda item:	22.2

Source: 	Samsung

Title: 	Further discussion on multi-hop for SL relay

Document for:	Discussion and decision

Introduction

Last meeting discussed the multi-hop SL relay, and the following WA is captured,

WA: Reuse the Peer UE ID IE as counter part information.

In this contribution, we further discuss this open issue.

Discussion

Recall that in R17/18, to support the single-hop relay, over F1AP,

- For the Relay UE context, one list of Uu RLC channel (between gNB and Relay UE) and one list of PC5 RLC channel (between Relay UE and Remote UEs) are needed to be coordinated between gNB-CU and gNB-DU.

- For the Remote UE context, one list of PC5 RLC channel (between Relay UE and Remote UE) is needed to be coordinated between gNB-CU and gNB-DU.

While for R19 intermediate Relay UE, two separate lists of PC5 RLC channel (i.e. one list for the PC5 RLC channels between intermediate Relay UE and its Parent UE, and one list for the PC5 RLC channels between intermediate Relay UE and its Child UE) are needed to be coordinated between gNB-CU and gNB-DU.

Proposal 1: Two separate lists of PC5 RLC Channel(s) are needed to be maintained for the intermediate relay UE context over F1AP, i.e. one list of PC5 RLC channels between intermediate Relay UE and its Parent UE, and one list of PC5 RLC channels between intermediate Relay UE and its Child UE.

In order to meet the requirement of P1, we need to further discuss the necessary F1AP enhancement.

It should be noted that last meeting we achieved a WA as follows which may have F1AP impact,

WA: Reuse the Peer UE ID IE as counter part information.

During online discussion last meeting, actually all companies did not have strong preference on defining a new IE as counterpart information, so it is ok to turning the above WA into agreement.

Proposal 2: Turn the WA into agreement: Reuse the Peer UE ID IE as counterpart information.

In our understanding, the above P2 means that there’s no need to define an additional set of PC5 RLC channels to be Setup/Modified/Released in the current F1AP UE CONTEXT SETUP/MODIFICATION REQUEST messages. Similar to what we did for U2U Relay UE in R18.

However, the current semantics of Peer UE ID IE restricts this IE to be used in U2U relay case; if we want to reuse this IE, we need to at least update the semantics of this IE.

| >>Peer UE ID | O |  | BIT STRING (SIZE(24)) | Corresponds to information provided in the sl-DestinationIdentityL2-U2U contained in the SL-TxResourceReqL2-U2U IE, defined in TS 38.331 [8]. This IE is included if the gNB-CU UE F1AP ID and/or gNB-DU UE F1AP ID are associated with a L2 U2U Remote UE or L2 U2U Relay UE. | YES | reject |
| --- | --- | --- | --- | --- | --- | --- |

Observation 1: The semantic description is needed to be updated for Peer UE ID to also include the multi-hop relay case.

Another thing to notice is that for U2U relay case, the Peer UE ID IE is only included in PC5 RLC Channel to be Setup List, but not included in PC5 RLC Channel to be Modified/Released List: the reason to not including the Peer UE ID in Modified/Released list is that for U2U Relay case the PC5 RLC channel can be uniquely identified by PC5 RLC Channel ID and Remote UE Local ID contained in the Modified/Released List.

However, for multi-hop relay case, especially for the Second Relay UE, it is possible that the established PC5 RLC channels are not established for only one Remote UE. So it is not appropriate to include Remote UE local ID in Modified/Released list in all cases.

Observation 2: For e.g. the Second Relay UE, it is possible that the established PC5 RLC channels are established for multiple Remote UEs. So it is not appropriate to include Remote UE local ID in PC5 to be Modified/Released List in F1AP UE CONTEXT MODIFICATION REQUEST message for the Second Relay UE context.

If we do not use Remote UE Local ID IE in Modified/Released List, in order to uniquely identify a PC5 RLC Channel by gNB-DU, the Peer UE ID IE is also needed to be included in Modified/Released List.

Proposal 3: Add Peer UE ID IE in PC5 RLC Channel to be Modified/Released List in F1AP UE CONTEXT MODIFICATION REQUEST message.

By following similar reason, the Peer UE ID IE is also needed to be included in RC5 RLC Channel related lists in F1AP UE CONTEXT MODIFICATION RESPONSE/REQUIRED/CONFIRM message.

Proposal 4: Add Peer UE ID IE in PC5 RLC channel related lists in F1AP UE CONTEXT MODIFICATION RESPONSE/REQUIRED/CONFIRM message.

Conclusion

In this paper, we provide discussion on multi-hop relay. The following proposals are provided,

Proposal 1: Two separate lists of PC5 RLC Channel(s) are needed to be maintained for the intermediate relay UE context, i.e. one list of PC5 RLC channels between intermediate Relay UE and its Parent UE, and one list of PC5 RLC channels between intermediate Relay UE and its Child UE.

Proposal 2: Turn the WA into agreement: Reuse the Peer UE ID IE as counterpart information.

Observation 1: The semantic description is needed to be updated for Peer UE ID to also include the multi-hop relay case.

Observation 2: For e.g. the Second Relay UE, it is possible that the established PC5 RLC channels are established for multiple Remote UEs. So it is not appropriate to include Remote UE local ID in PC5 to be Modified/Released List in F1AP UE CONTEXT MODIFICATION REQUEST message for the Second Relay UE context.

Proposal 3: Add Peer UE ID IE in PC5 RLC Channel to be Modified/Released List in F1AP UE CONTEXT MODIFICATION REQUEST message.

Proposal 4: Add Peer UE ID IE in PC5 RLC channel related lists in F1AP UE CONTEXT MODIFICATION RESPONSE/REQUIRED/CONFIRM message.

Reference