3GPP TSG-RAN WG3 Meeting #128		R3-253291

Malta, MT, 19th - 23rd May, 2025

Source:	CATT

Title:	Discussion on the remaining issues to support regenerative payload

Agenda Item:	14.3

Document for:	Discussion and Decision

- Introduction

In the last RAN3 meeting, we further discussed the issues on regenerative payload. According to the discussion, some agreements and conclusions are achieved, as below:

Support RRC_INACTIVE UE in NTN by implementation in R19.

No enhancements on location based CHO for NTN in R19.

In case of the TNL addresses are changed due to feeder link switch over, existing procedures may be used.

No new NGAP suspend/resume procedures in R19.

In this contribution, we will continue discuss the remaining issues on regenerative payload for NR NTN and work on the stage2/3 details, our views will be provided accordingly.

- Discussion

In case of NG Connection interruption due to hard feeder link switch over, the last RAN3 meeting has agreed to introduce a new IE for NG downlink transmission suspension/resumption. The corresponding description is as follows in BLCR 38.300:

For regenerative payload, an indication to suspend and resume data and signaling procedures over NG interface may be sent to AMF from gNB in the RAN Configuration Update procedure when the hard feeder link switch over occurs.

The new indicator to suspend o resume NG connnection is limited in case of hard feeder link switch over. But in our opinion, the specified indication shall be generically applicable, not restricted to hard feeder link switchover case. For example, when the on-board gNB coverage leaves the AMF service area, the NG connection could be suspended, by indicating the AMF that the current NG connection is temporarily unavailable, and the logical NG interface still keeps. The gNB and AMF stores the corresponding NG context information. And when the on-board gNB comes back to the AMF service area, the NG connection could be resumed through the new indicator.

Observation 1: When the on-board gNB coverage leaves or re-enters the AMF service area, the NG connection could be suspended or resumed through the new indicator.

Both NG Removal/Setup and NG suspend/resume can be supported, and the two mechanisms can be used flexibly or can be used in combination. For example, if on board gNB may not return to the service area of an AMF for a long time in the future, it can choose NG Removal procedure. If on board gNB will return to the service area of an AMF in a shorter period of time, it can suspend the NG connection.

Observation 2: Support of NG Suspend/Resume could make the NG connection establishment/recovery faster, no need to frequently remove and setup the NG interfaces between on board gNB and the AMFs on ground.

In this case, in addition to the DL data/ signalling which should be kept and suspended, the UE Context should be also kept in the AMF. Therefore, revise the description in BL CR 38.300 and remove the restriction on suspending and resuming NG connection, not limited to hard feeder link switchover scenarios.

As above, the corresponding stage 2 TP for 38.300 BL CR is provided in [2]. RAN3 is requested to discuss and agree the TP.

Proposal 1: Revise the description of BL CR 38.300 and remove the restriction on suspending and resuming NG connection. Corresponding TP to BL CR for TS38.300 is provided in [2].

The “Downlink NG Transmission Control” IE was introduced in the RAN Configuration Update message in BL CR 38.413. It’s beneficial to rename this new added IE to make it more general, e.g. rename it to “NG Connection Control”. Corresponding stage 3 TP for 38.413 BL CR is provided in [3]. RAN3 is requested to discuss and agree the TPs.

Proposal 2: Rename IE “Downlink NG Transmission Control” to “NG Connection Control” to make it more general for other purpose(s). Corresponding TP to BL CR for TS38.413 is provided in [3].

- Conclusion

Based on the discussion in section 2, we have the following observations and proposals:

Observation 1: When the on-board gNB coverage leaves or re-enters the AMF service area, the NG connection could be suspended or resumed through the new indicator.

Observation 2: Support of NG Suspend/Resume could make the NG connection establishment/recovery faster, no need to frequently remove and setup the NG interfaces between on board gNB and the AMFs on ground.

Proposal 1: Revise the description of BL CR 38.300 and remove the restriction on suspending and resuming NG connection. Corresponding TP to BL CR for TS38.300 is provided in [2].

Proposal 2: Rename IE “Downlink NG Transmission Control” to “NG Connection Control” to make it more general for other purpose(s). Corresponding TP to BL CR for TS38.413 is provided in [3].

- Reference

- RAN3_127_agenda_20250221_1300_EOM

- R3-253292 (TP to BL CR for TS38.300) Support for Regenerative Payload, CATT, ZTE Corporation, Samsung, CMCC, NEC

- R3-253237 (TP to BL CR for 38.413) Support of regenerative payload, ZTE Corporation, CATT, Samsung, NEC, CMCC