3GPP TSG-RAN WG3 #128                                            	R3-253296

Malta, MT, 19th – 23rd May, 2025

Source:	CATT

Title:	Further Consideration on locating of A-IoT device

Agenda Item:	16.3

Document for:	Discussion and Decision

- Introduction

On support of A-IoT, a study item has been completed in the last RAN WG meetings. Corresponding solutions on the architecture, signallings, locating, etc. are captured in the TR38.769 [1]. In RAN#106, a new work item for Ambient IoT has been approved(WID in [2]), aimed to standardize RAN aspects of Ambient IoT. In the WID, we will only focus on Topology 1, the major RAN3 scope are highlighted below:

- RAN3 scope:

  - Specify necessary architectural aspects, and signaling and procedures between A-IoT RAN and A-IoT CN to support the A-IoT functions, assuming an architecture of aggregated gNB, including:

    - Inventory and command operations

    - Device location reporting at reader ID granularity

Note: The above A-IoT functions are supported over the existing NG interface, based on architecture(s) defined by RAN3/SA2.

In the last RAN3 meeting, we discussed how to report location of A-IoT device. Latest progress is shown below:

-   Device location reporting at reader ID granularity

The A-IoT RAN node always sends the AIoT device’s location at reader ID granularity to the AIoTF in Inventory Report.

AIoT RAN node sends the Inventory Report aggregated per Reader.

“Global gNB ID + Reader index” is used to uniquely identify the Reader globally.

RAN3#127bis:

Reader indexes and the gNB ID are mandatory included in Inventory Report Transfer IE.

Continue to discuss the open issues....

In this contribution, we will further discuss device location reporting at reader ID granularity, and provide the observations and proposals accordingly.

- Discussion

In the last meeting we discussed and agreed that Reader indexes and the gNB ID are mandatory included in Inventory Report Transfer IE. We understand that the basic design to report device location with reader granularity is done.

Observation 1: Inventory Report with reader ID could provide reader granularity.

In Release 19, if a device is under the coverage of multiple readers, and received the Inventory Request from different readers:

- RAN2 is discussing whether and how to avoid duplicated reply from the device for a given A-IoT Inventory Request.

- For different A-IoT Inventory services, e.g. Inventory #1 for device #x in Reader #1,#2,#3; Inventory #2 for device #x in Reader #4, #5, #6. If device #x receives the Inventory Request #1 in Reader #3, and Inventory Request #2 in Reader #4, and responses Inventory #1 and #2 separately to the AIoTF, with the reader ID.

As above, if AIoTF receives Inventory Reports for a specific device in different readers, no matter in one or multiple Inventory procedures, AIoTF may associate the Reader(s) for locating purpose, up to SA2 design. From RAN3 perspective, device ID in the inventory report is in the NAS PDU, which is transparent to the gNB, such kind of association could not be performed in the gNB, no extra design is needed, at least in this release.

Proposal 1: If a device reports multiple Inventory Reports via different Readers (for a single Inventory procedure, or multiple Inventory Procedures), AIoTF may use the info for locating purpose, details up to SA2. No RAN impact is expected in this release.

In the TR 22.840 [3], there’re several use cases on A-IoT which have different requirements on Positioning accuracy. E.g. for the automated warehousing (section 5.1), and the medical instruments inventory management and positioning (section 5.2),  the positioning accuracy expected are 2~3m, and 3~5m.

5.7	Positioning accuracy

The design target of absolute positioning accuracy when performed by the cellular network (including assisting nodes when present) is:

-	1~3 meters @ 90% indoor location.

-	Several tens of meters @ 90% outdoor location.

The design target of relative ranging accuracy for topology 4 is:

-	1~3 meters @ 90% indoor and outdoor location

In the TR 38.848 [4], the requirement on Positioning accuracy is also mentioned below:

6.1.7	Positioning accuracy

Feasibility assessment for this aspect has been reported by reference to technologies of a similar complexity level, such as UHF RFID achieving 2-3 m accuracy in [27], [28], [29] indoors, and ultra-narrow IoT achieving from several tens to 100 or 150 m in [30], [31], [32] outdoors. It is also observed that Device A and B need to have a carrier wave source in an appropriate distance to be able to transmit signals for positioning.

In the SID, it’s not clearly specified what’s the expected positioning requirement to be achieved in Rel-19.

Observation 2: in the TR [3][4], different use cases have different requirements on Positioning accuracy, e.g. for some indoor use cases, 1~3m positioning accuracy is expected, and several tens of meters accuracy for outdoor cases.

We understand that locating of A-IoT device is essential for several use cases, we should further discuss how to satisfy the locating services with finer granularity (e.g. 1~3m indoor, tens of meters outdoor) the next release, coordination with other RAN/SA WGs may be needed.

Proposal 2: RAN3 should further discuss whether and how to improve the finer locating accuracy on top of reader granularity in the next release, coordination with other RAN/SA WG(s) seems needed.

- Conclusion

Based on the discussion in section 2, the following observations and proposals are provided:

Observation 1: Inventory Report with reader ID could provide reader granularity.

Proposal 1: If a device reports multiple Inventory Reports via different Readers (for a single Inventory procedure, or multiple Inventory Procedures), AIoTF may use the info for locating purpose, details up to SA2. No RAN impact is expected in this release.

Observation 2: in the TR [3][4], different use cases have different requirements on Positioning accuracy, e.g. for some indoor use cases, 1~3m positioning accuracy is expected, and several tens of meters accuracy for outdoor cases.

Proposal 2: RAN3 should further discuss whether and how to improve the finer locating accuracy on top of reader granularity in the next release, coordination with other RAN/SA WG(s) seems needed.

- Reference

- TR 38.769 Study on solutions for ambient IoT (Internet of Things)

- RP-243326 New Work Item: Solutions for Ambient IoT (Internet of Things) in NR

- TR 22.840 Study on Ambient power-enabled Internet of Things

- TR 38.848 Study on Ambient IoT (Internet of Things) in RAN